# Section 1	Specification

Contract Ref: 	NLS161202

Contract Title: 	Supply and Installation of Compact Mobile Bottle 				Shredders

Part A: Technical Specification

- The Shredder

  - This contract specifies for the supply delivery, installation and commissioning of two compact mobile shredding systems, one each to be delivered to our laboratory sites at Starcross (Staplake Mount, Starcross, Exeter, Devon, EX6 8FD) and Nottingham (Meadow Lane, Nottingham, NG2 3HN)

  - The shredders will be used primarily to shred plastic bottles used to hold environmental water samples.

In most cases the plastic will contain a volume of water and the shredders must be capable of dealing with ‘wet shredding’ safely and effectively.

The shredder should also be proof against small amounts of other materials being mixed with the plastic bottles such as paper, glass and wood. The systems will be used daily for short periods of less than 3 hours.

  - Some of the plastic sample containers to be shredded will also contain water which has been acidified with nitric and hydrochloric acids up to a combined concentration of approximately 3% acid. The shredder must be proofed to corrosion and degradation by acid solutions of this strength,

  - The systems will be used both indoors and outdoors (covered) and must be suitable for use in both situations. The shredders must be mobile and capable of being easily disconnection from the power source.  The system should come complete with a 3m power cable with a ‘plug’ appropriate to the power required, already fitted. Details of the ‘plug’ type used and the socket required must be provided as part of your return. –

  - The system may be used in several locations at each site and must include suitable wheels with lock brakes. The shredder’s dimensions must allow for it to pass easily through a door width of 1000mm.

  - The shredder must be supplied with a hopper with a minimum capacity of 0.5 cubic metres. The hopper must come supplied with a suitable lid. The hopper should have a safety device which prevents activation of shredder when the hopper lid is raised. The hopper should be easy to reach and load and have an option for a manual pressing device to be installed to assist in feeding the contents through the shredder. There must be an option to lock the hopper lid when the shredder is not in use.

  - The shredder must be compatible with the use of a 240 litre wheelie bin as the receptacle for the shredded materials. The shredder must allow for the easy removal and replacement of the wheelie bin when the materials being shredded are changed.  The shredder must include a direction chute to ensure the shredded materials always enter the wheelie bin.

- Health and Safety

The shredders must be supplied with appropriate safety cut outs to prevent incorrect operation and to cease operation once the hopper has been emptied or the shredder has been operating for an unnecessarily long period.

The shredders must meet all relent safety legislation relating to its build and operation, in particular the noise whilst operating must be below 75db to facilitate safe indoors use.

The shredders must be equipped with an easy to reach emergency shutdown device.

- Additional Information

Please provide the following information with your quotation

- Power requirements and loading of the mobile shredder units including details of the sockets that will be required in the usage areas.

- Full technical details of the system offered.

- Details of the support packages you offer for the routine preventative maintenance and repair of the shredders

- Details of any replacement or consumable parts that will be required to operate the shredders.

- Details of 2 existing users with similar requirements. (we may call these users for references)

Part B: Special Conditions

- Introduction

1.1	The following conditions of contract relate to the installation of the equipment and subsequent post-sales services and are supplementary to the standard conditions of contract for goods.

  - In addition to the interpretations listed in the standard conditions for Goods the following applies :-

1.2.1	"Acceptance Date" - means the date on which the Agency gives written notice in accordance with Condition 7.

1.2.2	"Installation Site"- means the place where the Goods are to be installed in the Agency's premises.

- Specification

  - The Goods shall be of the qualities and kinds described and equal in respects to the descriptions, specifications, patterns and Contractor's samples, which form part of the Contract or are otherwise relevant for the purpose of the Contract.  Except in so far as may otherwise be indicated by such descriptions, specifications, patterns or Contractor's samples, the Goods shall be strictly in compliance with the latest relevant International or British Standards where such exist.

  - All goods which customarily bear any mark, tab, brand, label or other device indicating place or origin, inspection by any Government or other body or standard of quality must be delivered with all the said marks, tabs, brands, labels or other devices intact.

- Packaging

  - The following details shall be shown with every delivery unless otherwise specified in the Contract:

Description of the Goods & quantity in package

Any special directions for storage/unpacking, which shall include the appropriate Health & Safety information

Expiry date of contents (where applicable)

Name of manufacturer and any other relevant information

  - The Contractor shall collect without charge any returnable containers (including pallets) within twenty-one days of the date of the relevant delivery note, unless otherwise instructed by the Agency named in the Contract.  Empty containers not so removed may be returned by the Agency at the Contractor's expense or otherwise disposed of at the Agency's discretion.  Charged containers shall be credited in full by the Contractor upon collection or return.

  - All packaging shall be removed by the contractor and recycled/reused where possible. Disposal of waste must comply with all relevant legislation.

- Delivery and Installation

  - All equipment detailed herein must be delivered to designated site(s), installed and accepted by the Agency on or before date specified.

  - A delivery note clearly marked with the Agency’s order number shall accompany each delivery.

  - The Goods shall be delivered free of charge to the Agency to the locations shown on the order and by the agreed delivery date. The Agency will make available access to the site as reasonably required by the supplier for the delivery and installation of the equipment.

- Tests and Acceptance

  - The following conditions supplement paragraph 12 (Rejection of Goods) of the Standard Conditions of Contract and identify that the acceptance of the instrument(s) need to satisfy the Agency’s nominated representative at post-commissioning stages.

  - On completion of installation the supplier shall verify full operation of the system to demonstrate that the performance meets the requirements of the specification to the satisfaction of the Agency's nominated representative.

  - The giving of the above notices by the Agency shall not in any way prejudice its rights if it is otherwise discovered later that the Goods do not meet the specification or otherwise fail to comply with the terms of the Contract.

- Price and Payment

  - The Contract Price shall be "net", that is, after deduction of all agreed discounts and fixed for the period of the contract. It shall include the cost of all royalties, licence fees, WEEE responsibilities, packaging, packing materials, addressing, labelling, loading, delivery to the addresses named in the Contract or orders, installation, commissioning and be in accordance with the amount set out in financial cost statement. All other costs, charges, fees and expenses of whatever kind for or arising out of or in connection with the provision of the Goods and  Services shall be paid by the Contractor unless otherwise agreed in writing by the Agency

  - Any early settlement or multiple purchase discounts shall be shown separately in the Contract.

  - The payment details for the support and maintenance after the warranty period to be agreed

- Environmental Consideration

  - The following Conditions supplement paragraph 26 (Environment) of the Standard Conditions of Contract and are specific for Contractors supplying chemicals or any other hazardous goods or equipment where such products are used.

  - The Contractor should ensure that all hazardous goods supplied to the Agency must be marked with the International Danger Symbol(s) and display the name of the material in English.  All documentation must include a declaration of the hazard and the name of the material in English.  Goods must be accompanied by emergency information in English in the form of written instructions, labels or markings.  The Contractor shall ensure that all information held by, or reasonably available to him, regarding any potential hazard known, or believed to exist in the transport, handling or use of the Goods supplied is notified to the Agency.

  - The Contractor shall notify the Agency of any Goods supplied for which there is a potential to cause environmental damage through their use and provide information to the Agency on the steps they are taking to minimise such damage.

  - The Contractor shall ensure that he complies with all relevant legislation relating to the "Duty of Care and Registration of carriers" as detailed in the Environmental Protection Act 1990.

- Training and Operating Manuals

  - The Contractor shall supply free of charge installation drawings and manuals containing instructions for the use and maintenance of the Goods.

  - The Supplier must provide full documentation and operational manuals (including software), for the operation and maintenance of the equipment.

- Maintenance and Spares

  - The Supplier must detail the routine operator maintenance required to keep the system performing to specification.

  - The Supplier must be capable of offering maintenance and support packages including preventative maintenance and repair services.

  - The Environment Agency makes no commitment to purchase maintenance cover and/or consumables under this contract and reserves the right to source form other parties to ensure value for money is achieved and shall not in any way prejudice its rights under the agreed contract terms.