Area 3 Asset Support Contract

Service Information

Annex 7

Part 1 - Employer’s Vehicles,

Part 2 – Severe Weather Plan Stocks

and

Part 3 - Employer’s Stocks

SERVICE INFORMATION FOR AREA 3 ASC

ANNEX 7

CONTENTS AMENDMENT SHEET

| Amend. No. | Issue Date | Amendments | Initials | Date |
| --- | --- | --- | --- | --- |
| 0 | August 2012 | ### First Issue | SOS | 21/08/12 |
| 1 | October 2012 | Paragraph 7.7.3 amended to include ‘SE’. | SOS | 24/10/12 |

LIST OF CONTENTS

PART 1 - EMPLOYER’S VEHICLES	4

7.1	Relevant Clauses	4

7.2	Use	4

7.3	Maintenance	6

PART 2 – SEVERE WEATHER PLAN MATERIAL STOCKS	11

7.4	Salt Stocks and Other Materials	11

7.5	Storage	11

7.6	Strategic Salt Stock Requirement	11

PART 3 – EMPLOYER’S STOCKS	12

7.7	Traffic Technology Equipment Procurement and Traffic Technology Logistics	12

7.8	Airwave Equipment	15

APPENDIX A: 	ACCIDENT REPORTING PROCEDURES	17

APPENDIX B:	 PLANNED PERIODIC MAINTENANCE	18

APPENDIX C: 	TECHNICAL LITERATURE	20

APPENDIX D:	 DRIVER OPERATOR TASKS	21

APPENDIX E:	MAINTENANCE FORMS AND RECORDS	23

# PART 1 - EMPLOYER’S VEHICLES

## Relevant Clauses

### The following section sets out the relevant clauses that apply to the Employer’s Vehicles. Maintenance terms used in this document are defined in the relevant Winter Maintenance Support Publications listed in Appendix C to this Annex.

## Use

### 7.2.1	Vehicles

      - Employer’s Vehicles are

        - Area Operational Winter Service Vehicles (including the  Operational Reserve)

        - National Reserve Winter Service Vehicles

        - Snow Blowers

      - The Area Operational Winter Service Vehicles (including the Operational Reserve) are the vehicles listed in the Network Information and summarised in the Severe Weather Plan at Appendix B.4, which the Provider may use to provide the Services.

      - The number of National Reserve Winter Service Vehicles is confirmed in the Network Information, the use of which shall be in accordance with Section 3.4 and Appendix B.4 of the Severe Weather Plan.

      - The Service Manager may substitute any vehicle with an equivalent vehicle at any time during the contract period.

      - The Employer provides excise licences for the Employer's Vehicles.

### 7.2.2 	Operation

- (1)	The de-icing material capacity and distribution patterns of the Employer’s Vehicles are designed for use on motorways and All-Purpose Trunk Roads. The Provider shall only deploy the Employer's Vehicles for winter and severe weather service delivery on the Area Network, unless otherwise approved by the Service Manager.

- (2)	The Provider shall rotate the use of all Employer’s Vehicles (including operational and national reserve vehicles) to achieve equal individual vehicle mileage as far as is reasonably practicable.

### 7.2.3	Fuel

      - The fuel to be used in the Employer’s Vehicles is winter grade Ultra Low Sulphur Diesel or winter grade Ultra Low Sulphur Gas Oil conforming with BS EN 590 requirements and providing protection from waxing to –15C. High sulphur content fuels shall not be used in the Employer’s Vehicles.

      - Requirements for recording fuel stocks and storage are set out in Appendix A.15 of the Severe Weather Plan

      - The Provider supplies fuel to be used in the Employer’s Vehicles.

### 7.2.4	Driver and Operatives

      - The Provider provides drivers and operatives for the Employer’s Vehicles which the Provider is using to provide the Services.

      - The Provider complies with the statutory and special licensing requirements for drivers of the Employer's Vehicles.

      - Area Operational Winter Service Vehicles (including the Operational Reserve) and National Reserve Winter Service Vehicles are classified as Category C; Large Goods Vehicles (LGV). Snow Blowers are classified as Category C vehicles.

      - Authorised drivers of the Employer's Vehicles are required to hold a valid UK driving licence, or the equivalent recognised licence issued by a state of the European Economic Area for the relevant category of vehicle.

### 7.2.5	Reporting of Vehicle Accidents

      - The Provider issues to all personnel employed by the Provider to drive and operate the Employer's Vehicles, copies of the Employer’s accident reporting procedures set out in Appendix A of this Annex.

      - The Provider reports all accidents in accordance with the requirements set out in the Severe Weather Plan.

### 7.2.6	Garaging

- (1)	The Provider garages the Employer’s Vehicles and component parts such as snow ploughs, ensuring that they are suitably covered and protected. Highways Agency owned depots should be used where possible.

### 7.2.7	Joint Inspections

      - The Provider inspects and assesses jointly with the Service Manager the condition of the Employer's Vehicles in accordance with the inspection standards and procedures contained within the Technical Literature shown in Appendix C to this Annex. The Provider prepares an inspection report within 14 days of the inspection.

      - The inspections are carried out:

    - within 28 days of the access date,

    - within 28 days before the end of the Contract Period,

    - at any other time as requested by the Service Manager.

## Maintenance

### 7.3.1.	General

      - The Provider appoints a Fleet Service Manager qualified to hold an ‘O’ Licence, and skilled mechanics who have served a recognised apprenticeship in the motor vehicle repair trade and have had a minimum of 5 years experience. The Provider maintains documentary evidence of qualifications and experience of his personnel.

### 7.3.2 	Maintenance Activities

      - The Provider maintains the Employer's Vehicles and maintenance is to comprise the following categories of work:

        - Driver/Operator tasks

- These are set out in Appendix D to this Annex.

        - Planned periodic maintenance

- The categories for planned periodic maintenance are highlighted in Appendix B and are described in detail in Appendix E to this Annex.

- During the first week of each calendar month, the Provider prepares a maintenance programme and attaches the appropriate specific WM11 forms for each individual service required.

- On completion of the work the Provider conducts a formal Out Inspection of the equipment to confirm that all works have been correctly completed. The Provider signs and certifies the equipment’s state of roadworthiness on individual WM11 forms. If he finds any Employer’s Vehicle is unroadworthy or unsafe to use he notifies the Service Manager.

- (c)  		Repairs and Modification

- The Provider carries out repairs to Employer's Vehicles up to the value of £1000 as quickly as possible. Repairs exceeding £1000 should be agreed with the Service Manager. In exceptional circumstances, where prior authorisation may not be possible, full justification and details of the work undertaken shall be provided to the Service Manager after the event.

- If the vehicle is still in warranty the Provider arranges such repairs immediately and reports to the Service Manager as soon as possible.

- The Provider does not make any modification to the Employer's Vehicles or fit any equipment to them without the prior approval of the Service Manager.

- (d)	Painting and preservation

- The Provider carries out repairs to paintwork or protective coatings to the Employer's Vehicles to prevent deterioration of vehicle components as agreed with the Service Manager, following an assessment prior to any C services.

### (e)	Breakdown and Recovery

- The Provider provides a suitably equipped and staffed specialised breakdown and recovery service, capable of undertaking all aspects of vehicle recovery and roadside repair for the Employer’s Vehicles, to be available at all times during the contract period.

- (2) 	In carrying out the maintenance work, the Provider uses methods and standards prescribed by current legislation, the Employer’s Winter Maintenance Support Publications and manufacturers’ Technical Literature as described in Appendix C to this Annex.

- (3)		The Provider takes all reasonable steps to complete maintenance works to each of the Employer’s Vehicles and to return the vehicle ready for operation at its appointed location at the end of each working day.

### Calibration

- (1) 	The Provider shall undertake vehicle calibration (either through the manufacturer or directly following suitable training) annually to comply with the manufacturer’s requirements/ specification using a competent mechanic/technician.  A dated and signed calibration certificate shall be produced and held by the Provider in the vehicle record file and must be available on request.

### Supply of Spare Parts and Materials

- (1) 	The Provider shall supply and fit vehicle parts offering the best value for money, whilst meeting appropriate specification and performance requirements.

- (2)	The Service Manager’s authorisation will be required for non-proprietary parts and for the supply of individual items greater than £1000.00. In exceptional circumstances, where prior authorisation may not be possible, full justification and details of the work undertaken shall be provided to the Service Manager after the event.

- (3)	The Service Manager may instruct the Provider to establish a stock of spare parts to ensure that critical components are available immediately.

### 7.3.6 	Retention of Defective Components

- (1)	The Provider retains for a period of one month for inspection by the Service Manager any components with a replacement value greater than £1000.00 removed from vehicles, plant or equipment and declared by the Provider as beyond economic repair or beyond repair.

      - The Provider shall also retain defective components where serious or repeated failure is apparent.

### 7.3.7 	Technical Literature

- (1)	Prior to the access date, the Employer issues to the Provider one copy of the technical documents listed in Appendix C to this Annex. These include Winter Maintenance Support Publications (WM11 Series), which define details of the maintenance and repair of the Employer’s Romaquip, Schmidt and Rolba vehicles. The copies of technical support documents are to be returned to the Employer at the end of the Contract Period.

### 7.3.8 	Defect Reporting

- (1)			Within one week of the access date, the Provider prepares a vehicle state report covering all of the Employer's Vehicles. Subsequently, the Provider updates this report whenever the state of any of the Employer's Vehicles changes. The Provider makes this report available for inspection by the Service Manager at all times, and provides copies of the report when requested by the Service Manager. The format of the report will be at the discretion of the Service Manager.

  - The Provider monitors defects occurring to the Employer's Vehicles, and raises vehicle and equipment defect reports where premature failure, excessive wear, unacceptable loss of performance or unreliability has occurred. The Provider will complete and maintain a daily register of individual vehicle defects, which will include confirmation of the number of ‘vehicles off the road’ (VORs).  The VOR definition covers those vehicles that are unusable through serious defect or malfunction for a period of more than 24 hours.  The Provider makes the register available weekly to the Service Manager from 1 October to 30 April.

### Pre-Winter Checks

      - In September of each year, the Provider shall:

        - check and calibrate the Employer’s Vehicles for full operational use including rates of spread of de-icing materials and spread patterns in accordance with 7.3.4.

        - fits and remove the plough to all the Employer’s vehicles. The Provider shall submit a written report to the Service Manager and within 3 days of this operation including details of time taken to fit the ploughs and any problems encountered.

### 7.3.10 	Vehicle Maintenance Records

- (1)	The Provider maintains an individual service, inspection and maintenance record for each of the Employer's Vehicles. The records are returned to the Employer at the end of the Contract Period. The Service Manager is entitled to examine the records and request copies at any time. The records contain the following documents generated during the Contract Period:

        - Original

          - Completed WM11 forms

          - Completed specific forms listed in Appendix E

          - Tachograph Calibration Certificates

          - Spreader Body Calibration Certificate

          - Workshop Job Cards

          - Spent copies of Vehicle Record Books WM31R (Rolba)

          - Re-call notices

          - Manufacturers warranties

          - Legal notices (GV9’s etc)

        - Copy

          - Invoices

          - Accident Reports

          - Damage Reports

          - Defects Reports

### 7.3.11	Deployment of National Reserve

- (1)	The Provider manages the deployment and recovery of the National Reserve Vehicles from and to any location within or outside the Area Network when instructed by the Service Manager, and in accordance with the Severe Weather Plan. The maximum response time from receipt of instruction from the Service Manager to the commencement of vehicle delivery is 1 hour.

## PART 2 – SEVERE WEATHER PLAN MATERIAL STOCKS

## 7.4	Salt Stocks and Other Materials

- (1)	The Provider supplies de-icing material for use with the Employer’s Vehicles.

- (2)		The Provider maintains the Salt Stocks set out in Appendix B.3 of the Severe Weather Plan comprising

        - Operational Salt Stock at Start of Season

- b)	Minimum Contract Level

- c)	Strategic Salt Stock Requirement

  - The Strategic Salt Stock Requirement is reserved for use only in accordance with Section 3.3 and Appendix B.3 of the Severe Weather Plan

  - The Provider maintains adequate stocks of other materials      required for the delivery of the Winter Service and Severe Weather Service.

## Storage

      - The Provider ensures that storage space is sufficient for maintaining Operational Salt Stock levels and other materials required for the delivery of the Winter Service and Severe Weather Service.

      - The Provider may use the Employer’s premises or other locations for this purpose. Salt and other materials shall normally be stored in the locations identified by the Provider for the Employer’s Vehicles and other vehicles required by the Provider for the delivery of the Winter Service and Severe Weather Service.

## 7.6	Strategic Salt Stock Requirement

      - The Service Manager identifies the Strategic Salt Stock Requirement and agrees the storage location(s) with the Provider

      - The Provider obtains an estimate of the cost of providing and maintaining the Strategic Salt Stock Requirement as required by the Service Manager. The Service Manager confirms the cost to be paid to the Provider as a Compensation Event.

      - Usage and replenishment of the Strategic Salt Stock Requirement is governed by the Employer as set out in Section 3.3 and Appendix B.3 of the Severe Weather Plan.

## PART 3 – EMPLOYER’S STOCKS

## 7.7	Traffic Technology Equipment Procurement and Traffic Technology Logistics

### 7.7.1 	Traffic Technology Equipment Procurement is a team within the Traffic Technology Division of the Employer that undertakes a range of supply chain activities with which the Provider can engage.  The range of activities undertaken includes:

      - the specification and central purchase of a range of parts and    equipment for major Schemes and routine maintenance,

      - setting of minimum and maximum levels for Provider stock replenishment,

      - the central coordination of the repair of a selected range of items,

      - the maintenance of an electronic catalogue detailing the Employer’s (Oracle Code) part number for all parts and equipment used on the Area Network.

- MCH2538 details the interaction between the Provider and the Employer in relation to the above activities.

### 7.7.2	Traffic Technology Logistics is a team within the Traffic Technology Division of the Employer that undertakes a range of supply chain activities with which the Provider can engage.  The range of activities undertaken includes:

- (1)	the provision of a National Distribution Centre (NDC), responsible      for:

  - the consolidation of equipment for delivery to road schemes,

  - the central storage of parts and equipment required for maintenance of the network,

  - the storage of parts and equipment taken out of service that can be reused and

  - cannibalisation of parts to support legacy equipment.

- The NDC is located in Telford.

- the operation of the Stock Management System including the capture of stock requests from the Provider.

- the transportation of stock from the NDC to the Provider’s store,

- the collection of stock to be returned to the NDC from the Provider’s store.

- MCH2538 details the interaction between the Provider and the Employer in relation to the above activities.

### 7.7.3 	At the access date the Provider will be supplied with a stock of parts and equipment to support the Area Network and the RTMC (SE). These parts and equipment will remain the property of the Employer and the Provider will be responsible for their safe keeping and correctly accounting for them.

### During and at the end of the Contract Period, the Provider must return all stocks of parts and equipment that remain the property of the Employer and will be liable for the cost of replacement for any missing items not correctly returned.

### The National Logistics Manager will undertake an audit of the Provider’s stores periodically.

### The procedures manual (MCH2538 – Technology Maintenance Stock Management Operating Procedures) details the audit regime and how to care and account for technology stock.

### MCH2538 also stipulates how the Stock Management System is to be used, including the mandatory use of Employer’s warehouse management software (Chess Empirica Lite) that must be utilised by the Provider to manage technology stock.

### MCH2538 must be adhered to as a condition of the Employer providing parts and equipment to support the Provider.

### During the Contract Period the Provider may, with authority from the Service Manager, reorder parts and equipment from Traffic Technology Logistics.

### The NDC will undertake one delivery per week per Provider based upon minimum-maximum stock replenishment calculations performed every Monday during the Contract Period.

### The time of delivery is agreed between the NDC, the National Logistics Manager and the Provider.  This day and time slot will be fixed unless application is made by the Provider to change it, such a change must be agreed by the National Logistics Manager.

### Any stock need that is required ahead of the next scheduled weekly delivery slot will be evaluated by the Traffic Technology Logistics Team as an emergency stock requirement.  Application must be made by the Provider’s Stores Manager to the Traffic Technology Logistics Team who will decide whether an extra delivery is warranted under the circumstances particular to the specified stock requirement described by the Provider.

### Where an emergency delivery is declined by the Traffic Technology Logistics Team, the requested stock, where available, will be added to the regular scheduled weekly delivery for the Provider.

### When agreeing the regular scheduled delivery date, the Provider is responsible for highlighting any particular delivery vehicle requirements for agreement with the National Logistics Manager such as:

  - vehicle with tail hoist,

  - vehicle with lorry mounted fork lift truck or

  - vehicle with lorry mounted crane.

### The Provider may use Employer supplied parts and equipment to change items of equipment on the Area Network but the faulty item replaced should be returned to the NDC Third Line Repair facility as soon as possible so that it may be put back into good stock.

### Application to return faulty items must be made through the Stock Management System to the Traffic Technology Equipment Procurement Team as outlined in MCH2538

### The Provider shall regularly review the stock held in its store and shall additionally:

- regularly review stock levels and return all surplus and unwanted stock to the NDC,

- ensure that faulty stock (identified as “Red” stock within the Stock Management System) is returned to the NDC following application to the Traffic Technology Equipment Procurement Team (process detailed in MCH2538),

- ensure the store is well laid out and tidy,

- ensure the store is open 24 hours a day, seven days a week and

- ensure records are up to date and accurate through the prompt and timely use of the warehouse management system component of the Stock Management System.

### The warehouse management system software Chess Empirica Lite, remote encryption systems (Citrix), and all required usernames are provided to the Provider by the Employer’s ICT service provider.

### Access to the above systems is provided through encrypted communications and each individual requiring access will require a user specific encryption dongle.  These will be provided to the Provider on request to the Employer’s ICT service provider.  It should be noted that the Employer’s IT security stipulations shall apply.

### A Windows PC with an ADSL broadband type internet connection must be sited within the stores and made available by the Provider on which the warehouse management software can be installed.  This software operates on a thin-client basis connecting to the Employer’s warehouse management servers alongside Oracle financial systems.  The PC has these minimum requirements:

- Windows XP / Vista / 7

- Intel Celeron 2GHz processor or equivalent

- 256 MB of RAM

- 20 MB of hard disc space

    - In addition to the PC and ADSL connection, a Zebra Label printer shall be procured by the Provider to be utilised in conjunction with the warehouse management software to enable printing of stock labels.  The recommended printer is the Zebra LP2844-Z label printer utilising labels of width 4” and height 2¼”.  The Provider is responsible for purchasing labels.  Alternative label printers may be used only by agreement with the Traffic Technology Logistics Team and the Employer’s ICT service provider.

### A laser printer compatible with (at minimum) PCL5 shall be procured by the Provider to enable printing of documentation from the warehouse management system.

### The PC, Printers and Internet Connection are the property of the Provider and all related costs such as purchase, maintenance and support will be covered by the Provider.

    - Warehouse management remote client software will be provided to the Provider for installation by the Provider’s internal ICT resource.  Management and support for this client software will be provided by the Employer’s ICT service provider.

## 7.8	Airwave Equipment

### 7.8.1	Airwave communication equipment and associated TRAKA cabinets as listed in Network Information Section 3.2 are made available for use by the Provider.

### 7.8.2	The Provider shall notify the Service Manager of any defective Airwave communication equipment within five working days of the defect being identified.

### 7.8.3	Where damage to the Airwave communication equipment is due to negligence by the Provider or any defective communication equipment returned for repair is not deemed a warrantable repair, the Provider shall be liable for the replacement or repair costs.

7.8.4	Any maintenance, repair, replacement costs or other costs incidental to safeguarding the equipment and the associated TRAKA cabinets shall be at the expense of the Provider.

# APPENDIX A: 	ACCIDENT REPORTING PROCEDURES

The Employer’s accident reporting procedures, and relevant forms, are as follows:

The Provider reports all accidents involving the Employer's Vehicles to the Service Manager in accordance with the requirements set out in Section 2.1.2.7 of the Severe Weather Plan.

The driver of the vehicle involved in an accident completes a Traffic Accident Report (Form HA 20001), a blank copy of which is kept in the cab of each vehicle. The form is to be completed so far as is possible at the scene of the accident and the driver prepares a signed written statement of the circumstances of the accident. The form, statement and any other signed statements that have been obtained from witnesses and any other relevant data are forwarded to the Service Manager soon as possible after the accident. The Provider provides all information relevant to the accident, including an estimate of the cost of repairs.

# APPENDIX B:		PLANNED PERIODIC MAINTENANCE

- Table 1 lists the types and estimated quantity of planned periodic maintenance of the Employer's Vehicles. The annual frequency of servicing and inspections may be determined and amended by the Service Manager as operational and economic requirements dictate. Such changes are to be notified on submission of the Monthly Maintenance Plan

- TABLE 1 - PLANNED MAINTENANCE

| Equipment Type | Task | Estimated Annual Frequency | Estimated time for servicing/ inspection | Maintenance Form Ref | Maintenance Procedure |
| --- | --- | --- | --- | --- | --- |
| Romaquip and Schmidt vehicles | A Service | 9 (Note 1) | 3hrs Fixed Price | WM11/MAN-Volvo A |  |
|  | C Service | 1 | 14hrs Fixed Price | WM11/MAN-Volvo C |  |
|  | Full Inspection | 1 | 6hrs Fixed Price | WM11/MAN-Volvo Full |  |
|  | Brake Test | 1 | Fixed Price | Test Station Form | By qualified personnel at testing station |
|  | Brake inspection - check and lubricate | After 1st year of use, every two years thereafter | Fixed Price |  | By qualified maintenance personnel |
|  | Spreader Body Calibration | 1 | Fixed Price | Manufacturer or HA supplied form | By Manufacturer or suitably qualified maintenance personnel |
|  | Tachograph Calibration | Every two years | Fixed Price | Test Station Form | By qualified personnel at testing station accredited by Vehicle Inspectorate |
| Snowplough Blade | Pre season Combined Service and Inspection | 1 (Note 2) | 1.5hrs Fixed Price | WM11/PL(S) and WM11/Rom-SP |  |
|  | Mid season Combined Service and Inspection | 1 (Note 2) | 1.5hrs Fixed Price | WM11/PL(S) and WM11/Rom-SP |  |
| Rolba Snowblower | B Service | 8 | 3hrs Fixed Price | WM11/Rolba-B/C |  |
|  | C Service | 1 | 16hrs Fixed Price | WM11/Rolba-B/C |  |
|  | Brake Test | 1 | Fixed Price |  |  |

Note 1.	This service and inspection is normally carried out when the vehicles are in use. Provider to recommend to the Service Manager the extent of work required taking into account the actual use of the vehicles. The frequency of the service and inspection shall be a minimum of 9 per year.

Note 2.	The frequency of the service and inspection is dependant on the usage of the plough blade.

Note 3.	Details of Periodic Maintenance undertaken on the vehicle are to be recorded by the maintainer in the Vehicle Defect Record Book.

Note 4.	Service sheets will be specific to different vehicle suppliers.

# APPENDIX C: 	TECHNICAL LITERATURE

- The technical documents to be issued by the Employer are as follows:

|  | Romaquip Operators manual |
| --- | --- |
|  | Romaquip Maintenance manual |
|  | Schmidt Operators manual |
|  | Schmidt Maintenance manual |
|  | Romaquip Spare Parts manual |
|  | Schmidt Spare Parts manual |
| Un-referenced WM31R | ROLBA 400 Operator’s Manual ROLBA 400 Operator Instructions |
| Un-referenced WM31R | ROLBA 1000 Operator’s Manual ROLBA 1000 Operator Instructions |
| WM11 series | Maintenance Forms (On demand) |
| HA 20001 | Traffic Accident Report Form (On demand) |

The Literature detailed above is available for inspection on request.

- [Compiler to insert address where documents and literature etc can be inspected.]

# APPENDIX D:		DRIVER OPERATOR TASKS

- The following table lists the documents, which describe the Driver Operator tasks to be carried out when the Provider operates the Employer's Vehicles under instruction from the Service Manager.

| Equipment | Reference Number | Title |
| --- | --- | --- |
| Romaquip and Schmidt vehicles | WM11/PL (S) WM11/ROM-SP WM11-D/OD | Operating Information Vehicle Record |
| ROLBA 400 | No reference number provided WM31R | ROLBA 400 Operator’s Manual ROLBA Operator Instructions |
| ROLBA 1000 | No reference number provided WM31R | ROLBA 1000 Operator’s Manual ROLBA Operator Instructions |

A reference in the documents to Highways Agency Area Managers or Vehicle Inspectors means the Service Manager.

The Provider’s Driver/Operators are to:

- Inspect the Vehicle Defect Record Book prior to operating the vehicle or equipment, and only proceed if the vehicle is serviceable.

- Enter details of all journeys undertaken on public highways, together with any fault occurring during periods of operation into the Vehicle Defect Record Book.

Operator Maintenance for the Employer's Vehicles is detailed in the Manufactures’ Handbook provided. The information in the table below is indicative and provided for tender purposes only. The Provider operates and maintains equipment to instructions that may be amended from time to time. Tasks to be carried out on the Rolba machines are contained within the operator’s manual.

DRIVER/OPERATOR CHECKS AND TASKS

| Driver/Operator Duties |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Item No | Check and report | Each Operation |  |  |  |  |
|  |  | Before | During | After |  |  |
| 1 | Outstanding defects from the driver defect book. |  |  |  |  |  |
| 2 | Engine oil, & coolant level, top up if required. |  |  |  |  |  |
| 3 | Windscreen/ headlight wash bottle levels, top up if required. |  |  |  |  |  |
| 4 | All lights, reflectors & mirrors (operation & condition) |  |  |  |  |  |
| 5 | All wheels & tyres, condition, security (visual check) & inflation, including plough when fitted. Check plough wheel bearings for noise & excess side play when in raised position. |  |  |  |  |  |
| 6 | Vehicle for accident damage, including reject screens & plough when fitted. |  |  |  |  |  |
| 7 | Hydraulic oil level, top up if required. |  |  |  |  |  |
| 8 | Presence of tax disc & in-cab fire extinguishers. |  |  |  |  |  |
| 9 | All switches & warning devices (including gauges) |  |  |  |  |  |
| 10 | Driving controls (steering, gear change, windscreen & wash wipe, etc.) |  |  |  |  |  |
| 11 | Oil & coolant leaks. |  |  |  |  |  |
| 12 | Body operation, conveyor, spinners, salt discharge door, pre-wet, chemical |  |  |  |  |  |
| 13 | Security, operation & condition of plough (when fitted), ensure marker poles are present. |  |  |  |  |  |
| 14 | Adjustment of plough (when fitted) |  |  |  |  |  |
| 15 | Condition of plough rubbers (when fitted) |  |  |  |  |  |
| 16 | Cab heater. |  |  |  |  |  |
| 17 | Access ladders security in raised position |  |  |  |  |  |
| 18 | Operation of all driving controls. |  |  |  |  |  |
| 19 | Operation of body and Chemical spray equipment (if fitted) |  |  |  |  |  |
| 20 | Operation of all gauges & warning devices. |  |  |  |  |  |
| 21 | Visibility of all lights & reflective surfaces |  |  |  |  |  |
| 22 | When fitted, adjustment & operation of plough |  |  |  |  |  |
| 23 | Overheating of components (wheels, hubs brakes), including plough when fitted. |  |  |  |  |  |
| 24 | Full wash down to remove all traces of salt, clean out cab, and carry out short road test to dry out brakes. |  |  |  |  |  |
| 25 | Complete entry in vehicle record book, including defect book if required. |  |  |  |  |  |
| 26 | Top up fuel tank and AdBlue tank (if fitted) |  |  |  |  |  |
| Every Two Weeks |  |  |  |  |  |  |
| 1 | Carry out 20km non-motorway maintenance run without the plough blade fitted, including full operation of body controls (including any data logging equipment), followed by a full detergent hot wash to remove all salt and road grime. The maintenance run should be carried out in line with vehicle manufacturers guidelines and with the approval of the Service Manager. This will ensure, for example, that the engine is operated at the appropriate temperature. (The maintenance run is not required if the vehicle has been used within the two-week period and has covered more than 20km on a single run.) |  |  |  |  |  |
| 2 | Clean out the vehicle cab. |  |  |  |  |  |
| 3 | Complete entry in vehicle record book. |  |  |  |  |  |
| Annually/End of Season |  |  |  |  |  |  |
| 1 | Empty, clean & flush out brine tanks / Chemical tanks (if fitted) & system with clean water. |  |  |  |  |  |
| 2 | Carry out full hot water wash down, including under vehicle |  |  |  |  |  |

WM11- D/OD Rev. 2010-06-02

# APPENDIX E:	MAINTENANCE FORMS AND RECORDS

The Table below lists WM11 forms to be used by the Provider.

| WM11 Form No | Equipment | Purpose |
| --- | --- | --- |
|  | General | Maintenance Programming |
| WM11/MAN-Volvo A | Romaquip and Schmidt vehicles | A Service |
| WM11/MAN-Volvo C |  | C Service |
| WM11/MAN-Volvo Full |  | Full Inspection |
|  | Romaquip and Schmidt | Spreader Calibration Certificate |
|  | General | Vehicle Brake Test |
| WM11/PL(S) WM11/Rom-SP | Snowplough | Pre season Combined Service and Inspection |
| WM11/PL(S) WM11/Rom-SP |  | Mid season Combined Service and Inspection |
|  | General | Issue and Receipt |
|  | Rolba 400 and 1000 | Safety Inspection |
|  |  | B Service |
|  |  | C Service |

Forms detailed above are available for inspection on request.

| I certify that all items listed have been inspected |  |  |  |  |  | Service Instructions for H.A. Salt Spreader |  |  | Reg. No. |  | H.A. ID No |  | Vehicle Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  | Romaquip Body |  |  |  |  | WS |  |  |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  | Speedo Reading |  |  | Date Carried Out |  |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  | A |  |  |
|  | Check & Top Up |  |  |  |  | 25 | Brake actuators & air valves |  |  | Check Operation |  |  |  |  |
| 1 | Engine oil |  |  |  |  | 26 | Air, fuel & hydraulic pipes. |  | 51 | All switches, instruments & warning devices |  |  |  |  |
| 2 | Engine coolant (50% antifreeze) |  |  |  |  | 27 | Suspension & dampers |  | 52 | All driving controls & lights |  |  |  |  |
| 3 | Power steering oil |  |  |  |  | 28 | Drive shafts, UJ’s, flanges & safety covers. |  | 53 | All wipers & washers |  |  |  |  |
| 4 | Hydraulic oil |  |  |  |  | 29 | Hydraulic valves (manual override at 0) |  | 54 | Cab heating systems |  |  |  |  |
| 5 | Clutch fluid (if applicable) |  |  |  |  | 30 | All exposed electrical cables & connections |  | 55 | Air pressure build up, check for leaks |  |  |  |  |
| 6 | Screen wash reservoir |  |  |  |  | 31 | Body & salt loading screens |  | 56 | Power steering & serviceability |  |  |  |  |
| 7 |  |  |  |  |  | 32 | Access ladders |  | 57 | Service & parking brake operation |  |  |  |  |
| 8 |  |  |  |  |  | 33 | Plough mounting frame & connections |  | 58 | Body operation |  |  |  |  |
|  | Check & Adjust if Required |  |  |  |  | 34 | Cameras & cables |  | 59 | Cameras operation & visibility |  |  |  |  |
| 9 | Engine drive belts |  |  |  |  | 35 | Brine tanks & equipment (clean filters) |  | 60 | Carry out 20k road test (operate body on test) |  |  |  |  |
| 10 | Brakes |  |  |  |  | 36 | Fuel tank |  | 61 | Discharge & rear door actuators |  |  |  |  |
| 11 | Conveyor belt & scrapers |  |  |  |  | 37 | AdBlue tank |  | 62 | Reflective materials, markers, signs & labels |  |  |  |  |
| 12 | Tyre pressures |  |  |  |  | 38 |  |  | 63 |  |  |  |  |  |
| 13 |  |  |  |  |  | 39 |  |  | 64 |  |  |  |  |  |
| 14 |  |  |  |  |  | 40 |  |  |  | After Road Test |  |  |  |  |
|  | Check Security & Condition |  |  |  |  |  | Clean, Protect & Lubricate |  | 65 | Check for oil, fuel & coolant leaks |  |  |  |  |
| 15 | Seats & seat belts |  |  |  |  | 41 | All grease points including pump drive |  | 66 | Check for overheating components |  |  |  |  |
| 16 | Glass & mirrors (view front & rear) |  |  |  |  | 42 | All cables, joints, pivots & levers |  | 67 |  |  |  |  |  |
| 17 | Driving controls & fire extinguisher |  |  |  |  | 43 | Windscreen wiper linkage |  | 68 |  |  |  |  |  |
| 18 | Road wheels, tyres & indicator flags |  |  |  |  | 44 | All hinges & locks |  |  | Check & Report Condition Of |  |  |  |  |
| 19 | Cab & body mountings |  |  |  |  | 45 | Brake clevis pins |  | 69 | Cab, all paintwork & preservation |  |  |  |  |
| 20 | Rear chute, springs & spinner ass. & catches |  |  |  |  | 46 | All exposed ram & actuator pistons |  | 70 | Bodywork ( damage, cracks etc.) |  |  |  |  |
| 21 | Salt sensor & cable |  |  |  |  | 47 | Hydraulic, electrical & brine connections |  | 71 | Rear & side reflectors & materials |  |  |  |  |
| 22 | All chassis mounted equipment |  |  |  |  | 48 |  |  | 72 | Vehicle cleanliness |  |  |  |  |
| 23 | Exhaust system, EGR valve & retarder |  |  |  |  | 49 |  |  | 73 |  |  |  |  |  |
| 24 | Steering linkage |  |  |  |  | 50 |  |  | 74 |  |  |  |  |  |

WM11/MAN-Volvo-A   Rev. 2010-06-02

| I certify that all items listed have been inspected |  |  |  |  |  | Service Instructions for H.A. Salt Spreader |  |  | Reg. No. |  | H.A. ID No. |  | Vehicle Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  | Romaquip Body |  |  |  |  | WS |  | MAN & Volvo |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  | Speedo Reading |  |  | Date Carried Out |  |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  | C |  |  |
|  | Renew |  |  |  |  |  | Lubricate |  | 49 |  |  |  |  |  |
| 1 | Engine oil & filter |  |  |  |  | 26 | All grease points |  | 50 |  |  |  |  |  |
| 2 | Fuel filters & fuel line pre filter |  |  |  |  | 27 | Cab door hinges & locks |  | 51 |  |  |  |  |  |
| 3 | Hydraulic oil return line filter (every 2 years) |  |  |  |  | 28 | All cables, pivots & joints |  | 52 |  |  |  |  |  |
| 4 | Hydraulic oil pressure filter (every 2 years) |  |  |  |  | 29 | Exposed ram & actuator pistons |  |  | Clean Protect and or Paint |  |  |  |  |
| 5 | Desiccant kit (every 2 years) |  |  |  |  | 30 | Seat runners & adjusters |  | 53 | All exposed electrical connections |  |  |  |  |
| 6 | Gearbox oil (MAN) (every 3 years) |  |  |  |  | 31 | All brake actuator clevis pins (remove pins) |  | 54 | All brake valves and actuators |  |  |  |  |
| 7 | Axle Diff. oils (every 3 years) |  |  |  |  | 32 | Conveyor adjusting bolts |  | 55 | Discharge actuators and drive motors |  |  |  |  |
| 8 | Hydraulic oil (every 3 years) |  |  |  |  | 33 |  |  | 56 | Any exposed aluminium or mild steel |  |  |  |  |
| 9 | Transfer box oil (4X4) (every 3 years) |  |  |  |  | 34 |  |  | 57 |  |  |  |  |  |
| 10 | Engine coolant (every 4 years) |  |  |  |  | 35 |  |  | 58 |  |  |  |  |  |
| 11 | *Volvo G/box oil & filters (* See below) |  |  |  |  | 36 |  |  | 59 |  |  |  |  |  |
| 12 | Primary air filter (every 2 years) |  |  |  |  | 37 |  |  | 60 |  |  |  |  |  |
| 13 | Secondary air filter (every 4 years) |  |  |  |  | 38 |  |  | 61 |  |  |  |  |  |
|  | *Change 1st year & every 2 years after |  |  |  |  |  |  |  | 62 |  |  |  |  |  |
|  | Check & Top U p |  |  |  |  |  | Adjust if Required |  | 63 |  |  |  |  |  |
| 14 | Gear box oil |  |  |  |  | 39 | Conveyor belt |  |  |  |  |  |  |  |
| 15 | Rear axle oil |  |  |  |  | 40 | Engine drive belts |  |  | Road Test |  |  |  |  |
| 16 | Screen wash reservoir, add wash fluid |  |  |  |  | 41 |  |  | 64 | Carry out 20k road test (non motorway) |  |  |  |  |
| 17 | Hydraulic oil reservoir |  |  |  |  | 42 |  |  |  | After Road Test |  |  |  |  |
| 18 | Engine coolant (50% antifreeze) |  |  |  |  |  | Remove Clean & Refit |  | 65 | Check for fuel, oil and coolant leaks |  |  |  |  |
| 19 | Battery electrolyte |  |  |  |  | 43 | G/box, axles & transfer box breathers |  | 66 | Check for overheating components |  |  |  |  |
| 20 | Tyre pressures |  |  |  |  | 44 | Drain moisture from air tanks |  | 67 |  |  |  |  |  |
| 21 | Clutch fluid |  |  |  |  | 45 | Conveyor:- check rollers for bearing wear, scraper |  | 68 |  |  |  |  |  |
| 22 | Conveyor drive box oil |  |  |  |  |  | condition, drive chain for wear, wash assembly |  | 69 |  |  |  |  |  |
| 23 | Transfer box oil (4X4) |  |  |  |  |  | & hopper base. (Romaquip every 2 years) |  | 70 |  |  |  |  |  |
| 24 |  |  |  |  |  | 46 | Cabin pollen filter (renew when required) |  | 71 |  |  |  |  |  |
| 25 |  |  |  |  |  | 47 |  |  |  |  |  |  |  |  |

WM11/MAN-Volvo-C   Rev. 2010-06-02

| Before starting read WM/RQ MAN-Volvo/AM-Rev/06-10 |  |  |  |  |  | Service Instructions for H.A. Salt Spreader |  |  | Reg. No. |  | H.A. ID No. |  | Vehicle Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I certify that all items listed have been inspected to the required |  |  |  |  |  | Romaquip Body |  |  |  |  | WS |  |  |  |
| standard. All defects have been recorded, repairs have been carried |  |  |  |  |  |  |  |  | Speedo Reading |  |  | Date Carried Out |  |  |
| out or further action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  | Full Inspection |  |  |
|  | Check Condition/Operation & Report |  |  |  |  | 27 | Conveyor belt & bearings |  | 54 | Rear suspension and mountings |  |  |  |  |
| 1 | Preliminary Checks |  |  |  |  | 28 | Conveyor drive motor & gear box |  | 55 | Axle 3 differential assembly (6X4) |  |  |  |  |
| 2 | Offside cab interior |  |  |  |  | 29 | Nearside chassis, body & catwalk (rear) |  | 56 | Axle 3 brakes (6X4) |  |  |  |  |
| 3 | Vehicle system checks |  |  |  |  | 30 | Nearside rear wing(s) & spray suppression |  | 57 | Rear cross members |  |  |  |  |
| 4 | Instrument panel |  |  |  |  | 31 | Nearside brine tanks & mountings |  | 58 | Wheels and tyres |  |  |  |  |
| 5 | All switches |  |  |  |  | 32 | Nearside of cab & cat walk |  | 59 | ‘X’ lock & diff. locks |  |  |  |  |
| 6 | All lights |  |  |  |  | 33 | Inside nearside of cab door |  | 60 | Paint & preservation condition |  |  |  |  |
| 7 | Cab heaters and ventilation systems |  |  |  |  | 34 | Nearside of cab |  | 61 | Operation of body systems |  |  |  |  |
| 8 | Body controls |  |  |  |  | 35 | Passenger seat & seat belt |  | 62 | Road test ( record in onboard documentation) |  |  |  |  |
| 9 | Driving controls |  |  |  |  | 36 | Din plate & plough connections |  | 63 | Check vehicle on return |  |  |  |  |
| 10 | Steering wheel & column |  |  |  |  | 37 | Bumper bar |  | 64 | Documentation |  |  |  |  |
| 11 | Driving seat, adjuster & seat belt |  |  |  |  | 38 | Cab front & centre panel |  | 65 | General cleanliness |  |  |  |  |
| 12 | View from Drivers seat (glass, mirrors, etc) |  |  |  |  | 39 | Under front panel |  | 66 |  |  |  |  |  |
| 13 | On board equipment |  |  |  |  | 40 | Radiator & fittings |  | 67 |  |  |  |  |  |
| 14 | Air pressure system |  |  |  |  | 41 | Cab tilt mechanism |  | 68 |  |  |  |  |  |
| 15 | Offside cab door |  |  |  |  | 42 | Engine mounted equipment & controls |  | 69 |  |  |  |  |  |
| 16 | Offside front of cab |  |  |  |  | 43 | Under nearside of cab |  | 70 |  |  |  |  |  |
| 17 | Rear of cab |  |  |  |  | 44 | Under offside of cab |  | 71 |  |  |  |  |  |
| 18 | Offside of chassis & body |  |  |  |  | 45 | Under front of cab |  | 72 |  |  |  |  |  |
| 19 | Fuel tank & mounting |  |  |  |  | 46 | Front suspension & mountings |  | 73 |  |  |  |  |  |
| 20 | Offside brine tanks & mountings |  |  |  |  | 47 | Front axle, steering, wheels & tyres |  | 74 |  |  |  |  |  |
| 21 |  |  |  |  |  | 48 | Front brakes |  | 75 |  |  |  |  |  |
| 22 | Offside rear wing(s) and spray suppression |  |  |  |  | 49 | Gearbox & transmission system |  | 76 |  |  |  |  |  |
| 23 | Rear lamps & brackets |  |  |  |  | 50 | Chassis rear of gearbox |  | 77 |  |  |  |  |  |
| 24 | Rear of hopper & discharge door |  |  |  |  | 51 | Axle 2 brakes |  | 78 |  |  |  |  |  |
| 25 | Salt & brine discharge units |  |  |  |  | 52 | Axle 2 differential assembly |  | 78 |  |  |  |  |  |
| 26 | Spinner & mounting plate |  |  |  |  | 53 | Inter axle propeller shaft (6X4) |  | 80 |  |  |  |  |  |

WM11/MAN-Volvo-Full   Rev. 2010-06-02

| I certify that all items listed have been inspected |  |  |  |  |  | Service Instructions for H.A. Snow Plough |  |  | Identification No. |  | Plough Type |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  | Romaquip Snow Plough |  |  |  |  | Romaquip |  |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  | Speedo Reading |  | Date Carried Out |  |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  | N/A |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  | Safety Service |  |  |
|  | Main Frame |  |  |  |  | 24 |  |  | 47 | Check condition of main electric coupling |  |  |  |
| 1 | Check all frame welds for cracks |  |  |  |  | 25 |  |  | 48 |  |  |  |  |
| 2 | Check security/condition of moulded boards |  |  |  |  | 26 |  |  | 49 |  |  |  |  |
| 3 | Check security/condition of lights & reflectors |  |  |  |  |  | Mounting Plate |  | 50 |  |  |  |  |
| 4 | Check condition of deflector pivots & pins |  |  |  |  | 27 | Check security/condition of plate |  |  | With Plough Mounted |  |  |  |
| 5 |  |  |  |  |  | 28 | Check security/condition of mounting bolts |  | 51 | Check operation of raise, lower & float |  |  |  |
| 6 |  |  |  |  |  | 29 | Check security/condition of thrust arms & pivots |  | 52 | Check operation of angling left & right |  |  |  |
|  | Angling Frame |  |  |  |  | 30 |  |  | 53 | Check operation of deflector plate |  |  |  |
| 7 | Check thrust frame for damage/distortion |  |  |  |  | 31 |  |  | 54 | Check all hoses, rams & connections for leaks |  |  |  |
| 8 | Check angling rams for damage/distortion |  |  |  |  | 32 |  |  | 55 | Check operation of all lights |  |  |  |
| 9 | Check condition of centre pivot & housing |  |  |  |  |  | Edging Rubbers |  | 56 | Check plough height adjusters, set plough height to |  |  |  |
| 10 | Check condition of pins & safety chains |  |  |  |  | 33 | Check security/condition of blade rubbers |  |  | recommended setting |  |  |  |
| 11 |  |  |  |  |  | 34 | Check security/condition of trip plates & springs |  | 57 | Check condition/operation of carrying stay |  |  |  |
| 12 |  |  |  |  |  | 35 |  |  | 58 |  |  |  |  |
| 13 |  |  |  |  |  | 36 |  |  | 59 |  |  |  |  |
|  | Castor Assemblies |  |  |  |  | 37 |  |  | 60 |  |  |  |  |
| 14 | Check condition of castor assemblies |  |  |  |  |  | Hydraulics |  |  | Lubricate/Protect |  |  |  |
| 15 | Check security of adjuster lock |  |  |  |  | 38 | Check security/condition of all rams |  | 61 | All pivots, pin, hinges & ‘R’ clips |  |  |  |
| 16 | Check wheel bearings for excessive wear/noise |  |  |  |  | 39 | Check security/condition of ram pins & pivots |  | 62 | Threads of mounting bolts & plough height adjusters |  |  |  |
| 17 | Check security of hub covers |  |  |  |  | 40 | Check security/condition of all hydraulic hoses |  | 63 | Quick release couplings |  |  |  |
| 18 | Check security/condition of mudguards |  |  |  |  | 41 | Check condition of quick release couplings & covers |  | 64 | Main electrical connector |  |  |  |
| 19 | Check security/condition of damper assemblies |  |  |  |  | 42 |  |  | 65 | Exposed ram pistons |  |  |  |
| 20 |  |  |  |  |  | 43 |  |  | 66 |  |  |  |  |
| 21 |  |  |  |  |  | 44 |  |  | 67 |  |  |  |  |
|  | Tyres & Wheels |  |  |  |  |  | Electrics |  | 68 |  |  |  |  |
| 22 | Check wheels for security/damage |  |  |  |  | 45 | Check security/condition of all cables & connections |  | 69 |  |  |  |  |
| 23 | Check tyres for pressure and wear |  |  |  |  | 46 | Check condition of all protective trunking |  | 70 | Check condition of plough stand |  |  |  |

WM11/Rom-SP   Rev. 2010-06-29

| I certify that all items listed have been inspected |  |  |  |  |  | Service Instructions for H.A. Salt Spreader |  |  | Reg. No. |  | H.A. ID No |  | Vehicle Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  | Applicable to Schmidt Body Type - A1, A1 Combi. B1, C, D |  |  |  |  | WS |  | MAN VOLVO |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  | Speedo Reading |  |  | Date Carried Out |  |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  | A |  |  |
|  | Check & Top Up |  |  |  |  | 25 | Brake actuators & air valves |  |  | Check Operation |  |  |  |  |
| 1 | Engine oil |  |  |  |  | 26 | Air, fuel & hydraulic pipes. |  | 51 | All switches, instruments & warning devices |  |  |  |  |
| 2 | Engine coolant (50% antifreeze) |  |  |  |  | 27 | Suspension & dampers |  | 52 | All driving controls & lights |  |  |  |  |
| 3 | Power steering oil |  |  |  |  | 28 | Drive shafts, UJ’s, flanges & safety covers. |  | 53 | All wipers & washers |  |  |  |  |
| 4 | Hydraulic oil |  |  |  |  | 29 | Hydraulic valves (manual override at 0) |  | 54 | Cab heating systems |  |  |  |  |
| 5 | Gearbox Oil |  |  |  |  | 30 | All exposed electrical cables & connections |  | 55 | Air pressure build up, check for leaks |  |  |  |  |
| 6 | Transfer Box Oil (4x4) |  |  |  |  | 31 | Body & salt loading screens |  | 56 | Power steering & serviceability |  |  |  |  |
| 7 | Screen wash reservoir |  |  |  |  | 32 | Access ladders |  | 57 | Service & parking brake operation |  |  |  |  |
| 8 | Batteries |  |  |  |  | 33 | Plough mounting frame & connections |  | 58 | Body operation including Pre-Wet function |  |  |  |  |
| 9 | Axle Oils and wheel bearings* (*if oil filled) |  |  |  |  | 34 | Cameras & cables |  | 59 | Linear actuator |  |  |  |  |
|  | Check and Adjust If Required |  |  |  |  | 35 | Brine tanks & equipment |  | 60 | Chemical spray bar operation |  |  |  |  |
| 10 | Engine drive belts |  |  |  |  | 36 | Brine valve Inlet Gauze |  | 61 | Camera operation & visibility |  |  |  |  |
| 11 | Brakes |  |  |  |  | 37 | Chemical Tanks/s and equipment |  | 62 | Reflective materials, markers, signs & labels |  |  |  |  |
| 12 | Conveyor belt & scrapers |  |  |  |  | 38 | Fuel Tank |  | 63 | Carry out 20k road test (operate body on test) |  |  |  |  |
| 13 | Tyre pressures |  |  |  |  | 39 | AdBlue Tank |  | 64 |  |  |  |  |  |
| 14 |  |  |  |  |  | 40 |  |  |  | After Road Test |  |  |  |  |
|  | Check Security & Condition |  |  |  |  |  | Clean, Protect & Lubricate |  | 65 | Check for oil, fuel & coolant leaks |  |  |  |  |
| 15 | Seats & seat belts |  |  |  |  | 41 | All grease points |  | 66 | Check for overheating components |  |  |  |  |
| 16 | Glass & mirrors (view front & rear) |  |  |  |  | 42 | All cables, joints, pivots & levers |  | 67 | Check for visible ECU fault codes |  |  |  |  |
| 17 | Driving controls & fire extinguisher |  |  |  |  | 43 | Windscreen wiper linkage |  | 68 |  |  |  |  |  |
| 18 | Road wheels, tyres & indicator flags |  |  |  |  | 44 | All hinges & locks |  |  | Check & Report Condition Of |  |  |  |  |
| 19 | Cab & body mountings |  |  |  |  | 45 | Brake clevis pins |  | 69 | Cab & all paintwork |  |  |  |  |
| 20 | Rear chute, springs & spinner ass. & catches |  |  |  |  | 46 | All exposed ram & actuator pistons |  | 70 | Bodywork (damage, cracks etc.) |  |  |  |  |
| 21 | Salt sensor & cable |  |  |  |  | 47 | Hydraulic, electrical & brine connections |  | 71 | Rear & side reflectors & materials |  |  |  |  |
| 22 | All chassis mounted equipment |  |  |  |  | 48 |  |  | 72 | Vehicle cleanliness |  |  |  |  |
| 23 | Exhaust system & retarder |  |  |  |  | 49 |  |  | 73 |  |  |  |  |  |
| 24 | Steering linkage |  |  |  |  | 50 |  |  | 74 |  |  |  |  |  |

WM11/MAN-VOLVO-A   Rev. 2010-06-02

| I certify that all items listed have been inspected |  |  |  |  |  | Service Instructions for H.A. Salt Spreader |  |  | Reg. No. |  | H.A. ID No. |  | Vehicle Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  | Applicable to Schmidt Body Type - A1, A1 Combi, B1, C, D |  |  |  |  | WS |  | MAN VOLVO |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  | Speedo Reading |  |  | Date Carried Out |  |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  | C |  |  |
|  | Renew |  |  |  |  |  | Lubricate |  | 50 |  |  |  |  |  |
| 1 | Engine oil & filter |  |  |  |  | 27 | All grease points |  | 51 |  |  |  |  |  |
| 2 | Engine Fuel filter/s and Water Separator filter |  |  |  |  | 28 | Cab door hinges & locks |  | 52 |  |  |  |  |  |
| 3 | Volvo G/box Oil and Filter ( note * below) |  |  |  |  | 29 | All cables, pivots & joints |  | 53 |  |  |  |  |  |
| 4 | Air Dryer Cartridge |  |  |  |  | 30 | Exposed ram & actuator pistons |  |  | Clean Protect and or Paint |  |  |  |  |
| 5 | Hydraulic oil return filter (every 2 years) |  |  |  |  | 31 | Seat runners & adjusters |  | 54 | All exposed electrical connections |  |  |  |  |
| 6 | Primary Air Filter (every 2 years) |  |  |  |  | 32 | All brake actuator clevis pins (remove pins) |  | 55 | All brake valves and actuators |  |  |  |  |
| 7 | Conveyor Gearbox Oil (every 2 years) |  |  |  |  | 33 | Conveyor adjusting bolts |  | 56 | Body Linear actuator and drive motors |  |  |  |  |
| 8 | AdBlue Filter (every 2 years) |  |  |  |  | 34 |  |  | 57 | Any exposed aluminium or mild steel |  |  |  |  |
| 9 | Gearbox Oil (MAN) (every 3 years) |  |  |  |  | 35 |  |  | 58 | Chemical spray bar, frame and nozzles |  |  |  |  |
| 10 | Transfer G/Box Oil (4x4) (every 3 years) |  |  |  |  | 36 |  |  | 59 |  |  |  |  |  |
| 11 | Axle 1 2 & 3 Diff. oils (every 3 years) |  |  |  |  | 37 |  |  | 60 |  |  |  |  |  |
| 12 | Hydraulic oil (every 3 years) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | Engine Coolant (every 4 years) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Secondary Air Filter (Volvo) (every 4 years) |  |  |  |  |  |  |  | 61 |  |  |  |  |  |
|  | * Ist year only, thereafter every 2 years |  |  |  |  |  | Adjust if Required |  | 62 |  |  |  |  |  |
|  | Check and Top Up |  |  |  |  | 38 | Conveyor belt |  | 63 |  |  |  |  |  |
| 15 | Gear box oil |  |  |  |  | 39 | Discharge door height |  |  | Road Test |  |  |  |  |
| 16 | Transfer Gearbox (4x4s) |  |  |  |  | 40 | Engine drive belts |  | 64 | Carry out 20k road test (non motorway) |  |  |  |  |
| 17 | Front/Rear axle oils |  |  |  |  | 41 |  |  |  | After Road Test |  |  |  |  |
| 18 | Screen wash reservoir, add wash fluid |  |  |  |  |  | Remove Clean & Refit |  | 65 | Check for fuel, oil and coolant leaks |  |  |  |  |
| 19 | Hydraulic oil reservoir |  |  |  |  | 42 | Cabin (pollen) filter (renew when required) |  | 66 | Check for overheating components |  |  |  |  |
| 20 | Engine coolant (50% antifreeze) |  |  |  |  | 43 | Conveyor g/box breather (renew when required) |  | 67 |  |  |  |  |  |
| 21 | Battery electrolyte |  |  |  |  | 44 | Gearbox Breather (every 2 years) |  | 68 |  |  |  |  |  |
| 22 | Tyre pressures |  |  |  |  | 45 | Rear Axle Breathers (every 2 years) |  |  | Calibration |  |  |  |  |
| 23 | Conveyor drive box oil |  |  |  |  | 46 | Drain moisture from air tanks |  | 69 | Carry out body onboard weighing calibration |  |  |  |  |
| 24 |  |  |  |  |  | 47 | Anti dribble valve |  |  | (as manufacturers requirements) |  |  |  |  |
| 25 |  |  |  |  |  | 48 | Brine/Chemical tank breathers |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  | 49 | Brine/Chemical inlet gauze filter |  |  |  |  |  |  |  |

WM11/MAN-VOLVO-C   Rev. 2010-06-02

| Before starting read WM/MAN/VOLVO/AM |  |  |  |  |  | Service Instructions for H.A. Salt Spreader |  |  | Reg. No. |  | H.A. ID No. |  | Vehicle Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I certify that all items listed have been inspected to the required |  |  |  |  |  | Applicable to Schmidt Body Type - A1,A1 Combi, B1, C, D. |  |  |  |  | WS |  | MAN VOLVO |  |
| standard. All defects have been recorded, repairs have been carried |  |  |  |  |  |  |  |  | Speedo Reading |  |  | Date Carried Out |  |  |
| out or further action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  | Full Inspection |  |  |
|  | Check Condition/Operation & Report |  |  |  |  | 27 | Conveyor belt & bearings |  | 54 | Axle 2 differential assembly |  |  |  |  |
| 1 | Preliminary Checks |  |  |  |  | 28 | Conveyor drive motor & gear box |  | 55 | Inter axle propeller shaft |  |  |  |  |
| 2 | Offside cab interior |  |  |  |  | 29 | Chemical spray bar and equipment |  | 56 | Rear suspension and mountings |  |  |  |  |
| 3 | Vehicle system checks |  |  |  |  | 30 | Nearside chassis, body |  | 57 | Axle 3 differential assembly |  |  |  |  |
| 4 | Instrument panel |  |  |  |  | 31 | Nearside rear wing(s) & spray suppression |  | 58 | Axle 3 brakes |  |  |  |  |
| 5 | All switches |  |  |  |  | 32 | Nearside brine tanks & mountings |  | 59 | Rear cross members |  |  |  |  |
| 6 | All lights |  |  |  |  | 33 | Nearside Chemical tank/s and mountings |  | 60 | Wheels and tyres |  |  |  |  |
| 7 | Cab heaters and ventilation systems |  |  |  |  | 34 | Nearside and rear of cab |  | 61 | Cross lock & diff. locks |  |  |  |  |
| 8 | Body and Plough controls |  |  |  |  | 35 | Inside nearside of cab door |  | 62 | Paint & preservation condition |  |  |  |  |
| 9 | Driving controls |  |  |  |  | 36 | Inside nearside of cab |  | 63 | Operation of body systems |  |  |  |  |
| 10 | Steering wheel & column |  |  |  |  | 37 | Passenger seat & seat belt |  | 64 | Road test ( record in onboard documentation) |  |  |  |  |
| 11 | Driving seat, adjuster & seat belt |  |  |  |  | 38 | Din plate & plough connections |  | 65 | Check vehicle on return from road test |  |  |  |  |
| 12 | View from Drivers seat (glass, mirrors, etc) |  |  |  |  | 39 | Bumper bar |  | 66 | Documentation |  |  |  |  |
| 13 | On board equipment |  |  |  |  | 40 | Cab front & centre panel |  | 67 | General cleanliness |  |  |  |  |
| 14 | Air pressure system |  |  |  |  | 41 | Under front panel |  |  |  |  |  |  |  |
| 15 | Offside cab door |  |  |  |  | 42 | Radiator & fittings |  |  |  |  |  |  |  |
| 16 | Offside of cab |  |  |  |  | 43 | Cab tilt mechanism |  |  |  |  |  |  |  |
| 17 | Rear of cab |  |  |  |  | 44 | Engine mounted equipment & controls |  |  |  |  |  |  |  |
| 18 | Offside of chassis & body |  |  |  |  | 45 | Under nearside of cab |  |  |  |  |  |  |  |
| 19 | Fuel and AdBlue tank & mountings |  |  |  |  | 46 | Under offside of cab |  |  |  |  |  |  |  |
| 20 | Offside brine tanks & mountings |  |  |  |  | 47 | Under front of cab |  |  |  |  |  |  |  |
| 21 | Offside Chemical tank/s and mountings |  |  |  |  | 48 | Front suspension & mountings |  |  |  |  |  |  |  |
| 22 | Offside rear wing(s) and spray suppression |  |  |  |  | 49 | Front axle, steering, wheels & tyres |  |  |  |  |  |  |  |
| 23 | Rear lamps & brackets |  |  |  |  | 50 | Axle 1 brakes |  |  |  |  |  |  |  |
| 24 | Rear of hopper & discharge door |  |  |  |  | 51 | Gearbox & transmission system |  |  |  |  |  |  |  |
| 25 | Salt & brine discharge units |  |  |  |  | 52 | Chassis rear of gearbox |  |  |  |  |  |  |  |
| 26 | Spinner & mounting plate |  |  |  |  | 53 | Axle 2 brakes |  |  |  |  |  |  |  |

WM11/MAN-VOLVO-Full   Rev. 2010-06-02

| I certify that all items listed have been inspected |  |  |  |  |  | Service Instructions for H.A. Snow Plough |  |  | Registration No. |  | Plough Type |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  |  |  |  |  |  | SCHMIDT SNK 30/34 |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  | Speedo Reading |  | Date Carried Out |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  | N/A |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  | Contractor |  | Service Type |  |
| Sign |  |  | Print |  |  |  |  |  |  |  | Safety Service |  |
| Item | Main Frame |  |  |  | Tick |  | Mounting Plate |  |  | Paintwork |  |  |
| 1 | Check all frame welds for cracking |  |  |  |  | 20 | Check for damage/distortion of plate incl. mounting lugs |  | 39 | Check and report on condition of paintwork and galvanizing of the carrying frame |  |  |
| 2 | Check security and condition of lights, marker poles and reflective strips |  |  |  |  | 21 | Check condition, fit, serviceability& nylon rollers of stowage frames |  | 40 |  |  |  |
| 3 | Check security and condition of the mould board |  |  |  |  | 22 |  |  | 41 |  |  |  |
| 4 | Check the security and condition of the serial plate |  |  |  |  |  | Edging Rubbers |  | 42 |  |  |  |
| 5 |  |  |  |  |  | 23 | Check security/condition of blade rubbers |  |  | With Plough Mounted |  |  |
| 6 |  |  |  |  |  | 24 | Check security/condition of kerb guards |  | 43 | Check operation of raise, lower & float |  |  |
|  | Angling Frame |  |  |  |  | 25 | Check condition/security of trip plates, springs and pivot rod |  | 44 | Check operation of angling left & right |  |  |
| 7 | Check thrust frames for signs of damage/distortion |  |  |  |  | 26 |  |  | 45 | Check all hoses, rams and connections for leaks |  |  |
| 8 |  |  |  |  |  | 27 |  |  | 46 | Check operation of all lights |  |  |
| 9 |  |  |  |  |  |  | Hydraulics |  | 47 | Set plough height to recommended setting |  |  |
|  | Pins, Safety Cables and “R” Clips |  |  |  |  | 28 | Check security/condition of all rams |  | 48 | Check condition/fit of carrying cable |  |  |
| 10 | Check the security and condition of all securing pins, safety cables and transport cable. |  |  |  |  | 29 | Check security/condition of all hoses |  | 49 |  |  |  |
| 11 | Check angling ram for damage/distortion |  |  |  |  | 30 | Check condition/operation of all self sealing couplings & covers |  | 50 |  |  |  |
| 12 |  |  |  |  |  | 31 |  |  | 51 |  |  |  |
| 13 |  |  |  |  |  | 32 |  |  |  | Lubricate & Protect |  |  |
|  | Castor and Wheel Assemblies |  |  |  |  |  | Electrics |  | 32 | All pivots, pins, ball joints and “R” clips |  |  |
| 14 | Check the security of the castor assemblies to the plough frame |  |  |  |  | 33 | Check security/condition of all cables & connections |  | 53 | Threads of mounting bolts and height adjusters |  |  |
| 15 | Inspect the condition and operation of the turret adjuster and locking strap |  |  |  |  | 34 | Check condition of all protective trunking |  | 54 | Quick release couplings |  |  |
| 16 | Check wheel bearings for condition, wear and noise. |  |  |  |  | 35 | Check condition of electrical coupling |  | 55 | Electrical plug and socket |  |  |
| 17 | Check tyre condition for cuts, damage and de-lamination |  |  |  |  | 36 |  |  | 56 | Exposed ram pistons |  |  |
| 18 | Check wheel for security |  |  |  |  | 37 |  |  | 57 | Castor wheels & king pins |  |  |
| 19 |  |  |  |  |  | 38 |  |  | 58 |  |  |  |

WM11/PL(S)   Rev. 2010-06-03

| I certify that all items listed have been inspected |  |  |  |  |  |  | Combined Service instructions for HA Rolba |  |  |  | Reg. No |  |  | Type |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| to the required standard. All defects have been |  |  |  |  |  |  | Applicable to Rolba B & C Services |  |  |  |  |  |  |  |  |  |  |  |  |
| recorded, repairs have been carried out or further |  |  |  |  |  |  |  |  |  |  | Speedo |  | Hours | Date |  |  |  |  |  |
| action arranged. Vehicle on board documentation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| has been completed. |  |  |  |  |  |  |  |  |  |  | Contractor |  |  | Service Type |  |  |  |  |  |
| Sign |  |  | Print |  |  |  |  |  |  |  |  |  |  | B |  | C |  |  |  |
|  | Renew |  |  |  | B | C |  | Lubricate | B | C |  |  |  |  |  |  | B |  | C |
| 1 | Engine oil & filter |  |  |  |  |  | 25 | All grease point, see chart in cab |  |  | 49 | All drive belts |  |  |  |  |  |  |  |
| 2 | Fuel Filter |  |  |  |  |  | 26 | All exposed ram pistons |  |  | 50 | Clutch release bearing free play (R400) |  |  |  |  |  |  |  |
| 3 | Engine coolant & antifreeze (R1000) |  |  |  |  |  | 27 | All cable pivots, joints & levers |  |  | 51 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  | 28 | Door and access cover hinges |  |  |  | Check operation |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  | 29 | Seat runners & adjusters |  |  | 52 | Foot & parking brakes |  |  |  |  |  |  |  |
|  | Remove Clean & Refit |  |  |  |  |  | 30 | Ejector chute & turntable |  |  | 47 | All driving controls |  |  |  |  |  |  |  |
| 6 | Air cleaner & dust trap (renew if req.) |  |  |  |  |  | 31 | Skid adjusters & all shear pins |  |  | 48 | Diff & cross locks |  |  |  |  |  |  |  |
| 7 | Steering/drive suction filter |  |  |  |  |  | 32 | Front & rear marker light slides |  |  | 49 | All hydraulic controls |  |  |  |  |  |  |  |
| 8 | Fuel tank filler breather & strainer |  |  |  |  |  | 33 |  |  |  | 50 | All lighting equipment & switches |  |  |  |  |  |  |  |
| 9 | Fuel pump strainer |  |  |  |  |  | 34 |  |  |  | 51 | All heating & demist systems |  |  |  |  |  |  |  |
| 10 | G/box & axle breather |  |  |  |  |  |  | Check Security & Condition |  |  | 52 | Screen wash & wipers |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  | 35 | Exhaust system & turbocharger |  |  | 53 | Snow cutter & ejection system (observe |  |  |  |  |  |  |  |
|  | Drain Sample & Report |  |  |  |  |  | 36 | Road wheels & tyres |  |  |  | All H&S requirements) |  |  |  |  |  |  |  |
| 12 | Fuel tank (1 ltr.) |  |  |  |  |  | 37 | Steering mechanism & joints |  |  | 54 | Impeller brake (R1000) |  |  |  |  |  |  |  |
|  | Hydraulic tank (1 ltr.) |  |  |  |  |  | 38 | Radiator & hoses (R1000) |  |  | 55 |  |  |  |  |  |  |  |  |
| 13 | Check & Top Up |  |  |  |  |  | 39 | All hydraulic pipes & hoses |  |  |  | Preserve |  |  |  |  |  |  |  |
| 14 | Engine oil |  |  |  |  |  | 40 | Engine cooling system, fan, fins (R400) |  |  | 56 | All lamp interiors & electrical junctions |  |  |  |  |  |  |  |
| 15 | Coolant (R1000) |  |  |  |  |  | 41 | Service brake hydraulic system |  |  | 57 | Battery carrier with battery removed |  |  |  |  |  |  |  |
| 16 | G/box oil |  |  |  |  |  | 42 | All electrical wiring & terminals |  |  |  | Road test |  |  |  |  |  |  |  |
| 17 | Axle oil including hubs |  |  |  |  |  | 43 | All engine drive belts |  |  | 58 | Carry out 20km road test |  |  |  |  |  |  |  |
| 18 | Battery electrolyte |  |  |  |  |  | 44 | Alternator & starter motor |  |  |  | After road test |  |  |  |  |  |  |  |
| 19 | Brake fluid reservoir |  |  |  |  |  | 45 | Brake cylinders & linings with back |  |  | 59 | Check for oil, fuel & coolant leaks |  |  |  |  |  |  |  |
| 20 | Hydraulic oil level |  |  |  |  |  |  | Plate removed |  |  | 60 | Check for overheating components |  |  |  |  |  |  |  |
| 21 | Screen wash |  |  |  |  |  | 46 |  |  |  |  | Check & Report Condition |  |  |  |  |  |  |  |
| 22 | Tyre pressures |  |  |  |  |  |  | Adjust If Required |  |  | 61 | Interior/exterior paintwork |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  | 47 | Foot brake |  |  | 62 | Bodywork damage |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  | 48 | Hand brake |  |  | 63 | Vehicle cleanliness |  |  |  |  |  |  |  |

WM11/Rolba-B/C   Rev. March 2011

| “S” Serviceable “R” Repair/Adjust “VOR” Vehicle OFF Road “O” Observe |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Item No. |  | Class | Item No |  | Class |
|  |  |  | Comments (cleanliness/paint & pres. etc) |  |  |