Schedule 1 Specification

Norden Toilet Block & Office  Specification

General Description: (Existing)

The toilet block currently consists of ladies, gents, baby changing room and disabled toilet facilities with a store room containing all cisterns and access to the foul drains. The office has one entrance with windows & highlight windows. The work consists of a refurbishment of the current facilities to the following specification. Alternative options may be considered but pricing of the tender must be based on the following specification. (See ITT 2.9)

- Preparation Toilet Block

  - Tenderer to install & maintain temporary toilet facilities for Ladies, Gents and Disabled during works.

  - Remove all existing cubicles & doors, sanitary ware, Wallgate wash units,

lights, fans and all other unnecessary material and dispose of from all toilets

including the disabled.

  - Strip out and isolate water supplies, remove all existing water cisterns.

  - All interior walls (excluding Store Room & Cobb Walls) to be dry lined using     12.5mm plasterboard sheets

- Flooring

2.1	Install 300 x 300mm non slip dark grey floor tiles to all interior areas (excluding the store room), overlaid over the existing floor.

3.  	Walls

3.1	Affix pearl marble white gloss shower walls to all interior walls and inside   cubicles (or equivalent with agreement), excluding store room.

4. 	Ceiling

4.1	To apply a wood preserver to the ceiling cladding and beams in all toilets.

5. 	Doors

5.1	Replace minimum of 5 x interior cubicles & doors doors, locks and interior D handles and coat hooks. Design to a maximum amount of Cubicles to be installed within the space available (type and finish to be agreed).

5.2 	Replace the 4 x exterior doors with suitable weather resistant exterior doors   to include all new locks and furniture. (type and finish to be agreed)

6. 	Plumbing

6.1 	All connections are to be made to existing water feeds and waste points

6.1	Strip out and isolate water supplies

6.2	Supply and install all 1st fix plumbing as needed including isolation valves, copper pipe and all necessary materials to connect to existing drainage.

7. 	2nd Fix plumbing

Supply and install

7.1	Design and install minimum of 5 x back to wall ceramic toilets including cisterns, eco flush and soft close seats. Ladies minimum of 3 cubicles. Gents minimum of 2 cubicles. Design to a maximum amount of Cubicles to be installed within the space available (Cisterns to be located in the rear store room).

7.2 	5 x Push to operated taps sets (2x Gents, 2x Ladies, 1x Baby changing room). The number of these may alter depending on total of cubicles that are included in the final design.

7.3        1 X DDA Disabled Toilet to include Pan, cistern and basin with sensor flush and operation (including electrical supply) Grip barriers X 3.

7.4	2 x ceramic square urinals (Gents) with auto flush.

8. 	Vanity units

Supply and Install

8.1	2 x solid surface vanity tops incorporating 2 solid surface basins in each 1  unit located in ladies, 1 in the gents.

8.2	1 x ceramic basin in the disabled toilet

8.3	1 x ceramic corner basin in the baby changing room with drop down changing table)

9. 	Electrical

9.1	Isolate all redundant electrical items including fan units and lights and dispose of.

Supply and install

9.1a      5 x new PIR extraction fans to all rooms replacing existing.

9.1b      3 x instantaneous water heaters or equivalent to feed all basins (Can be agreed later)

9.1c      4 x Frost protection heaters (can be agreed later)

Supply and install:

9.2 	10 x 15W LED Light fittings to be switched by PIR motion sensors (location to be agreed)

9.3	3 x bulkhead lamps outside gents, ladies, disabled toilets switched by PIR motion sensors.

9.4 	4 x low energy blow down hand dryers (1 per gents, ladies, disabled toilets and baby changing room).

10            Roof Window and highlights

10.1      Inspect repair or replace window highlights where necessary throughout length including into office.

11. 	Sundries

Supply and install:

11.1	3 x individual safety backed mirrors 60cm x 60cm (1 per Gents, Ladies, Disabled)

11.2	6 x stainless steel liquid soap dispenser (1 per basin)

11.3	6 x ‘Not drinking Water’ signs above each basin

11.4	Replace existing external Gentleman, Ladies and Disabled Toilets signs.

12. 	Paint & Decorate

12.1	Supply and apply wood preserver all interior woodwork in each block.

12.2	Inspect and replace where necessary all exterior fascia boards.

12.3	Undercoat and paint all exterior woodwork, fascias etc.

12.4	Clean (or replace where necessary) all guttering and downpipes.

13.      Office Refurbishment

13.1	Remove existing sink and unit isolate all supplies including redundant items, shelving and carpet.

13.2      Make good walls and all surfaces and prepare for decorating.

Supply and install

14.1      1 x Sink unit with double base unit for storage to include all required fixtures including taps, drainage pipe to connect to sewer.

14.2      1 x Over sink 1.5kw water heater and associated supply and isolation.

14.3      Paint all walls with emulsion to a good standard and any main frames and supply wood preserver to wooden ceiling and beams.

14.4     Replace the 1 x exterior door with suitable weather resistant exterior doors to include all new locks and furniture.(type and finish to be agreed)

14.5     Replace flooring with a durable surface carpet/lyno/tile to be discussed.

14.6     Build a cupboard to cover all electrical distribution boards and electrical meters making easy accessibility.

14.7     2 x   Install new 15w Lighting and two additional 13amp sockets.

Commissioning:

15.1	Test and commission all electrical and plumbing work to ensure all is in working order.

15.2	Clean down and wipe all windows and other interior surfaces leaving in clean condition.