BRIEF TECHNICAL DESCRIPTIONS – TOYOTA AMBULANCES:-

23 x TOYOTA LAND CRUISER AMBULANCES - BASE MODEL: HZJ78L-RJMRS

External colour: All white with White Cross/Green Background stickers on side doors; Left hand drive, 4x4; No. of doors: 3 (with central locking); Diesel engine, (Petrol V8 model not permitted); Transmission 5 speed, Manual. Engine Model 1HZ - 4164cc, Diesel (Fuel tank capacity litres – 90 + 90L), 6 cylinders, BHP: 129, KW: 96, Cooling (Water), electrics 12 Volts with air conditioning included; Brakes: Front: Disc, Rear: Drum; All wheel rims are steel, inclusive of two spare wheels - Mud tyres (NO ALLOYS). Vehicles to be fitted with bullbars, winches and long jacks. Ambulance conversion requirements for these vehicles:- - Air Bags, Siren and blue LED Beacon Light Bar fitted to roof; - Codan HF Mobile Radio, NGT SRx with 9350 Antenna, 400 Channels; - Oxygen Concentrator fitted to the Oxygen piping system and powered by ambulance; - Comprehensive emergency first aid kit; - Stretcher with backrest, complete with restraints, 4 wheels and mattress; - Contour Mattress; - Ceiling mounted IV track with 2 bottle hooks (drip hooks); - Swivel attendant seat with three-point seat belt; - Handheld pulse oximeter (including case); - Ambu MKIII manual adult resuscitator including facemask (adult, paediatric and neonate); - Medical suction unit (1 electric and 1 manual); - Hand wash electric unit 4 litre capacity – provides hot water; - LED Interior Lighting and 12v 2md lighter socket mounted in rear; - Tinted Heat reflective film for windows; - Metal locker with 3 lockable drawer unit; - Wipe clean interior panelling with partition window; - Transit safety box; - Rear Ambulance compartment fan assisted vent with division only; - 2 x lap type seatbelts Warranty must be provided on the vehicles. Vehicles must be compliant with road regulations in South Sudan, suitable for the local terrain/environment, climate and the engine must be capable of operating with the quality of diesel fuel available within this country.