HM Land Registry Croydon Lift Replacement Project Additional Notes

Ref:

Contents

Version Control

| Version | Date | Author | Details |
| --- | --- | --- | --- |
| 0.1 |  | Luke Carman | First Draft |

Requirement

Design of new Lift system must be fully compliant and fit-for-purpose for 8-floor Head Office location (with approx. 400-450 staff and regular visitors) and include application of all relevant Industry Best Practice

Project scope will include the necessity to ensure continuous operation of two Lift units during the Works period (including consideration whether Works contractor assumes PPM responsibility for Works period), and mitigation of impact on Building users

Further Detail on Existing Lifts

The lift was installed in 2006, a mid-range Univers machine roomless lift manufactured and design by ThyssenKrupp (from France factory).

Since installation the lift has been maintained by three major lift companies, ThyssenKrupp, Kone and Otis. Maintenance was given back to the OEM ThyssenKrupp in 2013

Existing fabric is concrete walls

Lift Details (Passenger/Fire-fighting and Goods)

Location/Client ID - L/H Passenger

Safe Working Load - 10 Persons/800kg

Drive System - Top Drive MRL Traction

Speed - 1.6m/s

Control System - Full Collective

Serving - G, 1, 2, 3, 4, 5, 6, 7 & 8

Doors - 2 Piece Centre Opening

Serial Number - AM058001

Year of Manufacture - 2006

Manufacturer – ThyssenKrupp

Location/Client ID - R/H Passenger Fire-Fighting

Safe Working Load - 10 Persons/800kg

Drive System - Top Drive MRL Traction

Speed - 1.6m/s

Control System - Full Collective

Serving - G, 1, 2, 3, 4, 5, 6, 7 & 8

Doors - 2 Piece Centre Opening

Serial Number - AM058002

Year of Manufacture - 2006

Manufacturer – ThyssenKrupp

Location/Client ID - Goods

Safe Working Load - 13 Persons/1000kg

Drive System - Top Drive MRL Traction

Speed - 1.6m/s

Control System - Full Collective

Serving - G, 1, 2, 3, 4, 5, 6, 7 & 8

Doors - 2 Piece Centre Opening

Serial Number - AM058003

Year of Manufacture - 2006

Manufacturer - ThyssenKrupp

Programme

Desired Outcome:

Installation of complete new Lift system by 31 March 2019.

Site Access

Minimal Car Park space available.

Container and compound space available for temporary storage/skips