Appendix three

Crossing Point and Culvert Repair Points Specification and logistics.

See table 1 below for full breakdown of what is required at each point.

Item 1.

Renovate existing crossing point

Grid reference: TL97441307		Compartment: 1/8

Suggested action:

- Install a 9m length of 300mm twinwall pipe with upstand and an appropriate simple timber and sandbag headwall.

- 16 Rock roll units installed on sandbags on south side only, covering up to the level of the crossing point, keyed into a profiled bankside.

- Estimated 20m3 of material (to be sourced from Bale works) required to widen crossing point back to original width and build up sides to support rock-roll

- Estimated 20m3 of type 1 required to raise surface of track

See figure 1 for detailed plan in the Appendix Two.

Photo above showing south side of crossing point

Item 2:

Renovate existing crossing point

Grid reference:	TL97581305			Compartment: Field 1/8

Suggested actions:

- Build up both sides of crossing point with approximately 10m3 of material gained from Bale works

- Raise area with type 1 by approximately 30cm

- Estimated 40m3 of type 1 required

Photos above showing left- and right-hand sides of low point

Item 3:

Renovate existing crossing point

Grid reference:	TL97761298	Compartment: Field 1/8

Suggested actions:

- Install a 9m length of 300mm twinwall pipe with upstand and an appropriate simple timber and sandbag headwall.

- 16 Rock roll units installed on sandbags on south side only, covering up to the level of the crossing point, keyed into a profiled bankside.

- Estimated 20m3 of material (to be sourced from Bale works) required to widen crossing point back to original width and build up sides to support rock-roll

- Estimated 20m3 of type 1 required to raise surface of track

See figure 1 for detailed plan in the Appendix Two.

Photo above showing south side of crossing point

Item 4:

Renovate existing crossing point

Grid reference:	TL98011284			Compartment: Field 1/8

Suggested actions:

- Install a 9m length of 300mm twinwall pipe with upstand and an appropriate simple timber and sandbag headwall.

- Approximately 1m3 of type one to cover up any excavations

See figure 1 for detailed plan in the Appendix Two.

Item 6:

Renovate existing crossing point (first area from south to north)

Grid reference:	TL97521213			Compartment: Field 6

Suggested actions:

- Build up crossing point with material sourced from Bale

- Total area approximately 12x9m

- Estimated 80m3 of material required

- Dress the surface with turf gained from Bale works

Item 7:

Renovate existing crossing point (second area from south to north)

Grid reference:	TL97511221		Compartment: Field 6

Suggested actions:

- Build up crossing point with material sourced from Bale

- Total area length approximately 15x5m

- Estimated 40m3 of material required

- Dress the surface with turf gained from Bale works

Item 8:

Renovate existing crossing point (third area from south to north)

Grid reference:			Compartment: Field 6 and 7

Suggested actions:

- Build up crossing point with material sourced from Bale

- Estimated 120m3 of material required to widen crossing point back to original width and height. Build up sides to support rock-roll

- 24 Rock roll units installed on sandbags on both sides, covering up to the level of the crossing point, keyed into a profiled bankside.

- Estimated 20m3 of Type 1 required to dress surface

See figure 2 for detailed plan in the Appendix Two.

View from the south (left) and north (right)

Item 9:

Renovate existing crossing point (fourth area from south to north)

Grid reference:	TL97511228				Compartment: Field 7

Suggested actions:

- Build up crossing point with material sourced from Bale to surrounding ground height

- Total area approximately 4x2m

- Estimated 10m3 of material required

- Top area with turf sourced from Bale works

Item 10:

Renovate existing crossing point (fifth area from south to north)

Grid reference:	TL97521229				Compartment: Field 7

Suggested actions:

- Build up crossing point with material sourced from Bale to surrounding ground height

- Total area approximately 4x2m

- Estimated 10m3 of material required

- Top area with turf sourced from Bale works

Table 1: Estimated Specification per crossing point

| Crossing Point | Pipe & control structure | Rock-Roll (units) | Sandbags (Number) | Spoil required (m3) | Type 1 required (m3) | Length (metres) | Width (meters) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 16 |  | 20 | 20 | 25 | 4 |
| 2 | 0 | 0 |  | 10 | 40 | 44 | 4 |
| 3 | 1 | 16 |  | 20 | 20 | 20 | 4 |
| 4 | 1 | 0 |  | 0 | 1 | 20 | 5 |
| 6 | 0 | 0 |  | 80 | 0 | 12 | 9 |
| 7 | 0 | 0 |  | 40 | 0 | 15 | 5 |
| 8 | 1 | 24 |  | 120 | 20 | 33 | 7 |
| 9 | 0 | 0 |  | 10 | 0 | 4 | 2 |
| 10 | 0 | 0 |  | 10 | 0 | 4 | 2 |
| TOTAL | 3 | 64 |  | 310 | 101 |  |  |