Statement of Service Requirements

Fire Risk Assessments

Contract Ref.  RBGKEW710

Version:  1.0

Date:  8th August 2019

- INTRODUCTION

The Royal Botanic Gardens Kew (“Kew”) are seeking to appoint a service provider (“Supplier”) to undertake Fire Risk Assessments (“FRA”) for Kew Gardens, Richmond, Surrey TW9 3AE and Wakehurst, Ardingly, Haywards Heath, Sussex, RH17 6TN.

Both sites are open to the general public throughout the year between 10:00 – 19:00 (BST) and 10:00 – 16:00 (GMT).

Both sites comprise the following building types including:

- Plant rooms containing higher risk assets eg. boilers, electrical LV panels adjacent to HV transformers, mechanical moving plant, etc.;

- Conservatories and greenhouses of glass construction;

- Agricultural outhouses;

- Office space;

- Science laboratories;

- Educational facilities and lecture theatres;

- Commercial and retail space;

- Underground basement areas;

- Archive stores for long term storage of specimens;

- Public restaurants and kitchens;

- Recreational facilities;

- Fuel storage facilities and on-site generators;

- Confined spaces and restricted access areas;

- Galleries housing valuable artworks; and

- Residential occupied buildings.

A number of buildings contain enclosed lift shafts.

A high number of buildings are listed between Grade I and II and are of significant architectural value.

- BACKGROUND

Kew is a world-famous centre for botanical and mycological knowledge. Our mission is to be the global resource for plant and fungal knowledge, building an understanding of the world's plants and fungi upon which all our lives depend.  With our two inspiring gardens at Kew in west London and Wakehurst in Sussex, we enchant our visitors with the wonder of plant diversity.

The Board of Trustees of RBG Kew is a non-departmental public body and an exempt charity.  It is a statutory incorporation established under the National Heritage Act 1983.  Defra (the Department for Environment, Food and Rural Affairs) is RBG Kew’s sponsor and principal regulator (for charity law purposes).

Kew Gardens was founded as a Royal Garden in 1759, and is a UNESCO World Heritage Site, inscribed in 2003.

Kew Gardens is Crown land occupied and managed by RBG Kew under Ministerial Direction.  Wakehurst consists of multiple titles – the gardens, woodland and mansion house are leased under a long lease from National Trust whilst adjacent land comprising the Millennium Seed Bank and Havelock are owned by RBG Kew.

The Kew site consists of 121 hectares (300 acres) of gardens and botanical glasshouses, four Grade I listed buildings, and 36 Grade II listed structures, all set in an internationally significant landscape. It is listed Grade I on the Register of Historic Parks and Gardens.

The Wakehurst site, in West Sussex, consists of 620 acres of gardens and parkland with 1 Grade I and 2 Grade II listed buildings.

More information can be found at www.kew.org.

Kew Gardens and Wakehurst Place site plans and description of buildings at both sites can be found at Appendix 5.

- SCOPE OF REQUIREMENTS

3.1	Principles

Kew require a full set of Fire Risk Assessments (hard copy and soft copy), for each building at both Kew & Wakehurst, to be undertaken compliant to the Regulatory Reform (Fire Safety) Order 2005 (“RRO”) and to enable compliance with Kew’s Fire Risk Management Strategy and System (See Appendices 2 and 3).

Kew’s aspiration is to comply with PAS 7 (BS 9997).

The FRAs will be Quality Assured, and the FRA company must QA their document and show that this process has been undertaken, prior to release to Kew’s Responsible Person’s, being “Head of Estates” and/or “The Project Manager” and/or “The Compliance Manager”.

Kew will, with the assistance of the Supplier, prioritise the significant findings and actions from the FRAs in accordance with their business criticality and Fire Risk Management Strategy and System.

Please see Appendix 1 for Kew’s prevailing Fire Policy.

Kew will provide to previous FRAs undertaken on site.

These FRAs are only to be used as an historic indication and shall not be simply updated.

The project start date will be 1st October 2019

The project completion date will be 31st January 2020

Both the dates above are to be confirmed by the tenderer and any variance agreed, in writing, by RBG Kew’s representative.

RBG Kew have provided a building list, see appendix 5.  Note RBG Kew may “cherry pick” from this list.

RBG Kew may also add buildings that require a Fire Risk Assessment and the supplier is to provide a £ / per square meter calculation with the tender return should this be necessary.

3.2	Building Safety Manual

All buildings that have a medium or high fire rated risk shall have a Building Safety Manual, in accordance with the Kew fire risk management system PAS7.

The supplier shall provide information for the manual via the Fire Risk Assessments, but not contribute directly to the manual.

A template Building Safety Manual can be seen in Appendix 4.

3.3	FRAs

The FRAs will satisfy the requirements of the RRO and will provide adequate information to resolve deficiencies, including indicative costs.

Kew will, in liaison with the Supplier, prioritise the significant findings from the FRAs.

3.4	Buildings / Floorplans

A full list of buildings to have an FRA is attached at Appendix 5. The Supplier shall confirm or mark up, using conventional symbols, the floorplans provided by Kew with the following information:

- Location of fire compartmentation and zones/fire wall ratings.

- Location, type and quantity of fire extinguishers (existing and required)

- Location, type and quantity of fire exit signage (existing and required)

- Location, type and quantity of fire alarm devices (existing and required)

- Location and type of emergency lighting (Maintained or Non-Maintained).

- Location and type of fire rated doors.

This information will be transferred to electronic drawings by Kew.

3.5	Compartmentation

The Supplier shall review fire compartmentation drawings (where available) against actual compartmentation.

Where remedial works are required (e.g. missing fire damper in duct work, door not adequately rated, hole in wall / ceiling, etc.) the Supplier shall:

- photograph the issue;

- mark the location on the relevant floorplan;

- clearly define a scope of works to resolve; and

- add the works to the FRA remedial works list in Excel spreadsheet format.

The Supplier shall list each issue separately in order that issues can be clearly identified by Kew.

If an electronic reporting process is in place, this will supersede the requirement for a spreadsheet.

3.6	Minor Remedial Items

The Supplier shall undertake minor remedial items e.g. replace a Fire Action Notice sign / Fire Extinguisher sign etc. if easy to undertake.

A single item value of less than twenty pounds (£20.00), including parts, is permitted for works to be undertaken by supplier without approval by the Kew representative.  Any works with a value above twenty pounds (£20.00) should be approved by the Kew representative.

This is acceptable until the cumulative spend reaches one thousand ponds (£1,000.00) at which point the Kew representative will be approached to agree further expenditure.

Where other minor remedial works are required (e.g. fire door not closing, extinguisher bracket loose, etc.) the Supplier shall contact the appropriate Help Desk (Kew or Wakehurst) and report the defect, obtaining a call reference number and inserting in the FRA report.

For clarity the works as indicated can be defined as minor / major remedial works. For minor remedial works the value shall be works with a total value of less than five hundred pounds (£500.00). Major remedial works shall be those works with a total value above this amount (£501.00).  Value shall be inclusive of all labour & materials.

The appropriate telephone number / email address for the Helpdesks will be provided by Kew.

3.7	Significant Findings & Major Remedial Works

Where major remedial works are required (e.g. fire door damaged, extinguisher required, fire compartment breach, etc.) the Supplier will photograph the issue, mark location on floorplan, clearly define scope of works, add works to FRA remedial works list on electronic system or in Excel spreadsheet format.

The Supplier shall list each issue separately in order that issues can be clearly identified and independently closed out.

3.8	Recording

It is Kew’s intention to record FRAs electronically.

Kew will provide the Supplier with either a tablet or access to an app for conducting the FRA.

The system to be used has not been confirmed by Kew, however the Supplier should be familiar with electronic system recording.

If an electronic system is not available, the Supplier shall submit an FRA template to Kew to agree an acceptable format for the assessments, typically MS Word & Excel format. The template will be submitted with the tender return.

3.9	FRA Feedback & Progress

The Supplier shall provide feedback to the Kew representative either at end of day or start of next working day.

In addition, monthly progress report/meetings are required. Also progress against programme of assessments and any access issues, additional works, recommended works, etc.

4.	Skills & Experience

The Assessor (“Assessor”) proposed by the Supplier shall have appropriate and recognised training together with experience of public venues as well as working knowledge of historic buildings and horticultural sites.

A current CV covering knowledge, experience & qualifications shall be provided for each assessor at tender stage.

The Assessor shall have sat a competency review and shall provide evidence of the same to Kew’s representative, also the assessor shall be registered on a Fire Risk Assessor register (e.g. IFE, IFSM, BAFE, etc.)

In addition, the Assessor shall possess a current Enhanced DBS.

- Constraints

Works shall be undertaken during normal working hours (08:00 – 17:00).

By exception, works may be undertaken outside these hours by prior approval from Kew.

Access to particular building will be via prior consent and may require full escort and supervision by a Kew representative.

Appendix 1 - Fire Policy

Appendix 2 - Fire Risk Management Strategy

[Currently in development by Kew.  Document shall be inserted either prior to contract signature of post contract signature via contract change note.]

Appendix 3 - Fire Risk Management System

[Currently in development by Kew.  Document shall be inserted either prior to contract signature of post contract signature via contract change note.]

Appendix 4 - Building Safety Manual Template

Appendix 5 – Site Plans and Building Information

Kew Gardens Site Plan

Wakehurst Place Site Plan

Building list for Kew & Wakehurst