SPSCM/01044

Schedule 11 - Statement of Technical Requirement (SoTR)

Issued by

SPECIAL PROJECTS SEARCH AND COUNTER MEASURES DELIVERY TEAM

(SPSCM DT)

Conditions for Release

The information provided within this document is supplied without commitment or prejudice and is SUBJECT TO CONTRACT.

This information is released by the UK Government for Defence purposes only. It must be accorded the same degree of security protection as that accorded thereto by the UK Government

# Foreword

THIS DOCUMENT IS THE PROPERTY OF HER BRITANNIC MAJESTY'S GOVERNMENT, and is for information of such persons as only required to know its contents in the course of their official duties.  Any persons finding this document should immediately hand it in to a British Consulate, British Forces Unit or to a UK Police Station for its safe return to the Ministry of Defence, D MoD Sy, London, SW1A 2HB, with particulars of how and where it was found.  THE UNAUTHORISED RETENTION OR DESTRUCTION OF THE DOCUMENT IS AN OFFENCE UNDER THE UK OFFICIAL SECRETS ACTS OF 1911-1989.  (When released to persons outside Government Service, this document is issued on a personal basis and the recipient to whom it is entrusted in confidence, within the provisions of the UK Official Secrets Acts 1911-1989 or national legislation, is personally responsible for its safe custody and for seeing that its contents are disclosed only to authorised persons).

If additional copies of this document are required, they must be obtained from either the SPSCM Project Manager or SPSCM DT.  The Project Manager should keep registered holders informed of any amendments subsequently issued.

References in this document to any other requirement, specification, drawing or document refer to the latest issues of those documents.

The contents of this document in no way absolve the supplier or the user from statutory obligations relating to health and safety at any stage of development, manufacture or use.

This document has been devised for use within the Ministry of Defence and by its Contractors in the execution of contracts for the Ministry and subject to the Unfair Contract Terms Act 1977, the Ministry will not be liable in any way whatever (including, but without limitation, negligence on the part of the Ministry, its servants or agents) where the plan is used for other purposes.

# DOCUMENT CONFIGURATION CONTROL

This document is managed by SPSCM DT DELIVERY TEAM 4b2 (DES LE STSP-SPSCM-4b2).

This document shall be amended by issue of complete main section, annex or appendix.  Amendment status shall be recorded in the footer information of affected pages.

| Version No | Date | Affected Pages | Description of Change | Amendment Incorporated by |
| --- | --- | --- | --- | --- |
| (a) | (b) | (c) | (d) | (e) |
| 1.0 | 3/7/17 |  | Version 1.0 | AW |

# CONTENTS

Foreword	1

DOCUMENT CONFIGURATION CONTROL	2

CONTENTS	3

Introduction	4

Purpose	4

SPSCM/01044 System Scope	4

Acceptance Methods	5

Statement of Technical Requirement	6

Glossary	11

Appendix A – Vibration Test Limits	13

## Introduction

The document has been written by the Special Projects Seach and Countermeasures (SPSCM) Delivery Team (DT) in support of the Requirement for a Rapid-DNA System.

## Purpose

To define the scope of the System.

The performance of the System is broken down by a series of Requirements with an Agreed Contractual Measure of Performance and Acceptance Method against each. The Agreed Contractual Measure of Performance is taken from the Contractor’s Stated Compliance in Annex E to DEFFORM 47 - Technical Compliance Matrix (TCM).

The Acceptance method is used by the Authority to validate that the Agreed Contractual Measure of Performance has been met. Further detail about each of the acceptance methods stated in the SoTR can be found in paragraph 8. This validation will be used by the Authority as part of the Acceptance process before the System Acceptance.

The System Requirements in the SoTR are extracted from the System Requirements in the System Requirements Document (SRD), however not all System Requirements in the SRD are present in the SoTR. Where this is the case these SRs are not deemed as a Contractor deliverable.

## SPSCM/01044 System Scope

The system boundary and associated services to deliver the outputs required are outlined withing Figure 1, the system boundary provides the basis for the structure of the SRs and attempts to highlight the interfaces with external systems and within the system itself. A glossary of all terms used in this document can be found on page 11.

Figure 1: System Scope

An overview of the services within the system boundary is as follows and flows along the expected process path:

  - DNA Sample Collection. The collection of a DNA sample is to be conducted by in-service items of equipment (PIA and PEK) and is external to the R-DNA System Boundary.

  - Interface between DNA Sample Collection and System. Once the DNA sample has been collected, it has to be converted into a format that can be accepted by the system.

  - Sample Registration Service. Each DNA sample entered into the system must be registered with the unique ID provided with collection kits. This will ensure that the sample can be associated with the contextual information gathered on collection.

  - Profile Generation Service. The generation of a digitised DNA profile from the corresponding DNA sample.

  - Profile Comparison Service. The digitised DNA profile will be stored within a local database and analysed for one to one or one to many match comparisons, with other DNA profiles in the local database.

  - Interface to Additional Profiles. The system will be able to import data sets to the local database for local comparisons and also export digitised DNA Profiles and raw DNA data for transmission to a more advanced facility for more detailed analysis.

  - Additional Profile Comparision Service. Provides a more comprehensive database of DNA Profiles for comparison.

## Acceptance Methods

The following methods will be used to ensure the System meets the agreed measure of performance:

  - Authority Trials: Evidence produced by a set of repeatable tests or trials. These trials will consist of tests such as running blood samples on the System and comparing the output to a ground truth profile of the same sample.

  - Design Review: Evidence that a function or a feature exists within the System, as requested. This is expected to be contained in evidence provided by the supplier, and will be reviewed by the Authority. This may also involve using the System in order to ascertain whether certain features are present and functioning, but does not involve repeatable tests or trials.

  - Certification: Independent verification of system performance from a reputable body. This is expected to be confirmed through provision of appropriate certification from the supplier, which will be reviewed by the Authority.

  - In-Service Support Evaluation: An assessment of the in-service support solution elements, this is expected to be evaluated at the Authority Logistic Demonstration.

  - Supplier Analysis:  Evidence produced by the Supplier and to be reviewed by the Authority, that may take the form of documents, modelling output etc. that infers compliance based on knowledge and no physical testing or trialling. An example might be a Supplier inferring compliance with the ability of their system to operate in high temperatures, based on known temperature performance of the equipment constituting the system.

The primary methods to be used for accepting the System against each System Requirement are detailed in the Table 1. Evidence from the assessment of the Systems during the Tender phase may also be used for acceptance purposes.

## Statement of Technical Requirement

| SR Number | System Requirement Description | Remarks | Agreed Measure of Performance | Primary Acceptance Method |
| --- | --- | --- | --- | --- |
| 1 | The system shall accept [DNA Sample]s that have been collected by [In-Service DNA Collection Kit]s. | Current in service collection kits are PEK for enrolment and PIA for post incident analysis. They both contain swabs for the collection of DNA. However, the aspiration for the user is that they are able to use any swab available to them. |  | Authority Trials |
| 2 | The system shall [Register] the [DNA Sample] with a unique ID for reference. | Current UK in-service collection kits are provided with a barcode, hence other in-service equipment utilises a barcode scanner to assign unique IDs to the items collected. |  | Authority Trials |
| 3 | The system shall produce a [DNA Profile] in an operational timescale. |  |  | Authority Trials |
| 4 | The system shall process multiple [DNA Sample]s at the same time. | The aspiration is that DNA samples can have their processing started independent of any other DNA sample process (ie. mid-process), to enable the system to be in continuous use. |  | Authority Trials Design Review |
| 5 | The system shall generate [DNA Profile]s using a large multiplex. |  |  | Authority Trials Design Review |
| 6 | The system shall produce a full [DNA Profile] from a buccal [DNA Sample]. |  |  | Authority Trials |
| 7 | The system shall produce a full [DNA Profile] from a wet blood [DNA Sample]. |  |  | Authority Trials |
| 8 | The system shall produce a [DNA Profile] from a [Touch] [DNA Sample]. | [DNA Profile] generated from a sample containing 100pg of DNA. |  | Authority Trials Design Review |
| 9 | The system shall produce a [DNA Profile] from a [DNA Sample] collected via saliva on an item. | [DNA Profile] generated from a sample containing 100pg of DNA An item with saliva on it that the user would like to capture DNA from, would be cigarette butts, chewing gum etc. |  | Authority Trials Design Review |
| 10 | The system shall inform the user of the [DNA Profile] quality. | An indication of DNA profile quality will aid the user in understanding whether to re-run the sample, or rely on Level 3 support. |  | Authority Trials Design Review |
| 11 | The system shall inform the user of the presence of DNA from more than one individual within a [DNA Sample]. |  |  | Authority Trials Design Review |
| 12 | The system shall store a quantity of [DNA Profile]s in a [Local Database]. | [DNA Profile]s will be exports from the MOD DNA Database. |  | Authority Trials Design Review |
| 13 | The system shall be able to import [DNA Profile]s to the [Local Database]. | [DNA Profile]s will be in a non-proprietary format, such as an excel spreadsheet or xml file. |  | Authority Trials Design Review |
| 14 | The system shall be able to export [DNA Profile]s. |  |  | Authority Trials Design Review |
| 15 | The system shall enable the deletion of [DNA Profile]s from the [Local Database]. |  |  | Authority Trials Design Review |
| 16 | The system shall conduct one to one [Biometric Match]es. |  |  | Authority Trials Design Review |
| 17 | The system shall conduct one to many [Biometric Match]es. |  |  | Authority Trials Design Review |
| 18 | The system shall provide a [User Interface] to control the equipment functions. |  |  | Design Review |
| 19 | The system shall present information via a visual display. |  |  | Design Review |
| 20 | The system shall present information in a format for a [non-specialist user]. |  |  | Design Review |
| 21 | The system shall protect the users' identities. |  |  | Authority Trials Design Review |
| 22 | The system shall not record its location. |  |  | Authority Trials Design Review |
| 23 | The system shall provide a system administration function. |  |  | Design Review |
| 24 | The system shall minimise the preparation time required between unpackaging and being available to start a [DNA Profile] process. |  |  | Design Review |
| 25 | The system shall be configurable for each deployment. |  |  | Authority Trials Design Review |
| 26 | The system shall provide a means to test the confidence in the equipment's performance. | Expect this to be generated through the use of positive and negative control samples, where the result is known. |  | Design Review |
| 27 | The system shall fully function with date-expired consumables. | Date-expired consumables will only be used for training purposes, allowing the user to practice operating the equipment to produce DNA profiles. |  | Authority Trials Design Review |
| 28 | The system shall be operable by a [non-specialist user]. |  |  | Authority Trials Design Review |
| 29 | The system shall operate globally with mains electrical power. |  |  | Design Review |
| 30 | The system shall be protected from loss of mains electrical power. | Expect this to be provided through the use of uninterruptable power supplies or onboard batteries. |  | Design Review |
| 31 | The system shall operate from onboard batteries. |  |  | Design Review |
| 32 | The system weight shall allow it to be transported. | Including ancillaries and transport packaging. |  | Design Review |
| 33 | The system dimensions shall allow it to be transported. | Including ancillaries and transport packaging. |  | Design Review |
| 34 | The system shall be safe and suitable for service. |  |  | Design Review Certification |
| 35 | The system shall be safe to store and transport. |  |  | Design Review Certification |
| 36 | The system shall be stored and operated no higher than the OFFICIAL Security Classification. | System to only store digitised DNA profiles, no contextual or user information (including DNA profiles) to be imported |  | Design Review Certification |
| 37 | The system shall provide a function to securely wipe data from any onboard memory. | See also SR-31. |  | Design Review Certification |
| 38 | All system consumables for the collection, storage and analysing of [DNA Sample]s shall be free from contamination with human DNA. |  |  | Certification |
| 39 | The system shall have a high probability of full functionality when used in an operational environment. | Reliability figures are for a benign environment, operational availability may be lower in agreement with The Authority. |  | Design Review In-Service Sp. Eval |
| 40 | The system shall have a continuous built in test (BIT) that assesses available system functionality on startup. |  |  | Design Review |
| 41 | The system shall require no more than Level 1 maintenance on task. |  |  | Design Review In-Service Sp. Eval |
| 42 | The system shall be provided with Level 4 maintenance and repair support by the supplier. | Where possible the supplier should apply corrective maintenance at the user's location. If the system has to be returned to the supplier, the Repair Turnaround Time does not include transportation time to and from the Supplier's site. |  | In-Service Sp. Eval |
| 43 | The system shall be supplied with a full Logistics Support Package. |  |  | In-Service Sp. Eval |
| 44 | The system shall be supplied with a full Supply Support Package. | Spares to be available to conduct Level 1 Maintenance. Consumables to be available to operate the system on task. |  | In-Service Sp. Eval |
| 45 | The system shall be supported by the Supplier for the duration of its service life. |  |  | Design Review |
| 46 | The system shall be provided with all required Support and Test Equipment (S&TE). |  |  | In-Service Sp. Eval |
| 47 | The system shall be packaged to {Def Stan 81-41} and {Def Con 129}. | Due to the protection of the equipment case, this is expected to be export trade packaging or military packaging level N, subject to agreement with the authority. |  | In-Service Sp. Eval |
| 48 | The system spares and consumables shall be packaged to {Def Stan 81-41} and {Def Con 129}. | This is expected to be export trade packaging or military packaging level N, subject to agreement with the authority. |  | In-Service Sp. Eval |
| 49 | The system packaging shall be labelled in accordance with {Def Stan 81-41} and {Def Con 129}. |  |  | In-Service Sp. Eval |
| 50 | The system spares and consumables packaging shall be labelled in accordance with {Def Stan 81-41} and {Def Con 129}. |  |  | In-Service Sp. Eval |
| 51 | The system packaging shall be clearly marked with the system handling instructions. |  |  | In-Service Sp. Eval |
| 52 | Each equipment within the system shall have a serial number that is unique within the system family. |  |  | In-Service Sp. Eval |
| 53 | The system shall be suitable for storage for extended periods of time. | Storage temperature range to be between 10 and 30degC. |  | In-Service Sp. Eval |
| 54 | The system, spares and consumables shall be codified in accordance with MOD requirements outlined in the {ILS SOW}. |  |  | In-Service Sp. Eval |
| 55 | The system shall be provided with sufficient technical documentation for the production of in-service publications to allow personnel to operate and maintain the capability throughout its service life. |  |  | In-Service Sp. Eval |
| 56 | The system shall be operable in a hot office environment. |  |  | Design Review Supplier Analysis |
| 57 | The system shall be operable in a cold office environment. |  |  | Design Review Supplier Analysis |
| 58 | The system and its spares and consumables shall be storable using in-service facilities. |  |  | Design Review Supplier Analysis |
| 59 | The system shall be capable of temporary storage in a hot environment. |  |  | Design Review Supplier Analysis |
| 60 | The system shall be capable of temporary storage in a cold environment. |  |  | Design Review Supplier Analysis |
| 61 | The system shall be transportable on fixed wing platforms. |  |  | Design Review Supplier Analysis |
| 62 | The system shall be transportable on rotary wing platforms. |  |  | Design Review Supplier Analysis |
| 63 | The system shall be transportable by wheeled vehicles. |  |  | Design Review Supplier Analysis |
| 64 | The system shall be transportable via ship. |  |  | Design Review Supplier Analysis |
| 65 | The system shall be protected from the effects of being dropped during transportation. |  |  | Design Review Supplier Analysis |
| 66 | The system shall be protected from dust and water during transportation. |  |  | Design Review Certification |
| 67 | The system shall be provided with a Training Needs Analysis (TNA) to {JSP 822}, The Defence Systems Approach to Training (DSAT) requirements. | TNA to be conducted by Authority third party Contractor, with assistance from the equipment Contractor. TNA to cover all Target Audience Descriptions (TAD). |  | In-Service Sp. Eval |
| 68 | The system shall be provided with a Train the Trainer (T3) Operator and level 1 maintenance training course with training material. | Initial training to be provided for development within current capability training. Training material to be provided with Instructor Specifications and Assessor Specifications. |  | In-Service Sp. Eval |

Table 1: System Requirements

## Glossary

| Term | Definition |
| --- | --- |
| AC | Alternating Current |
| ALARP | As Low As Reasonably Practicable |
| BDR | Battle Damage Repair |
| BFM | Battlefield Mission |
| Biometric Match | A comparsion decision stating that the system generated DNA Profile and the DNA Profiles it is being compared against are from the same source. |
| BIT | Built In Test |
| BMR | Biometric match Report |
| BS | British Standard |
| CIS | Command Information Systems |
| CODIS | Combined DNA Index System and is the general term used to describe the FBI's programme of support for ciminal justice DNA databases, as well as the software used to run them. |
| DNA | Deoxyribonucleic Aced |
| DNA17 | DNA17 is the DNA profiling technologies used by the National DNA Database (NDNAD), which includes 32 DNA sites (plus gender indicator). |
| DNA Profile | A DNA profile is a small set of DNA variations that are unique to individuals. |
| DNA Sample | A sample of a person's DNA taken either directly from them or retrieved from a site. |
| Enrolment | The assigning of personal contextual information to a DNA Profile for future biometric matches. |
| Equipment Case | The Equipment Case is used to protect the equipment during strategic movemements (i.e. Inter-theathre). |
| ESSL | European Standard Set of DNA Database Loci |
| Familial Member | A blood relative of a person. |
| Fully Automated | A biometric match decision that requires no input from specialist personnel. |
| ID | Identification |
| In-Serivce DNA Collection Kit | The current MOD in-service DNA bilogical material collection kits are the Post Incident Analysis (PIA) kits and Personal Enrolment Kit (PEK) |
| ISO | International Standards Organisation |
| JSP | Joint Service Publication |
| Level 3 | Level 3 is the rigourous scientific and technical examination of materiel under controlled conditions at national facilities in the UK. |
| Local Database | A database of DNA Profiles that is stored within the system. |
| Minor System Failure | The system fails but after conducting ML1 maintenance, the user makes the system operational. The BFM continues after a delay. |
| MOD | Ministry of Defence |
| MPE | Materiel and Personnel Exploitation |
| NATO | North Atlantic Treaty Organisation |
| Non-specialist User | A person who's primary role is not in the analysis and interpretation of DNA forensics. |
| PEK | Personal Enrolment Kit |
| PIA | Post Incident Analysis |
| Register | The assignment of a unique ID to a DNA Sample. |
| Removable Media | Any type of storage device that can be removed from a system whilst it is still operating, making it easier to transfer data from one system to another. Examples include DVDs and USB drives. |
| SME | Subject Matter Expert |
| SOW | Statement of Work |
| T3 | Train the Trainer |
| TAD | Target Audience Description |
| TNA | Training Needs Analysis |
| Trace | Trace DNA is DNA that has been left behind at a site by a person through touch or by coming into contact with items. |
| UK | United Kingdom |
| Unpackaged | Unpackaged is define as without any form of packaging and in a fully operational state. |
| US | United States |
| User Interface | The interface that allows the user to make system inputs and control the operation of the system. |
| Yes/No | A binary visual indication that the DNA Sample is either suitable or not. |

## Appendix A – Vibration Test Limits

## Basic Vibration

- Defence Standard 00-35, Part 3, Chapter 2-01, Test M1 Random Vibration/ Mixed Mode (also see BS EN 60068-2-6, Tests Fh and Fi). Parameters as specified below.

## Wheeled Vibration

Broadband Spectral Density of 0.015 gn2/Hz from 10 to 50 Hz, reducing to 0.001 gn2/Hz at 500 Hz. Duration: 0.25 hours per axis per 18000 km of expected vehicle use (based on average speed of 50 km/hr) (Subject to a maximum of 10 hours per axis).

Spectral Density of 0.015 gn2/Hz from 10 to 50 Hz, reducing to 0.001 gn2/Hz at 1000 Hz. Duration: 0.75 hours per axis per 18000 km of expected vehicle use (based on average speed of 50km/hr) (Subject to a maximum of 10 hours per axis).

## Helicopter

## Note: For Chinook or Merlin Helicopters, seek specialist advice

Broadband Spectral Density of 0.004 gn2/Hz from 5 to 100 Hz, reducing to 0.001 gn2/Hz at 500 Hz.

Narrow band random components of:

| Component | Swept Frequency (Hz) | Bandwidth Maximum (Hz) | Vertical Amplitude (gb2) | Lateral Amplitude (gb2) | Longitudinal Amplitude (gb2) |
| --- | --- | --- | --- | --- | --- |
| L1 | 15 to 25 | 2 | 1.5 | 1.5 | 0.5 |
| L2 | 30 to 50 | 4 | 1.5 | 1.5 | 0.5 |

For amplitude of narrow band peaks (gn2/Hz) = gb2/B, where B Ω 10% of f1. The exact value of B will depend upon the frequency of resolution, i.e. the number of control lines, of the vibration test control equipment used. The maximum bandwidths shall be 2 and 4 Hz respectively.

Duration: 1 hour per axis per 200 hours of service flight (Minimum of 30 minutes per axis. 2 hours per axis is sufficient for most applications).

Sweep rate of one octave per minute.

## Helicopter – Underslung Load

Broadband Spectral Density:

| Frequency (Hz) | PSD g2/Hz |
| --- | --- |
| 5 | 0.005 |
| 100 | 0.005 |
| 1000 | 0.0001 |

1 hour of test is equivalent to 12 hours of flight time.

## Fixed Wing A/C (Propeller Driven)

## Light A/C (e.g. Islander) / Transport A/C (e.g. C130J/C130K)

Broadband random vibration specturm shall comprise of 0.0025 gn2/Hz from 10 to 90 Hz, reducing to 0.000125 gn2/Hz at 100 Hz, and then maintaining this level to 2000 Hz. Functional testing is to be conducted throughout the test.

Sine tone components of:

| Component | Frequency (f1) (Hz) | Vertical Amplitude (gb2) | Lateral Amplitude (gb2) | Longitudinal Amplitude (gb2) |
| --- | --- | --- | --- | --- |
| L1 | 78 | 0.5 | 0.5 | 0.25 |
| L2 | 156 | 0.375 | 0.375 | 0.125 |
| L3 | 234 | 0.25 | 0.25 | 0.125 |

Amplitudes of the Sine tones are calculated by gpk = √(2gb2)

Duration 30 minutes per axis.

## Fixed Wing A/C (Jet)

Broadband Spectrial Density:

| Frequency (Hz) | Axis | PSD (g2/Hz) |  |
| --- | --- | --- | --- |
|  |  | Vertical | Longitudinal/ Lateral |
| 10 |  | 0.002 | 0.001 |
| 1000 |  | 0.002 | 0.001 |
| 2000 |  | 0.0007 | 0.0005 |

1 hour of test is equivalent to 100 hours of flight time.

## Ship Borne

Broadband Spectrial Density:

| Frequency (Hz) | Axis | PSD (g2/Hz) |
| --- | --- | --- |
|  |  | All Axes |
| 5 |  | 0.00005 |
| 10 |  | 0.001 |
| 50 |  | 0.001 |
| 200 |  | 0.00005 |

Duration: 1 hour/axis = 3 months of transport.

## Rigid Inflatable Boats

Broadband Spectrial Density:

| Frequency (Hz) | Axis | PSD (g2/Hz) |
| --- | --- | --- |
|  |  | Vertical Axis |
| 5 |  | 0.0017 |
| 500 |  | 0.0004 |
| Frequency (Hz) | Axis | PSD (g2/Hz) |
|  |  | Fore/Aft Axis |
| 5 |  | 0.002 |
| 15 |  | 0.002 |
| 200 |  | 0.020 |
| 230 |  | 0.020 |
| 250 |  | 0.002 |
| 420 |  | 0.002 |
| 430 |  | 0.010 |
| 450 |  | 0.010 |
| 460 |  | 0.001 |
| 500 |  | 0.001 |
| Frequency (Hz) | Axis | PSD (g2/Hz) |
|  |  | Beam Axis |
| 5 |  | 0.023 |
| 130 |  | 0.100 |
| 200 |  | 0.004 |
| 500 |  | 0.001 |

Duration: 1 hour/axis = 180 hours of use.