Project PETREL

ANNEX G – Integrated Test and Evaluation (ITEAP)

ANNEX G to SSP/000192

19 Feb 18

PETREL VERIFICATION & VALIDATION REQUIREMENTS MATRIX (VVRM)

A1.	The following table of Systems Acceptance Tests constitutes the VVRM for Project PETREL.

| ID | System Acceptance Test | Test Category | Determination Details | Evaluation Criteria | SR Link | Responsibility | References | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # 1 Tenderers Tests |  |  |  |  |  |  |  |
|  | ## 1.1 Protection |  |  |  |  |  |  |  |
| SAT-5 | Fragmentation Protection and Area Coverage. | Technical Review | Annex B Paragraphs 1 & 3 | Pass/Fail This SAT consists of multiple parts. A fail in any of these tests will result in CPE Clothing being rejected from the competition. Fragmentation Protection PASS OMoP. V50 as stated in the SRD SR-1. PASS TMoP. V50 as stated in the SRD SR-1. In addition to providing test results for the TMoP you shall provide the V50 results for this material tested against a 1.1g FSP, this will be used for objective scoring assessment as detailed in Annex B Paragraph 3. FAIL. Failure to meet the Minimum V50 as stated in the SRD SR-1 Fragmentation protection is to be tested in accordance with NATO AEP 2920, with the following additional requirements: - For testing using chisel nosed fragment simulators less than 1.1g the Authority does not require the control or measurement of yaw - For chisel nosed 1.1g fragment simulators the yaw measurement shall be measured for every shot - A minimum of 22 V50 results are required as detailed in NATO AEP 2920, with the V50 calculated using the Probit method defined in AEP 2920 Annex H.3 - All samples are to be tested wet, using de-ionised water Testing is to be conducted at a facility accredited by UKAS to ISO 17025 and scoped for NATO AEP 2920 or an equivalent body signatory to the ILAC Mutual Recognition Agreement Area Coverage: PASS OMoP. Area coverage pass the OMoP as specified in Annex B Paragraph 3 PASS TMoP. Area coverage pass the TMoP as specified in Annex B paragraph 3 FAIL. Area coverage has failed to meet the requirements of Annex B Paragraph 3 Areal Density: PASS. Areal density is no more than or equal to 0.300 kg·m-2 FAIL. Areal density is greater than 0.300 kg·m-2 | SR-1 | Tenderers | [NATO AEP 2920] | A copy of STANAG 2920 and NATO AEP 2920 test protocol can be obtained through the DSTAN In order to gain access to the documents, Tenderers must register for a StanMIS account www.dstan.mod.uk/StanMIS/Account/Login Once Tenderers have successfully registered for an account they can search for and download the STANAG 2920 and the testing protocol (AEP 2920). |
| SAT-90 | Scalable Fragmentation Protection and Area Coverage | Technical Review | Annex B Paragraphs 2 & 3 | Pass/Fail This SAT consists of multiple parts. A fail in any of these tests will result in CPE Clothing being rejected from the competition. Fragmentation Protection OMoP PASS OMoP. V50 as stated in the SRD SR-80. FAIL. Failed to meet the Minimum V50 as stated in the SRD SR-80 Areal Density: PASS OMoP. Areal density is no more than 4.3 kg·m-2 FAIL. Areal density is greater than 4.3 kg·m-2 TMoP PASS TMoP. V50 as stated in the SRD SR-80 In addition to providing test results for the TMoP you may provide the V50 results for this material tested against a 1.1g FSP, this will be used for objective scoring assessment as detailed in Annex B Paragraph 3. FAIL. Failed to meet the Minimum V50 as stated in the SRD SR-80 Areal Density: PASS TMoP Areal density is no more than 0.300 kg·m-2 FAIL. Areal density is greater than 0.300 kg·m--2 Fragmentation protection is to be tested in accordance with NATO AEP 2920, with the following additional requirements: - For testing using chisel nosed fragment simulators less than 1.1g the Authority does not require the control or measurement of yaw - For 1.1g chisel nosed fragment simulators the yaw measurement shall be measured for every shot - A minimum of 22 V50 results are required as detailed in NATO AEP 2920 - All samples are to be tested wet, using de-ionised water with the V50 calculated using the Probit method defined in AEP 2920 Annex H.3 Testing is to be conducted at a facility accredited by UKAS to ISO 17025 and scoped for NATO AEP 2920 or an equivalent body signatory to the ILAC Mutual Recognition Agreement Area Coverage: PASS OMoP. Area coverage pass the OMoP as specified in Annex B Paragraph 3 PASS TMoP. Area coverage pass the TMoP as specified in Annex B paragraph 3 FAIL. Area coverage has failed to meet the requirements of Annex B Paragraph 3 | SR-80 | Tenderers | [NATO AEP 2920] |  |
| SAT-6 | Protection from Flame and burning liquids | Technical Review | Annex B Paragraph 4 | Scored: This will be tested against the Tenderers submission and results of Manufacturers Raw Material Data Sheets or test results from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. PASS OMoP as per Annex B Paragraph 4 PASS TMoP as per Annex B Paragraph 4 0 = Below TMoP | SR-3 | Tenderers | [BS EN ISO 11612:2015] |  |
| SAT-7 | Knee Protection | Technical Review | Annex B Paragraphs 5 & 6 | Scored: This will be tested against the Tenderers submission and results of Manufacturer's Raw Material Data Sheets or test results from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. PASS OMoP. Integrated, removable knee pad meeting the performance requirements of Annex B Paragraph 5 and 6. The protection is to be MTP or to blend with a colour within the MTP print. PASS TMoP. CPE Clothing is interoperable with the VIRTUS Knee pads. 0 = Below TMoP. CPE Clothing neither integrates with the VIRTUS Knee Pads of offers an integrated knee pad | SR-4 | Tenderers | [BS EN 14404:2004] |  |
| SAT-8 | Elbow Protection | Technical Review | Annex B Paragraphs 5 & 6 | Scored: This will be tested against the Tenderers submission and results of Manufacturer's Raw Material Data Sheets or test results from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. PASS OMoP. Integrated, removable elbow pad meeting the performance requirements of Annex B Paragraph 5 and 6. The protection is to be MTP or to blend with a colour within the MTP print. PASS TMoP. CPE Clothing affords protection to the elbows above that afforded by the shirt material 0 = Below TMoP. CPE Clothing does not afford any protection to the elbows | SR-66 | Tenderers | [DD CEN/TS 1526:2005] [BS EN 14404:2004] |  |
| SAT-10 | Multi-Terrain Pattern (MTP) IRR capability of main material for CPE clothing. | Technical Review | Annex B Paragraph 6 Table 16 | Pass / Fail IRR Main Material This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. PASS. MTP main material performance passes 70% of the measurements. (249/356) FAIL. Not passing 70% of the measurements. Standard Pattern (MAT0006B) The design of the image on the disc 125 MPT/01 is correct for print on material width of 150cm according to the Standard Pattern. Pattern repeat size 75cm - 80cm. | SR-6 | Tenderers | [MAT0006B] | IRR REQUIREMNTS All results MUST be submitted in the Wavelength data calculator provided VISUAL ASSESSMENT Samples of all MTP Main Materials will be required at tender return. These are to be the full width of the material and showing a full repeat of the MTP print Material Standard Pattern MAT0006B and Disc 125 MTP/01 are available from Defence Clothing Technical Information Office LEIDOS-TVS-DC-TIM@mod.uk |
|  | ## 1.2 Personnel |  |  |  |  |  |  |  |
| SAT-11 | Clothing size range accommodates the military population | Technical Review |  | Scored: Assessment of size and fit guides against anthropometric data. Percentile data is contained within the Survey; percentile data is stated in the SRD. The sizing of the items is to be as follows: The long sleeve shirts shall be available in a minimum of 10 sizes The trousers shall be sized in accordance with NATO sizing, based on the following 3 measurements: • Waist • Outside Leg • Seat The tier 1 pelvic protection shall be available in a minimum of 5 sizes PASS OMoP. Provision of sizing information for all sized components. Information contains size of wearer and/or suggested size range of wearer against the size description of item. 100% of the military population is fitted with all items of clothing. PASS TMoP. Provision of sizing information for all sized components. Information contains size of wearer and/or suggested size range of wearer against the size description of item. 1-99% of the military population is fitted with all items of clothing. CPE Clothing shall have an adjustable mechanism to ensure optimum fit to the wearer. 0 = Below TMoP. No provision of sizing information and/or size of components is not representative of anthropometric data provided. | SR-9 | Tenderers | [Def Stan 00-251] [STANAG 2335] |  |
|  | ## 1.3 Usability |  |  |  |  |  |  |  |
| SAT-12 | Mass of main materials | Technical Review | Annex B Tables 17 & 18 | Scored: This will be tested against the Tenderer submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Materials PASS TMoP. As per Annex B Table 2 0 = Above TMoP Knitted Materials PASS TMoP. As per Annex B Table 3 0 = Above TMoP | SR-58 | Tenderers | [BS EN 12127] |  |
| SAT-13 | Abrasion Resistance of main materials | Technical Review | Annex B Tables 17 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Materials PASS TMoP. As per Annex B Table 2 0 = Below TMoP | SR-59 | Tenderers | [12947-2] |  |
| SAT-14 | Air Permeability of main materials | Technical Review | Annex B Tables 17 & 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Materials (as received) PASS OMoP. As per Annex B Table 2 PASS TMoP. As per Annex B Table 2 0 = Below Threshold Knitted Materials PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-60 | Tenderers | [BS EN ISO 9237] |  |
| SAT-15 | Dimensional Stability of main materials | Technical Review | Annex B Tables 17 & 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Materials PASS TMoP. As per Annex B Table 2 0 = Below TMoP Knitted Materials PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-61 | Tenderers | [BS EN ISO 5077] |  |
| SAT-16 | Tensile Strength of main materials | Technical Review | Annex B Table 17 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Fabrics (as received) PASS OMoP. As per Annex B Table 2 PASS TMoP. As per Annex B Table 2 0 = Below TMoP | SR-62 | Tenderers | [BS EN ISO 13934-1] |  |
| SAT-17 | Tear Strength of main materials | Technical Review | Annex B Table 17 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Fabrics (after washing) PASS OMoP. As per Annex B Table 2 PASS TMoP. As per Annex B Table 2 0 = Below TMoP | SR-63 | Tenderers | [BS EN ISO 13937-3] |  |
| SAT-18 | Burst strength of main material | Technical Review | Annex B Table 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Knitted Fabrics (as received) PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-71 | Tenderers | [BS EN ISO 13938-1] |  |
| SAT-19 | Pill Rating of main materials | Technical Review | Annex B Tables 17 & 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Fabrics (as received) PASS TMoP. As per Annex B Table 2 0 = Below TMoP Knitted Fabrics PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-64 | Tenderers | [BS EN ISO 12945-1] [BS EN ISO 12945-2] |  |
| SAT-20 | Colour Fastness of main materials | Technical Review | Annex B Paragraph 7 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. All Fabrics PASS TMoP. As per Annex B Paragraph 6 0 = Below TMoP | SR-65 | Tenderers | [BS EN ISO 105 B02] - Light [BS EN ISO 105 C06] – Washing [BS EN ISO 105 E04] – Perspiration Acid & Alkaline [BS EN ISO 105 X12] – Rubbing |  |
| SAT-21 | Snagging of main material | Technical Review | Annex B Table 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Knitted Fabrics PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-72 | Tenderers | [PD 6527 Part 19] |  |
| SAT-22 | Solar Protection of main materials | Technical Review | Annex B Table 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Knitted Fabrics PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-73 | Tenderers | [BS EN 13758-1] |  |
| SAT-23 | Surface Resistivity of main materials | Technical Review | Annex B Table 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Knitted Fabrics PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-74 | Tenderers | [BS EN 1149-1] |  |
| SAT-24 | Charge Decay of main materials | Technical Review | Annex B Table 18 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Knitted Fabrics PASS TMoP. As per Annex B Table 3 0 = Below TMoP | SR-75 | Tenderers | [BS EN 1149-3] |  |
| SAT-25 | Absorption Rate of main materials | Technical Review | Annex B Paragraph 8 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Knitted Fabrics PASS TMoP. As per Annex B Paragraph 7 0 = Below TMoP | SR-76 | Tenderers | [BS 3449] |  |
| SAT-26 | Evaporation and Wicking Rates of main materials | Technical Review | Annex B Paragraph 8 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. Woven Fabrics (After Washing) PASS TMoP. As per Annex B Paragraph 7 0 = Below TMoP Knitted Fabrics (After Washing) PASS TMoP. As per Annex B Paragraph 7 0 = Below TMoP | SR-77 | Tenderers | [BS EN ISO 6330] [BS 3449] BS 3424-18] [BS 4554 |  |
| SAT-27 | Permethrin Treatment of main material | Technical Review | Annex B Paragraphs 9 & 10 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. All Fabrics PASS TMoP. As per Annex B Paragraphs 8 & 9 0 = Below TMoP | SR-13 | Tenderers | [TM27:2003] |  |
| SAT-89 | Anti-Microbial properties of main materials. | Technical Review | Annex B Paragraph 11 | Scored: This will be tested against the Tenderers submission and test results of Manufacturer's Raw Material Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement. All materials (After Washing) PASS - TMoP. As per Annex B Paragraph 10. 0 = Below TMoP | SR-82 | Tenderers | [ASTM E 2149-10] |  |
|  | ## 1.4 Sustainability |  |  |  |  |  |  |  |
| SAT-29 | Integrated Logistics Support Statement of Work | Technical Review |  | Pass/Fail A review of the technical documentation supplied with CPE Clothing provides assurance that it complies with the ILS Statement of Work. | SR-18 SR-20 SR-21 | Tenderers |  |  |
| SAT-28 | Care and Maintenance Instructions | Technical Review |  | Pass/Fail A review of the technical documentation supplied with the CPE Clothing System includes the following information: - A user fitting guide / care and maintenance manual. - The method of storage, evidence that the spatial envelope is minimised, and that CPE Clothing will not deteriorate when stored in the stated climatic conditions for up to 10 years. - The labelling is in accordance with the NATO sizing regime, unless otherwise stated, and includes the NSN, size, usage information and provides space for the wearer’s name and ID number. - The labelling is to include all wash/care instructions for each garment, clearly stating if any components of the garment are to be removed prior to washing i.e. Scalable protection as defined in SAT-90 - The risk of obsolescence of each component due to potential supply chain changes. | SR-17 SR-19 SR-22 SR-24 | DE&S | [BS EN ISO 3758:2012] |  |
|  | ## 1.5 Legislative |  |  |  |  |  |  |  |
| SAT-36 | Environmental | Technical Review |  | Pass/Fail Evidence is presented by the Tenderers in their Environmental Features Matrix that their CPE Clothing System, including its packaging, complies with the Secretary of State for Defence's policy for environmental protection. | SR-25 | Tenderers | JSP 418 | May be presented in the form of an Environmental Features Matrix. |
| SAT-37 | Safety | Technical Review |  | Pass/Fail Evidence is presented by the Tenderers in their Safety Case Report that the CPE Clothing System, including all packaging and supporting elements, is safe and suitable for operational use throughout its service life, is in full compliance with the references, and demonstrates that all risks are reduced So Far As Is Reasonably Practicable. | SR-26 | Tenderers | Def Stan 00-56 JSP 375 JSP 454 JSP 815 JSP 418 |  |
|  | # 2 IUAP Inspection |  |  |  |  |  |  |  |
| SAT-43 | Positioning of Protection Components | Visual Inspection |  | Scored An assessment made by the IUAP through a visual inspection of the CPE Clothing System indicates that it is likely that each protection component provided will remain correctly and securely positioned for all wearers. | SR-2 | DE&S |  | The assessment will be made in the form of a numerical score in answer to questions. |
| SAT-44 | Carriage of personal equipment | Visual Inspection |  | Scored The IUAP will put each of the military personal equipment listed in the SRD in to the appropriate pocket while either off the body or on a mannequin, and make an assessment of the ease with which it was accomplished. | SR-68 | DE&S |  | The assessment will be in the form of a numerical score. |
| SAT-45 | Bodily Functions | Visual Inspection |  | Scored The IUAP will make a subjective assessment on the ease with which a wearer of the PETREL CPE clothing can both urinate and defecate. | SR-70 | DE&S |  | This assessment will be in the form of a numerical score using the Likert scale. |
| SAT-46 | Fastenings and Mechanisms | Visual Inspection |  | Scored The IUAP will observe and manipulate any fastenings and adjustment mechanisms on CPE Clothing while off the body and make an assessment as to the durability and usability of them. | SR-12 | DE&S |  | The assessment will be in the form of a score. |
| SAT-48 | Simplicity of Use | Visual Inspection |  | Scored The IUAP will examine CPE Clothing and make an assessment as to the ease with which it is possible to put on and take off the clothing from the body. | SR-15 | DE&S |  | The assessment will be in the form of a score. |
| SAT-49 | Maintenance Req uirements | Visual Inspection |  | Scored Based on their examination of each item, the IUAP assess that the ease of carrying out any maintenance that may be required on the CPE Clothing System, such as the re-fixing of buttons, is acceptable. | SR-17 | DE&S |  | The assessment will be in the form of a numerical score. |
| SAT-50 | Supporting Documentation | Visual Inspection |  | Scored A review by the IUAP of the supporting information issued with the CPE Clothing System confirms that the operator information is considered sufficient to inform a wearer with no specific prior knowledge of CPE Clothing about the sizing, fitting, general usage and maintenance of the CPE Clothing System throughout its operational life. | SR-18 | DE&S |  | The assessment will be in the form of a numerical score. |
| SAT-51 | Conformance to PHS&T. | Visual Inspection |  | Pass / Fail A review by the IUAP of the CPE Clothing System's packaging and transportation solution confirms that CPE Clothing conforms to the MOD's packaging, handling, storage and transportation (PHS&T) requirements. | SR-20 | DE&S | Def Stan 81-41 Part 1, Issue 6). |  |
| SAT-52 | Traceability | Visual Inspection |  | Pass / Fail A visual inspection of CPE Clothing when packed, by the IUAP, confirms that: a. each component of the CPE Clothing System can be visually identified by type and, when unpacked, by the owner; b. each component can be easily marked by the owner to indicate ownership; c. each component can be identified by type when packed for storage or transportation. | SR-21 | DE&S |  |  |
| SAT-53 | UA-Labelling | Visual Inspection |  | Scored A visual inspection of CPE Clothing when packed, by the IUAP, confirms that: a. each component of the CPE Clothing System is labelled in accordance with the NATO sizing regime, including usage information, NSN, and size and has space for the wearer's name and ID number; b. the labelling is likely to remain attached and readable throughout the lifetime of the component. | SR-22 | DE&S |  |  |
|  | # 3 HFI Tests |  |  |  |  |  |  |  |
| SAT-54 | Kit Issue and Fitting | HFI Test |  | Results of the qualitative assessment following the issue and fitting of the PETREL CPE clothing system confirms that: a. the size, fit and coverage of the PETREL clothing amongst the trials personnel is found to be acceptable to the wearer (SR-9); b. the wearer is able to fit and adjust the CPE Clothing System easily using both bare and gloved hands under the conditions of the day (SR-12). | SR-9 SR-12 | Dstl |  | This test will take place on the first day of the PETREL trials and comprise kit issue and fitting, ensuring that all protection components fit well, and checking that each wearer has freedom of manoeuvre. |
| SAT-55 | Functional assessment and Common Battlefield Test Facility (CBTF) | HFI Test |  | The results of the run through the CBTF obstacles shows that: a. All protection components (knee & elbow) remained securely positioned for trial participants (SR-2); b. The knee protection provided adequate protection to the knees from impact to hard ground, rocks, and objects experienced during the CBTF test (SR-4); c. The elbow protection provided adequate protection to the elbows from impact with hard ground and obstacles experienced during the CBTF test run (SR-66); d. The wearer indicates that the thermal burden experienced during the CBTF run-through was not significantly worse than that experienced with the PCS (SR-5); e. The wearer was able to undertake kinematic actions when traversing obstacles in the CBTF (SR-11); f. The wearer was able to adjust the CPE Clothing System satisfactorily with bare and gloved hands under the conditions of the day (SR-12); g. The CPE Clothing System did not snag on any obstacle during the CBTF test run (SR-72). | SR-2 SR-4 SR-5 SR-11 SR-12 SR-66 SR-72 | DSTL |  | A run through of all CBTF obstacles. A questionnaire will be completed after all obstacles using a combination of Likert Scale and verbal responses to questions from DSTL Staff. |
| SAT-56 | Practice Firing in the DCCT | HFI Test |  | Following a live fire practice using the Dismounted Close Combat Trainer answers from the participating troops provide evidence that: a. The operator was able to undertake the operation of weapon systems and equipment in field orders of dress and in varying positions (range of movement) (SR-8); b. That all fastenings and adjustment mechanisms were found to be intuitive and simple to use, and operable with hands/gloves/ cold/wet (SR-12); c. The wearer was able to perform the required kinematic actions while adopting or changing between the lying prone, kneeling, sitting, squatting, and standing fire positions within the DCCT (SR-11); d. The wearer was able to retrieve military tools carried 'on the person' without assistance while lying prone, kneeling, sitting, squatting, and standing (SR-69); e. The CPE Clothing System did not snag on any obstacle (SR-72). | SR-8 SR-11 SR-12 SR-69 SR-72 | DSTL |  |  |
| SAT-57 | Final Comfort Assessment | HFI Test |  | The results of a questionnaire upon completion of all HFI tests while wearing PETREL CPE clothing system confirm that the wearer has not experienced any pain or rash from friction, heat retention, or pressure at localised points on the body, and that overall comfort was acceptable (SR-14). | SR-14 | DSTL |  | Question to be presented using Pain Map. |
|  | # 4 User Tests |  |  |  |  |  |  |  |
|  | ## 4.1 ITDU Trial |  |  |  |  |  |  |  |
| SAT-70 | Battlefield Mission Serial in Urban Environment. | ITDU Trial |  | Conduct Military training serials in an Urban Environment on a MilitaryTraining Area confirms that: a. All protection components (knee & elbow) have remained securely in place throughout the serial (SR-2); b. the knee protection adequately protects the wearer's knees from impact to hard ground, rocks and buildings (SR-4); c. the elbow protection adequately protects the wearer's elbows from hard impact with the ground, buildings and other obstacles (SR-66); d. the thermal burden on the wearer is not significantly worse than the current PCS (SR-5); e. the wearer is able to retrieve military tools carried on the person without undue difficulty (SR-69) f. the Clothing System does not snag on objects (SR-72), and; g. the fastenings and adjustment mechanisms are Intuitive and simple to use. (SR-12). - Quiet. - Operable with bare hands - Operable when wearing combat gloves. - Operable with cold hands. - Operable when wet. | SR-2 SR-4 SR-5 SR-66 SR-69 SR-72 | ITDU |  | Questionnaire to use Likert Scale & Verbal responses. The ITDU Trails will consist of five Battlefield Missions (BFM) on Salisbury Plain, each lasting 48 hours duration. One BFM will be undertaken by each soldier with a different tenderers clothing system, plus one baseline. Similar BFMs will also be conducted under testing field conditions in a hot climate with British Army Training Unit Kenya (BATUK) |
| SAT-71 | Battlefield Mission serial in Rural Environment. | ITDU Trial |  | A run through of Military training serial in a 'rural environment' within the Salisbury Plain Training Area confirms that: a. All protection components (knee & elbow) have remained securely in place throughout the serial (SR-2); b. the knee protection adequately protects the wearer's knees from impact to hard ground and rocks (SR-4); c. the elbow protection adequately protects the wearer's elbows from hard impact with the ground and other obstacles (SR-66); d. the thermal burden on the wearer is not significantly worse than the current PCS (SR-5); e. the wearer is able to retrieve military tools carried on the person without undue difficulty (SR-69), f. the CPE Clothing System does not snag on objects (SR-72), and; g. the fastenings and adjustment mechanisms are Intuitive and simple to use. (SR-12). - Quiet. - Operable with bare hands - Operable when wearing combat gloves. - Operable with cold hands. - Operable when wet. | SR-2 SR-4 SR-5 SR-66 SR-69 SR-72 | ITDU |  | Questionnaire to use Likert Scale & Verbal responses. |
| SAT-72 | End of Day's Serials Questionnaire | ITDU Trial |  | The results of a questionnaire at the end of the day's serials confirm that: a. the thermal burden experienced by the participating troops was not significantly worse than that with the PCS system (SR-5). b. the wearer experienced no difficulty operating the weapons, sensors and specialist infantry equipment used during the day (SR-7); c. the wearer was able to undertake the full range of military activities ordered during the day, including operating equipment while wearing the field order of dress (SR-8); d. the wearer indicates that the CPE Clothing System fitted them satisfactorily throughout the day (SR-9); e. the wearer experienced no difficulty with urinating and defecating while wearing the CPE Clothing System (SR-70); f. the wearer was able to fit and adjust the CPE Clothing System with both bare and gloved hands in the conditions experienced in the field with no issues (SR-12); g. the wearer's level of comfort during the day's training was at least as good as with PCS and that no friction points were experienced (SR-14). | SR-5 SR-7 SR-8 SR-9 SR-12 SR-14 SR-70 | ITDU |  | Questionnaire to use Likert Scale & verbal responses. |
|  | ## 4.2 Other TDU Trials |  |  |  |  |  |  |  |
| SAT-84 | Compatibility | ITDU Trial |  | Troops from non-infantry units wearing the PETREL CPE clothing and any other task-related clothing provide written evidence that they did not experience any compatibility issues with respect to the PETREL CPE clothing when using the equipment or vehicles listed in the designated Integration List (SR-7) and that the CPE Clothing System did not snag on any surface (SR-72). | SR-7 SR-72 SR-12 | Other TDU DCBRNC |  | Questionnaire to use Likert Scale & verbal responses. |
| SAT-85 | Bodily Functions | ITDU Trial |  | The PETREL CPE Clothing System does not hinder or prevent the use of specialist toilet facilities in vehicles or any other toilet facility used by the represented Corps. | SR-70 | Other TDU |  |  |
| SAT-86 | Airborne Certification | ITDU Trial |  | The PETREL CPE Clothing System is certified by SQEP to be suitable for use in aircraft and for conducting parachute operations, and is granted the Airborne Forces Equipment Release Certificate of use. | SR-10 | JADTEU GM Cap |  |  |

ANNEX B	TEST DEFINITIONS

B1.	The following table defines the procedures to be used in the Tenderers Tests detailed in Annex A.

- SAT-5 Area of Coverage

NECK 1.1 Threshold Coverage - Minimum length (total height) horizontally from the suprasternal notch for front, rear and sides shall be 65mm. - Minimum width of coverage shall be a horizontal line from the suprasternal notch along the clavicle and ending at the acromion(s). - Tenderers shall ensure continuous coverage to encompass the neck. 1.2 Objective Coverage - Coverage to be the same as threshold coverage.

UPPER ARM 1.4 Threshold Coverage - Minimum lengths for the upper arm coverage from the acromion (see figure 1) are provided in Table 1. - Tenderers must meet the minimum lengths specified in Table 1. - Tenderers must ensure continuous coverage that encompasses the entire upper arm including underarm. 1.5 Objective Coverage - Minimum lengths for the upper arm coverage from the acromion are provided in Table 1. - Tenderers must meet the minimum lengths specified in Table 1. Figure 1: Landmarks of the upper arm. Coverage of the arm is defined from the three landmarks depicted in Figure 1. 1. Acromion 2. Axillary fold 3. Deltoid insertion

Table 1 – Arm protection length from the acromion Threshold arm protection length below acromion (mm). Percentile; Male; Female. 1; 150; 141. 2; 153; 144. 3; 156; 145. 5; 158; 147. 10; 162; 152. 15; 165; 155. 20; 167; 158. 25; 169; 160. 30; 171; 161. 35; 172; 163. 40; 174; 165. 45; 175; 166. 50; 176; 168. 55; 178; 169. 60; 179; 170. 65; 180; 172. 70; 182; 173. 75; 184; 175. 80; 186; 177. 85; 188; 179. 90; 191; 183. 95; 195; 186. 97; 198; 189. 98; 200; 190. 99; 201; 192.

TORSO 1.6 Threshold Coverage - Minimum lengths for torso coverage (lower border of rib cage to iliac crest) shall be derived from the data provided in Table 2 (Suprasternal Notch to Iliac Crest) and Table 3 (VIRTUS Soft Armour Filler Lengths). - Tenderers shall ensure integration with VIRTUS STV. Coverage is to be maintained during physical activities - Tenderers shall ensure continuous coverage to the (360°) of the garment. 1.7 Objective Coverage - Coverage to be the same as threshold coverage. Figure 2: Landmarks of the torso. Coverage of the torso is defined from the three landmarks defined in Figure 3. 1. Suprasternal notch. 2. Lower border of ribcage (10th rib). 3. Iliac crest. Distance A is suprasternal notch to lower border of ribcage (10th rib); distance B is suprasternal notch to iliac crest.

Table 2 – Measurements from suprasternal notch to iliac crest Suprasternal notch to iliac crest length (mm). Percentile; Male; Female. 1; 333; 282. 2; 334; 289. 3; 337; 294. 5; 342; 297. 10; 346; 305. 15; 351; 310. 20; 355; 315. 25; 358; 319. 30; 362; 322. 35; 363; 325. 40; 367; 328. 45; 369; 330. 50; 371; 332. 55; 373; 334. 60; 375; 336. 65; 377; 339. 70; 380; 342. 75; 385; 346. 80; 392; 349. 85; 399; 354. 90; 405; 362. 95; 411; 368. 97; 414; 374. 98; 418; 375. 99; 422; 377.

Table 3 – VIRTUS STV Soft Armour Filler Lengths Soft armour length (mm). Vest Size; Front; Rear. S; 338; 363. M; 334; 383. MW; 339; 383. ML; 347; 401. L; 367; 425. LW; 372; 425. XL; 397; 461. XLW; 402; 461. Note – The Small and Medium Front measurements are correct.

PELVIS and THIGH - Threshold Coverage - Minimum lengths (from iliac crest Figure 3) for pelvis and thigh coverage is provided in Table 4. - Tenderers shall meet the minimum length specified in Table 4. 185mm shall be the minimum length for any size female less than the 1st percentile male - Tenderers shall ensure that the coverage is continuous to include the gusset (underneath). - Minimum width of coverage specified in Table 5 for males and Table 6 for females. - Objective Coverage - Minimum lengths (from iliac crest) for pelvis and thigh coverage is provided in Table 4. - Tenderers shall meet the minimum length specified in Table 4. 185mm shall be the minimum length for any size female less than the 1st percentile male - Tenderers shall ensure that the coverage is continuous to include the gusset (underneath). - Objective width specified in Table 7. Figure 3: Pelvis and leg landmarks. Coverage of the pelvis/leg is defined from the two landmarks in Figure 3. 1. Iliac crest 2. Ischial tuberosity.

Table 4 – Measurement Lengths from Iliac Crest Threshold pelvis/leg protection length below iliac crest (mm). Percentile; Male; Female. 1; 185; Not available. 2; 196; Not available. 3; 198; Not available. 5; 201; Not available. 10; 207; Not available. 15; 208; Not available. 20; 210; Not available. 25; 212; Not available. 30; 216; Not available. 35; 219; Not available. 40; 219; Not available. 45; 221; Not available. 50; 222; Not available. 55; 224; Not available. 60; 225; Not available. 65; 227; Not available. 70; 230; Not available. 75; 231; Not available. 80; 233; Not available. 85; 236; Not available. 90; 238; Not available. 95; 243; Not available. 97; 243; Not available. 98; 243; Not available. 99; 246; Not available.

Table 5 – Threshold Width Coverage Measurements for Males Size; Front (mm); Rear (mm). Small; 220; 240. Medium; 270; 290. Large; 320; 340. X Large; 370; 390. XX Large; 390; 440. Table 6 – Threshold Width Coverage Measurements for Females Size; Front (mm); Rear (mm). X Small; 210; 250. Small; 260; 300. Medium; 310; 350. Large; 360; 400. X Large; 410; 450. Note – These measurements in Tables 5 & 6 are taken when the garment is relaxed and not stretched.

Table 7 – Objective Width Coverage Measurements Note – These measurements are taken when the garment is relaxed and not stretched. Front and rear measurements are identical. Size; Width (mm). X Small; 500. Small; 600. Medium; 700. Large; 800. X Large; 900. XX Large; 1000.

- SAT-90 Scalable (Removable) Area of Coverage

All scalable levels of protection must be capable of being removed from the garments, when the higher level of protection is not required NECK – - Threshold Coverage – - Minimum length (total height) horizontally from suprasternal notch for front, rear and sides shall be 65mm. - Minimum width of coverage shall be a horizontal line from the suprasternal notch along the clavicle and ending at the acromion(s). - Tenderers shall ensure continuous coverage to encompass the neck. 2.2 Objective Coverage – - Minimum length (height) from suprasternal notch for front and sides shall be 75mm and for the rear shall be 31mm. - Tenderers can provide increased coverage (up to 100mm (suprasternal notch to lower border of mandible, figure 4) whilst ensuring integration of neck coverage with VIRTUS STV, nape protector and helmet. Fig 4 Landmarks of the neck Coverage of the neck is divided into 3 zones as depicted in Figure 4. These are defined as: • Zone 1: Suprasternal notch to cricoid cartilage. • Zone 2: Cricoid cartilage to lower border of mandible. • Zone 3: Lower border of mandible to base of skull

UPPER ARM – - Threshold Coverage – - Minimum lengths for the upper arm coverage (from the acromion) specified in Table 8. - Tenderers must meet the minimum length specified in Table 8. - Tenderers must ensure that the coverage encompasses the upper arm and underarm. - Objective coverage – - Tenderers shall provide coverage to the upper arm to a maximum length specified in Table 8. - Tenderers to note that whilst no minimum area or length is specified by the authority the coverage shall remain within the anthropometric landmarks of specified (acromion/lower border of coverage) Fig 5. - Tenderers shall ensure integration with VIRTUS STV and the upper arm requirements detailed in SAT 5. Figure 5: Landmarks of the upper arm. Coverage of the arm is defined from the three landmarks depicted in Figure 2. 1. Acromion. 2. Axillary fold 3. Deltoid insertion

Table 8 Measurements of arm protection length from the acromion Threshold arm protection length below acromion (mm). Percentile; Male; Female. 1; 150; 141. 2; 153; 144. 3; 156; 145. 5; 158; 147. 10; 162; 152. 15; 165; 155. 20; 167; 158. 25; 169; 160. 30; 171; 161. 35; 172; 163. 40; 174; 165. 45; 175; 166. 50; 176; 168. 55; 178; 169. 60; 179; 170. 65; 180; 172. 70; 182; 173. 75; 184; 175. 80; 186; 177. 85; 188; 179. 90; 191; 183. 95; 195; 186. 97; 198; 189. 98; 200; 190. 99; 201; 192.

TORSO – - Threshold coverage – - Minimum lengths for torso coverage (lower border of rib cage to iliac crest, figure 6) shall be derived from the data provided in Table 9 (Suprasternal Notch to Iliac Crest) and Table 10 (VIRTUS Soft Armour Filler Lengths). - Tenderers to note that this coverage requirement applies to the front and rear of the garment, but the rear shall integrate with the VIRTUS STV and lower back protection held within the VIRTUS Chassis/hip belt. - Objective coverage – - Tenderers shall provide coverage to the torso up to a maximum length as specified in Table 9 - Tenderers to note that whilst no minimum area/length is specified by the Authority that the coverage must remain within the anthropometric landmarks of lower border rib cage to iliac crest. - Tenderers shall ensure that the coverage shall integrate with the VIRTUS STV and lower back protection held within the VIRTUS Chassis/hip belt. Figure 6: Landmarks of the torso. Coverage of the torso is defined from the three landmarks defined in Figure 3. 1. Suprasternal notch. 2. Lower border of ribcage (10th rib). 3. Iliac crest. Distance A is suprasternal notch to lower border of ribcage (10th rib); distance B is suprasternal notch to iliac crest.

Table 9 – Measurement lengths from suprasternal notch to iliac crest Suprasternal notch to iliac crest length (mm). Percentile; Male; Female. 1; 333; 282. 2; 334; 289. 3; 337; 294. 5; 342; 297. 10; 346; 305. 15; 351; 310. 20; 355; 315. 25; 358; 319. 30; 362; 322. 35; 363; 325. 40; 367; 328. 45; 369; 330. 50; 371; 332. 55; 373; 334. 60; 375; 336. 65; 377; 339. 70; 380; 342. 75; 385; 346. 80; 392; 349. 85; 399; 354. 90; 405; 362. 95; 411; 368. 97; 414; 374. 98; 418; 375. 99; 422; 377.

Table 10 – VIRTUS STV Soft Armour Filler Lengths Soft armour length (mm). Vest Size; Front; Rear. S; 338; 363. M; 334; 383. MW; 339; 383. ML; 347; 401. L; 367; 425. LW; 372; 425. XL; 397; 461. XLW; 402; 461. Note – The Small and Medium Front measurements are correct.

| PELVIS AND THIGH – - Threshold Coverage – - Minimum lengths from iliac crest in Table 11. - Tenderers must meet the minimum length specified in Table 11. 185mm shall be the minimum length for any size female less than the 1st percentile male - Tenderers must meet the minimum width specified in Table 12 for males and Table 13 for females. - Objective Coverage – - Tenderers shall provide coverage to the pelvis and thigh up to a maximum length specified in Table 11. - Tenderers to note that whilst no minimum area of coverage width/length is specified by the authority, the coverage shall remain within the anthropometric landmarks specified (iliac crest to lower border of coverage Fig 7. - Tenderers shall ensure integration with the VIRTUS STV and the pelvis and thigh requirements detailed in SAT 5. Figure 7: Pelvis and leg landmarks. Coverage of the pelvis/leg is defined from the two landmarks in Figure 7. 1. Iliac crest 2. Ischial tuberosity. |  |
| --- | --- |
| Table 11 – Measurement lengths from iliac crest Threshold pelvis/leg protection length below iliac crest (mm). Percentile; Male; Female. 1; 185; Not available. 2; 196; Not available. 3; 198; Not available. 5; 201; Not available. 10; 207; Not available. 15; 208; Not available. 20; 210; Not available. 25; 212; Not available. 30; 216; Not available. 35; 219; Not available. 40; 219; Not available. 45; 221; Not available. 50; 222; Not available. 55; 224; Not available. 60; 225; Not available. 65; 227; Not available. 70; 230; Not available. 75; 231; Not available. 80; 233; Not available. 85; 236; Not available. 90; 238; Not available. 95; 243; Not available. 97; 243; Not available. 98; 243; Not available. 99; 246; Not available. |
| Table 12– Threshold Width Coverage Measurements for Males Size; Front (mm); Rear (mm). Small; 220; 240. Medium; 270; 290. Large; 320; 340. X Large; 370; 390. XX Large; 390; 440. Table 13 – Threshold Width Coverage Measurements for Females Size; Front (mm); Rear (mm). X Small; 210; 250. Small; 260; 300. Medium; 310; 350. Large; 360; 400. X Large; 410; 450. Note – These measurements in Tables 12 & 13 are taken when the garment is relaxed and not stretched. |

- SAT-5 & SAT-90 Coverage Scoring

- SAT 5 Basic armour Basic armour will be scored for protection and coverage levels with the split: - 80% protection - 20% coverage Basic armour coverage must meet the Threshold requirements described in SAT-5 Paragraph 1. Systems will be rewarded as follows up to a maximum score of 10: - Between 0 and 8 points for protection level on a linear scale based on the V50 of the protection from a 1.1g fragment on a scale from 50m/s up to 300m/s. The Protection Score (PS) is defined as: PS = ( (V50 – 50) / 250 ) x 8 - If the basic armour has different protection levels in different areas then the protection score will be calculated as a percentage of the entire protected area. This will be based off CPE Clothing that fits a 50th percentile male. - For example, if PS1 and PS2 are the Protection Scores for areas A1 and A2 (defined as a proportion of the total coverage area) then: PS = (PS1 x A1) + (PS2 x A2) - Between 0 and 0.5 points for extra coverage around the legs. This will be awarded on a linear scale based on the extra area of protection needed to cover the entire width of the region. - The Threshold Undershorts Area (TUA) coverage as defined above, the Whole Undershorts Area (WUA) is the entire area of the garment up to the threshold leg length and Extra Undershorts Area (EUA) coverage is the extra area between the front and rear widths of threshold protection in the undershorts; the Leg Coverage Score (LCS) is defined as: LCS = ( EUA / (WUA - TUA) ) x 0.5 - The score will be evaluated on the garment that fit the 50th percentile male. - Between 0 and 1.5 points for coverage above 65mm height in the neck. This will be awarded on a linear scale based on the extra area of protection provided up to a total amount of 14,000mm2. - The Threshold Neck Area (TNA) coverage as define above and the Extra Neck Area (ENA) coverage is the extra area in the Collar over 65mm above the suprasternal notch, then the Neck Coverage Score (NCS) is defined as: NCS = ( ENA / 14000 ) x 1.5 - The score will be evaluated on the garment that fit the 50th percentile male. The Coverage score for the Basic armour (BC), out of a maximum score of 10, is defined as: BC = PS + LCS + NCS - SAT-90 Removable Scalable armour COAT assessments will be undertaken on the scalable armour solutions submitted to determine the level of coverage afforded, this score will be multiplied by the protection level afforded to determine a Scalable Armour Coverage (SAC) score. - COAT Assessments Figure 8: Example of COAT with the Baseline VIRTUS coverage. The baseline coverage score is the current VIRTUS soft armour coverage detailed in Table 15. The upper score that solutions are assessed against will be total coverage of these structures in assessments (100%). The structures assessed in each region are detailed in Table 14 and shown in figure 9. Neck; Torso/Abdomen; Arms; Pelvis/legs. Aortic arch; Heart; Axillary Arteries; Iliac Arteries. Oesophagus; Spinal Cord; Brachial Arteries; Femoral arteries. Spinal Cord; Aorta; Iliac Veins. Carotid Arteries; Vena Cava. Vertebral Arteries; Liver. Larynx; Oesophagus. Pharynx; Bronchial Arteries. Pulmonary Arteries. Pulmonary Veins. Spleen. Subclavian Artery. Subclavian Vein. Table 14: Structures assessed in COAT assessments. Figure 9: Structures to be assessed. - COAT results The VIRTUS soft armour was assessed in COAT; the results are displayed in Table 15 and represent the Baseline Coverage Score (BCS). Area; % covered. Neck; 51.1. Torso/Abdomen; 77.3. Arms; 14.6. Pelvis/legs; 23.3. Table 15: Baseline COAT results. For each area of the body the coverage score is determined by assessing how close to full coverage (100) from the Baseline Coverage Score CPE Clothing achieves. If CS is the COAT Score for CPE Clothing, this is defined as: ( (CS - BCS) / (100-BCS) ) x 2.5 This is calculated for each of the areas assessed and we name these: NCS, TCS, ACS and LCS for the neck, torso/abdomen, arm and leg/pelvis respectively. Hence a system that offers full coverage to all areas will result in a coverage score of 10. - Protection level scores The protection level afforded by the scalable armour will be determined on a linear scale based on the V50 of the protection from a 1.1g FSP on a scale from 50m/s up to 490m/s. This will then be applied as a Protection Multiplier (PM) defined by: PM = ( (V50 – 50) / 440 ) Hence PM will be between 0 and 1. If the scalable armour solution has different protection levels then each will have an associated PM. - Scalable Armour Coverage score The Scalable Armour Coverage score is defined as: SAC = PM x (NCS + TCS + ACS + LCS) This has a maximum score of 10. If the scalable armour solution has different protection levels then RAS will be determined for each protection level and the Scalable Armour Coverage score will be the total sum of each individual score, this still has a maximum score of 10.

- SAT- 6 Flame and Burning Liquids

4.1 Testing Main Material The main materials are to be tested for Flaming Debris only, in accordance with BS EN ISO 11612:2015 Paragraph 6.3 Limited Flame Spread Objective: Tested in accordance with BS EN ISO 11612:2015 Procedure A1 and A2. Flaming debris; No Specimen shall give flaming or molten debris. Threshold: Tested in accordance with BS EN ISO 11612:2015 Procedure A1. Flaming debris; No Specimen shall give flaming or molten debris.

4.2 Proof Testing • Proof Testing is required for Tender Sample Submissions. • Tested in accordance with Annex B Paragraph 3 of the PETREL ITEAP/38452/002. • Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

4.3 Batch Testing • Batch testing will be required for production quantities for the PETREL contract SSP/00192 • Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. • Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance wit4h Annex B Paragraph 3 of the PETREL ITEAP/38452/002 version correct at contract let. • If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested. • If either re-test sample does not meet a requirements, the whole batch is rejected. • If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken.

- SAT- 7 & 8 Knee and Elbow Protection

5.1 The knee and elbow pads must comply the following test in order to meet the OMoP: OMoP – Performance against Level 2 BS EN 14404:2004 = A1:2010 (type 1 or 2). Clause 5.2.5 - Penetration resistance, resistance penetration at a force for at least 250N ±10N. Internal face shall not deflect by more than 5mm. Clause 5.2.6 - Force distribution, mean force recorded by each transducer shall be less than 30N. Clause 5.2.7 - Peak transmitted force in impact under impact energy of 5J peak force shall not exceed Mean 3kN highest value 4kN

5.2 Proof Testing • Proof Testing is required for Tender Sample Submissions. • Tested in accordance with Annex B Paragraph 4 of the PETREL ITEAP/38452/002 • Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

5.3 Batch Testing • Batch testing will be required for production quantities for the PETREL contract SSP/00192 • Knee/elbow pads – test every batch. Maximum batch size is to 5000 items. Only the first batch is required to be tested. • Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 16 of the PETREL ITEAP/38452/002 version correct at contract let. • If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested • If either re-test sample does not meet a requirements, the whole batch is rejected • If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken

| - SAT- 10 Multi Terrain Pattern (MTP) IRR capability for Main Material |  |
| --- | --- |
| 6.1 Standard Pattern No. MAT0006B may be obtained from the DCT Technical Information Office. - The design of the image on Disc 125 MPT/01 is correct for print on material width of 150cm as per the Standard Pattern. - The visual image must not be shrunk or elongated to fit narrower or wider widths of material. The image must be cropped to fit. - The visual image must not be shrunk for items that have a small surface area such as and collars. - If the image needs to be cropped to fit the width of material for print the balance of shade needs to be maintain for the overall visual image of the camouflage to be maintained. |  |
| 6.2 Colour link |  |
| Written specification | TIF file |
| Cream | Bone |
| Light Brown | Desert Pink |
| Khaki | Beige |
| Light Green | Olive |
| Dark Green | Green |
| Medium Brown | Medium Brown |
| Dark Brown | Dark Brown |
| 6.3 Determination of Near Infra-red reflectance Apparatus - A scanning spectrophotometer fitted with a diffuse reflectance attachment, working and calibrated over the range 650 to 1,150nm - Calibration to be traceable to National Standards - Reflectance from behind the test sample is to be less than 5% reflectance over the range of 650 to 1150 nm (Carbon Black filled rubber has been found to be suitable). Sampling - Any pre-treatments must be completed prior to colour samples being cut - Measurements to be taken on two different samples of each colour in the pattern. The samples must be a minimum of 7mm diameter but large enough to fill the test port. (If a composite sample is tested this must be stated in the test report.) - Unless the incident light is depolarised the two samples of each colour must be tested such that the fabric orientation is rotated through 90 degrees between measurements. The reported result shall be an average of the two readings at each wavelength - For example: if the first dark brown sample was tested with the warp direction vertical on the test instrument, the second dark brown sample should be taken with the weft direction vertical on the test instrument. Only two readings are required for each colour. - Base cloth to be thoroughly de-sized and scoured before being heat set - Scoured cloth must not contain any impurities which may adversely affect the subsequent coating process - Selvedge’s are to be firm and straight and not of such thickness as may lead to unacceptable build up during layering up for end item cutting - The dyed cloth is to be uniformly coated on one side with an un-pigmented hydrolysis resistant polyurethane elastomer - The coating is to be free from pinholes, thin places, or any other coating defect. |  |
| 6.4 Testing - Test results must be entered onto the Wavelength Data Calculator - Calibrate the spectrophotometer in accordance with the manufacturer’s instructions - Check that the reflectance of the instrument without a sample is less than 5% - If the reflectance is greater than 5% no testing can be carried out - Mount one layer of cloth, on the port of the integrating sphere Note the warp direction and ensure the second sample is rotated through 90 degrees - Scan the face side on each sample for each colour |  |
| 6.5 Proof Testing - Proof Testing is required for Tender Sample Submissions - Tested in accordance with Annex B Paragraph 5 PETREL ITEAP/38452/002 - Test reports must be either Manufacturers Raw Technical Data Sheets or from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement - Test results must be entered into the Wavelength Calculator |  |
| 6.6 Batch Testing - The Tenderer will provide the Authority at the beginning of system acceptance stage, a preproduction sample with IRR test report for compliance approval. This initial sample shall serve as reference for all subsequent production. - Batch testing will be required for production quantities for the PETREL CPE Clothing System contract SSP/00192 - Tested for every dye batch, Maximum batch size is 5000m. The first 3 batches are to be test as per the stated batch size. - On acceptance of the first 3 batches, all subsequent batch sizes will be 20,000m - Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 6 of the PRETREL ITEAP/38452/002 version correct at contract let. - Test reports must be provided for each batch of main material used on contract SSP/00192 - Each batch sample no less than 20m from the end piece - Are to be representative of each dying or finishing batch - Large enough to provide material for all the tests required - Each sample is to be identified with the piece from which it is taken. |  |

| Table 16 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| TMoP Multi-Terrain Pattern: Upper and Lower Limits |  |  |  |  |  |  |
|  | Area 1 |  | Area 2 |  | Area 3 |  |
| Wavelength nm | Cream & Khaki |  | Light Green, Dk Green & Light Brown |  | Mid Brown & Dark Brown |  |
|  | Upper | Lower | Upper | Lower | Upper | Lower |
| 650 | 40 | 20 | 25 | 15 | 15 | 7 |
| 660 | 43 | 22.5 | 28 | 17 | 16 | 8 |
| 670 | 46 | 25 | 31 | 21 | 17 | 9 |
| 680 | 49 | 27.5 | 33.5 | 23.5 | 18 | 10 |
| 690 | 52 | 30 | 35 | 25 | 19 | 11 |
| 700 | 55 | 32.5 | 36 | 26 | 20 | 12 |
| 710 | 58 | 35 | 37 | 27 | 21 | 13 |
| 720 | 61 | 37.5 | 38 | 28 | 21.5 | 13.5 |
| 730 | 64 | 40 | 39 | 29 | 22 | 14 |
| 740 | 67 | 42.5 | 40 | 30 | 22 | 14 |
| 750 | 70 | 45 | 41 | 31 | 22 | 14 |
| 760 | 70 | 47.5 | 42 | 32 | 22 | 14 |
| 770 | 70 | 50 | 43 | 33 | 22 | 14 |
| 780 | 70 | 50 | 43.5 | 33.5 | 22 | 14 |
| 790 | 70 | 50 | 44 | 34 | 22 | 14 |
| 800 | 70 | 50 | 44.5 | 34.5 | 22 | 14 |
| 810 | 70 | 50 | 45 | 35 | 23 | 15 |
| 820 | 70 | 50 | 45 | 35 | 23 | 15 |
| 830 | 70 | 50 | 45 | 35 | 24 | 16 |
| 840 | 70 | 50 | 45 | 35 | 24 | 16 |
| 850 | 70 | 53. | 45 | 35 | 25 | 17 |
| 860 | 70 | 53 | 45 | 35 | 25 | 17 |
| 870 | 70 | 53 | 45 | 35 | 26 | 18 |
| 880 | 70 | 53 | 45 | 35 | 26 | 18 |
| 890 | 70 | 53 | 45 | 35 | 27 | 19 |
| 900 | 70 | 53 | 45 | 35 | 27 | 19 |
| 910 | 70 | 53 | 45 | 35 | 28 | 20 |
| 920 | 70 | 53 | 45 | 35 | 28 | 20 |
| 930 | 70 | 53 | 45 | 35 | 28 | 20 |
| 940 | 70 | 53 | 45 | 35 | 28 | 20 |
| 950 | 70 | 57 | 45 | 35 | 28 | 20 |
| 960 | 70 | 57 | 45 | 35 | 29 | 21 |
| 970 | 70 | 57 | 45 | 35 | 29 | 21 |
| 980 | 70 | 57 | 45 | 35 | 30 | 22 |
| 990 | 70 | 57 | 45 | 35 | 30 | 22 |
| 1000 | 70 | 57 | 45 | 35 | 30 | 22 |
| 1010 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1020 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1030 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1040 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1050 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1060 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1070 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1080 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1090 | 70 | 57 | 45 | 35 | 31 | 23 |
| 1100 | 70 | 57 | 45 | 35 | 32 | 24 |
| 1110 | 70 | 57 | 45 | 35 | 32 | 24 |
| 1120 | 70 | 57 | 45 | 35 | 32 | 24 |
| 1130 | 70 | 57 | 45 | 35 | 32 | 24 |
| 1140 | 70 | 57 | 45 | 35 | 32 | 24 |
| 1150 | 70 | 57 | 45 | 35 | 32 | 24 |

| Table 17 – Physical Characteristics of Woven Fabrics |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SAT | 12 | 13 | 14 | 15 | 16 | 17 | 19 |
| Test | Mass g/m2 | Abrasion Resistance using 9kPa pressure | Air permeability Mm/sec | Dimensional stability After washing * % max Warp and Weft | Tensile strength | Tear strength After washing* | Pilling 36,000 cycles After washing* |
|  | BS EN 12127 | BS EN ISO 12934-1 | BS EN ISO 9237 | BS EN ISO 5077 | BS EN ISO 13934-1 | BS EN ISO 13937-3 | BS EN ISO 12945-1 |
|  | ±10g/m2 | min | Min | nominal | N min | N min | Rating min |
| OMoP | 185g/m2 | No breakdown after 45,000 cycles | 200 | +1% to -4% | Warp – 1250 Weft – 750 | Warp – 28 Weft – 20 | 3 |
| TMoP | Same as OMoP | No breakdown after 30,000 cycles | 100 | Same as OMoP | Warp – 640 Weft – 460 | Warp – 25 Weft – 20 | Same as OMoP |
| - Samples are tested as received unless otherwise stated - * To be determined on samples after 5 wash and dry cycles to BS EN ISO 6330 Process 6N, Drying Procedure F (high). |  |  |  |  |  |  |  |
| Proof Testing - Proof Testing is required for Tender Sample Submissions - Tested in accordance with Annex B Table 2 of the PETREL ITEAP/38452/002 - Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets. |  |  |  |  |  |  |  |
| Batch Testing - Batch testing will be required for production quantities for the PETREL contract SSP/00192 - Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. - Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Table 2 of the PETREL ITEAP/38452/002 version correct at contract let. - If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested - If either re-test sample does not meet a requirements, the whole batch is rejected - If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken |  |  |  |  |  |  |  |

| Table 18 – Physical Characteristics of Knitted Fabrics |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SAT | 12 | 13 | 15 | 18 | 19 | 21 | 22 | 23 | 24 |  | 25 |
| Test | Mass | Air permeability Mm/sec | Dimensional stability After washing * % max Warp and Weft | Burst strength Diameter of Specimen 30.5mm | Pilling After 36,000 cycles | Snagging After 400 revolutions | Solar Protection | Surface Resistivity Measured on face or back | Charge Decay |  | Absorption |
|  |  |  |  |  |  |  |  |  | Shielding Factor | Half Decay time t50 |  |
|  | BS EN 12127 | BS EN ISO 9237 | BS EN ISO 5077 | BS EN ISO 13938-1 | BS EN ISO 12945-1 | PD 6527 Part 19 | BS EN ISO 13758-1 | BS EN 1149-1 | BS EN 1149-3 |  | BS 3449 |
|  | ±10g/m2 | Min | nominal | kPa min | Max | Rating min | UPF min | Ω must be less than | Seconds min | % max | % min |
| OMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP | Same as TMoP |
| TMoP | 150g/m2 | 1000 | -3% | 500 | 4-5 | 4-5 | 40 | 1 x 109 Ω | 0.2 | 4 | 120 |
| - Samples are conducted as received unless otherwise stated - * To be determined on samples after 5 wash and dry cycles to BS EN ISO 6330 Process 6N, Drying Procedure F (high). |  |  |  |  |  |  |  |  |  |  |  |
| Proof Testing - Proof Testing is required for Tender Sample Submissions - Tested in accordance with Annex B Table 3 of the PETREL ITEAP/38452/002 - Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets. |  |  |  |  |  |  |  |  |  |  |  |
| Batch Testing - Batch testing will be required for production quantities for the PETREL contract SSP/00192 - Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. - Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Table 3 of the PETREL ITEAP/38452/002 version correct at contract let. - If a sample does not meet any requirement, 2 further samples from two different. |  |  |  |  |  |  |  |  |  |  |  |

- SAT- 20 Colour Fastness of Main Materials

Method of Test. BS EN ISO 105. B02; C06: C2S; E04; X12. Light; Washing; Perspiration Acid & Alkaline; Rubbing. Change 5-6; Change 4-5; Stain 4-5; Change 4-5; Stain 4-5; Change 4-5; Stain 4-5. 7.1 Testing Main Material - In accordance with BS EN ISO 105 Textiles - Tests for Colour Fastness - Part B02 Colour fastness to Artificial Light: Xenon Arc Fading Lamp - Part E04 Colour fastness to Perspiration - Part C06 Colour fastness to domestic and commercial laundering - Part X12 Colour fastness to Rubbing - As MTP has varied sizes of colours making up the pattern the following guidance is to help with sampling for colour fastness testing; For testing to BS EN ISO 105 methods where Multi-Fibre Strip is used: 3 test specimens should be cut: 1 predominantly dark brown 1 predominantly mid brown 1 predominantly dark green Assessment should be in accordance with the relevant test method For testing to BS EN ISO 105: X12 – Wet and Dry Rubbing 3 test specimens should be cut with as much of the predominant colour along the line of the rubbing finger as possible: 1 predominantly dark brown 1 predominantly mid brown 1 predominantly dark green Assessment should be in accordance with the relevant test method

7.2 Proof Testing - Proof Testing is required for Tender Sample Submissions - Tested in accordance with Annex B Paragraph 6 of the PETREL ITEAP/38452/002 - Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

7.3 Batch Testing - Batch testing will be required for production quantities for the PETREL contract SSP/00192 - Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. - Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 6 of the PETREL ITEAP/38452/002 version correct at contract let. - If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested - If either re-test sample does not meet a requirements, the whole batch is rejected - If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken.

- SAT- 25 & 26 Absorption, Evaporation and Wicking Rates of Main Materials

8.1 Testing Main Materials All main materials to be tested after 5 Wash/Dry cycles – 5M @50oC / Low Tumble Dry in accordance with BS EN 6330 Woven Materials: Absorption Rate (%); Evaporation Rate (%); Wicking Rate (mm). ≥ 50; ≥37; ≥20. Knitted Materials: Absorption Rate (%); Evaporation Rate (%); Wicking Rate (mm) Longitudinal Method Length & Width; Wicking Rate Horizontal Method Back. ≥ 120; ≥50; ≥80; ≥2. Absorption Rate to be tested in accordance with BS 3449 Evaporation Rate Specimens; - After the Absorption test to BS 3449 retain the four specimens to conduct this test.. Method; - The mass of the wet specimen as measured in the Absorption test to BS 3449 should be noted. - Place the specimens on a drying rack in the Standard temperate atmosphere for testing textiles as defined in BS EN ISO 139 i.e. 65 2% rh/20 2 o C, for 3 hours. - Re-weigh the samples as described in BS 3449 using a lightweight airtight container. Calculation and expression of results; - Calculate the evaporation rate (in %) for each test specimen using the following formula: - M1 – mass of container (in g) - M2 – mass of container plus specimen after 3 hours conditioning (in g) - M3 – mass of wet specimen after Absorption test (in g) - Evaporation Rate = M3 – (M2 – M1) x100 ### M3 - Calculate the mean percentage evaporation rate for the four specimens to the nearest 1%. Wicking – Longitudinal Method Specimens; - Prepare four specimens each in the Warp and Weft direction as described in BS 3424-18 Method 21A. - (Although this test is for coated fabrics it is appropriate to evaluate the wicking effect on the knitted cloth of this specification). Method; - Follow the procedure as detailed in BS 3424-18 Method 21A except: - Use distilled water in place of the dye liquor detailed in 3.2.6 - Evaluate the wicking effect after 5 minutes on each specimen. Calculation and expression of results; - Calculate the mean of the two highest wicking results in the Warp and Weft direction and record this as the wicking distance in millimetres (mm). Wicking – Horizontal Method Specimens; - Prepare three specimens as described in BS 4554.. Method; - Following the procedure as detailed in BS 4554 using distilled water - The specimens should be used to test the back of the material i.e the side that will be next to skin. Calculation and expression of results; - Calculate the mean of 15 measurements applying the appropriate correction for temperature as detailed in BS 4554 - Report the results to the nearest second.

8.2 Proof Testing • Proof Testing is required for Tender Sample Submissions • Tested in accordance with Annex B Paragraph 7 of the PETREL ITEAP/38452/002 • Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

8.3 Batch Testing • Batch testing will be required for production quantities for the PETREL contract SSP/00192 • Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. • Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 7 of the PETREL ITEAP/38452/002 version correct at contract let. • If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested • If either re-test sample does not meet a requirements, the whole batch is rejected • If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken

- SAT- 27 PERMETHRIN TREATMENT – QUANTITATIVE ANALYSIS

9.1 Testing Main Material Method of Test. Woolmark test TM27:2003 – Using HPLC method. As Received; After 50 wash/dry cycles to BS EN ISO 6330 process 6N, drying Procedure F (high). mg/m2; mg/m2. Min; min. 1250; 400.

9.2 Proof Testing • Proof Testing is required for Tender Sample Submissions • Tested in accordance with Annex B Paragraph 8 of the PETREL ITEAP/38452/002 • Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

9.3 Batch Testing • Batch testing will be required for production quantities for the PETREL contract SSP/00192 • Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. • Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 8 of the PETREL ITEAP/38452/002 version correct at contract let. • If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested • If either re-test sample does not meet a requirements, the whole batch is rejected • If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken

- SAT- 27 PERMETHRIN TREATMENT – BIOGENTS* CAGE TEST

10.1 Testing Main Material *This test is available through Biogents AG, Test Labor, Universitarsstrasse 31, D-93053 Regensburg, Germany. Other tests may be used provided the Tenderer proves a correlation with the Biogents test Specimens; Samples are to be laundered for 50 wash and dry cycles to BS EN ISO 6330 Process 6N, Drying Procedure F (high) before the exposure test. Method; The test is to be human exposure test using 30 female Aedes aegypti mosquitoes in a cage 41cm x 41cm x 16cm. The test subject wraps their arm in the test fabric and places their arm in the cage for 2 minutes and the number of bites recorded. Results; There should be two or less bites during the 2 minutes exposure.

10.2 Proof Testing • Proof Testing is required for Tender Sample Submissions • Tested in accordance with Annex B Paragraph 9 of the PETREL ITEAP/38452/002 • Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

10.3 Batch Testing • Batch testing will be required for production quantities for the PETREL contract SSP/00192 • Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. • Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 9 of the PETREL ITEAP/38452/002 version correct at contract let. • If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested • If either re-test sample does not meet a requirements, the whole batch is rejected • If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken

- SAT- 89 Anti-Microbial

11.1 Testing Main Material METHOD OF TEST. Non-migrating, Non-leaching anti-microbial agent compliant with ASTM E 2149-10. Using Klebsiella Pnueumoniae (American type culture collection No.4352) as the test organism. After 20 Wash-dry cycles to BS EN ISO 6330 Wash Procedure 6N Dry F at high setting. %. No less than 95.

11.2 Proof Testing • Proof Testing is required for Tender Sample Submissions • Tested in accordance with Annex B Paragraph 10 of the PETREL ITEAP/38452/002 • Test reports are to be either a UKAS accredited facility or an equivalent body signatory to the ILAC Mutual Recognition Agreement or Manufacturers Raw Technical Data Sheets.

11.3 Batch Testing • Batch testing will be required for production quantities for the PETREL contract SSP/00192 • Main Material – test every batch. Maximum batch size is 5000m. Only the first batch is required to be tested. • Test reports must be from a facility accredited by UKAS or an equivalent body signatory to the ILAC Mutual Recognition Agreement and in accordance with Annex B Paragraph 10 of the PETREL ITEAP/38452/002 version correct at contract let. • If a sample does not meet any requirement, 2 further samples from two different pieces in the same batch are to be tested • If either re-test sample does not meet a requirements, the whole batch is rejected • If both re-test samples meet all the requirements, the batch can be accepted for the piece from which the failed sample was taken

This area has been determined as an estimate based on the area of a cylinder of circumference 400mm and height of 35mm; however protection can be offered in any area above 65mm it is not restricted to a uniform height.

30 degree steps of azimuth, and 0, -20 and 20 degrees elevation; all with 5mm resolution in each of the body areas.