Date:

Name of Company

Address

Address

Address

Post Code

Dear Sir

INVITATION TO TENDER FOR THE SUPPLY AND INSTALLATION OF PLAY AREA EQUIPMENT

Newport Pagnell Town Council is taking over ownership and management of play areas in Newport Pagnell from Milton Keynes Council in July 2019.  A number of these are overdue for refurbishment.  We invite your company to tender for the supply and installation of play equipment and associated leisure furniture as below.

LOCATION:  		Chicheley Play Area, Chicheley Road, Newport Pagnell

OS REFERENCE:		52.084693, -0.709917

TIMELINE:

Deadline for bids to be received by the Town Council by: 6th August

Formal notification of acceptance of bid: 7th August

Works to commence either:  September 2019 or November/December 2019. Prices for each start date required.

The Play Area in question is to be split into four areas of interest; a junior area, which must be enclosed due to proximity to the road and a culvert running through the park, a senior area, which is to contain the climbing equipment and outdoor exercise equipment, a separate area to incorporate a MUGA and finally a picnic area.  Equipment to be steel framed rather than wooden.  Wetpour surfacing to link, so that a continuous route can be undertaken by visitors to the park.

SPECIFICATION:

JUNIOR AREA – toddler to age 6:

This area to be completely enclosed by an ‘anti-trap’ black bow top fencing at 1.2m height, appropriate to the age group 1 – 6, to completely encircle the Junior Play Area, and to include two suitable gates, unable to be left opened by children of this age range.

Supply and installation of one new Toddler climber and slide, similar to Komplan Toddler’s Castle, Model No MSC5417 with platform steps as opposed to ladder steps.

Refurbishment and repositioning of existing baby springer, to across the culvert within the Junior Play Area on Wetpour surfacing.  Existing surfacing where springer is taken out to be removed and to be flattened, or built up as necessary and laid to turf.

Refurbishment and moving existing rocket ship equipment to opposite side of culvert to be included in fenced area, to be completed with a new Wetpour surface.  Existing surfacing where rocket ship is taken out to be removed and to be flattened, or built up as necessary and laid to turf.

Refurbishment and moving of existing car, to be included in Junior Play fenced area, to be completed with a new Wetpour surface. Existing surface where car is removed to be flattened or built up as necessary and laid to turf.

Removal of existing swings on far side (Keynes Close side) of park.  Existing surfacing in the area from which swings are removed to be removed, then area to be tarmacked, for picnic area.

Refurbishment and replacement of the above existing swings, for reinstallation in the Junior Area.  New swing seating to be provided, including two flat seats of varying heights and two buckets seats. Suitable for ages 1-6.

Refurbishment and moving of the existing junior slide and climbing frame to be placed inside the Junior Play Area with Wetpour surfacing.  Removal of existing surfacing in the area from which the junior slide and climbing frame is removed, then area to be flattened or build up as necessary and laid to turf.

Removal and refurbishment of existing climbing blocks, moving to Junior Play area on Wetpour base.  Removal of existing surfacing, and area flattened, or built up as necessary and laid to turf.

Suitable Wetpour surfacing for all equipment to a BS EN standard.  Please identify depth for each piece of equipment supplied in your tender.

Removal of existing gravel path leading over culvert.  Area flattened or built up as necessary, and laid to turf.

Refurbishment, moving and replacement of two seats in the junior area.  Backs of seats to culvert, inside fenced area.

One waste bin provided near to gate, similar to Slingsby Hooded Top outdoor bins 75L.

SENIOR AREA:

Supply and installation of new climbing equipment with a steel frame, suitable for ages 8 – 15, similar to but not necessarily identical to the Gaheris Modular Play System offered by Wicksteed.  Slide to be included within structure and must be at least 2.0m from the ground at its highest standing platform height.

Wetpour surfacing to slide to be provided which must meet BS EN standards including catering for a fall from the height of the structure.  Please identify depth and colour in your tender.

Supply and installation of an inclusive basket swing, minimum diameter of swing to be 1.2 m.  Minimum height of swing seat from ground 70 cm. Wetpour surfacing.

Supply and installation of outdoor fitness equipment similar to but necessarily the exact model Sutcliffe Playzone hand over hand monkey bars. Model No. PZW 580. Wetpour surfacing.

Removal of existing goal posts, removal of existing fixings for goal posts, and relaying of area to turf if necessary.

MUGA:

Supply and installation of a 35 X 17m MUGA with steel end walls and rebound fencing.

Fencing must be acoustic inhibiting, and must be 3.8 m all around the MUGA. At least two chicane entrances to be provided, with one on long side of MUGA leading to Tarmac path as shown on diagrams.

Tarmac surfacing to BS EN standard to be included, suitable for ages 8 - adult, similar to but not necessarily identical to the Standard/Classic MUGA provided by Playdale Playgrounds.

MUGA should allow for basketball play with suitable baskets as well as having two goal mouths.  A larger (36 x 18m) MUGA may be quoted for, if this fits within the design in the area specified,

PICNIC AREA:

Supply and installation of two units of communal seating/tables furniture provision which can seat a minimum of 6 people each and are suitable for inclusive (wheelchair) use.  Wooden provision is acceptable for seating/tables only, although steel is preferred.  Seating base to be tarmac to join new path

One waste bin provided between picnic tables similar to Slingsby Hooded Top outdoor bins 75L.

Existing dog bin provision on site to be retained. One on each side of existing path. Reposition if necessary.

Refurbishment of existing bench seat near picnic area, install tarmac hard standing.

PATHS:

Three Tarmac paths, leading from the existing path, one to junior play area, one to picnic area and one to MUGA to be provided.  Paths to be DDA compliant.

Install a new tarmac path from the existing tarmac path to junior area

Install a new tarmac path from the existing tarmac path to the new picnic area.

Install a new tarmac path from the existing tarmac path to the MUGA.

In addition to these three paths, either additional Wetpour or a further Tarmac path to link the senior play area to the MUGA.  Specify which in your quotation.

All Wetpour to be provided in circular patterns with varying colours as per attached diagrams.

REQUIREMENTS:

The Contractor shall fully complete the Form of Tender, together with a Pricing Schedule (Bill of Quantities) itemising all items, rates and areas including the costs of every item, surfacing, and other costs. This document should also clarify the specification of each individual item.  Preliminary costs must be shown separately. A plan of the site and of the 4 areas shall be provided by the Contractor as part of the tender submission.

Public indemnity of £10,000,000.00

Full supply and fix contract

Full responsibility for ensuring all Health and Safety requirements are in place for

duration of installation and up to hand over following snagging.

All installations, materials and ground/play surfacing to conform to relevant

BS EN regulations.

Price quoted on tender to be fixed for 60 days after written acceptance of quote

Agreement to participate in publicity if requested.

Disclosure of last accounting records and disclosure of any legal proceedings taken

against your company.

To accommodate a ‘snagging period’ of at least two weeks prior to completion, for any defects to be rectified.

A works contract is attached.

The bid to be accompanied by a to-scale drawing of the proposal.

Should trees, or branches need removal within your quotation, please specify this and indicate pricing associated with this.

Pricing is to identify each piece of equipment individually and each area of surfacing individually.

Bidders are to indicate whether there is a discount for completed delivery in November/December 2019 versus delivery in September 2019.

Contractors to indicate proposed timetable for completion with bids.

The evaluation criteria for this tender is attached.

If you haven’t already done so, all interested parties should visit the area to get an idea of scale and work required for this project.

Please send your completed bid electronically to Shar Roselman, Town Council Clerk at townclerk@newport-pagnell.org.uk

Should you require any further information about this project, please do not hesitate to contact Shar Roselman, on townclerk@newport-pagnell.org.uk or 01908 618756.