| FURNITURE CATEGORY MANAGEMENT TEAM (FCMT) TECHNICAL SPECIFICATION RESIDENTIAL FURNITURE RANGE consisting of Service Families Accommodation (SFA) # Specification Numbers 1000 |  |  |
| --- | --- | --- |
| FCMT Poplar Level 1 # 2119 Abbey Wood North Bristol BS34 8QJ Commencement Date: 2020 Issue No 1 |  |  |
| # Amendments to Specification |  |  |
| # Issue No | # Clause | # Amendment |

# Service Families Accommodation (SFA) Range (Specification 1000 Series)

| Part A | Introduction and Scope |
| --- | --- |
| Part B | Generic Technical Information |
| Part C | Product Specification |

Part A - Introduction and Scope

- This specification sets out the Customer’s requirements for a range of Service Families Accommodation (SFA) furniture as follows:

| PRODUCT NO | PRODUCT DESCRIPTION | IMAGE & DRAWING NO | NATO STOCK NO |
| --- | --- | --- | --- |
| 1001 | Sideboard Unit (Large) Oak Veneer | 1001 + 1169 | 7105 99 5932920 |
| 1001a | Sideboard Unit (Large) Oak MFC | 1169a | 7105 99 2979499 |
| 1002 | Sideboard Unit (Small 3 Drawer) Oak Veneer | 1002 + 1178 | 7105 99 1334025 |
| 1002a | Sideboard Unit (Small, 3 Drawer) Oak MFC | 1178a | 7105 99 9588716 |
| 1003 | Bookcase (Large) Oak Veneer | 1003 + 1170 | 7105 99 3354263 |
| 1003a | Bookcase (Large) Oak MFC | 1170a | 7105 99 3431990 |
| 1004 | Bookcase (Small) Oak Veneer | 1004 + 1177 | 7105 99 7482784 |
| 1004a | Bookcase (Small) Oak MFC | 1177a | 7105 99 7482784 |
| 1005 | Dining Table (Large) Oak Veneer | 1005 + 1171 | 7105 99 1767832 |
| 1005a | Dining Table (Large) Oak MFC | See Clause 5 | 7105 99 5110053 |
| 1006 | Dining Table (Small) Oak Veneer | 1172 | 7105 99 8525865 |
| 1006a | Dining Table (Small) Oak MFC | See Clause 5 | 7105 99 3321390 |
| 1007 | Dining Chair (Velour Fabric) | 1007 + 1173 | 7105 99 6663661 |
| 1008 | Dining Chair (Vinyl Fabric) | 1008 + 1173a | 7105 99 1271881 |
| 1009 | Nest of Tables (3) Oak Veneer | 1009 + 1174 | 7105 99 3761813 |
| 1009a | Nest of Tables (3) Oak MFC | 1174a | 7105 99 1537337 |
| 1011 | Occasional Table Oak Veneer | 1011 + 1176 | 7105 99 7309281 |
| 1011a | Occasional Table Oak MFC | 1176a | 7105 99 3057292 |
| 1012 | Dressing Table/Desk Oak Veneer | 1012 + 1179 | 7105 99 3578491 |
| 1012a | Dressing Table/Desk Oak MFC | 1179a | 7105 99 1537339 |
| 1013 | Bedside Cabinet/Side Table Oak Veneer | 1013 + 1180 | 7105 99 6089972 |
| 1013a | Bedside Cabinet/Side Table Oak MFC | 1180a | 7105 99 2755543 |
| 1014 | Chest of Drawers (3 Drawer) Oak Veneer | 1014 + 1181 | 7105 99 3405124 |
| 1014a | Chest of Drawers (3 Drawer) Oak MFC | 1181a | 7105 99 5539341 |
| 1016 | Dressing Stool (Velour Fabric) | 1016 + 1183 | 7105 99 2640497 |
| 1017 | Free Standing Mirror c/w base | 1017 + 1184 | 7105 99 9112158 |
| 1018 | Wardrobe (Double) Oak Veneer | 1018 + 1185 | 7105 99 4002071 |
| 1018a | Wardrobe (Double) Oak MFC | 1185a | 7105 99 1537351 |
| 1019 | Wardrobe (Single) Oak Veneer | 1019 + 1186 | 7105 99 6663670 |
| 1019a | Wardrobe (Single) Oak MFC | 1186a | 7105 99 6168385 |
| 1020 | Wardrobe (Double, Self-Assembly) Oak Veneer | 1020 + 1187 | 7105 99 6021615 |
| 1020a | Wardrobe (Double, Self-Assembly) Oak MFC | 1187a | 7105 99 7175917 |
| 1021 | Wardrobe (Single, Self-Assembly) Oak Veneer | 1021 + 1188 | 7105 99 9580530 |
| 1021a | Wardrobe (Single, Self Assembly) Oak MFC | 1188a | 7105 99 1537353 |
| 1022 | Bed Headboard Double Oak Veneer | 1022 + 1189 | 7105 99 3323023 |
| 1022a | Bed Headboard Double Oak MFC | 1022 + 1189a | 7105 99 1537353 |
| 1023 | Bed Headboard Single Oak Veneer | 1023 + 1190 | 7105 99 2127773 |
| 1023a | Bed Headboard Single Oak MFC | 1023 + 1190a | 7105 99 7027136 |
| 1024 | Bed Base Double Divan | Trade Pattern | 7105 99 1578893 |
| 1026 | Bed Base Single Divan | Trade Pattern | 7105 99 7433096 |
| 1028 | Bunk Bed (Children only) | Trade Pattern | 7105 99 9859154 |
| 1029 | Armchair (use with Covers) | 1026 | 7105 99 7338034 |
| 1030 | Settee (2 Seat, use with Covers) | 1027 | 7105 99 3957903 |
| 1031 | Settee (3 Seat, use with Covers) | 1028 | 7105 99 1293741 |
| 1032 | Replacement Seat Cushion | 1026 | 7105 99 6157621 |
| 1033 | Replacement Back Cushion | 1026 | 7105 99 2223270 |
| 1034 | Replacement Back Cushion | 1027 | 7105 99 2705411 |
| 1035 | Replacement Back Cushion | 1028 | 7105 99 1093032 |
| 1036 | Armchair (Velour Fabric) | Trade Pattern | 7105 99 3124224 |
| 1037 | Settee (2 Seat, Velour Fabric) | Trade Pattern | 7105 99 1317944 |
| 1038 | Settee (3 Seat, Velour Fabric) | Trade Pattern | 7105 99 3211197 |
| 1039 | Foot Stool (Velour Fabric) | Trade Pattern | 7105 99 8404653 |
| 1043 | Castor Cup (Pack of 4) | Trade Pattern | 5340 99 9678786 |
| 1044 | Cot (Drop Side in Beech) | Trade Pattern | 7105 99 3182001 |
| 1045 | Cot Mattress | Trade Pattern | 7210 99 8030292 |
| 1046 | High Chair (c/w Harness) | Trade Pattern | 7105 15 1522421 |
| 1047 | Replacement Harness (High Chair) | Trade Pattern | 7105 99 1301464 |
| 1048 | Play Pen Folding (c/w mat) | Trade Pattern | 7105 99 6797717 |
| 1049 | Replacement Mat (for Play Pen) | Trade Pattern | 7105 99 5623030 |
| 1050 | Stair & Door Gate | Trade Pattern | 7290 99 1665696 |
| 1051 | Stair & Door Gate Extension | Trade Pattern | 7290 99 3925341 |
| 1052 | Wall Cups (Pack of 4) | Trade Pattern | 5340 99 2429992 |
| 1053 | Stair & Door Gate (Newel/Wall fix) | Trade Pattern | 7105 99 2127778 |
| 1054 | Step Stool (Folding) | Trade Pattern | 7105 99 8739181 |
| 1055 | Bathroom Cabinet (Mirrored) | 1055 + 1100 | 7125 99 8600175 |
| 1056 | Bathroom Stool | Trade Pattern | 7105 99 2122644 |
| 1057 | Linen Container | Trade Pattern | 7125 99 5624979 |
| Not Used | 1010, 1015, 1025, 1040, 1041 & 1042 |  |  |

- The Customer’s previous specifications for these products are superseded.

- The range includes Bespoke items (manufactured in line with the Customer’s drawings and constructional details) and Trade Pattern items manufactured to the provider’s own design but conforming to the performance levels stated in this specification. In reference to MFC drawings ending with letter (a) e.g. 1169a were it states 25mm MFC board to be replaced by a 18mm MFC board.

- Bespoke aspects such as dimensions, appearance and construction (usually detailed on the drawing) are provided and should be closely followed provided it is feasible and practical to do so; performance standards need to be met.

- The construction of the MFC (or similar finished) items shall be panel based, with the exception of items 1005a and 1006a Dining Tables where a solid timber under-frame is required, the overall appearance of the finished items shall resemble those in oak solid/veneer. Performance (strength and durability) shall be the same as that required of the oak solid/veneer items.

- Any mention of fittings by manufacturer’s reference number or name implies a type and/or performance level only; alternatives may be offered. It is the responsibility of the furniture Provider to ensure that products supplied comply with this specification.

- The appearance/styling of the Trade Pattern furniture shall be traditional in design and construction; if dimensions are stated the tolerance is +/- 5mm.

- Knock Down (KD) also known as Flat-Pack or Self-Assembly products shall be supplied in KD form with a component list and comprehensive assembly instructions (illustrated if possible).

- All storage furniture over 600 mm in height (e.g. wardrobes, filing units etc) shall be supplied with a wall tethering device.

- Product Samples

  - After contract award product samples may be requested by the Customer for evaluation against the technical specification and drawings (if applicable). The following products represent the SFA range, evaluations may be made against some or all of them:

| 1002 | Sideboard Unit (Small) |
| --- | --- |
| 1002a | Sideboard Unit (Small) MFC version |
| 1005 | Table Dining (Large) |
| 1005a | Table Dining (Large) MFC version |
| 1008 | Chair Dining (Vinyl Fabric) |
| 1018 | Wardrobe Double |
| 1018a | Wardrobe Double MFC version |
| 1020 | Wardrobe Double (Self Assembly) |
| 1020a | Wardrobe Double (Self Assembly) MFC version |
| 1022 | Bed Headboard Double |
| 1022a | Bed Headboard Double MFC version |
| 1024 | Bed Base Double Divan |
| 1028 | Bunk Bed (Children only) |
| 1031 | Settee (3 Seat, use with covers) |
| 1038 | Settee (3 Seat, Velour Fabric) |
| 1039 | Foot Stool (Velour Fabric) |
| 1044 | Cot (Drop Side in Beech |
| 1045 | Cot Mattress |
| 1046 | High Chair (c/w Harness) |
| 1048 | Play Pen Folding (c/w mat) |
| 1050 | Stair & Door Gate |
| 1051 | Stair & Door Gate Extension |
| 1053 | Stair & Door Gate (Newel/Wall fix) |
| 1054 | Step Stool (Folding) |
| 1057 | Linen Container (Large) |

  - Cost of samples shall be borne by the provider; these may be from first production.

  - Approved samples are to be retained for the contract period and will be the standard manufacture will adhere to.

  - At the end of the contract the approved samples can be included in the final orders subject to it being in a new, unused condition.

Part B - Generic Technical Information

- Timber Products

    - All items of furniture shall be of sound construction, joints neatly and accurately cut, fully closed and fixed with component parts machined square, flat and true.

    - Timber products are made from materials such as boards, veneers and solid timber.

    - All wood particleboard used in the construction of these items shall comply with the requirements specified in ‘BS EN 312:2010 Particleboards Specifications Type P2 - Requirements for boards for interior fitments (including furniture) for use in dry conditions.  Additionally, particleboard shall meet requirements for Class E1 formaldehyde release as specified in this standard.

    - Any MDF used in the construction of these items shall comply with the Type MDF LA requirements specified in BS EN 622-5:2009 ‘Fibreboards Specifications Part 5 - Requirements for dry process boards (MDF)’. Additionally, MDF shall meet requirements for Class E1 formaldehyde release as specified in BS EN 622-1:2003 Fibreboards Specifications Part 1 General Requirements.

    - Cabinets, Cupboards, Chairs, Stools and Tables shall stand square; doors and drawers fitted correctly with accurate and parallel spacing.

    - In so far as the Bespoke range is concerned, the solid lipping's and veneered surface top shall be of a matching timber.

    - The underside of all panels shall be veneered with material of equal strength to the top surface; all surfaces shall be finished with a clear lacquer to give a natural appearance.

    - Timber drawers (MDF or Plywood) shall have sufficient coats of lacquer, sanded between each application, to produce a smooth, “snag-free” finish to all surfaces.

    - “Rift” cut veneers, 0.6mm thick (min), shall either be book matched or slip matched to give a visually balanced effect with all veneers trimmed to exclude sap.

    - Surface finishes on wood; wood-based sheet materials or other substrate, plastic laminates, paper and PVC foil shall satisfy the ‘Severe’ performance requirements of FIRA Standard 6250: 2018, Table 2 Finish performance.

    - The adhesion of surface and edging materials shall satisfy the requirements of FIRA standard 6250: 2018, Table 3 – Adhesion Performance (General).

  - Metal Products

    - Metal items manufactured from sheet steel, tube or square section shall be sound and fit for purpose with all folds and welds neatly and correctly executed.

    - Welds shall be free from distortion, oxidation arcing, splash, undercutting, blowholes, excessive build-up or any other defects.

    - Metalwork shall be completely free from rust/scale, properly degreased and phosphated prior to application of the final finish (stove enamel or epoxy-powder).

    - The surface finish shall be non-reflective, flat, smooth and free from imperfections with high durability and resistance to impact and abrasion.

  - Upholstery

    - Foam shall comply with BS 3379:2005 + A1:2011, for classification refer to individual products.

    - Upholstered furniture shall be fire retardant in accordance with BS 7176:2007 + A1:2011; for protection levels refer to individual products.

Note: Foam must be combustion modified as defined in schedule 1 to statutory instrument:1988 No. 1324, consumer protection. The Furniture and Furnishing (Fire)(Safety) regulations 1988 (as amended in 1989,1993 and 2010).

    - The upholstery shall be accurately and neatly fitted, staples fully driven home, spaced evenly and to the highest commercial standards.  All products shall be free from loose ends of sewing, protrusions of material or fastenings which may cause injury or discomfort. The underside of each chair/seat platform shall be covered in a black scrim.

    - Products designed and specified for use with stretch or tailored loose covers are be covered with a Fire Retardant “Schedule 3” Interliner.

11.3. 6        Detailed below is the performance criteria for upholstery fabrics:

| Upholstery Fabric Type - Velour |  |  |  |
| --- | --- | --- | --- |
| Table A | Test | Standard | Performance |
|  | Fabric Classification | BS 2543:2004 | Severe Contract |

| Upholstery Fabric Type - Interliner |  |  |  |
| --- | --- | --- | --- |
| Table B | Test | Standard | Performance |
|  | Flammability (preferably inherently Fire Retardant | The interliner shall conform to Schedule 3 of the Furniture (Fire) and (Safety) Regulations 1988 (1989) (1993). BS 7176:2007. Medium Hazard. |  |

| Upholstery Fabric – Vinyl |  |  |  |
| --- | --- | --- | --- |
| Table C | Test | Standard | Performance |
|  | Flammability (preferably inherently Fire Retardant | BS 7176:2007 + A1:2011 | Medium Hazard |

11.3.7         See clause 1058 for fabric colour options

- Packaging, Marking and Pallets

  - All items of furniture shall be packaged to commercial standards.

    - Each pack shall be labelled with the following:

Manufacturers/Suppliers Name

Order Number

Date of Manufacture or Supply

Product Code

NATO Stock No.

Contract Number

Package Quantity

Weight of Package (kg)

Any statutory hazard markings and any handling markings

Chain of Custody number/details (if applicable)

  - Marking of Products

    - Marking of items is as follows.

Manufacturers/Suppliers Name

Product Code

NATO Stock No

Contract Number

Date of Manufacture or Supply

Guarantee Period

Batch Number

Chain of Custody number/details (if applicable)

Part C- Product Specification

| 1001 | Sideboard Unit Oak Veneer (Large) | Drawing No 1169 | NSN: 7105 99 5932920 |
| --- | --- | --- | --- |

| 1001a | Sideboard Unit Oak MFC (Large) | Drawing No 1169a | NSN: 7105 99 2979499 |
| --- | --- | --- | --- |

  - For specification details refer to Drawing No 1169 and 1169a

| 1002 | Sideboard Unit Oak Veneer (Small) | Drawing No 1178 | NSN: 7105 99 1334025 |
| --- | --- | --- | --- |
| 1002a | Sideboard Unit Oak MFC (Small) | Drawing No 1178a | NSN: 7105 99 9588716 |

  - For specification details refer to Drawing No 1178 and 1178a.  Note: 1002 this is now a three drawer unit not a four drawer as shown on the drawing; the three drawers to be of equal size/depth.

| 1003 | Bookcase Oak Veneer (Large) | Drawing No 1170 | NSN: 7105 99 3354263 |
| --- | --- | --- | --- |
| 1003a | Bookcase Oak MFC (Large) | Drawing No 1170a | NSN: 7105 99 3431990 |

  - For specification details refer to Drawing No 1170 and 1170a

| 1004 | Bookcase Oak Veneer (Small) | Drawing No 1177 | NSN: 7105 99 7482784 |
| --- | --- | --- | --- |
| 1004a | Bookcase Oak MFC (Small) | Drawing No 1177a | NSN: 7105 99 9688837 |

  - For specification details refer to Drawing No 1177 and 1177a

| 1005 | Dining Table (Large, Self-Assembly) Oak Veneer (Closed Length 1700 Extended 2200mm) | Drawing No 1171 | NSN: 7105 99 1767832 |
| --- | --- | --- | --- |
| 1005a | Dining Table (Large, Self-Assembly) Oak MFC (Closed Length 1700 Extended 2200mm) | Drawing No 1171a | NSN: 7105 99 1767832 |

  - For specification details refer to Drawing No 1171 and see Clause 5 for 1005a

| 1006 | Dining Table (Small, Self-Assembly) Oak Veneer (Closed Length 1200, Extended 1700mm) | Drawing No 1172 | NSN: 7105 99 8525865 |
| --- | --- | --- | --- |
| 1006a | Dining Table (Small, Self-Assembly) Oak MFC (Closed Length 1200, Extended 1700mm) | Drawing No 1172a | NSN: 7105 99 332190 |

  - For specification details refer to Drawing No 1172 and see Clause 5 for 1006a

| 1007 | Dining Chair (Velour Fabric Upholstery) | Drawing No 1173 | NSN: 7105 99 6663661 |
| --- | --- | --- | --- |

  - For specification details refer to Drawing No 1173

  - Chair to be upholstered to fabric conforming to Table A in a minimum of three colour-ways, the Demanding Customer for each order shall select the fabric colour. (See clause 2454 for fabric colour options).

  - Upholstery composites shall conform to BS 7176:2007 + A1:2011 (Medium Hazard).

| 1008 | Dining Chair (Vinyl Fabric) | Drawing No 1173a | NSN 7105 99 1271881 |
| --- | --- | --- | --- |

1008.1       For specification details refer to Drawing No 1173.

1008.2       Chair to be upholstered in fabric conforming to Table C in a minimum of three colour-ways, the Demanding Customer for each order shall select the fabric colour. (See clause 2454 for fabric colour options).

1008.3       Upholstery composites shall conform to BS 7176:2007 + A1:2011 (Medium Hazard). Antimicrobial and anti-fungal.

| 1009 | Nest of Tables Oak Veneer (Qty 3) | Drawing No 1174 | NSN: 7105 99 3761813 |
| --- | --- | --- | --- |
| 1009a | Nest of Tables Oak MFC (Qty 3) | Drawing No 1174a | NSN: 7105 1537337 |

  - For specification details refer to Drawing No 1174 and 1174a

| 1011 | Occasional Table Oak Veneer | Drawing No 1176 | NSN: 7105 99 7309281 |
| --- | --- | --- | --- |
| 1011a | Occasional Table Oak MFC | Drawing No 1176a | NSN: 7105 99 3057292 |

1011.1	For specification details refer to Drawing No 1176 and 1176a

| 1012 | Dressing Table/Desk Oak Veneer (complete with 3 Drawer Pedestal) | Drawing No 1179 | NSN: 7105 99 3578491 |
| --- | --- | --- | --- |
| 1012a | Dressing Table/Desk Oak MFC (complete with 3 Drawer Pedestal) | Drawing No 1179a | NSN: 7105 99 1537339 |

  - For specification details refer to Drawing No 1179 and 1179a

| 1013 | Bedside Cabinet/Side Table Oak Veneer (Single Drawer) | Drawing No 1180 | NSN: 7105 99 6089972 |
| --- | --- | --- | --- |
| 1013a | Bedside Cabinet/Side Table Oak MFC (Single Drawer) | Drawing No 1180a | NSN: 7105 99 2755543 |

  - For specification details refer to Drawing No 1180 and 1180a

| 1014 | Chest of Drawers Oak Veneer (3 Drawer) | Drawing No 1181 | NSN: 7105 99 3405124 |
| --- | --- | --- | --- |
| 1014a | Chest of Drawers Oak MFC (3 Drawer) | Drawing No 1181a | NSN: 7105 99 3405124 |

  - For specification details refer to Drawing No 1181and 1181a.  Note: 1014 this is now a three-drawer unit not a four drawer as shown on the drawing; the three drawers to be of equal size/depth.

| 1016 | Dressing Stool (Velour Fabric Upholstery) | Drawing No 1183 | NSN: 7105 99 2640497 |
| --- | --- | --- | --- |

  - For specification details please refer to Drawing No 1183.

  - Stool to be upholstered to fabric conforming to Table A in a minimum of three colourways, the Demanding Customer for each order shall select the fabric colour. (See clause 2454 for fabric colour options).

  - Upholstery composites shall conform to BS 7176:2007 + A1:2011 (Medium Hazard).

| 1017 | Free Standing Mirror Oak (complete with base) | Drawing No 1184 | NSN: 7105 99 9112158 |
| --- | --- | --- | --- |

  - For specification details please refer to Drawing No 1184.

| 1018 | Wardrobe, Double Oak Veneer (Fully Assembled) | Drawing No 1185 | NSN: 7105 99 4002071 |
| --- | --- | --- | --- |
| 1018a | Wardrobe, Double Oak MFC (Fully Assembled) | Drawing No 1185a | NSN: 7105 99 1537351 |

  - For specification details please refer to Drawing No 1185 and 1185a

  - The shelf shall have the capability of being removed so that the hanging rail can be secured at a higher position to allow the hanging of long garments.

  - The 18mm back panel may be replaced by a light/medium oak foil (inner surface only) high density fibre board; strength & rigidity of the wardrobe must not be compromised.

| 1019 | Wardrobe, Single Oak Veneer (Fully Assembled) | Drawing No 1186 | NSN: 7105 99 6663670 |
| --- | --- | --- | --- |
| 1019a | Wardrobe, Single Oak MFC (Fully Assembled) | Drawing No 1186a | NSN: 7105 99 6663670 |

  - For specification details please refer to Drawing No 1186 and 1186a

  - The shelf shall have the capability of being removed so that the hanging rail can be secured at a higher position to allow the hanging of long garments.

  - The 18mm back panel may be replaced by a light/medium oak foil (inner surface only) high density fibre board; strength & rigidity of the wardrobe must not be compromised.

| 1020 | Wardrobe, Double Oak Veneer (Self-Assembly) | Drawing No 1187 | NSN: 7105 99 6021615 |
| --- | --- | --- | --- |
| 1020a | Wardrobe, Double Oak MFC (Self-Assembly) | Drawing No 1187a | NSN: 7105 99 7175917 |

  - For specification details please refer to Drawing No 1187.and 1187a

  - A component list and assembly instructions, illustrated if necessary, shall be provided.

| 1021 | Wardrobe, Single Oak Veneer (Self-Assembly) | Drawing No 1188 | NSN: 7105 99 9580530 |
| --- | --- | --- | --- |
| 1021a | Wardrobe, Single Oak MFC (Self-Assembly) | Drawing No 1188a | NSN: 7105 99 1537353 |

  - For specification details please refer to Drawing No 1188 and 1188a

  - A component list and assembly instructions, illustrated if necessary, shall be provided.

| 1022 | Bed Headboard Double Oak Veneer (for use on Bed Base Divan NSN 7105 99 1578893) | Drawing No 1189 | NSN: 7105 99 3323023 |
| --- | --- | --- | --- |
| 1022a | Bed Headboard Double Oak MFC (for use on Bed Base Divan NSN 7105 99 1578893) | Drawing No 1189a | NSN: 7105 99 2904828 |

  - For specification details please refer to Drawing No 1189 and 1189a

| 1023 | Bed Headboard Single (for use on Bed Base Divan NSN 7105 99 7433096) | Drawing No 1190 | NSN: 7105 99 2127773 |
| --- | --- | --- | --- |

  - For specification details please refer to Drawing No 1190.

| 1024 | Bed Base Double Divan (2 Parts) (1375mm Wide) | Trade Pattern | NSN: 7105 99 1578893 |
| --- | --- | --- | --- |

  - The dimensions for the contract quality divan base shall be L1900 x W1375 x H300mm.

  - The bed base shall be a two-part upholstered, solid (cardboard re-enforced sides, platform top) un-sprung divan; four castors shall support each half of the bed.

  - Brackets and fastening plates complete with wing nuts shall be provided to securely lock the two halves together; “C” shaped clamps are not acceptable.

  - Standard headboard mountings and fixings, quantity 4, shall be provided to accommodate the headboard as specified at product 1022.

  - Damask or woven ticking shall be in accordance with Table 2 & 3 of BS EN 14976:2005.

  - The bed base shall comply with:

- BS 7177:2008 + A1:2011 (Medium Hazard)

- BS EN 1725: 1998

| 1026 | Bed Base Single Divan (915mm Wide) | Trade Pattern | NSN: 7105 99 7433096 |
| --- | --- | --- | --- |

  - The dimensions for the divan base shall be L1900 x W915 x H300mm.

  - Standard headboard mountings and fixings, quantity 4, shall be provided to accommodate the headboards as specified at product 1023 and 2331 (SLA range).

  - All other aspects shall be as item 1024.

| 1028 | Bunk Bed (Self-Assembly, children only) | Trade Pattern | NSN: 7105 99 9859154 |
| --- | --- | --- | --- |

  - Complying with the current European standard, and the General Product Safety Regulations, the domestic quality Bunk Bed shall be sufficiently robust to withstand repeated assembly, disassembly and re-issue (a minimum of 6 times).

  - The bunk bed shall be suitable for use with standard single (1900mm x 900mm x 100mm) foam interior mattresses (not supplied).

  - It shall be constructed from timber (finish - clear A/C lacquer) or fabricated steel.

  - Supplied with comprehensive assembly instruction and all fittings and fixings, the bunk bed shall be in knock down format, typically consisting of:

- Bed ends

- Base & Safety rails

- Mattress supports

- Ladder

  - The bunk bed shall conform to:

- BSEN 747-1:2012 + A1:2015

| 1029 | Armchair (For use with Stretch & Loose Covers) | Drawing No 1026 | NSN: 7105 99 7338034 |
| --- | --- | --- | --- |
| 1030 | Settee, 2 Seat (For use with Stretch & Loose Covers) | Drawing No 1027 | NSN: 7105 99 3957903 |
| 1031 | Settee, 3 Seat (For use with Stretch & Loose Covers) | Drawing No 1028 | NSN: 7105 99 1293741 |

  - The overall dimensions of what are essentially bespoke contract quality armchairs and accompanying settees shall be as per drawings 1026, 1027 & 1028.

  - All seat cushions are to be foam designation CM-HB-S-170(I) (Density 35kg/m2), the back cushions are to be hollowfill fibre (6 Denier).

  - The armchair and settee carcases shall be constructed from 18mm board material (e.g. chipboard, plywood as appropriate).

  - All cross members and corner braces shall be beech or equivalent hardwood, glued and screwed using a PVA adhesive, other joints shall be fully glued and stapled.

  - Arms shall be bolted to seat and back frame using M8 bolts and T-nuts.

  - Each product shall be fitted with heavy-duty (replaceable) 50mm dia “shepherd” castors which shall be fitted to beech or equivalent hardwood blocks.

  - Each individual seat cushion shall be supported on at least 5 equally spaced interlaced heavy-duty 50mm wide elasticated webbing securely stapled to the armchair or settee base. The webbings shall be covered with 18mm foam.

  - The underside of the seat and settee bases shall be covered and neatly stapled with an appropriate quality scrim complying to BS 2543:2004 general contract or higher.

  - The back-suspension system will provide a flexible support for the back cushion and will be covered in the FR Interliner.

  - Fillings must not creak or squeak.

  - All products shall be fully upholstered in a flame retardant Interliner, in accordance with Table B; interliner can be of a neutral or dark colour.

  - The FR Interliner shall be fitted over all Combustion Modified High Resilient (CMHR) foam, all foam shall conform to BS 3379:2005.

  - Upholstery composites shall conform to BS 7176:2007 + A1:2011 (Medium Hazard).

  - Seating shall meet the performance requirements of BS EN 161393:2013 (Test Level 2).

| 1032 | Replacement Seat Cushion - Armchair (For use with Stretch & Loose Covers) | Drawing No 1026 | NSN: 7105 99 6157621 |
| --- | --- | --- | --- |
| 1033 | Replacement Back Cushion – Armchair (For use with Stretch & Loose Covers) | Drawing No 1026 | NSN: 7105 99 2223270 |
| 1034 | Replacement Back Cushion - 2 & 3 Seat (For use with Stretch & Loose Covers) | Drawing No 1027 | NSN: 7105 99 2705411 |
| 1035 | Replacement Back Cushion-Centre 3 Seat (For use with Stretch & Loose Covers) | Drawing No 1028 | NSN: 7105 99 1093032 |

  - Seat & back cushions used on items 1029,1030&1031 shall be available as spare parts.

  - All materials and performance levels shall be as stated for items 1029, 1030 and 1031.

| 1036 | Armchair (Fully-Upholstered) | Trade Pattern | NSN: 7105 99 3124224 |
| --- | --- | --- | --- |
| 1037 | Settee (2 Seat, Fully-Upholstered) | Trade Pattern | NSN: 7105 99 1317944 |
| 1038 | Settee (3 Seat, Fully-Upholstered) | Trade Pattern | NSN: 7105 99 3211197 |
| 1039 | Foot Stool (with storage, Fully-Upholstered) | Trade Pattern | NSN: TBC |

  - Overall dimensions of these Trade Pattern contract quality armchairs, settees and foot stools shall be:

| Aspect | Armchair | Settee (2 Seat) | Settee (3 Seat) | Foot Stool |
| --- | --- | --- | --- | --- |
| Width | 850mm | 1450mm | 2000mm | 600mm |
| Depth | 900mm | 900mm | 900mm | 600mm |
| Height | 800mm | 800mm | 800mm | See 1036.4 |

  - The design of each product is to be compatible with one another, be of a contemporary modern style and of straight ‘slab’ appearance rather than curved.

  - All items shall be fitted with timber bun feet.

  - The Foot Stool height shall match that of the Armchair/Settee seat height.

  - The Foot Stool shall have a hinged top with storage underneath.

  - Upholstery fabric shall be inherently flame-retardant conforming to Table A, available in three colours as selected by the Demanding Customer. (See clause 2454 for fabric colour options).

  - Foam shall conform to BS 3379:2005 + A1:2011 (Class V).

  - Upholstery composites shall conform to BS 7176:2007 + A1:2011 (Medium Hazard).

  - Seating shall meet the requirements of BS EN 16139:2013 (Test Level 2)

| 1043 | Castor Cup (Pack of 4) | Trade Pattern | NSN: 5340 99 9678786 |
| --- | --- | --- | --- |

  - The Castor Cup for use with Armchairs and Settees fitted with “Shepherd” type castors and shall be manufactured in High Density Polyethylene; colour Brown or Black.

| 1044 | Cot without Mattress | Trade Pattern | NSN: 7105 99 3182001 |
| --- | --- | --- | --- |

  - The internal dimensions of the cot shall be approximately 1200mm x 600mm.

  - The cot shall be beech framed with a beech slatted base and feature a single drop-side.  It shall be finished in a clear A/C lacquer, all surfaces left smooth and shall meet the performance requirements of clause 10.1.10.

  - The Cot shall be supplied in flat pack form complete with clear and illustrated assembly instructions and all fittings and fixings packs; excludes mattress.

  - The cot shall meet the performance requirements of:

- BSEN 716-1:2017

- BSEN 716-2:2017

| 1045 | Cot Mattress | Trade Pattern | NSN: 7210 99 8030292 |
| --- | --- | --- | --- |

  - The mattress dimensions shall be 1200mm long x 600mm wide x 100mm thick.

  - The mattress shall have a Non-allergenic Combustion Modified polyurethane foam core or polyester fibre filling.

  - Conforming to BS 3379:2005 the foam shall have the following designation “CM-HB-A-100(I)” and have a density of 28kg/m² complying with SI No. 1324 The Furniture and Furnishings (Fire) (Safety) Regulations 1988 (1989) (1993) (2010) - Schedule 5 Part 1.

  - The mattress outer cover shall be a non-woven Polypropylene, colour White, Air Permeable, Water and Stain Resistant, Wipe Clean and Washable features.

  - The mattress shall conform to BS EN 16890-2017

  - Mattress and package labelling shall be in accordance with BS EN 16890-2017 and SI No.1324 The Furniture & Furnishings (Fire)(Safety) Regulations 1988(1989) (1993) (2010).

  - Mattress shall meet the requirements of BS 7177:2008 + A1:2011 (Low Hazard).

| 1046 | High Chair (Complete with Harness) | Trade Pattern | NSN: 7105 15 1522421 |
| --- | --- | --- | --- |

  - The high chair shall be a folding, single height and tubular steel framed complying with BSEN 14988-1:2017.

  - The chair shall be supplied fully assembled, complete with a moulded plastic tray.

  - The chair shall be upholstered in a wipe-clean fabric over foam on a board substrate.

  - The chair shall be fitted with a removable crotch strap.

  - The tubular frame shall be finished in the manufacturer’s standard colour.

  - The high chair and harness shall meet the requirements of:

- BSEN 14988-1:2017;

- The Furniture and Furnishings (Fire) (Safety) Regulations 1988 (1989) (1993).

| 1047 | Replacement Harness (for High Chair) | Trade Pattern | NSN: 7105 99 1301464 |
| --- | --- | --- | --- |

  - The replacement harness assembly shall be the same type and design as the original fitted (item 1046).

  - The harness assembly shall comply with BSEN 14988: 2017

| 1048 | Play Pen Folding (complete with mat) | Trade Pattern | NSN: 7105 99 6797717 |
| --- | --- | --- | --- |

  - Conforming to BSEN 12227: 2010, the footprint of the playpen shall be approx. 900 x 900mm.

  - The playpen shall be supplied in flat pack form complete with assembly instructions.

  - The playpen shall be a 'Rigid Sided' collapsible unit.

  - The playpen shall be supplied with a replaceable wipe clean mat.

  - The mat shall consist of a Combustion Modified High Resilient (CMHR) foam core and tie tabs for securing to the corners of the playpen.

  - The playpen shall meet the performance requirements of:

- BSEN 12227:2010;

- The Furniture and Furnishings (Fire) (Safety) Regulations 1988 (1989) (1993).

| 1049 | Replacement Mat (for Play Pen) | Trade Pattern | NSN: 7105 99 5623030 |
| --- | --- | --- | --- |

  - The replacement playpen mat shall be the same type and design as the original supplied (item 1048).

| 1050 | Stair & Door Gate | Trade Pattern | NSN: 7290 99 1665696 |
| --- | --- | --- | --- |

  - Conforming to BS EN 1930:2011 the gate shall feature a hinged gate section.

  - The unit shall be constructed in steel (tubular and solid), coated in a tough, knock and scratch resistant finish.

  - In addition, the gate shall be adjustable in width between 700mm and 850mm (approx.).

  - Width adjustment shall be by means of threaded spindles (nominally M10).

  - Spindles on the hinge side of the frame shall be removable to permit the fitting of an extension piece (product 1051).

  - An installation kit shall be provided containing wall cup fastenings and fitting instructions.

  - The stair and door gate shall meet the performance requirements of BSEN 1930:2011.

| 1051 | Extension (for Stair & Door Gate) | Trade Pattern | NSN: 7290 99 3925341 |
| --- | --- | --- | --- |

  - The gate side extension shall be the matching accessory for the Stair and Door Gate and shall have a common appearance and common materials.

  - When fitted to the stair/door gate, the extension shall increase the width of the assembly by 65mm (giving a range of adjustment between 850mm & 980mm when fitted in pairs).

  - The threaded spindles from the hinge side of the stair and door gate shall fit and function on the external side of the extension.

  - Solid rods shall firmly anchor the extension into the gate spindle holes.

  - The stair and door gate shall continue to meet the performance requirements of BS EN 1930:2011 when fitted with the extension.

| 1052 | Wall Cups (Stair & Safety Gate, Pack of 4) | Trade Pattern | NSN: 5340 99 2429992 |
| --- | --- | --- | --- |

  - The wall fixing sets referred to clause 1050.9 shall be available separately.

  - Each set shall contain four plastic retaining cups, four fixing screws and four wall plugs and fitting instructions.

  - The set shall be sealed in a transparent poly bag and identified.

| 1053 | Stair & Door Gate (Secured to Staircase Newel or Wall) | Trade Pattern | NSN: 7105 99 2127778 |
| --- | --- | --- | --- |

  - The unit shall feature a hinged gate section capable of being secured to either a staircase newel or wall.

  - The unit shall be constructed in steel (tubular and solid), coated in a tough, knock and scratch resistant finish.

  - This adjustable gate shall be capable of fitting into openings measuring between 620mm and 1020mm in width in accordance with BSEN 1930:2011.

  - An installation kit shall be provided containing fastenings and comprehensive self-assembly and fitting instructions.

  - The stair and door gate shall meet the performance requirements of BSEN 1930:2011.

| 1054 | Step Stool | Trade Pattern | NSN: 7105 99 8739181 |
| --- | --- | --- | --- |

  - The step stool shall consist of a two-tread frame which folds away.

  - The top step shall also double as a seat.

  - The two-tread frame shall provide 240mm high steps with a non-slip covering; overall size 380mm x 260mm.

  - The tubular steel frame shall be capped with durable plastic inserts and Stove Enamelled or Powder Coated in the manufacturer’s standard colour.

  - The step stool shall meet the performance requirements of BS EN 14183: 2003.

| 1055 | Bathroom Cabinet (with Mirrored Door) | Drawing No 1100 | NSN: 7125 99 8600175 |
| --- | --- | --- | --- |

  - For specification details please refer to Drawing No 1100.

  - The cabinet (inside and outside) is to be finished in a white pigmented lacquer conforming to the finish requirements as detailed at clause 10.1.10.

  - The cabinet is to be supplied with a wall fixing kit.

| 1056 | Bathroom Stool | Trade Pattern | NSN: 7105 99 2122644 |
| --- | --- | --- | --- |

  - A plastic four-legged stool in a neutral colour, seat height a minimum of 300mm.

| 1057 | Linen Container | Trade Pattern | NSN: 7125 99 5624979 |
| --- | --- | --- | --- |

  - The plastic linen container shall have a removable lid and both lid and body shall be perforated to allow air circulation.

  - The height of the container shall be approximately 600mm and be of a neutral colour.

| 1058 | RM6119 MOD Residential Fabric Colour Chart |
| --- | --- |

1058.1 	Plain Fabric

Colours available in Camira fabric Xtreme or similar

Panama       Curacao      Havana

YS078	         YS005	     YS009

1058.2 	Vinyl/PVC Fabric

Colours available in Chieftain fabrics Just Colour or similar

Sapphire Blue       Cobalt               Mulled Wine

1058.3 	24/7 Range Fabric

Colours available in Camira Flax 24/7 or similar

FYR02                FYR18            FYR21

Vernal                  Eternity           While

1058.4 	Velour Fabric

Colours available in AGUA Libra collection or similar

Tan		      Claret	         Midnight

1058.5 	Windowless Accommodation

All windowless Accommodation to Crib 7 High Hazard when tested as a composite of fabric and foam and the Fire Label to note that the upholstery meets Crib 7 High Hazard

------------------------------------------------------------------PAGE 17 – END OF SPECIFICATION---------------------------------------------------------------