# Waste Management Forklift Truck and rotator forks specification

Council of the Isles of Scilly	October 2024

Specification:

- Li ION Counterbalance forklift truck

- Single speed powershift automatic transmission

- Class II Carriage

- Two LED OHG mounted lights

- Rear LED work light

- Rear view mirror

- Amber strobe light

- Rear LED combination lamp & turn signals

- Power steering

- Tiltable steering column

- Neutral shift

- Headlight guards

- Draw bar pin.

- Capacity				2500kg

- Mast					4800mm (Triplex)

- Battery 	Li-Ion 				280AH /80V

- Transmission				Powershift

- Tyres					Solid puncture proof

- Forks					1150*122*40mm

- Overhead guard			Standard

- Valves					4 way

- Hoses					4 way

- Attachment				Integral side shift

- Beacon					Amber

- Back Up alarm				Yes

- Charger					3 phase 130A

- Fully enclosed cabin: steel doors

- Heater

- Telematic data feed

- Seat Belts

- Vinyl seat

(1) One New class 2 ATIB 360° Rotator

Capacity to 2200kg@500mm

Mounting: FEM 2A

Bolt on bottom hooks.

360° continuous rotation

Hydraulic Drive 6700Nm of torque p-100 b

Castellated carriage

Clear view center

Carriage Width: 1100mm

4xfork positioning blocks.

Mass: 187Kg+ forks

Centre of Gravity: 79mm

Lost Load Thickness: 167mm

Maximum pressure 175 bar

Recommended oil flow 25Lp-min

1 hydraulic function required. (already fitted to the EP ?)

New rotator forks fitted.

Rotator Forks installed.

100x40x1200mmRF

Capacity 2500kg@500mm (2100kg@500mm inverted capacity)

Mounting Fem 2A

Load bearing top and bottom hooks.

Warranty	3 years