West Malling Parish Council

Norman Road Playing Field, Norman Road – Tennis Courts

Works Specification

The main elements of the works required will consist of the following:

Tennis Courts

- Provide a safe and practicable working environment

- Provide a suitable means of waste removal from the site; and ensure all waste is correctly disposed of

- Spike courts with 30mm holes at approximately 450mm centres and backfill with pea-shingle

- Chase a 150mm channel around present edging kerbs and feather new surface down to old kerb edge should new edging kerbs not be required

- To supply and lay a bitumen emulsion tack coat

- Hand lay and screed along 40mm steel bars 6mm open textured Supersport Tennis tarmacadam wear course to a minimum compacted depth of 30mm laid in accordance with SAPCA, The Lawn Tennis Association and BS4987.  Thoroughly roll and cross roll with suitable power roller until an even close knit surface occurs

- Apply 2 directional coats of the highest quality Tennis anti slip Acrylic Court Paint.  Colours TBC

- Mark out and paint 2 sets of tennis lines and with quartz sand slip-resistant acrylic line paint

- On completion of works all surplus arising to be removed and the site left clean and tidy

2No Sets of Tennis Equipment

- To replace 2no sets of existing posts and sockets with new nylon coated posts with all brass winding mechanism, complete with heavy duty sockets and nets

Rigid Double Wire Weldmesh Fencing System

- Supply and install 67 linear metres of Sports System, or equivalent, comprising of 80x40 RHS. Galvanised Powder Coated intermediate and corner posts (over length 3.75m) at 2.52 centres at 3m high and 34 linear metres of Sports System comprising of 80x40 RHS.  Galvanised Powder Coated intermediate and corner posts (overall length 3.75m) at 2.52 centres at 3.6m high

- All rigid panels are coated with double lift panel 200mm x 50mm x double 5mm horizontal single 6mm vertical wires.  Inclusive of threaded inserts, 40x5mm clamp bar secured with anti-tamper M8 bolts all galvanised and coated standard RAL 6005 Green colour.

- To supply and install 1 gate 1.2m x 2m high with lintel infill above

- Existing single gate to be reused and moved to double gate site

(WMPC plan to work in partnership with the LTA and their gate access system provider, to install a lite

smart access mechanism on the pedestrian gate to allow WMPC to remotely manage the renovated

facility. WMPC still require a pedestrian gate to be quoted for as part of this project. Specification details for

this lite smart access mechanism are available upon request)

- To include the removal of old fencing and double gate from site

- On completion of works all surplus arising to be removed and the site left clean and tidy

Site Clearance

The contractor shall collect and remove all contractor’s rubbish and debris off site

Materials arising are to be removed from site to an appropriately licensed tip that the contractor shall provide at his own expense for all extraneous matter arising from the execution of the works

On completion of the works the site must be left clear and clean; no rubbish, debris or any other such items shall be left on site

Site Preparation

The site will be protected from damage by the contractor

Any damage caused by the contractor or its subcontractors must be repaired and returned to its original state

An area will be allocated for the contractor to keep a waste container, welfare unit and equipment.  Parking is available on site free of charge

Timescale of Works

The contractor will need to make suitable arrangements with West Malling Parish Council as to commencement of the works.  Works should start as soon as conditions allow

Safety

The contractor must provide a method statement and full risk assessment of works before an contact is accepted.  The contractor must ensure that it protects its employees, any council staff and any members of the public visiting the site

The contractor must always use appropriate PPE in accordance with regulations

No item is to be left that could result in a slip, trip or fall to any person.  All equipment used must conform to PUWERS 1998 regulations

The contractor must provide a safe and suitable work area.  All work must be carried out in such a way as to ensure that no employees, council staff or members of the public visiting the area are exposed to any risk of falling objects.  Also that no escape routes are blocked or access limited in any way