- SPECIFICATION

and

- PRE TENDER HEALTH & SAFETY PLAN

for

- WORKS TO BE EXECUTED AND MATERIALS TO BE USED IN

BREAKOUT AND RE-INSTATEMENT OF CONCRETE SLAB DRIVEWAY ADJACENT TO THE CONTROL TOWER

CONTRACT NO:

IWM/DUX/1573

for

- IMPERIAL WAR MUSEUM

DUXFORD AIRFIELD

DUXFORD

CAMBS

- Client:

- Imperial War Museums

Duxford

Cambs

CB22 4QR						Date September 2016

SECTION 1

Preliminary Matters

1.0 	Introduction

1.1	PRELIMINARY PARTICULARS ETC.

1.2	GENERAL MATTERS

1.3	STATUTORY/GENERAL OBLIGATIONS6

1.4	MANAGEMENT/ADMINISTRATION PROCEDURES

1.5	RESOURCES/TEMPORARY WORKS AND SERVICES

1.6	THE WORKS GENERALLY

1.7	CONTRACTUAL MATTERS

1.8	CONTINGENCY/ PROVISIONAL/ PRIME COST SUMS

1.9	APPROVED SUB-CONTRACTORS

1.10	PROVISIONAL DAYWORKS

1.11	WORKS UNDERTAKEN CONCURRENTLY BY OTHERS

1.12	LIAISON

1.13	PHASING OF WORKS

1.14	TEMPORARY SIGNAGE

1.15	SITE RULES

1.16	SECURITY CLEARANCE

APPENDIX 1

SECTION 2

Health & Safety

PRE TENDER HEALTH & SAFETY PLAN

2.0	GENERAL REQUIREMENTS

2.2	Checking/Handling and Site Storage of Materials and Components

2.3	Loading.

2.4	Working at Heights

2.5	Proximity to Public

2.6	Method Statements

2.7	Pre-Tender Health & Safety Plan

2.8	Client’s own Safety Representative

2.9	Health & Safety File

SECTION 3

Trade Preambles

3.01	MATERIALS		WORKMANSHIP

SECTION 4

SCHEDULE OF WORKS

SECTION 5

TENDER SUBMISSION

5.0	TENDER SUBMISSION

5.1	TENDER ACCEPTANCE

5.2	BASIS OF PRICES

5.3	AWARD CRITERIA (other than price)

5.4	PRICING SCHEDULE

SECTION 6

POINTS OF CONTACT

SECTION 7

CONTRACT TIMETABLE

7.0	CONTRACT TIMETABLE

SECTION 1

Preliminary Matters

1.0 	Introduction

  - An Act of Parliament founded the Imperial War Museum in 1920 following the Great War to act as a National War Museum to collect and display material relating to that War. Due to the general interest displayed by the Dominion Governments this led to the title “Imperial War Museum” (IWM).

  - The Imperial War Museum was opened in 1920 at a site in Crystal Palace, but eventually acquired its current home at Lambeth Road in July 1936 on the site of the former Bethlem Royal Hospital.

  - At the commencement of the Second World War the Museum’s terms of reference were extended to cover both the Great War and the Second World War, and again in 1953 to cover all military operations involving British & Commonwealth forces since August 1914.

  - In addition, IWM provides exhibits of educational and general interest to members of the public.

  - IWM Duxford is a former RAF fighter station originally dating from the First World War. It has a long and distinguished history from 1924 until its closure as an RAF Station in 1961 and is particularly famous for its role in the Battle of Britain in 1940.

  - Today, IWM Duxford is one of the world's leading aviation museums, housing some 200 aircraft ranging from flimsy biplanes to modern day jets. Many of the aircraft at IWM Duxford still fly. The airworthy aircraft at the Museum belong to private collectors and operators and are an important part of IWM Duxford's unique living atmosphere. IWM Duxford is internationally renowned for its annual programme of world-class air shows.

  - The American Air Museum stands as a memorial to the 30,000 Americans who died flying from bases in the UK including IWM Duxford, during the Second World War. This outstanding example of modern British architecture houses IWM Duxford's unique collection of historic American combat aircraft including the SR-71 Blackbird, the only one of its kind on display in Britain and the B-24 Liberator. The arrival of these new stunning exhibits led to the reconfiguration of the American Air Museum, and the Rededication Ceremony on 27 September 2002, with former president of the US, George Bush and HRH the Prince of Wales present.

  - IWM Duxford is not just about aircraft and the Museum houses an outstanding collection of tanks, military vehicles and artillery displayed in walk through battle scenes in the Land Warfare Hall.

  - As of 2016, Imperial War Museums consists of 4 other major public tourist attractions:

IWM London, Lambeth Road

HMS Belfast moored in the Pool of London

Churchill War Rooms

IWM North, Trafford, Manchester

1.1	PRELIMINARY PARTICULARS ETC.

1.1.1	Employer

- Imperial War Museums

Lambeth Road

London

SE1 6HZ

1.1.2	Contract Administrator

- J P Banks

- Capital Projects Manager

Imperial War Museums

Duxford

CB22 4QR

T: 01223 497243

E: jbanks@iwm.org.uk

1.1.3 	Principal Designer

IWM represented by J P Banks as above.

1.1.4	General Description of the Works

The works comprise of breaking up the existing concrete slab adjacent to the Control Tower and replacing with a new Structural Engineered designed concrete slab.

1.1.5	Location and Access to Site

The site is located immediately to the east of the M11/A505 junction in South Cambridgeshire.

Access into the site is from the A505 to the south side and via the staff entrance/guardroom.

1.1.6	Tender Drawings

The following drawings form part of this specification and will become contract documents.

No tender drawings are provided.

1.2	GENERAL MATTERS

1.2.1	Limits of site operations

The contractor shall limit his operations to the immediate area of the works.

The contractor shall be responsible for storing materials, arranging for loading/unloading materials and organizing his works in a manner that does not interfere with other parts of the site and access to same.

1.2.2	Preliminary investigation

The Contractor shall be deemed to have visited and inspected the site and to have examined drawings (if any) and contract documents and to have adequately acquainted himself with local conditions, accessibility of the works and site, the nature of the ground and subsoil, supply of and conditions affecting labour, the availability and supply of materials, water, electricity and telephones, all in relation to the execution of the works as no claim on the ground of want of knowledge in such respect will be entertained.

1.2.3	Abbreviations and references

- The following abbreviations, references or terms are used in the Specification.

CP 	- 	British Standards Code of Practice

BS 	- 	British Standard Specification

The term ‘Contractor’ used throughout this document relates to the Contractor who is contracted to carry out and complete the works. The contractor shall be the ‘Principal Contractor’ under the CDM regulations.

Reference to CA means the Contract Administrator or other consultant, when appointed; with authority under the terms of the contract to act is such a capacity.

The reference to Building Inspector means Building Control Officer, District Surveyor or any other person with authority under statute to require compliance with any regulation or byelaw.

The terms ‘Approved’, ‘Selected’, or ‘Directed’ means the approval, selection, or direction of the client/CA.

The term ‘Provide’ means that the item(s) is to be supplied, delivered and fixed at the expense of the builder.

The term ‘Day Work’ is applicable to the work of an unknown or unforeseen nature that could not be specified and therefore estimated for during the preparation of the schedule of works.  Such work is usually paid for on a time and material basis with the Contractor submitting evidence of hours of work and invoices for materials, plant, etc.

1.3	STATUTORY/GENERAL OBLIGATIONS

1.3.1	Safety/Health and Welfare

Allow for complying with all enactments, regulations and working rules relating to safety, health and welfare of work people.

The Contractor’s attention is drawn to Section 2 of this Specification, which covers Health & Safety matters specifically.

These works are subject to CDM Regulations 2015.

1.3.2	Working Hours

Monday to Friday - 0730 - 1700.  Weekend working will only be allowed with the prior agreement of Employer. The contractor may extend working hours on weekdays subject to approval of Employer.

1.3.3	Statutory Regulations

The works must be undertaken in compliance with Statutory Regulations, Building Regulations, Gas Safety Regulations, Water Bye Laws and IEE Regulations.  Contractor is to allow in his tender for compliance, particularly in respect of service installations, which require contractors design input.

1.3.4	Noise

Fit all compressors, percussion tools and vehicles with effective silencers of a type recommended by manufacturers of the compressors, tools or vehicles.

1.3.5	Public and Private Services

Adequately protect, uphold, maintain and prevent damage to all services.  Do not interfere with their operation without consent of the service authorities or private owners or the CA as appropriate.

1.3.6	Existing Features

Prevent damage to existing buildings, fences, gates, walls, roads, paved areas, other of the site features, which are to remain in position during the execution of the works.

The contractor shall be responsible for properly making good to the satisfaction of the C.A. all areas disturbed during the execution of the works. Contractor is to plan the works and in particular the maneuvering of plant and machinery so as to cause minimal disturbance to adjacent areas.

1.3.7	Structural Fabric

The contractor is advised the hangars are grade 2* listed buildings. Works must be undertaken in a manner that causes the minimum of damage/disturbance to the fabric of the building.

1.4	MANAGEMENT/ADMINISTRATION PROCEDURES

1.4.1	Supervision

Accept responsibility for co-ordination, supervision and administration of the works, including subcontractors. Arrange and monitor a programme with each subcontractor, supplier, Local Authority and statutory undertaker and obtain and supply information as necessary for co-ordination of the work.

The Contractor is to provide a competent person to constantly supervise the works and shall inform the CA of the name of such person.

The person is to be able to adequately receive directions given to him by the CA and is to remain on site throughout the duration of the contract and shall not be changed without prior approval from the CA.

The person is to implement the requirements of the Construction (Design & Management) Regulations 2015 as they apply to this project, and must assist the Planning Coordinator in maintaining the health and safety file for this project.

1.4.2	Insurances

Before starting work on site submit to the employer (through the CA) documentary evidence and/or policies and receipts for all insurances which are required to be taken out by the Contractor.

As soon as possible and prior to starting work on site, prepare, in an approved form, a programme for the works that should make allowance for all:

- Subcontractors works including completion of drawings etc.

2.  Work resulting from instructions issued with regard to the expenditure of provisional sums.

3.  Other work concurrent with the Contract.

4.  All contractor and sub-contractor site operatives are to carry up to date CSCS cards.

1.4.4	Commencement of Work

Inform CA as least seven working days before proposed date for commencement of the work on site.

1.4.5	Monitoring

Record progress on site and keep a copy of the programme on site.  Update or re-draft without delay if circumstances arise which affect the progress of the works with copies of all revisions to CA.

1.4.6	Site Meetings

Hold regular site meetings as necessary for the proper management and co-ordination of the contract and as required by the CA.  Meetings will normally be held every four weeks.  Attend all meetings and inform subcontractors and suppliers when their presence is required.  Notify all consultants.  The CA will take the Chair and minutes will be taken and distributed by the CA.

1.4.7	Notice of Completion

Give CA at least one week’s notice of anticipated dates of practical completion of the whole or parts of the works.

1.4.8	Measurements

Give reasonable notice to the CA before covering up work, which the CA requires to  be measured.

1.4.9	Covering Up

Give not less than one working days notice to the CA or Clerk of Works before covering up.

1.5	RESOURCES/TEMPORARY WORKS AND SERVICES

1.5.1	Accommodation

Provide necessary temporary sheds, offices, mess rooms, sanitary accommodation, and other temporary buildings required for Contractor’s own and domestic subcontractor’s use.

Routes from fire escape staircases away from the building must not be obstructed at any time.

Scaffolding/access platforms must not be placed so as to restrict access into/out of the building by any of the users, private, or commercial.

1.5.2	Protection

Provide temporary fencing, hoardings, screens, vans, planked footways, guard rails, gantries and the like as may be necessary for protecting the public and others, for the proper execution of the works and for meeting the requirements of the Local or other Authority.

The contractor will be required to fence the works to prevent unauthorized persons entering into the working area. Contractor’s materials storage areas will need to be enclosed by Herras fencing, which is to be secured closed at the end of each working day.

1.5.3 	Water

Water for the works is available on site free of charge. Contractor is to make temporary arrangements for storing and distributing around the site.

1.5.4	Electricity

The contractor is to make his own arrangements for the supply of power he may require.

1.6	THE WORKS GENERALLY

- GENERALLY

1.6.1	Good Practice

Where and to the extent the materials, products and workmanship are not fully specified they are to be:

1.	Suitable for the purposes of the work stated in or reasonably to be inferred from the contract documents, and

2.	In accordance with good building practice including the relevant provisions or current BSI documents.

1.6.2	Manufacturers Recommendations

Handle, store, prepare and use, or fix each product in accordance with manufacturers printed or written instructions/recommendations.  Inform CA if this conflict with any other specified requirement.

1.6.3	Workmanship

The works to be carried out by, or under the close supervision of, experienced tradesmen, skilled in the particular type of work.

1.6.4	Service Runs

Make adequate provisions for services, including unobstructed routes and fixtures.  Wherever possible, ducts, chases and holes to be formed during construction rather than cut.

- PRODUCTS/MATERIALS GENERALLY

1.6.5	Products to be New

Unless otherwise specified.  Ensure that the whole quantity of each product and material required to complete the work is of consistent kind, size, quality and overall appearance.

Handle, store and fix products with care to ensure that they are not damaged when incorporated into the works.

1.6.6	Or Equivalent Approved

Means that the products of different manufacturers may be substituted if prior approval has been obtained, but the CA reserves the right to insist on a named product(s) the rates or prices will be held to be based on the product(s) specified, unless agreed otherwise.

1.6.7	Proprietary Names

The phrase ‘or equivalent’ is to be deemed included whenever products   are specified by proprietary names.

1.6.8	Single Source

Where a choice of manufacturer or source of suppliers is allowed for any particular product or material, the whole quantity required to complete the works must be the same type manufacturer or source.  Do not change without approval.  Produce written evidence or sources of supple requested by CA.

- ACCURACY/SETTING OUT

1.6.9	Accuracy

- The contractor is to set out the route of the path and obtain CA approval prior to commencement of excavations.

- PROTECTION/CONDITION OF WORK/DRYING OUT

1.6.10	Site administration and Security

Adequately safeguard the site, products, materials, plant, the works and any existing buildings affected by the works from damage and theft.  Take all responsible precautions to prevent unauthorized access to the site, the works and adjoining property.

1.6.11	Stability

Accept responsibility for stability and structural integrity of the works during the contract, and support as necessary.  Prevent overloading/damage.  Details of design loads may be obtained from the CA.

1.6.12	Inclement Weather

Adequately protect the works from damage by inclement weather and use all reasonable and approved building aids and methods to prevent or minimize delays during cold or inclement weather.

1.6.13	Rubbish

- Remove rubbish and debris as it accrues and keep the site and works clean and tidy.

1.6.14	Drying Out

Control the drying and humidity of the works and the application of heat to prevent:

1.	Blistering and failure of adhesion.

2.	Damage due to trapped moisture.

3.	Excessive movement.

- ALTERATIONS/EXTENSIONS/MAINTENANCE WORK

1.6.19	Make Good

Means carry out local remedial work, including the following as appropriate and necessary to leave the work sound and neat to approval:

1.	Remove defective parts of existing finishes and components and around any stated features.

2.	Fill, dress down, piece in, patch, extending existing finishes, make minor repairs and adjustments.

3.	Refix or re-stick.

4.	Redecorate.

5.	Consequent upon alteration, extension and maintenance work is deemed to be included in such items.

- SAMPLES/APPROVALS/TESTING/INSPECTION

1.6.22	Samples - materials

Where approval of products or materials is specified, submit samples or other evidence of suitability.  Do not confirm orders or use materials until approval has been obtained.  Retain approved samples on site for comparison with products and materials used in the works.  Remove when no longer required.

1.6.23	Samples - workmanship

Where samples of finished work are specified, obtain approval of stated characteristics before proceeding with the works.

Retain approved samples on site for comparison with the works.  Remove samples which are not part of the finished works when no longer required.

1.6.24	Approvals

Inspection of any other action by the CA must not be taken as approval of materials, products or work unless the CA confirms in writing the express terms referring to:

1.	Date of inspection.

2.	Part of the work inspected.

3.	Respect or characteristics which are approved.

4.	Extent and purpose of the approval.

5.	Any associated conditions.

- FIXING/FASTENINGS/ADHESIVES/MORTAR

1.6.25	Fixings Generally

Use fixings and jointing methods and types and spacings of fastenings which are suitable having regard to:

1.	Nature of compatibility with products/materials being fixed and fixed to.

2.	Recommendations of manufacturers of fastenings and manufacturers of products/material being fixed and fixed to.

3.	Materials and loads to be supported.

4.	Conditions expected in use.

5.	Appearance - being subject to approval.

- WORK AT COMPLETION/MAKING GOOD DEFECTS

1.6.26	Cleaning

Cleaning materials and methods are to be as recommended by manufacturer of product being cleaned.

In the absence of such recommendations cleaning materials and methods are to be approved by CA.

1.7	CONTRACTUAL MATTERS

1.7.1	Form of Contract

The Form of Contract will be the JCT Agreement for Minor Works 2016 Edition, together with amendments to date, including April 2015 amendments incorporating the CDM Regulations 2015 .

The Contractor should include for all costs incurred in compliance with the Terms and Conditions of the Contract.

- The Specification and drawings (if any) will become contract documents.

The Contractor must include for all works shown on the drawings, whether referred to in the Specification or not, and for all works which are not included in the Specification, or on the drawings which could reasonably be expected to be included in the works.

1.7.2	Appendix 1 to these Preliminaries

Appendix 1 at the end of these Preliminaries describes the contractual details that will be included in the Contract Conditions.

1.8	CONTINGENCY/ PROVISIONAL/ PRIME COST SUMS

1.8.1	Contingency Sum

- Not used

1.8.2	Provisional Sum

- Refer to section 4 for details of provisional sums allowed for the various sections of work.

1.8.3	Provisional Repairs

- Refer to section 4 as above.

1.8.4	Prime Cost Sums

- Not used.

1.9	APPROVED SUB-CONTRACTORS

Not used

1.10	PROVISIONAL DAYWORKS

Not used.

1.11	WORKS UNDERTAKEN CONCURRENTLY BY OTHERS

Building services and fabric are maintained on the site by an FM contractor appointed by IWM. (Engie).  This contractor will be permitted access to the buildings at all times. The normal activities of this contractor are not expected to interfere with these contract works.

1.12	PHASING OF WORKS

N/A

1.13	TEMPORARY SIGNAGE AND FENCING.

The working area is to be enclosed by temporary fencing – ‘Herras’ interlocking fencing or similar, adequately braced to ensure it is robust and will not blow over. Fencing must be arranged so that is does not obstruct pedestrian access/egress from the building.

Contractor is to provide appropriate warning signs and notices in order to warn the public or site users.

- Contractor/subcontractor advertising will not be permitted.

1.14	SITE RULES

The Museum operate specific rules for ‘Safe Working Conditions and Protection of  its Staff, Visitors and Collection’. Please see IWM’s ‘Control of Contractors’ document attached.

These are available as a small booklet from the CA The contractor will be required to sign the form included in the booklet to confirm he has read the rules and will ensure all work is carried out in accordance with same.

The CA will carry out a site induction for all contractors’ operatives on the morning of the first day of starting on site.

1.15	SECURITY CLEARANCE

1.15.1	The Contractor is responsible for the ensuring that all their employees and any sub-contractors, agents etc. who are due to work at the Premises, for longer than four weeks in duration in the delivery of the Contract, whether in a continuous period, or over the duration of the Contract, have been security vetted to Basic level, as defined by Disclosure Scotland1.

1     Please note that Disclosure Scotland is the agency that all security vetting providers will approach to undertake the security vetting.

1.15.2	For Contractor’s with a small employee base, IWM will be prepared to undertake the security clearance on behalf of the Contractor, provided that this is made known at the time of the appointment, and that this agreed by IWM, and that the Contractor agrees to pay the administration charge of £41 per clearance, that IWM is charged for this service.

1.15.3	This requirement will apply to an employee who has not worked at an IWM branch, within the previous 12 months.

1.15.4 A minimum of 48 hours before any individual commences work on the Premises, the Contractor is required to complete and submit the Security Notification Form (see Appendix 2) to the Technical Security Manager at IWM London.

1.15.5	Upon arrival at the Premises, each new employee must report individually to the Control Room with a copy of their certificate of proof of vetting, and some form of ID. IWM will note the details of the certificate of proof of vetting, but will not retain any documents.

1.15.6	IWM will accept a certificate of proof of vetting which is dated within 12 months of the date of their site commencement at IWM, although IWM reserves the right to request they are security vetted, at its discretion, which IWM agrees not to action unreasonably.

1.15.7	Failure to comply with this requirement could result In the employee not being granted access to IWM.

1.16	NEW SUPPLIER

The successful contractor if not on the museum’s approved supplier system, will be required to complete a ‘New Suppliers Form’ as contained within the tender pack and return it to the CA within 5 days of award of contract.

APPENDIX 1

- Contract:

The form of Contract will be the JCT Agreement for Minor Building Works 2016 together with any amendments to date.

- ARTICLES OF AGREEMENT:

- Recitals:

- First.   The work required is the breakout and re-instatement of concrete slab driveway adjacent to the Control Tower

- Second. Documents comprise the contract specification.

Third. The contractor shall supply a fully priced copy of the specification to the employer.

Fourth. The employer is not a contractor under the CIS rules.

Fifth. This project is considered not to be notifiable under the CDM Regulations 2015.

Sixth. Not applicable.

Seventh. Applies, excepting paragraphs 3,4 and 5.

Articles :

Article 3 	The Contract Administrator shall be IWM Duxford

Article 4	Not applicable.

Article 5	Not applicable.

Article 7 (and Schedule 1) the arbitration provisions of article 7 and schedule 1

apply.

- Contract Particulars:

- Base date					01 January 2016.

- Clause 1.1					2 weeks. Beginning on the date of receipt by the contractor of the tender documentation.

Clause 2.2	Commencement		To be agreed

Completion

Clause 2.10	Rectification Period	12 months

  - Percentage is 95%

  - Percentage is 95%

    - 3 months

  - Deleted

5.3.2	Contract Insurance required is £10million minimum

5.4A & 5.4B	Clause 5.4A applies % addition to cover fees – 20%

7.2	Nominator shall be President of the RICS

Schedule 1	Appointment of arbitrator shall be the President of The Royal Institute of Chartered Surveyors

SECTION 2

Health & Safety

PRE CONSTRUCTION INFORMATION.

GENERAL REQUIREMENTS

The works that are the subject of this Specification are subject to the Construction (Design & Management) Regulations 2015. Specifically Part 4 – General Requirements for all construction sites. The principal contractor will be required to comply with the relevant regulations contained in part 4.

The Contractor will be required to operate as the ’Principal contractor’ under those regulations and co-ordinate and manage Health & Safety issues during construction work, and, liaise with the Contract administrator during the course of these works.

Prior to commencement the contractor must ensure a construction phase health and safety plan has been produced, and that adequate provision is made on site for provision of welfare.

The Contractor will be responsible for ensuring that subcontractors employed by him operate in a safe manner appropriate to the work they are employed to undertake.

2.0	INFORMATION ON BUILDING/ENVIRONMENT/RISKS.

The following aspects of the work are considered to represent a risk, either to the person engaged in undertaking that work directly, or to other persons in the area of the works. These are to be addressed in the Principal contractor’s Construction Phase Plan.

2.1	The building fabric of Control Tower/Watch Office (Building 209).

The Control Tower at Duxford (Building No. 209) was one of 160 wartime Watch Office or All Commands control tower buildings erected by the Air Ministry’s Directorate of Works and Buildings. It is constructed of rendered brickwork with an asphalt roof, and was prominently sited in front of the main hangar group. The building is Grade 2 listed.

2.2	Proximity to Public and staff

The work will involve working in an area where public have access in close proximity around, and adjacent attractions. Staff similarly will have access to and be in occupation of buildings during the works.

2.3	Checking/Handling and Site Storage of Materials and Components

Contractor is to pay particular attention to how paint and associated materials are handled and stored.

Manufacturer’s COSSH data sheets should be followed and available on site for inspection by CA.

Some products are likely to be flammable - specific provisions for safe storage and handling must be identified by Contractor.

2.4	Lead

N/A

2.5	Working at Heights

N/A

2.6	Risk Assessments and Method Statements

The Contractor shall provide, with his tender, an outline Method Statement setting out how the work is to be approached, organized and monitored in order to ensure the works are undertaken both efficiently and safely.

2.7	Construction phase plan.

Before works commence on site, the Principal contractor must draw up a Construction Phase Plan. This must set out the health and safety arrangements and site rules, taking account of the nature of the site and use of the building.

Throughout the construction phase the principal contractor is to ensure the plan is reviewed, and updated, or revised so that it continues to be sufficient to ensure construction work is so far as is reasonably practicable carried out without risks to health or safety.

The ‘Principal Designer’ for this project is The IWM, represented by J P Banks Project Manager. The Principal contractor shall liaise and cooperate with the Principal designer in preparing the plan and providing information for inclusion into the Health and safety File.

2.9	Health & Safety File

The Principal Designer will prepare the Health and Safety file. The Principal Contractor is to provide information on request to enable the file to be prepared. Such information is likely to include, but not be limited to

a.	The contractor’s risk assessment and method statements.

b.	Copies of manufacturer’s current technical literature and COSHH data sheets for all materials, plant and equipment selected by the contractor.

.

SECTION 3

Trade Preambles

- To be read in conjunction with Preliminaries

and General Conditions

3.01	MATERIALS

0.01	Expansion Joint/Contraction Joint

Joint construction to comply with SHW series 1000 and Clause 1009

0.02	Reinforcement

BS MESH REF. C503

0.03	Dowel Bars

Plain, round, mild steel bars, grade 250 steel to BS 4449 and Department of Transport Clause 1011

0.04	Plastic Sheath

Approved manufacturer. Proprietary de-bonding agent may be used

0.05	Joint sealant

Cold poured sealant to BS 5212 and SHW Clauses 1016 and 1017, and manufacturer’s instructions

0.06	Dowel bar cradles

Proprietary manufacturer to SHW Clause 1011

0.07	Expansion Joint filler

Joint filler board ton SHW Clause 1015, 25mm thick

0.08	Surfacing

Air trained in-situ reinforced concrete paving to BS:5328 parts 1 to 4: Flexural strength 4.5N/mm2, Characteristic strength C40, 20mm Nominal size aggregate, Paving quality concrete (PQC) To SHW SERIES 1000,

  - Air Entrainment

Approved Chloride Free Air Entrainment Agent 4.5% +/- 1.5% Department of Transport Clause 1002

  - Polythene Membrane Underlay

Not less than 0.250mm thick (1000 gauge) Department of Transport Clause 1007

  - Sub-Base

MOT Type 1 Compacted in Maximum 150mm thick layers

0.11  Subgrade

Formation to be 50mm into hard chalk

3.02	WORKMANSHIP

0.01	Generally

Comply also with the requirements of BS 8000 Part 12 1989  ‘Workmanship on Building Sites’.

0.02	Ordering of Materials

Ordering and storing of materials are to be completed to manufacturer’s recommendations and instructions.

0.03	Order of Working

Work in a sequence that ensures the finished work is not spoilt by dust and debris arising from subsequent preparation work.  When planning the work consider the need for cleaning, washing down, heating or drying equipment.

0.04	Initial Preparation of Surfaces

Remove all dirt and any contaminants before commencing with installation of compressible sub base material

0.05	Defective Backgrounds

Advise CA of defective areas and seek clarification on how to proceed.  Areas of unstable or incompatible components will usually be subject to replacement or repair on the direction of the CA.

0.06	Preparation of Reinforcement and Dowel Bars

Reinforcement to terminate 300mm +  50mm from CL of joint. Dowel bars are to be 400mm long @ 300 centres set 10mm from top of surface of slab. Bar spacing may be varied so that no bar is within 150mm of joint parallel to bars.

0.07	Inspections

CA will make frequent site inspections to judge the preparation and quality of the work.

A final visual inspection will be made and the general quality will be judged as follows:

- Satisfactory excavation and preparation of subsoil surface.

- Satisfactory installation of reinforcement mesh and dowel bars as per Structural Engineers design.

- Quality of workmanship when concrete is poured

- Final finish as stipulated by CA

- General cleanliness and an overall appearance of the work.

SECTION 4

SCHEDULE OF WORKS

WORKS.

These works are to be commenced on or before TBC and scheduled to complete TBC.

PROVISIONAL REPAIRS

Note:- The repairs described below are an estimate of the repair work likely to be needed. The contractor will be required to liaise with the C.A. for confirmation of specific areas and extent of works required. The C.A. will raise a contract instruction to confirm the actual extent of the work.

Heras-Fencing.

Erect HERAS READYHOARD ON-GROUND SYSTEM for the duration of the breakout of existing concrete slab and excavation. Also to be used for compound adjacent to proposed project area. Panels to be painted to a CA approved colour.

Breaking out existing concrete slab.

Carefully breakout existing concrete slab and place material in a lockable skip.

Controlling dust: The Control of Substances Hazardous to Health 2002 (COSH) covers activities which may expose workers to construction dust. There are three key areas:

: Assess (the risks)

: Control (the risk)

: Review (the controls)

The most important action is to stop the dust getting into the air. The two main ways are:

: Water- water damps down dust clouds, however it needs to be used correctly. This means enough water supplied at the right levels for the whole time the project is being done. Just wetting the material is not enough.

On-tool extraction-removes dust as it is being produced.

Soakaways.

2m x 2m x 800mm deep soakaway to be excavated. Use H.D. CRATES with minimum 600mm cover to manufacturer’s instructions. Minimum 5m from concrete slab.

Drainage Channels.

Install CHARCON (DBM125) HD DRAINAGE CHANNEL. Slot 30mm wide with ductile iron insert.

Reinforcement.

Install C503 Mesh top and bottom, 50mm cover, 300mm laps and laid in direction of travel. To terminate 350mm +/- 50mm from CL of joint.

N.B. Joint construction to comply with SHW SERIES 1000 and Clause 1009.

Dowel Bars.

Install plain, round, mild steel bars, grade 250 steel to BS 4449, 25mm diameter, 600mm long @ 300mm centres, set D/2 +/- 20mm from top of surface of slab. Set out dowel bar centres from CL of joint +/- 25mm Department of Transport Clause 1011

Plastic Sheath.

Approved manufacturer. Proprietary de-bonding agent may be used.

Joint Sealant.

Cold poured sealant to be installed to BS 5212 and  SHW Clauses 1016 and 1017 as well as manufacturer’s instructions. Apply to

Dowel Bar Cradles.

Install to SHW Clause 1011.

Dowel Cap.

Install approved manufacturer, tight fitting 100mm long waterproof cap containing 25mm compressible material.

Expansion Joint Filler

Install Joint filler board to SHW Clause 1015, 25mm thick.

Re-concreting/Surfacing.

Lay air trained in-situ reinforced concrete, paving grade to: BS5328: Part 1 to 4: Flexural strength 4.5N/mm2, characteristic strength C40, 20mm nominal size aggregate, paving quality concrete (PQC) to SHW Series 1000. Surface finish to be of an exposed aggregate type and to CA approval.

Reinforcement.

Lay C503 Mesh (In accordance with BS: 4483) top and bottom, 50mm cover, 300mm laps. Main reinforcement to be laid in direction of travel.

Air Entrainment.

Apply approved Chloride free Air Entrainment agent 4.5% +/- 1.5% Department of Transport Clause 1002.

Polythene Membrane Underlay.

Install, not less than 0.250mm thick, 1000 gauge polythene membrane to Department of Transport Clause 1007.

Sub-Base.

Lay MOT Type 1 compacted material in 150mm thick layers.

Subgrade.

Formation is to be 50mm into hard chalk.

Plant tools and machinery.

The Contractor is responsible for ensuring the suitability and condition of all plant, tools, equipment and PPE, and for ensuring compliance with all relevant legislation and regulations.

The Contractor is also responsible for ensuring that the above are properly checked, inspected and maintained. All equipment that may give rise to significant risk is to be inspected and tested before it comes on site. Proof of inspection must accompany the equipment, and will be checked if IWM managers are in doubt as to the suitability of the equipment.

Work equipment must be maintained in good repair and in an efficient state. Any equipment which becomes damaged must be withdrawn from use. Any portable electrical equipment must be within its certified PAT test period.

Only qualified, competent persons are to use, erect, install, etc., any equipment. Proof of training may be requested by IWM staff if the personnel are not using the equipment in a competent manner.

Guards must be fitted to all equipment: including saws, as necessary and guards must be functional. The attachment of handsaws to boards to convert them into bench saws is not permitted. Bench saws must be braked and the emergency stop must be readily accessible.

Roads and Traffic.

Contractors are not to block roadways. A15 mph speed limit applies to all Duxford site areas. Transgression of the site speed limit will lead to disciplinary procedures and may lead to prohibition from the site. Please note that the normal ‘rules’ of the road apply on site. Traffic signs, signals and one way systems, if in operation, must be followed.

Lorries and other large vehicles (vans etc. with restricted rear vision) are not permitted to reverse without a banksman in attendance.

Drivers of all vehicles must not use walkie-talkies or mobile phones while their vehicle/machine is moving (unless they are using a hands free device).

All vehicles are to have side lights or hazard beacons switched on while moving around the site. In poor visibility conditions these should be supplemented with head lights.

Restricted Areas.

Contractors operatives are not allowed in the Aircraft Manoeuvring (Airside) areas. These areas must not be accessed at anytime without the permission of the CA.

Electrical Supply.

All electrical equipment is to be inspected and tested before it comes on site. Proof of inspection must accompany the equipment, and will be checked if IWM managers are in doubt as to the suitability of equipment.

Only 110 volt or battery powered equipment is to be used on site. Contractors are responsible for providing their own 110 volt power tools, leads and transformers. All 110 volt equipment is to be protected with a secondary winding Centre Tapped to Earth.

Any equipment which becomes damaged must be withdrawn from use, particular attention should be paid to leads and equipment casings.

Under no circumstances are trailing leads allowed to cross general traffic routes, either pedestrian or vehicle.

Housekeeping.

Contractors must make allowances for the necessary labour and plant to maintain the required standards of house- keeping and site tidiness.

Materials held on site must be kept to a minimum. Materials must be stored in agreed areas and under no circumstances are to be permitted to block access, vehicle or emergency routes.

Materials must not be stacked to an excessive height. Steel and nylon banding used to contain bundles of materials must be safely disposed of. Materials must be removed from site when finished with.

Other responsibilities.

Contractors must make allowance for the costs of safeguarding the safety, health and welfare of their own staff and others.

SECTION 5

Tender Submission

Tenderers are required to comply with the following in order for their tender to be considered. If tenderers do not comply fully, then their tender may be rejected by the Museum.

5.1    Site Visit

As part of the process tenderers shall visit site to acquaint themselves with the site, the extent of the work and the conditions under which it is to be carried out.

Tenderers are to contact Mr. J Banks at the Duxford site on 01223 497243 to make arrangements.

5.2    Tender Acceptance

The ‘Tender Declaration Form’ included in this specification must be submitted as part of the tender. Electronic submissions or e-mails will not be considered.

The tender envelope should be marked TENDER DOCUMENT REF IWM/DUX/1573. Other than this envelopes and/or packages should be plain and bear no reference to the tenderer's name; franking machines that automatically print the company name should not be used.

5.3    Basis of Prices

All prices must be quoted on the basis of lump sum fixed price based on no re-measurement, as indicated in the accompanying documents. Quantities given are for guidance only and prices entered should exclude VAT.

5.4    Supporting Information

The contractor shall as part of tender submission include the following information:

- An outline Method Statement describing how the work are to be undertaken and what arrangements will be put in place to manage health and safety.

- The names & contact details of two references where the company has carried out similar work.

- Copy of the company’s Health & Safety Statement

.

- This tender document complete, with each page initialed to indicate reading.

- The schedule of work items priced up.

- The tender form properly completed and signed.

- Proposed programme of works.

5.5    Award Criteria (other than price)

The employer does not bind himself to accept the lowest or any tender; when awarding the contract in addition to price the Employer will have regard to the following.

- Accuracy of submission.

- Compliance with tender instruction for submission of documentation.

- Suitability of the Method Statement and Risk assessment referred to above.

- Contractor’s ability to demonstrate his competence in carrying out works of this nature.

SECTION 6

CONTRACT TIMETABLE

6.0	CONTRACT TIMETABLE

# The following schedule represents the proposed timetable for the appointment of a consultant and commencement of the contract for the services in relation to various external building projects as detailed within the tender documents on behalf of the Trustees of the Imperial War Museum.

| CT 1.0 | Issue of Tenders: | 27 September 2016 |
| --- | --- | --- |
| CT 1.1 | Tender Period | 3 weeks |
| CT 1.2 | Tender Submission Date: | 17 October 2016 |
| CT 1.3 | Tender Submission Time: | Noon |
| CT 1.4 | Tender Opening Time: | TBA |
| CT 1.6 | Tender Evaluation Period: | One week |
| CT 2.0 | Contract Award: | 24 October 2016 |
| CT 2.1 | Contract Lead-in Time | To be advised |
| CT 2.2 | Contract Start Date: | On, or before 31 October 2016 |
| CT 2.3 | Contract Completion Date: | On or before 12 December 2016 |

- Tender Declaration; Contract Reference: IWM/Dux/1573

Project.	Breakout and re-instatement of concrete slab driveway adjacent to the control tower.

To:		The Trustees of the Imperial War Museum

I/We having read the following documents delivered to me/us in respect of the above Contract:

Do hereby offer to execute and complete in accordance with the Conditions of Contract the whole of the works as described for the sum of: -

£_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ (excluding VAT)

(_ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ _ amount in words)

- Company Name:	___________________________________________________

Address:			___________________________________________________

___________________________________________________

___________________________________________________

Tel. No.		_____________________

Fax. No.		_____________________

Signature:			___________________________________________________

Print Name:		___________________________________________________

In the Capacity of:	___________________________________________________

(The above named person is duly authorised to sign tenders for and on behalf of the Company)

# Dated this . . . . . .day of  . . . . . . . . . . . . . . . . . . . . . . . 2016

TENDER BREAKDOWN.						(£)

Preliminaries (excluding scaffold).					________

Contin	gency sum.

Breakout of existing concrete slab				________

Preparation of sub-base works                 			________

Installation of reinforcement mesh and dowel bars		________

Laying of new concrete slab				           ________

Total Tender figure.						_________

(Exclusive of VAT)

Appendix 2

STAFF SECURITY CLEARANCE

NOTIFICATION FORM

CONTRACTOR:

|  |  |  | Level of CRB |  |  |
| --- | --- | --- | --- | --- | --- |
| IWM Branch | Start Date | Staff Name | Basic | Standard | Enhanced |

| Signed |  |
| --- | --- |
| Name |  |
| Job Title |  |
| Date |  |

Please complete all sections of the Form and forward to IWM’s Technical Security Manager at jpawley@iwm.org.uk or addressed to Technical Security Manager, CWR, Clive Steps, King Charles Street, London SW1A 2AQ, a minimum of 48 hours prior to the individual(s) commencing work at IWM.