Schedule 12 – Rates for the Conversion of Mk VI EOD Suits into Mk VI EOD Training Suits and for the Repair of EOD Training Suit Elements

Year 1

| Part Number | NATO Stock Number | Description | Part | Inspection Level | Tests/Operations | Failure Criteria | Firm Price | Firm Delivery Time (from receipt of authorised Tasking Form) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Strip and Survey of EOD Suit |  |  |  |  |  |  |
|  |  | Mk VI A1 Operational to A1 Training Suit Conversion |  |  |  |  |  |  |
| TBC | TBC | Ventilation System & Electronic Control Modules | Tenderer to complete | Tenderer to complete | Tenderer to complete | Tenderer to complete |  |  |
| TBC | TBC | Training Visor | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage |  |  |
| 15411-133/RT1 | TBC | EOD Helmet Refurb Tier 1 | Shell paint, straps, pads, ear plugs, chin guard stop block and battery lid | Visual inspection | Strip liner, straps and fittings. Inspect stop block, battery lid and shell paint. Stencil Train | Basic refurbishment level, required for all helmets showing signs of use. |  |  |
| 15411-133/RT2 | TBC | EOD Helmet Refurb Tier 2 | Visor Chin Guard if training visor already fitted | Visual inspection | Perform visor demister test from control power source, to check operation | Faulty demister. Screen chipped, scratched more than 10%, cracked & water ingress. |  |  |
| 15411-133/RT3 | TBC | EOD Helmet Refurb Tier 3 | Battery Box | Visual inspection of battery box and lid | Fill with fresh batteries, plug into control visor to check operation. | Box scratched, chipped or cracked etc. Faulty operation. |  |  |
| 15411-133/RT4 | TBC | EOD Helmet Refurb Tier 4 | Visor Mounts and Catch and covers | Check operation | Use screwdriver to make sure fasteners are tight. | Excessive play, won’t fasten (spring broken), fixtures loose. |  |  |
| 15411-133/RT5 | TBC | EOD Helmet Refurb Tier 5 | Block Latch & Lever Lock | Visual inspection. Inspect operation | Open and close visor | Excessive play, spring broken, rivets broken, block bent or worn. |  |  |
| 15411-133/RT6 | TBC | EOD Helmet Refurb Tier 6 | Box | Visual inspection. Check operation | Open and close lid, checking hinges and locks. Inspect inner foam for damage. | Excessive damage to exterior. Hinges and locks faulty. Interior foam damaged. |  |  |
| 15412-133/RT1 Small 15413-133/RT1 Medium 15414-133/RT1 Large | TBC | Plate Refurbishment | Buckles straps and D-Rings’ Guides and Hinges’ Foam, mesh and paint. | Visual inspection and physical test. | Check operation of the buckles, releasing and clipping in. Check operation of the hinges and guides. Inspect foam, mesh and paint for defects. | Damage to foam and mesh. Faulty or harsh/stiff action. Damage, bent snapped components and cosmetic damage. Ripped, torn, frayed straps. Bent/damaged D rings. Worn/bent or damaged buckle. |  |  |
| 15462-133/RT1 | TBC | Plate Cover | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components |  | Any seam in the cover that needs re-threading. |  |  |
| 15418-133/RT1 | TBC | Jacket Cover Tier 1 | 30 min Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15418-133/RT2 | TBC | Jacket Cover Tier 2 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15417-133/RT1 | TBC | Jacket Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to the waterproof layer. |  |  |
| 15415-133/RT1 | TBC | Jacket Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15415-133/RT2 | TBC | Jacket Outer Tier 2 | Upper Plate Buckles | Check Function with a male connector. Check seams and webbing. |  | Buckle does not function or visually damaged, webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT3 | TBC | Jacket Outer Tier 3 | Lower Plate Buckles | Check Function making sure buckle opens and snaps shut. Check seams and webbing. |  | Buckle does not function or visually damaged, Webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT4 | TBC | Jacket Outer Tier 4 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15423-133/RT1 | TBC | Trouser Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15421-133/RT1 15422-133/RT1 | TBC | Trouser Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 15421-133/RT2 15422-133/RT2 | TBC | Trouser Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 15421-133/RT3 15422-133/RT3 | TBC | Trouser Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15421-133/RT4 15422-133/RT4 | TBC | Trouser Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT1 | TBC | Trouser Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15416-133/RT2 | TBC | Trouser Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT3 | TBC | Trouser Outer Tier 3 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15416-133/RT4 | TBC | Trouser Outer Tier 4 | Buckles | Check operation |  | Damaged, faulty, doesn’t operate or cosmetic damage. |  |  |
| 15420-133/RT1 | TBC | Shorts Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro |  | Any seam in the garment that needs re-threading. |  |  |
| 15419-133/RT1 | TBC | Shorts Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT1 | TBC | Summer Legs Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT2 | TBC | Summer Legs Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 17991-133/RT3 | TBC | Summer Legs Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 17991-133/RT4 | TBC | Summer Legs Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears. Wear or any other damage. |  |  |
| 24244-133/RT1 | TBC | Summer Legs Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 24244-133/RT2 | TBC | Summer Legs Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |

Year 2

| Part Number | NATO Stock Number | Description | Part | Inspection Level | Tests/Operations | Failure Criteria | Firm Price | Firm Delivery Time (from receipt of authorised Tasking Form) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Strip and Survey of EOD Suit |  |  |  |  |  |  |
|  |  | Mk VI A1 Operational to A1 Training Suit Conversion |  |  |  |  |  |  |
| TBC | TBC | Ventilation System & Electronic Control Modules | Tenderer to complete | Tenderer to complete | Tenderer to complete | Tenderer to complete |  |  |
| TBC | TBC | Training Visor | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage |  |  |
| 15411-133/RT1 | TBC | EOD Helmet Refurb Tier 1 | Shell paint, straps, pads, ear plugs, chin guard stop block and battery lid | Visual inspection | Strip liner, straps and fittings. Inspect stop block, battery lid and shell paint. Stencil Train | Basic refurbishment level, required for all helmets showing signs of use. |  |  |
| 15411-133/RT2 | TBC | EOD Helmet Refurb Tier 2 | Visor Chin Guard if training visor already fitted | Visual inspection | Perform visor demister test from control power source, to check operation | Faulty demister. Screen chipped, scratched more than 10%, cracked & water ingress. |  |  |
| 15411-133/RT3 | TBC | EOD Helmet Refurb Tier 3 | Battery Box | Visual inspection of battery box and lid | Fill with fresh batteries, plug into control visor to check operation. | Box scratched, chipped or cracked etc. Faulty operation. |  |  |
| 15411-133/RT4 | TBC | EOD Helmet Refurb Tier 4 | Visor Mounts and Catch and covers | Check operation | Use screwdriver to make sure fasteners are tight. | Excessive play, won’t fasten (spring broken), fixtures loose. |  |  |
| 15411-133/RT5 | TBC | EOD Helmet Refurb Tier 5 | Block Latch & Lever Lock | Visual inspection. Inspect operation | Open and close visor | Excessive play, spring broken, rivets broken, block bent or worn. |  |  |
| 15411-133/RT6 | TBC | EOD Helmet Refurb Tier 6 | Box | Visual inspection. Check operation | Open and close lid, checking hinges and locks. Inspect inner foam for damage. | Excessive damage to exterior. Hinges and locks faulty. Interior foam damaged. |  |  |
| 15412-133/RT1 Small 15413-133/RT1 Medium 15414-133/RT1 Large | TBC | Plate Refurbishment | Buckles straps and D-Rings’ Guides and Hinges’ Foam, mesh and paint. | Visual inspection and physical test. | Check operation of the buckles, releasing and clipping in. Check operation of the hinges and guides. Inspect foam, mesh and paint for defects. | Damage to foam and mesh. Faulty or harsh/stiff action. Damage, bent snapped components and cosmetic damage. Ripped, torn, frayed straps. Bent/damaged D rings. Worn/bent or damaged buckle. |  |  |
| 15462-133/RT1 | TBC | Plate Cover | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components |  | Any seam in the cover that needs re-threading. |  |  |
| 15418-133/RT1 | TBC | Jacket Cover Tier 1 | 30 min Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15418-133/RT2 | TBC | Jacket Cover Tier 2 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15417-133/RT1 | TBC | Jacket Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to the waterproof layer. |  |  |
| 15415-133/RT1 | TBC | Jacket Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15415-133/RT2 | TBC | Jacket Outer Tier 2 | Upper Plate Buckles | Check Function with a male connector. Check seams and webbing. |  | Buckle does not function or visually damaged, webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT3 | TBC | Jacket Outer Tier 3 | Lower Plate Buckles | Check Function making sure buckle opens and snaps shut. Check seams and webbing. |  | Buckle does not function or visually damaged, Webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT4 | TBC | Jacket Outer Tier 4 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15423-133/RT1 | TBC | Trouser Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15421-133/RT1 15422-133/RT1 | TBC | Trouser Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 15421-133/RT2 15422-133/RT2 | TBC | Trouser Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 15421-133/RT3 15422-133/RT3 | TBC | Trouser Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15421-133/RT4 15422-133/RT4 | TBC | Trouser Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT1 | TBC | Trouser Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15416-133/RT2 | TBC | Trouser Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT3 | TBC | Trouser Outer Tier 3 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15416-133/RT4 | TBC | Trouser Outer Tier 4 | Buckles | Check operation |  | Damaged, faulty, doesn’t operate or cosmetic damage. |  |  |
| 15420-133/RT1 | TBC | Shorts Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro |  | Any seam in the garment that needs re-threading. |  |  |
| 15419-133/RT1 | TBC | Shorts Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT1 | TBC | Summer Legs Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT2 | TBC | Summer Legs Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 17991-133/RT3 | TBC | Summer Legs Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 17991-133/RT4 | TBC | Summer Legs Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears. Wear or any other damage. |  |  |
| 24244-133/RT1 | TBC | Summer Legs Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 24244-133/RT2 | TBC | Summer Legs Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |

Year 3

| Part Number | NATO Stock Number | Description | Part | Inspection Level | Tests/Operations | Failure Criteria | Firm Price | Firm Delivery Time (from receipt of authorised Tasking Form) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Strip and Survey of EOD Suit |  |  |  |  |  |  |
|  |  | Mk VI A1 Operational to A1 Training Suit Conversion |  |  |  |  |  |  |
| TBC | TBC | Ventilation System & Electronic Control Modules | Tenderer to complete | Tenderer to complete | Tenderer to complete | Tenderer to complete |  |  |
| TBC | TBC | Training Visor | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage |  |  |
| 15411-133/RT1 | TBC | EOD Helmet Refurb Tier 1 | Shell paint, straps, pads, ear plugs, chin guard stop block and battery lid | Visual inspection | Strip liner, straps and fittings. Inspect stop block, battery lid and shell paint. Stencil Train | Basic refurbishment level, required for all helmets showing signs of use. |  |  |
| 15411-133/RT2 | TBC | EOD Helmet Refurb Tier 2 | Visor Chin Guard if training visor already fitted | Visual inspection | Perform visor demister test from control power source, to check operation | Faulty demister. Screen chipped, scratched more than 10%, cracked & water ingress. |  |  |
| 15411-133/RT3 | TBC | EOD Helmet Refurb Tier 3 | Battery Box | Visual inspection of battery box and lid | Fill with fresh batteries, plug into control visor to check operation. | Box scratched, chipped or cracked etc. Faulty operation. |  |  |
| 15411-133/RT4 | TBC | EOD Helmet Refurb Tier 4 | Visor Mounts and Catch and covers | Check operation | Use screwdriver to make sure fasteners are tight. | Excessive play, won’t fasten (spring broken), fixtures loose. |  |  |
| 15411-133/RT5 | TBC | EOD Helmet Refurb Tier 5 | Block Latch & Lever Lock | Visual inspection. Inspect operation | Open and close visor | Excessive play, spring broken, rivets broken, block bent or worn. |  |  |
| 15411-133/RT6 | TBC | EOD Helmet Refurb Tier 6 | Box | Visual inspection. Check operation | Open and close lid, checking hinges and locks. Inspect inner foam for damage. | Excessive damage to exterior. Hinges and locks faulty. Interior foam damaged. |  |  |
| 15412-133/RT1 Small 15413-133/RT1 Medium 15414-133/RT1 Large | TBC | Plate Refurbishment | Buckles straps and D-Rings’ Guides and Hinges’ Foam, mesh and paint. | Visual inspection and physical test. | Check operation of the buckles, releasing and clipping in. Check operation of the hinges and guides. Inspect foam, mesh and paint for defects. | Damage to foam and mesh. Faulty or harsh/stiff action. Damage, bent snapped components and cosmetic damage. Ripped, torn, frayed straps. Bent/damaged D rings. Worn/bent or damaged buckle. |  |  |
| 15462-133/RT1 | TBC | Plate Cover | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components |  | Any seam in the cover that needs re-threading. |  |  |
| 15418-133/RT1 | TBC | Jacket Cover Tier 1 | 30 min Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15418-133/RT2 | TBC | Jacket Cover Tier 2 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15417-133/RT1 | TBC | Jacket Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to the waterproof layer. |  |  |
| 15415-133/RT1 | TBC | Jacket Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15415-133/RT2 | TBC | Jacket Outer Tier 2 | Upper Plate Buckles | Check Function with a male connector. Check seams and webbing. |  | Buckle does not function or visually damaged, webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT3 | TBC | Jacket Outer Tier 3 | Lower Plate Buckles | Check Function making sure buckle opens and snaps shut. Check seams and webbing. |  | Buckle does not function or visually damaged, Webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT4 | TBC | Jacket Outer Tier 4 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15423-133/RT1 | TBC | Trouser Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15421-133/RT1 15422-133/RT1 | TBC | Trouser Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 15421-133/RT2 15422-133/RT2 | TBC | Trouser Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 15421-133/RT3 15422-133/RT3 | TBC | Trouser Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15421-133/RT4 15422-133/RT4 | TBC | Trouser Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT1 | TBC | Trouser Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15416-133/RT2 | TBC | Trouser Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT3 | TBC | Trouser Outer Tier 3 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15416-133/RT4 | TBC | Trouser Outer Tier 4 | Buckles | Check operation |  | Damaged, faulty, doesn’t operate or cosmetic damage. |  |  |
| 15420-133/RT1 | TBC | Shorts Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro |  | Any seam in the garment that needs re-threading. |  |  |
| 15419-133/RT1 | TBC | Shorts Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT1 | TBC | Summer Legs Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT2 | TBC | Summer Legs Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 17991-133/RT3 | TBC | Summer Legs Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 17991-133/RT4 | TBC | Summer Legs Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears. Wear or any other damage. |  |  |
| 24244-133/RT1 | TBC | Summer Legs Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 24244-133/RT2 | TBC | Summer Legs Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |

Year 4

| Part Number | NATO Stock Number | Description | Part | Inspection Level | Tests/Operations | Failure Criteria | Firm Price | Firm Delivery Time (from receipt of authorised Tasking Form) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Strip and Survey of EOD Suit |  |  |  |  |  |  |
|  |  | Mk VI A1 Operational to A1 Training Suit Conversion |  |  |  |  |  |  |
|  |  | Ventilation System & Electronic Control Modules | Tenderer to complete | Tenderer to complete | Tenderer to complete | Tenderer to complete |  |  |
|  |  | Training Visor | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage |  |  |
| 15411-133/RT1 | TBC | EOD Helmet Refurb Tier 1 | Shell paint, straps, pads, ear plugs, chin guard stop block and battery lid | Visual inspection | Strip liner, straps and fittings. Inspect stop block, battery lid and shell paint. Stencil Train | Basic refurbishment level, required for all helmets showing signs of use. |  |  |
| 15411-133/RT2 | TBC | EOD Helmet Refurb Tier 2 | Visor Chin Guard if training visor already fitted | Visual inspection | Perform visor demister test from control power source, to check operation | Faulty demister. Screen chipped, scratched more than 10%, cracked & water ingress. |  |  |
| 15411-133/RT3 | TBC | EOD Helmet Refurb Tier 3 | Battery Box | Visual inspection of battery box and lid | Fill with fresh batteries, plug into control visor to check operation. | Box scratched, chipped or cracked etc. Faulty operation. |  |  |
| 15411-133/RT4 | TBC | EOD Helmet Refurb Tier 4 | Visor Mounts and Catch and covers | Check operation | Use screwdriver to make sure fasteners are tight. | Excessive play, won’t fasten (spring broken), fixtures loose. |  |  |
| 15411-133/RT5 | TBC | EOD Helmet Refurb Tier 5 | Block Latch & Lever Lock | Visual inspection. Inspect operation | Open and close visor | Excessive play, spring broken, rivets broken, block bent or worn. |  |  |
| 15411-133/RT6 | TBC | EOD Helmet Refurb Tier 6 | Box | Visual inspection. Check operation | Open and close lid, checking hinges and locks. Inspect inner foam for damage. | Excessive damage to exterior. Hinges and locks faulty. Interior foam damaged. |  |  |
| 15412-133/RT1 Small 15413-133/RT1 Medium 15414-133/RT1 Large | TBC | Plate Refurbishment | Buckles straps and D-Rings’ Guides and Hinges’ Foam, mesh and paint. | Visual inspection and physical test. | Check operation of the buckles, releasing and clipping in. Check operation of the hinges and guides. Inspect foam, mesh and paint for defects. | Damage to foam and mesh. Faulty or harsh/stiff action. Damage, bent snapped components and cosmetic damage. Ripped, torn, frayed straps. Bent/damaged D rings. Worn/bent or damaged buckle. |  |  |
| 15462-133/RT1 | TBC | Plate Cover | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components |  | Any seam in the cover that needs re-threading. |  |  |
| 15418-133/RT1 | TBC | Jacket Cover Tier 1 | 30 min Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15418-133/RT2 | TBC | Jacket Cover Tier 2 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15417-133/RT1 | TBC | Jacket Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to the waterproof layer. |  |  |
| 15415-133/RT1 | TBC | Jacket Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15415-133/RT2 | TBC | Jacket Outer Tier 2 | Upper Plate Buckles | Check Function with a male connector. Check seams and webbing. |  | Buckle does not function or visually damaged, webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT3 | TBC | Jacket Outer Tier 3 | Lower Plate Buckles | Check Function making sure buckle opens and snaps shut. Check seams and webbing. |  | Buckle does not function or visually damaged, Webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT4 | TBC | Jacket Outer Tier 4 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15423-133/RT1 | TBC | Trouser Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15421-133/RT1 15422-133/RT1 | TBC | Trouser Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 15421-133/RT2 15422-133/RT2 | TBC | Trouser Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 15421-133/RT3 15422-133/RT3 | TBC | Trouser Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15421-133/RT4 15422-133/RT4 | TBC | Trouser Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT1 | TBC | Trouser Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15416-133/RT2 | TBC | Trouser Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT3 | TBC | Trouser Outer Tier 3 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15416-133/RT4 | TBC | Trouser Outer Tier 4 | Buckles | Check operation |  | Damaged, faulty, doesn’t operate or cosmetic damage. |  |  |
| 15420-133/RT1 | TBC | Shorts Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro |  | Any seam in the garment that needs re-threading. |  |  |
| 15419-133/RT1 | TBC | Shorts Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT1 | TBC | Summer Legs Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT2 | TBC | Summer Legs Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 17991-133/RT3 | TBC | Summer Legs Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 17991-133/RT4 | TBC | Summer Legs Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears. Wear or any other damage. |  |  |
| 24244-133/RT1 | TBC | Summer Legs Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 24244-133/RT2 | TBC | Summer Legs Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |

Year 5

| Part Number | NATO Stock Number | Description | Part | Inspection Level | Tests/Operations | Failure Criteria | Firm Price | Firm Delivery Time (from receipt of authorised Tasking Form) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Strip and Survey of EOD Suit |  |  |  |  |  |  |
|  |  | Mk VI A1 Operational to A1 Training Suit Conversion |  |  |  |  |  |  |
|  |  | Ventilation System & Electronic Control Modules | Tenderer to complete | Tenderer to complete | Tenderer to complete | Tenderer to complete |  |  |
|  |  | Training Visor | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage | To be completed at a later stage |  |  |
| 15411-133/RT1 | TBC | EOD Helmet Refurb Tier 1 | Shell paint, straps, pads, ear plugs, chin guard stop block and battery lid | Visual inspection | Strip liner, straps and fittings. Inspect stop block, battery lid and shell paint. Stencil Train | Basic refurbishment level, required for all helmets showing signs of use. |  |  |
| 15411-133/RT2 | TBC | EOD Helmet Refurb Tier 2 | Visor Chin Guard if training visor already fitted | Visual inspection | Perform visor demister test from control power source, to check operation | Faulty demister. Screen chipped, scratched more than 10%, cracked & water ingress. |  |  |
| 15411-133/RT3 | TBC | EOD Helmet Refurb Tier 3 | Battery Box | Visual inspection of battery box and lid | Fill with fresh batteries, plug into control visor to check operation. | Box scratched, chipped or cracked etc. Faulty operation. |  |  |
| 15411-133/RT4 | TBC | EOD Helmet Refurb Tier 4 | Visor Mounts and Catch and covers | Check operation | Use screwdriver to make sure fasteners are tight. | Excessive play, won’t fasten (spring broken), fixtures loose. |  |  |
| 15411-133/RT5 | TBC | EOD Helmet Refurb Tier 5 | Block Latch & Lever Lock | Visual inspection. Inspect operation | Open and close visor | Excessive play, spring broken, rivets broken, block bent or worn. |  |  |
| 15411-133/RT6 | TBC | EOD Helmet Refurb Tier 6 | Box | Visual inspection. Check operation | Open and close lid, checking hinges and locks. Inspect inner foam for damage. | Excessive damage to exterior. Hinges and locks faulty. Interior foam damaged. |  |  |
| 15412-133/RT1 Small 15413-133/RT1 Medium 15414-133/RT1 Large | TBC | Plate Refurbishment | Buckles straps and D-Rings’ Guides and Hinges’ Foam, mesh and paint. | Visual inspection and physical test. | Check operation of the buckles, releasing and clipping in. Check operation of the hinges and guides. Inspect foam, mesh and paint for defects. | Damage to foam and mesh. Faulty or harsh/stiff action. Damage, bent snapped components and cosmetic damage. Ripped, torn, frayed straps. Bent/damaged D rings. Worn/bent or damaged buckle. |  |  |
| 15462-133/RT1 | TBC | Plate Cover | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components |  | Any seam in the cover that needs re-threading. |  |  |
| 15418-133/RT1 | TBC | Jacket Cover Tier 1 | 30 min Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15418-133/RT2 | TBC | Jacket Cover Tier 2 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15417-133/RT1 | TBC | Jacket Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to the waterproof layer. |  |  |
| 15415-133/RT1 | TBC | Jacket Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15415-133/RT2 | TBC | Jacket Outer Tier 2 | Upper Plate Buckles | Check Function with a male connector. Check seams and webbing. |  | Buckle does not function or visually damaged, webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT3 | TBC | Jacket Outer Tier 3 | Lower Plate Buckles | Check Function making sure buckle opens and snaps shut. Check seams and webbing. |  | Buckle does not function or visually damaged, Webbing pulling off jacket at seams, webbing cut or fraying. |  |  |
| 15415-133/RT4 | TBC | Jacket Outer Tier 4 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15423-133/RT1 | TBC | Trouser Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro. |  | Any seam in the garment that needs re-threading. |  |  |
| 15421-133/RT1 15422-133/RT1 | TBC | Trouser Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 15421-133/RT2 15422-133/RT2 | TBC | Trouser Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 15421-133/RT3 15422-133/RT3 | TBC | Trouser Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15421-133/RT4 15422-133/RT4 | TBC | Trouser Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT1 | TBC | Trouser Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 15416-133/RT2 | TBC | Trouser Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |
| 15416-133/RT3 | TBC | Trouser Outer Tier 3 | Zip | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 15416-133/RT4 | TBC | Trouser Outer Tier 4 | Buckles | Check operation |  | Damaged, faulty, doesn’t operate or cosmetic damage. |  |  |
| 15420-133/RT1 | TBC | Shorts Cover Tier 1 | Cut and Sew | Check every seam, zip and Velcro |  | Any seam in the garment that needs re-threading. |  |  |
| 15419-133/RT1 | TBC | Shorts Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT1 | TBC | Summer Legs Filler Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and waterproof panels. |  | Any seam in the garment that needs re-threading. Damage to waterproof layer. |  |  |
| 17991-133/RT2 | TBC | Summer Legs Filler Tier 2 | Brace Replacement | Check brace elastic, joins, seams and buckles. Check the join onto trousers. |  | Rips, tears and cosmetic damage to straps, damaged/faulty buckles. Join to trousers torn. |  |  |
| 17991-133/RT3 | TBC | Summer Legs Filler Tier 3 | Zip Replacement | Check the operation of the zip |  | Visual damage, zip doesn’t function or is notchy and irregular. |  |  |
| 17991-133/RT4 | TBC | Summer Legs Filler Tier 4 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears. Wear or any other damage. |  |  |
| 24244-133/RT1 | TBC | Summer Legs Outer Tier 1 | Cut and Sew | Check every seam, zip, Velcro, and auxiliary components such as pockets |  | Any seam in the garment that needs re-threading. |  |  |
| 24244-133/RT2 | TBC | Summer Legs Outer Tier 2 | Hem Reinforcement | Inspect the hems at the bottom of each trouser leg. |  | Rips, tears, wear or any other damage. |  |  |