##### AB

.

Construction (Design and Management) Regulations 2015

PRE CONSTRUCTION INFORMATION PACK Draft V1

Demolition and associated Works

Northminster Car Park

Peterborough

.

Prepared by:

Peterborough City Council

Health and Safety Team

1.0 DESCRIPTION OF PROJECT

1a - Project Description and Programme Details

1b - Project Directory – Key Participants

1c - Workplace (Health, Safety and Welfare) Regulations 1992

1d - Existing Information, Plans and Records

2.0 CLIENT CONSIDERATIONS AND MANAGEMENT

2a - Arrangements

2b - Third Party Considerations

3.0 ENVIRONMENTAL RESTRICTIONS AND EXISTING RISKS

3a - Safety Hazards

3b - Health Hazards

4.0 SIGNIFICANT DESIGN AND CONSTRUCTION HAZARDS

4a - Design Assumptions, Work Methods and Control Measures

4b - On-going Design Work

4c - Significant Risks

4d - Materials Requiring Special Precautions

5.0 THE HEALTH & SAFETY FILE

5a - Format and Contents

1.0 DESCRIPTION OF PROJECT

1a - Project Description and Programme Details

Site Information

The details of the site to which this Pre-Construction Information relates are as follows:

Peterborough City Council

Northminster Multistorey Car Park

Peterborough

PE1 1HB

Nature of the Project

The works comprise of Suicide Prevention Scheme in the form of proprietary barrier installation on all floors including the roof deck.

Sectional Completion Details / Phasing Arrangements

The contractor is to submit a bar chart programme indicating the demolition phasing of the works.

There will be no other contracts associated with this site during the course of the demolition and associated works

Planning and Preparing for Construction

A minimum of 4 weeks is provided to allow the Principal Contractor adequate time to plan for

Construction works.

1.0 DESCRIPTION OF PROJECT

1b - Project Directory – Key Participants

| Company Name | Address | Phone Number | Contact |
| --- | --- | --- | --- |
| Client |  |  |  |
| Peterborough City Council | The Eco Innovation Centre, Peters Court, Peterborough | 07715 802489 | Stuart Macdonald |
| E-Mail Address: stuart.macdonald@peterborough.gov.uk |  |  |  |

| Principal Designer (PD). |  |  |  |
| --- | --- | --- | --- |
| NPS | The Eco Innovation Centre, Peters Court, Peterborough | 07813 785953 | Rebecca Close |
| E-Mail Address: rebecca.close@peterbrough.gov.uk |  |  |  |

| Client Appointed Safety Adviser |  |  |  |
| --- | --- | --- | --- |
| Peterborough City Council | Town Hall Bridge Street Peterborough PE1 1HQ > | 01733 563526 | Andy Baker |
| E-Mail Address: andy.baker@peterborough.gov.uk |  |  |  |

| Principal Contractor |  |  |  |
| --- | --- | --- | --- |
| ******** | <Address Line 1> <Address Line 2> <Address Line 3> <Address Line 4> | ************ | ************ |
| E-Mail Address: ************ |  |  |  |

1.0 DESCRIPTION OF PROJECT

1c - Workplace (Health, Safety and Welfare) Regulations 1992

Relevant Requirements

The completed works will not be used as a workplace; consequently the finished design will not need to comply with the requirements of the Workplace (Health, Safety and Welfare) Regulations 1992.

1.0 DESCRIPTION OF PROJECT

1d - Existing Information, Plans and Records

Existing Information

The following information has been provided in relation to the works:

Type							Document Location

Asbestos Register/ Survey				Tender Documentation

Existing drawings					Tender Documentation

Structural Survey / Report				Tender Documentation

This information must be considered as a base of information only and is to be checked on site for accuracy, and not relied upon.

Existing Health & Safety File

There is not an Existing Health and Safety File for this Site, so far as we are aware

2.0 CLIENT CONSIDERATIONS AND MANAGEMENT

2a – Arrangements

Planning and Management of Construction

The Principal Contractor will be responsible for the planning and management of the demolition phase and associated works. Prior to commencement of works, the Principal Contractor will prepare a Health and Safety Plan for submission.

All stages of planning and management of the works should be undertaken giving due regard to health and safety, through liaison with; and engagement of, the Client Appointed Safety Adviser (CASA). The CASA will review the development of the Health and Safety Plan, and will confirm when it is adequate, in order for works to commence.

During the demolition and associated works phase, the Principal Contractor will review and update the HS Plan and make available any part of the HS plan to contractors as is necessary.

Health and Safety Goals

The main health and safety objective is to ensure that adequate actions and precautions are to be taken to prevent harm being caused to those carrying out demolition and associated works and others who may be affected.

Communication and Liaison

The Principal Contractor is to detail his proposals for the following items of site management;

- The method, and by whom, that health and safety information will be conveyed to all contractors and site operatives, and for monitoring the proposed systems, i.e. regular site health and safety inspections.

2.0 CLIENT CONSIDERATIONS AND MANAGEMENT

2a – Arrangements

- How sub-contractors are to be co-ordinated*

- Arrangements for consultation with employees and sub-contractors in accordance with current legislation.

- To maintain a named supervisor on site whenever works are being carried out and supply a named contact officer and emergency telephone number, primarily for health and safety purposes.

- To confirm procedures for dealing with accidents, ill health and dangerous occurrences arising from the work and associated activities and their notification to the Health and Safety Executive (HSE) under the Reporting of Injuries, Diseases and Dangerous Occurrences Regulations 2013 (RIDDOR).

The Principal Contractor will also ensure that;

- All communications in respect of the works will be channelled through the Client/ representative.

- A site induction is carried out for all personnel / operatives attending site.

- Adequate information, instruction and training are received by all operatives as required depending on the activity.

- The Client/ representative  and / or CASA are contacted on the above numbers at all times.

- Close liaison is achieved between all parties where the proposed works will affect the means of escape plans, deliveries to site, parking etc.

- PC, or a representative, will attend the initial pre-start meeting, and any further required progress meetings organised by the Client/ representative

- PC and any site operatives to fully co-operate with any personnel attending site for the purpose of inspection, e.g. the CASA

- All relevant and requested documentation, such as risk assessments and method statements (RAMS), be available within the site office for inspection or review.

* The Principal Contractor should note that sub-contractors must supply risk assessments and method statements (RAMS) for any hazard in respect of their activities which may be required for inclusion within the Health and Safety Plan. Where sub-contractors have not yet been appointed at the start of the demolition phase, the Principal Contractor, in his Health and Safety plan, shall detail his selection criteria for those sub-contractors. When selected, the Principal Contractor should update his HS Plan with their risk assessments and method statements (RAMS) once appointed.

Site Security

The site is situated within a busy city centre. The Principal Contractor is to ensure there is a high level of security in place around his designated site to prevent access by unauthorised persons during the course of works, particularly out of working hours.

The Principal Contractor will be responsible for security of the site throughout the contract and is expected to provide details of his proposals as far as are reasonably practicable. Regard should be given to the following;

- Securing equipment outside of working hours

- Risk of trespass by unauthorised persons onto working areas of the site

- Method of access and parking contractors vehicles /  plant.

Welfare Provision

Welfare arrangements are to be in accordance with Schedule 2, Regulation 4(2), 13(4) and 15(11) of the Construction (Design and Management) Regulations 2015 and commensurate with the site undertaking.

The Principal Contractor is to confirm proposed arrangements and location for site welfare facilities, which should consider and include the following;

- Sanitary convenience.

- Washing facilities, to include hot and cold running water.

2.0 CLIENT CONSIDERATIONS AND MANAGEMENT

2a – Arrangements

- Secure storage for clothing and PPE.

- Adequate supply of drinking water.

- Detail arrangements for first aid, including name and qualifications of site first aider and emergency arrangements.

- Fire evacuation procedures (see 2b below)

The existing premises welfare facilities are available??????

Site Access and Egress Points

Access to the site will be (See site/ location traffic management plan ref: ************).

The Principal Contractor will as part of his submission supply a site Traffic Management plan taking into account the specific risks on site. Vehicular traffic and pedestrian movement and their impact on safety will need to be taken on an ongoing day to day basis throughout the works.

Safe access and egress is to be maintained at all times and appropriate warning signs must be displayed.

Location of Temporary Site Accommodation, Unloading Areas etc.

The Principal Contractor’s site area and layout of temporary site accommodation, material storage etc, is to be agreed on site with the Client / representative , prior to set up of site. The Principal Contractor will also have to identify the location of any unloading, parking, location of skips. And  appropriate warning signs to be displayed.

Site Rules

(To be agreed with the Client / representative prior to works commencing).

No smoking Site

The Principal Contractor will be expected to display and communicate site rules through appropriate signage and site inductions. All visitors to site should be inducted into these rules and made to sign the site register.

Correct Personal Protective Equipment (PPE) and Respiritory Protective Equipment (RPE)should be worn at all times by all those on site, and enforced if necessary.

Horseplay, abusive language nor offensive banter will not be tolerated

2.0 CLIENT CONSIDERATIONS AND MANAGEMENT

2b - Third Party Considerations

Localised Hoarding Requirements

Suitable solid close boarded hoardings, no lower than 2000 mm high, shall be in place prior to commencement of site works to ensure protection of works from all possible unauthorised access.

Externally, (see drawing ref:*******************************)

All fencing should be maintained and inspected throughout the project.

‘No Go’ Areas and Other Authorisation Requirements

Where the works require the Principal Contractor to work outside his agreed site boundary or out of hours, this is to be agreed in advance with the Client / representative giving at least 48 hours notice.

Site Transport Arrangements and Vehicle Movement Restrictions

The Principal Contractor will be responsible for the safe management of vehicle and pedestrian movements during the contract. A Traffic Management Plan will need to be prepared by the Principal Contractor, and agreed with the Client / representative, prior to the commencement of works. Things to be considered are;

2.0 CLIENT CONSIDERATIONS AND MANAGEMENT

2b - Third Party Considerations

- Maintaining access and egress to the existing car park, as per programme,

- Ensuring emergency and fire exit routes are kept clear

- Timings of vehicle movements are to be agreed in advance with the Client / representative, taking into account peak times for access and egress from the premises.

Fire Precautions

The Principal Contractor will assume total responsibility for the setting up, maintaining and monitoring of all fire preventative measures on site. He must ensure that suitable fire protection equipment is in place throughout the construction works to ensure no risk of damage. To this end the Principal Contractor should prepare a suitable Fire and Emergency Plan, and submit it to the Client / representative, for approval prior to work.

Emergency Procedures and Means of Escape

The Principal Contractor is responsible for having in place a detailed Emergency Procedure for the site which should be updated during the various stages of demolition and levels of risk involved through such. Means of escape as the construction forms must also be protected to provide swift egress from the building in the event of an emergency. The Emergency Plan should be submitted to the Client / representative, for approval prior to work.

The Principal Contractor should always ensure that any compounds, fencing etc does not in any way restrict the emergency evacuation of any persons from the premises.

Parking Restrictions

The Principal Contractor is to ensure that all construction site vehicles are restricted to only parking within the agreed designated areas.

Site Visitors

All visitors to site should be inducted to the site rules and made to sign the site register. Correct Personal Protective Equipment (PPE) should be worn at all times, and enforced if necessary. The Principal Contractor is not expected to supply visitors with PPE.

Nuisances/ Disturbance

The Principal Contractor will take all reasonable steps to avoid excessive noise, dust and other disturbances. The Principal Contractor is to give the Client / representative, notice of any exceptional disturbance and hazardous procedures, and continually liaise with the Client / representative for the management of any risks to the general public.

3.0 ENVIRONMENTAL RESTRICTIONS AND EXISTING RISKS

3a - Safety Hazards

Boundaries and Permanent / Temporary Access

Consideration should be given to the parking of site operative’s vehicles due to the restrictions of, and only using, the contractor’s allocated site.

Risk assessment and method statement (RAMS) to be provided with respect to operating in a live car park.

DEMOLITION

The Principal Contractor will include in his tender submission a risk assessment and method statement (RAMS)  including the following;

- Evidence of competency of Principal Contractor and any nominated sub contractor

- The Principal Contractor’s written health and safety policy statement

- An assurance that the works will be carried out in a safe and acceptable standard, in accordance with BS Code of Practice for Demolition.

3.0 ENVIRONMENTAL RESTRICTIONS AND EXISTING RISKS

3a - Safety Hazards

- A list of senior staff who will be involved with the project with details of their qualifications and experience.

- Evidence of sufficient demolition insurance cover.

- Measures for the mitigation of foreseeable significant hazards.

- Measures to restrict foreseeable nuisances e.g. dust, noise, vibration.

In relation to vibration any other information related to equipment that may cause vibration, affecting neighbouring structures*********

Waste

A copy of the Principal Contractor’s disposal licence, or disposal arrangements, must be available on site for inspection.

Delivery, Collection or Storage Restrictions

Collection / Delivery Times: No deliveries to site or other major vehicle movement are permitted during these times.

Deliveries and collections that are to be carried out outside the vehicle movement restriction times must be agreed with the Client / representative, prior to them commencing.

Existing Structures Fabric and Condition Information/  Pre-stressed or Post-tensioned Structures

See Structural Engineers Report

PC  to DETAIL AS APPROPRIATE information about existing structures - stability, structural form, , anchorage points for fall arrest systems

Existing Services (Utilities)

All existing services and temporary services brought on to site to be managed appropriately by competent persons

(see drawing ref:*******************************)

Plant and Equipment

The Principal Contractor will be responsible for securing the site out of working hours and preventative measures to ensure restricted access to site equipment by unauthorised persons.

Any plant or equipment brought on to site by the Principal Contractor, whether hired or the contractors own should be;

- Suitable for its intended use

- Be safe for use

- Maintained in a safe condition and inspected to ensure this remains the case

- Should only by used people who have received adequate information, instruction and training

- Should be accompanied by suitable safety measures, e.g. protective devices, markings, warnings, etc.

- Copies of certification and maintenance records where appropriate, should be available for inspection or review.

PC to DETAIL AS APPROPRIATE any difficulties relating to plant and equipment in the CAR PARK, height restricts access. also issues to be considered with consideration to plant & equipment used during construction

3.0 ENVIRONMENTAL RESTRICTIONS AND EXISTING RISKS

3b - Health Hazards

Asbestos Containing Materials

In compliance with The Control of Asbestos Regulations, the Principal Contractor must refer to the Asbestos Information provided which will be provided

The Principal Contractor should ensure that all operatives are informed, prior to the commencement of works, that it may be present, and if discovered the Client/ representative must be informed immediately and it’s safe removal planned and carried out, in accordance with HSE recommendations

4.0 SIGNIFICANT DESIGN AND CONSTRUCTION HAZARDS

4a - Design Assumptions, Work Methods and Control Measures

The Principal Contractor, where applicable, shall take cognisance of information provided by all stakeholder and develop safe systems of working. Risk Assessments and Method Statements (RAMS) shall be incorporated within the Health and Safety Plan prior to executing any particular activity.

4.0 SIGNIFICANT DESIGN AND CONSTRUCTION HAZARDS

4b - On-going Design Work

Under CDM, each element of design including temporary works, designed access equipment, individual specialist contractor and specialist designer packages, is required to be developed with due consideration for the health and safety of all concerned both during construction and subsequent occupancy and maintenance.

Each element of design is to be co-ordinated for health and safety aspects with Client / representative.

4.0 SIGNIFICANT DESIGN AND CONSTRUCTION HAZARDS

4c - Significant Risks

The Principal Contractor should address the following significant hazards associated with the design. It is essential that the Construction Phase Health & Safety Plan properly addresses the issues related below in the form of safe systems of work before the work actually commences on site.

Summary of Significant Risks:

The Principal Contractor should address the following significant hazards associated with the design. It is essential that the Construction Phase Health and Safety Plan properly addresses the issues related below in the form of safe systems of work before the work actually commences on site.

WORKING AT HEIGHT / ROOF ACCESS WORK

Suitable safety systems must be incorporated within the Principal Contractor’s Health and Safety Plan to protect personnel carrying out such work, and must comply with The Work at Height Regulations 2005. The Principal Contractor will include in his tender submission details of;

- How a safe place of work will be provided and the methods of protection, e.g. safety nets

- How falls are to be prevented

- How danger to those below and to the public from falling materials is to be controlled

- What equipment will be used

- What competence / training is needed

4.0 SIGNIFICANT DESIGN AND CONSTRUCTION HAZARDS

4c - Significant Risks

All scaffolding shall be installed by a competent person who is CITB approved.  The Principal Contractor should check and ensure that evidence of competency and records of all scaffold inspections are available on site. The Principal Contractor is to provide a method statement describing type, extent, erection, maintenance and removal of scaffolding.

If lifting equipment is to be used please refer to section ‘LIFTING ON SITE’ below.

CONTACT WITH OTHERS

Interaction with the general public and existing site users must be reduced with suitable health and safety systems in place to restrict access to the construction area from possible access points from within the overall site. Similar consideration must also be given to safely plan the movement of deliveries and waste disposal to and from the site so as not to impact on car park users.

HAZARDOUS MATERIALS - See also Section 4d

The Principal Contractor is to carry out a suitable and sufficient risk assessment/s for all activities as defined under the Control of Substances Hazardous to Health Regulations 2002 (COSHH). The Principal Contractor should ensure that all substances are accompanied by a corresponding Safety Data Sheet and that all operatives have been made aware of any dangers. All flammable materials are to be suitably and securely stored.

.

The Principal Contractor must satisfy himself of any information he may require to ensure the safety of all persons and the works as the accuracy and sufficiency of information provided by the employer can not be guaranteed.

LIFTING ON SITE

All processes involving lifting within the construction must be suitably controlled and planned to reduce the risks inherent in such operations. A lifting plan should be prepared, as appropriate, for complicated lifts using mechanical aids, such as cranes. The Principal Contractor must ensure that the Lifting Operations and Lifting Equipment Regulations (LOLER) 1998 are complied with.

All manual-handling activities should also be risk assessed as required under the Manual Handling Regulations 1992 (see main contract documents for length and weights of structural components).

4.0 SIGNIFICANT DESIGN AND CONSTRUCTION HAZARDS

4d - Materials Requiring Special Precautions

Any materials that are to be incorporated within the construction, which may present a risk to those who will work with them, must not be utilised without accompanying documentation, training and information.

This includes Material Data Sheets which should be presented to all personnel, so to give an awareness of risks involved. Substances that are to be utilised must be backed up by suitable COSHH Assessments.

5.0 THE HEALTH & SAFETY FILE

5a - Format and Contents

| Item Required | No. of Copies | Supplied By (If Applicable) | Required (Yes/No) |
| --- | --- | --- | --- |
| Services Location and details of all new incoming services mains (water, gas, electricity, BT). | 1 | DESIGNER PRINCIPAL CONTRACTOR |  |
| # Existing Environment - Original Drawings | 1 | DESIGNERS |  |
| Contract Details - Contract Commencement - Date of Practical Completion - End of Defects Liability Period - Certificate of Practical Completion | 1 | CLIENT / REPRESENTATIVE |  |
| # Design Information - Specification Details - As-built Drawings or Certified As-Built | 1 | DESIGNERS / PRINCIPAL CONTRACTOR |  |
| # Project Participants - List of all domestic and named sub- contractors - List of all direct contractors | 1 | CLIENT REPRESENTATIVE/ PRINCIPAL CONTRACTOR |  |
| # Mechanical - As-built drawings - Health and Safety information - De Commissioning and Inspection Certificates | 1 | CONTRACTOR |  |
| # Electrical - As-built drawings - De Commissioning and Inspection certificates | 1 | CONTRACTOR |  |
| # Fire and Emergency - Fire retardancy certificates e.g. test certificates for upholstery fabrics, curtains, carpets etc. - Fire Plan Drawing | NA | NA | NA |
| # Product Information - Product data sheets for materials requiring a COSHH assessment | NA | NA | NA |
| # Hazards in relation to future work - Residual hazards left on completion of the works | 1 | PRINCIPAL CONTRACTOR DESIGNERS |  |