BUSHBOARD WASHROOMS

NBS SPECIFICATION

PROJECT:		GREAT DUNMOW PUBLIC WCS

SPECIFIER:		EMMERSON BARNETT

Prepared by:	DAVID HARLEY

28 July 2026

K32		PANEL CUBICLES

To be read with Preliminaries/General conditions.

120/01 	FULLY FRAMED PANEL CUBICLES

Manufacturer: Bushboard Washroom Systems, Unit 1400, 1st Floor Montagu Court, Kettering Parkway, Kettering Venture Park, Kettering, Northants, NN15 6XR, Tel: 01536 533620

www.bushboard-washrooms.co.uk

  - Product Reference: Quadro SGL

Frame: Robust 13mm thick pilasters.

  - Finish: Solid grade laminate.

  - Colour: Refer to Bushboard standard colour range.

Panels / Doors / Partitions:

  - Overall Height: Pilasters 2100mm.

  - Floor Clearance: 100mm to Doors and partitions.

  - Core Material: 13mm Solid Grade Laminate

  - Finish: Decorative facing to both sides

  - Edge Treatment: Polished black radiused edges

  - Colour: Refer to Bushboard standard colour range.

Ironmongery / Accessories:

  - Headrail / Pilaster Shoe: Satin anodised aluminium pilaster shoe and head-rail in Light Grey.

  - Indicator Bolt: ABS lock body with nylon slider and soft feel door buffer knob in Light Grey.

  - ABS lock face-plate with external emergency release function.

  - Hinges / Channels: Satin anodised anti-finger-trap hinges and wall channels in Light Grey

Other Requirements:

-     Through Fixings to hinges

K32		PANEL CUBICLES/DUCT AND WALL LININGS/SCREENS

To be read with Preliminaries/General conditions.

160/01 	PREPLUMBED PANELS AND PROPRIETARY FRAMES

Manufacturer: Bushboard Washroom Systems, Unit 1400, 1st Floor Montagu Court, Kettering Parkway, Kettering Venture Park, Kettering, Northants, NN15 6XR, Tel: 01536 533620

www.bushboard-washrooms.co.uk

  - Product Reference: Ready Plumbed Module (RPM) SGL Pre-plumbed Washroom System

Panels

  - Board / Panel Type: Solid grade laminate.

  - Thickness: 13mm overall.

  - Core Material: Solid grade laminate.

  - Facing: Solid grade laminate.

  - Edge Treatment: Polished black radiused edges.

-     Colour: Refer to Bushboard standard colour range.

Installation:

  - Method of Fixing Panels: As drawing.

Included Features:

-     Rigid lightweight “Ezeeduct” aluminium frame, access panels and sanitary-ware all preassembled with associated plumbing and brassware.

Accessories:

-    Lift-off, push fit clips and all associated fixings and brackets.

Other Requirements:

-     Hinges and Duct locks.

-     SGL Flash-gaps

N13		SANITARY APPLIANCES AND FITTINGS

To be read with Preliminaries/General conditions.

300/01 	PRE-PLUMBED WCS AND CISTERNS

Type: Back to wall, concealed cistern

Manufacturer: Bushboard Washroom Systems, Unit 1400, 1st Floor Montagu Court, Kettering Parkway, Kettering Venture Park, Kettering, Northants, NN15 6XR, Tel: 01536 533620

www.bushboard-washrooms.co.uk

IPS Panel System: As clause 160/01. Sanitary appliances pre-fixed and sealed to RPM unit by Bushboard Washroom Systems.

WC:

Manufacturer : Wallgate

-     Product reference: BTW WC pan ref: CWC-250-AST

Material: Solid Surface, white.

Seat and Cover: Integral moulded solid surface seat

Pan Connector:

  - Standard: To BS 5627

  - Manufacturer: client choice

  - Product Reference: client choice

  - Colour: White.

Cistern:

  - Product Reference: 6ltr pneumatic valve cistern for sensor control

  - Material: Plastic

  - Colour and finish: Not applicable

Flushing Arrangement:

  - Product Reference: No Touch Sensor flush

  - Operating control: Sensor flush

  - Water supply connection: As recommended by manufacturer

  - Flush volume: 6ltr

Flush pipe: Concealed

  - Manufacturer: As cistern manufacturer

  - Product Reference: As cistern manufacturer

  - Material: Plastic

Accessories: Floor fixing kit supplied by manufacturer

N13		SANITARY APPLIANCES AND FITTINGS

To be read with Preliminaries/General conditions.

315/01 	PRE-PLUMBED URINALS AND AUTO FLUSHING CISTERNS

Manufacturer: Bushboard Washroom Systems, Unit 1400, 1st Floor Montagu Court, Kettering Parkway, Kettering Venture Park, Kettering, Northants, NN15 6XR, Tel: 01536 533620

www.bushboard-washrooms.co.uk

IPS Panel System: As clause 160/01. Sanitary appliances pre-fixed and sealed to RPM unit by Bushboard Washroom Systems.

  - Product reference: Concealed trap urinal ref: CWU-01

Manufacturer: Wallgate

  - Material: Solid Surface, white.

Size: 435mm H x 310mm W x 270mm D

Waste:

  - Standards: To BS EN 274-1, 2 & 3

  - Manufacturer: Sanceram

  - Product Reference:  URIN116

  - Size 1 ½”

  - Material: Plastic

  - Tail: Unslotted

Traps:

  - Standards: To BS EN 274-1, 2 & 3

  - Manufacturer: McAlpine

  - Product Reference: P trap ref L10 and elbow ref T1M

  - Size: 1 ½”

  - Material: Plastic

  - Depth of seal: 75mm

Cistern, complete with automatic siphon, petcock and lid

  - Manufacturer: SanCeram

  - Product Reference: Concealed auto cistern and fittings.

  - Material: Plastic

  - Flush volume: 4.5ltr

  - Operating control: Infra-red urinal control kit with sensor ref: URIN109

Accessories:

  - Air-break ref: URIN109

N13		SANITARY APPLIANCES AND FITTINGS

To be read with Preliminaries/General conditions.

335/01 	WASH BASINS WITH INTEGRAL DRYER AND SOAP DISPENSER

Manufacturer: Wallgate Ltd. Crow Lane, Salisbury, Wiltshire, SP2 0HB, Tel: 01722 744594

www.wallgate.com

- Product reference: Thrii All in one Hand Wash Dryer ref: THRII-SS

Size 450mm W x 870 H mm

Material: Solid Surface

ACCESSIBLE WC

K32		PANEL CUBICLES/DUCT AND WALL LININGS/SCREENS

To be read with Preliminaries/General conditions.

160/01 	PREPLUMBED PANELS AND PROPRIETARY FRAMES

Manufacturer: Bushboard Washroom Systems, Unit 1400, 1st Floor Montagu Court, Kettering Parkway, Kettering Venture Park, Kettering, Northants, NN15 6XR, Tel: 01536 533620

www.bushboard-washrooms.co.uk

  - Product Reference: Ready Plumbed Module (RPM) SGL Pre-plumbed Washroom System

Panels

  - Board / Panel Type: Solid grade laminate.

  - Thickness: 13mm overall.

  - Core Material: Solid grade laminate.

  - Facing: Solid grade laminate.

  - Edge Treatment: Polished black radiused edges.

-     Colour: Refer to Bushboard standard colour range.

Installation:

  - Method of Fixing Panels: As drawing.

Included Features:

-     Rigid lightweight “Ezeeduct” aluminium frame, access panels and sanitary-ware all preassembled with associated plumbing and brassware.

Accessories:

-    Lift-off, push fit clips and all associated fixings and brackets.

Other Requirements:

-     Hinges and Duct locks.

-     SGL Flash-gaps

N13		SANITARY APPLIANCES AND FITTINGS

To be read with Preliminaries/General conditions.

311/01 	PRE-PLUMBED BACK TO WALL DOC M PACK

Manufacturer: Bushboard Washroom Systems, Unit 1400, 1st Floor Montagu Court, Kettering Parkway, Kettering Venture Park, Kettering, Northants, NN15 6XR, Tel: 01536 533620

www.bushboard-washrooms.co.uk

  - Product Reference: Doc M Rimless BTW  Pack

  - IPS Unit: As clause 160/01. Sanitary appliances pre-fixed and sealed to RPM unit by Bushboard Washroom Systems.

  - Product Description: Back to wall rimless pan with lever operated flush, handed basin with sequential mixer tap, waste and trap, and all required grab rails and support arms.

  - Product Code(s): SCDMBTW

  - White: LW/RW

  - Blue: LB/RB

  - Stainless Steel: LSS/RSS

  - Dark Grey: LG/RG