RFQ049 Appendix B2 Lot 2 Chemistry Equipment

Please find the specifications for the following equipment

### Portable Balance (500g) 0.01g

- Nuclear Magnetic Resonance Spectrometer

- Mass spectometer

- Alpha-P FTIR Spectrometer

- Biochrom WPA Lightwave II  80-3003-72  + software

Portable Balance

- RS232 Interface

- Weighing modes: Grams and Newtons

- Multiple application modes and units

- Weigh-below hook

- Integral security bracket

- integral menu lock

- AC Adaptor

Nuclear Magnetic Resonance Spectrometer

- 60 MHz rare-earth permanent magnet

- Cryogen-free system

- Benchtop NMR system - no dedicated room required

- Fully auto shimming - no manual shimming required

- 19F or 1H measurements on single probe

- 13C option

- Software and hardware locks to give optimised performance for every experiment

- 2D homo- and heteronuclear sequences including COSY, J-resolved, TOCSY, DEPT and HMQC

- Perpetual license for Mnova software from Mestrelab

- Perfect for teaching departments - easily operated by undergraduate students

Mass spectrometer

- Type: MAX-500HT

- Mass Range: 1-500amu

- Relative Transmission 60%

- Resolution (M/ΔM FWHM): 2000

- Sensitivity (mA/Torr): 2

Comprehensive range of application specific configurations for atomic and molecular physics and chemistry experiments including;

- Temperature programmed desorption

- Static and dynamic SIMS/SNMS

- Laser molecule interactions

- Photoionization

- UHV and XHV Outgassing studies

- Molecular beam studies

- Helium scattering

- Cluster analysis

19mm triple filter quadrupole rod set, delivering far superior sensitivity, abundance sensitivity, transmission and resolution characteristics than the smaller quadrupole rod sets available

- 9.5mm or 19mm high transmission and resolution triple filter quadrupole

- Mass range options from 60amu to 16,000amu

- Positive ion and negative ion measurement

- Appearance potential and electron attachment modes for neutrals and radicals

- Dual analogue and ion counting detector for highest dynamic range

- Conversion dynode

- Pole bias mid-axis potential

- Einzel lens optics

- Axial and right angle sampling configurations

- 1E-16mbar detection capability

- Tandem source with high transmission energy filter with <0.5eV energy resolution

- Photoionization cross beam source

- Enclosed CI source

Alpha-P FTIR Spectrometer

- One reflection diamond crystal

- Small footprint 22x30cm

- Spectral range 375-7,500cm-1

- One finger clamp mechanism

- Pressure Control

- max Sample height 20mm

- Easy cleaning

- Exchangable ATR crystal plate with electronic recognition

- Spectral resolution better than 2cm-1

- RockSolid Interferometer , gold mirrors

- Detector DTGS

- Source SiC globar

- Opus/Mentor software package

- Opus/Search software in spectra libraries

- Warranty Extension for 10 years for ATR module, Interferometer, and Laser

- 5 Year Warranty Extension for MIR source

- 12 months on-site warranty, User training, Delivery, Installation

- Dimensions	22cmx30cm

Biochrom WPA Lightwave II 80-3003-72  + software

- Min wavelength: 190nm

- Max wavelength: 1100nm

- Bandwidth: 5nm

- Scanning: yes

- Beam format: Split

- Cell holder: Single

- Optical System Monochromator Holographic Grating

- Detector 1024 CCD array

- Photometric Range -0.300 to 2.500A, 0 to 199%T

- Photometric Accuracy +/- 0.008Abs or 1.3% of the reading, whichever is greater between 0 and 2.5A

- Photometric Reproducibility 0.002A at 0 - 0.5A at 546nm

Dimensions

- Width	260mm

- Depth	390mm

- Height	100mm