| Orchard Street Kidderminster Worcestershire DY10 2JA |  |  |  |
| --- | --- | --- | --- |

Project Scope

Planned maintenance & external refurbishment works at Arch Hill Court & Orchard Street.

The buildings are located in Orchard Street, Kidderminster, DY10 2JA.

Arch Hill Court and Orchard Street together provide retirement and extra care living accommodation in the town of Kidderminster.

Arch Hill Court, the main building, is a 2 storey building containing 38 individual flats, a communal dining hall and various communal spaces.

Orchard Street comprises 13 identical blocks each containing 2 ground floor and 2 first floor flats.

The refurbishment project will briefly comprise:

At Arch Hill

- Overhaul roof(s). Carry out any localised roof repair / replacement tiles in patch (Assume 5% repair/replacement tiles)

- Supply & fit Solar Panel fixing rails to suit 3 full size solar panels (solar to be provided by others).

- Install dry hip, ridge, valley & verge systems

- Replacement roofline, 1m strip and reinstate tiles and install a GRP eaves protector.

- Remove existing, profiled section box gutters and replace creating a soffit & facia and install a deeplflow gutter system.

- Take out & refix lead flashing

- Remove existing & renew high level cladding (in hardy plank or similar system) to manufacturers specification, compartmentation and fire regulations to be considered, observed, and if/where necessary addressed and colours to be decided by customer consultation,

At Orchard St

There are 13 blocks in total, each block identical, one end has a hipped roof and the other a gable.

Each block comprises 2 small wooden porches, 2 larger wooden porches and 2 white PVCu porches.

The refurbishment project will briefly comprise:

- Overhaul roof(s). Carry out any localised roof repair / replacement tiles in patch (Assume 5% repair/replacement tiles)

- Install dry hip, ridge & verge system

- Replacement roofline, 1m strip and reinstate tiles and install a GRP eaves protector.

- Remove existing, profiled section box gutters and replace creating a soffit & facia and install a deeplflow gutter system.

- Renew high level cladding to 3 sides (in hardy plank or similar system) to manufacturers specification, compartmentation and fire regulations to be considered, observed, and if/where necessary addressed and colours to be decided by customer consultation,

- NOTE: Localised repair to timber clad porches has been completed by others, so should not be included in price for over-cladding.

- Timber clad porches to be overclad (in hardy plank or similar system).

- Timber doors to porches to be painted in a suitable Class 0 Paint system (colour to be determined)

- White PVCu porches and polycarbonate roofs to be removed.

- Porches to be replaced in Brown PVCu with composite door.

- Create new roof structure to PVCu porches to match the style of those on the timber clad porch roof - with tiles (will involve removal of top of party wall that has been [poorly capped with flashband] between porches)

General layout

Arch Hill Roof Layout & Drone shot

Orchard St General photos and porches

PVC porches to be removed and replaced

timber porches to be overclad