Rosemount  X-STREAM Enhanced General Purpose Process Gas Analyser (XEGP) - 2 channels

- Enhanced specification required

  - Detection limit 0.5% of full scale deflection

- Two channel IR detection for sulphur hexafluoride

  - Bench 1: Sulfur Hexafluoride (SF6) - range 0-5 ppm with (Tstat 200mm IR SS bench; daily zero; Process Gas & Air Quality)

  - Bench 2: Sulfur Hexafluoride (SF6) 0-5000 ppm

- Web-browser interface

- X-STREAM Enhanced analyzer software

- 2x Analog Outputs = 1 for each channel (4 Status relays included for all options): Terminal strips: 2 analog outputs individually optically isolated

- Gas Path Tubing: Viton Tubing

- Gas Path Fittings: PVDF Fittings, 6/4 mm, gas path in series

- Rack Mount with Tstat control (dew point up to 42°C / 108°F)

This item will need to be delivered and invoiced before 20th March 2020.

Please provide the expected lead time with your quote.

Quotes should be sent by email to tenders@hse.gov.uk and quote reference HSE/T3549.

The closing date for quotes is 1pm, Monday 23rd December 2019.