| SPECIFICATION / SCOPE OF WORKS |  |  |  |
| --- | --- | --- | --- |
|  | ORGANISATION DETAILS |  |  |
|  | Full Name of Organisation |  |  |
|  | Registered Address |  |  |
|  | Postcode |  |  |
|  | Company Registration Number |  |  |
|  | Registered VAT Number |  |  |
|  | SAPCA Member | Yes: | No: |
|  | CONTACT DETAILS |  |  |
|  | Name: | Position: |  |
|  | Address (if different from main address) Postcode |  |  |
|  | Telephone Number | Email address |  |
|  | Please provide an itemized quotation for the works listed below |  |  |
|  | Employer’s requirements: MUGA refurbishment with 3-year warranty, minimum 10-year design life. |  |  |
| 1 | Site set up including, security, welfare, storage and fencing and removal on completion, allowing for leaving site clear and clean. |  |  |
| 2 | Lift, strip and remove existing synthetic turf and shockpad. Take away from site for recycling |  |  |
| 3 | Remove existing timber kickboards and take away from site for recycling |  |  |
| 4 | To arrange with J B Corrie Ltd for the removal and storage of dividing net and wires for duration of the works. (Botley Parish Council to provide storage facilities) The existing full perimeter fence is in good condition and is assumed to remain. |  |  |
| 5 | Repair existing macadam layer where root damage has occurred, removing root and reinstating surface |  |  |
| 6 | MUGA perimeter: Trench down and remove existing tree roots and install tree root protection membrane. Rebuild back to existing level. |  |  |
| 7 | Power clean existing macadam surface and provide tack spray coat |  |  |
| 8 | Install new performance shockpad |  |  |
| 9 | Install sand dressed Astro-turf:OR sand filled Astro-turf matting and any necessary gripper system. (Please quote for both options) |  |  |
| 10 | Permanent line markings for 2no 5-a-side football pitches |  |  |
| 11 | Install new timber kickboards |  |  |
| 12 | To arrange with JB Corrie Ltd for the reinstatement of pitch dividing wires and net |  |  |
| 13 | Preliminaries - items as required by Conditions of Contract and on/off site overheads. |  |  |
|  | FIXED SUM TOTAL exc. VAT |  |  |