REQUEST FOR QUOTATION

Renovation of Playgrounds

Ref. 2015/HPS/045/HCS

Contractors Brief

- Introduction

Hackney Council wishes to appoint an experienced contractor to undertake renovation work in 20 of the Borough’s playgrounds, as outlined in Appendix B. This brief sets out the background to this project, its aims, proposed methodology, timescales and management arrangements.

- London Borough of Hackney Vision

Hackney Council has outlined a clear vision for the future of:

- Making Hackney a Better Place:  “To achieve balanced, sustainable communities and neighbourhoods, which celebrate their diversity and share in London’s growing prosperity, to enable a good quality of life for all”

The vision will be realised through working towards achieving the Mayor’s three key priorities:

- Improving services and increasing opportunities for all, raising the life chances of the most disadvantaged

- Making sure the Council is high performing and efficient

- Providing effective community leadership and involving the whole borough in what we do.

- Within this context the Council has long recognised that parks and green spaces have an integral part to play in the achievement of this vision and priorities.

- Background to the Project

- The Health and Community Service Directorate is responsible for a wide range of parks and green spaces.  Historically there has been a lack of planned and preventative maintenance in this service area; in 2013 a sum of approximately £120K was invested in address a number of issues related to deterioration and health and safety thus reducing the risk to visitors to the Green Spaces. The Parks Service has now secured funding to carry out additional works which need to be completed by the end of May 2015.

- Aims

- The overall aim of the project is to undertake renovation works within our playgrounds so that they are fit for purpose and no longer pose a health and safety risk.

- Scope of works

Photos can be viewed / downloaded via the following link from which the RFQ document was downloaded: http://www.hackney.gov.uk/f-tenders-828.htm#current

| Equipment | Works Required | Photo |
| --- | --- | --- |
| Allen Gardens |  | 282 |
| Fence - Steel Flat Top | Replace and re-weld 3 No steel bars to panel near gate | 283 |
|  | Re-weld panel next to Pro-safe gate |  |
| Single Gate No 2 | Re-align each gate correctly to prevent shear hazard when closed |  |
| Picnic Tables No3 | Smooth off burnt edges of table | 285 |
| Richter Timber 2 Cradle Seat Swing | Replace 2 shackles |  |
|  | Patch repair area of .5sq.m hole in wear pad | 287 |
| Wicksteed Hen | Secure loose retaining plate | 288 |
| Adventure Trail by Hand Made Places |  |  |
| Static Timber Dragon | Reset I no loose wing section into the ground | 290 |
| Rope Walk | Replace rope | 280 |
| Zig Zag Rope Walk | Replace worn bottom rope 3No | 281 |
| Aske Gardens |  |  |
| Wicksteed Leisure/Football/Basketball Combination | Treat and repaint section of rust by removing flakes, sand and apply I x primer coat, 1 x undercoat and 1 x gloss | 40 |
|  | Replace 4 no missing bolts to panel and re-secure |  |
|  | Cut back protruding bolts threads to rear of 2 signs to the side panels | 36 |
| Broadway Market |  |  |
| Sutcliff 2 bay 4 Flat seat swing | replace 1 no flat seat |  |
| RSS Pathfinder Commanche Multi Play Unit | Pattern and replace 2 no worn climbing Rope | 96 |
| Richter Cantilever Tyre Swing | Inspect top chin and service as required | 98 to 102 |
| Unknown Burma Bridge and Platform | Break out and reset 2no post and secure to the platform | 103 |
| Butterfield Green 1 |  |  |
| Single Gate No 2 | Weld lug on 1 gate hinge post to remove crush hazard | 83 |
| Wicksteed 8ft 1 Bay 2 Flat Seat Swing | Replace 2 flat seat | 87/88 |
| Wicksteed 3 Tower & Slide Rainbow Multi Unit | Replace 6 no missing caps to upright | 90 |
| Ping Pong Table | Remove Technicks tiles, de-compact ground, level and relay new Technicks tiles 12sq.m | 91/92 |
| Butterfield Green 2 |  |  |
| RSS Plus-Play Mushroom | Remove current item and replace with similar type of play equipment. |  |
| Clapton Square |  |  |
| Kompan Moments Billy Goat Gruff | Replace 1 loose slat and provide costing to replace each additional slats | 169/170 |
| Kompan Moments See-Saw | Replace 1 plastic handle | 168 |
|  | Re-secure 2no loose section Head | 168 |
|  | Replace 1 no Plastic cap | 167 |
| Kompan Elements Multi Unit MQA 201000 | Inspect Dented slide run out and recommend required works and provide cost to rectify | 163 |
| Kompan Elements 8ft 2 Flat Seat Swing | Replace missing end cap | 171/172 |
|  | Replace 1no Flat seat |  |
| Kompan Elements 8ft 2 Cradle Seat Swing | Replace 2 cradle seats | 173/174 |
| Safer Surface | Jet wash approximately 170sq.m of wet pour to remove slipping hazard | 178 |
| Richter Cantilever Tyre Swing | Inspect top chain links and serviced as required |  |
|  | Lift and remove worn Technicks matting, remove from site to appropriate dump excavate new sub-base, supply 100mm of DOT type material to base and consolidate, install kiln dried treated timber to edging 150 x 35mm, supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 70sq.m to BSEN 1176 standards, lay approximately 70sq.m of artificial grass safer surface | 176/177 |
| Clapton Common |  |  |
| 2 Bay Swing | Remove 75sq.m of Technicks tile from underneath swing remove from site to appropriate dump. Check sub-base and if required supply excavate new sub-base 100mm DOT type material to base and consolidate. Supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 75sq.m to BSEN 1176 standards. Lay approximately 75sq.m of artificial grass safer surface. Tighten all loose fitting to swing Supply and fit rubber hydraulic hose to chain to make equipment compliant. Sand smooth any rough edges on timber frame | 226/227 |
| Large Wooden Multi Play Unit | Remove 30sq.m of Technicks tile from underneath slide run out. Remove from site to appropriate dump. Check sub-base and if required excavate new sub-base, supply 100mm DOT type material to base and consolidate. Supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 30sq.m to BSEN 1176 standards. Lay approximately 30sq.m of artificial grass safer surface Remove 9sq.m of Technicks tile from base of Fireman Pole Remove from site to appropriate dump Check sub-base and if required excavate new sub-base, supply 100mm DOT type material to base and consolidate. Supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 9sq.m to BSEN 1176 standards Lay approximately 9sq.m of artificial grass safer surface | 219/220 |
| Bench/Seating Area | Supply and lay approx. 40sq.m of Technicks grass mat tiles on the Right hand side | 223/224/225 |
|  | Supply and lay approx. 60sq.m of Technicks grass mat tiles on the left hand side | 223/224/225 |
| Daubney Fields |  |  |
| Sutcliff Toddler Zone Multi Play Unit | Replace 2no black bolt cover to panel | 180/181 |
|  | File off edge and replace damaged rubber to slide deck | 184 |
| Kompan Blazer | Replace rubber grips x 2 | 186 |
| Kompan 1 Bay 2 Cradle Swing | Replace worn top u hook hangers | 197 |
|  | Replace 1 end cap to top bar | 198 |
| Kompan Galaxy Sarin Multi Unit | Replace 1 no missing plastic cap to half round handle | 200 |
|  | Replace the dumbbell grips to the item | 199 |
| Kompan Super Nova | Replace missing rubber seal under item | 192/194 |
|  | Patch hole in blue wet pour of approximately .2sq.m | 192 |
| DeBeauvoir Square |  |  |
| General Surface | Grub out and refill gaps in Safer surface | 131 to 136 |
| RSS Playmaker Charokee Multi play Unit | Replace pull up rope | 137/138 |
|  | Re-secure screw to cargo net end fitting |  |
|  | Cut back safer surface where it is lifting, remove from site to appropriate dump | 139/140/141 |
|  | Relay safer surface material in gap as required |  |
| RSS Playmaker 1 bay 2 Cradle Seat Swing | Replace 1 cradle seat | 142 |
|  | Replace top corroded end bar bracket | 145 |
|  | Cut back safer surface where it is lifting, remove from site to appropriate dump | 146 |
|  | Relay safer surface material in gap as required |  |
| Hackney Downs |  |  |
| Park Leisure Roundabout | Inspect shaft and replace | 204 |
|  | Patch holes in blue wet pour safer surface measuring approximately 0.1sq.m | 205 |
| Unknown Large timber Multi Play Unit with Spiral Slide | Replace 2 burn deck slats to main deck with treated timber |  |
|  | Knock flush nail heads on deck slats |  |
|  | Re-weld and fettle off splits on rolled edge of 1 no small slide run out section (under deck) | 211 |
| Hackney Downs Trim Trail |  |  |
| Press Up | Replace main horizontal log and small upright | 216 |
| Over Head Ladder | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area 30m2 | 214 |
| Haggerston Park |  |  |
| Fence | Grind back projecting bolt thread to the rear of 1 no sign | 49 |
| Gate | Adjust 1 closer spring | 48 |
| Eibe Spring See-Saw 3 Way | Backfill with top soil, level, turf area and lay rubber Technics matting | 50 |
| Richter Cradle Seat Swing | inspect splits and recommend the cost for corrective action to prevent further splitting | 51/52/53/55 |
|  | Replace 2no cradle seats | 54 |
|  | Inspect chain and provide cost to replace if required | 58 |
| Richter 1 Bay Nest Seat Swing | Smooth off rough Edges to uprights | 69 |
|  | re-secure loose top shackles fitting to top bar | 68 |
| Embankment Slide | Re-secure raised tile edges at slide run out+B99 | 63 |
| Park Leisure 4 Way Climbing Frame | Replace rope to unit | 64/65/66 |
| Park Leisure Daisy Wheel Roundabout | Inspect internal working mechanism in accordance with manufacturer’s specification and recommend action and cost. | 59 |
| Haggerston Park Trim Trail |  |  |
| 3 No Step on logs | Reinstall log correctly | 77 |
| Double Balance Beam | Replace 2no small rotten upright and 1no missing horizontal log | 80 |
| Leap Frog | Replace 1no missing log | 78 |
| Balance Beam | Replace 1no burn horizontal log | 81 |
| London Fields 1 |  |  |
| Wicksteed 2Bay 4 Cradle Seat Swing | Replace 1 cradle seat | 112 |
| Wicksteed 2 Bay 4 Flat Seat Swing | Replace 2 flat seats | 113/114/115 |
|  | Inspect bushes and recommend works and provide cost for any repairs |  |
| Balance Beam Walk | Re-secure 1no end balance beam to post | 116 |
|  | Jet wash to remove slipping hazard | 116 |
| FSC Swing Alley | Replace missing plastic cap | 117 |
| London Fields 2 |  |  |
| Wicksteed 2 Bay 4 Flat Seat Swing | Replace 4 set of chains and shackles | 126 |
| Park Leisure outbound unit | Replace missing 3no egg fixing to cargo net | 123/124/125 |
|  | Jet wash approximately 424sq.m wet pour to remove slipping hazard | 128 |
| Millfields Trim Trail |  |  |
| Ladder Walk | Lift Technicks matting, de-compact ground, level, lay new turves and new Technicks matting |  |
| Chinning Bars | Lift Technicks matting, de-compact ground, level, lay new turves and new Technicks matting |  |
| 3 no Leaping Frog | Smooth off rough edges to 1no upright |  |
| Rowley Gardens |  |  |
| Proludic Vivarea Toddler Multi Play unit | Replace missing cap |  |
| St Johns Churchyard |  |  |
| Sutcliff Toddler Zoner Secret House Multi Play Unit | Replace 1no damage rubber edging to the top of panel | 180 |
| Shepherdess Walk |  |  |
| 3no Eibe Sheep Springer | Replace missing front section | 004/005 |
| Shoreditch Park |  |  |
| Kompan Bellatrix Multi Unit | Replace plastic section to green spiral seat | 16 |
|  | Replace rope to equipment | 006/008/009 |
|  | Replace 1no damaged triangle | 16 |
| Kompan Supernova | Replace 7no damaged plastic segments | 10 |
|  | Replace rubber seal to the underside of item on one side. | 14 |
| Kompan TR20122 Tower & Slide Multi Unit | Replace rope to equipment | 15 |
| Kompan Hammock | Replace damage Hammock | 22/23 |
| Kompan 1 Bay 2 Flat Seat Swing | Replace 1no plastic end cap to framework | 28 |
|  | Replace 1no flat seat | 27 |
|  | Repair approximately 0.5sq.m black wet pour under seat | 29 |
| Kompan 1 Bay 2 Cradle Seat Swing | Replace 1 no cradle seat and associated chain/shackles | 25 |
| Richter Cantilever Nest Swing | Inspect all mechanical parts and serviced as required | 18/19 |
|  | Replace all chains/shackles to the basket | 19 |
|  | Back fill over exposed foundations around one side of the base | 21 |
| 5no Cargo Net Climber | Straighten and re-install 1no upright correctly | 31 |
| Springfield Park |  |  |
| Single Gate | Weld Lug to 1no gate post near equipment to remove potential crush hazard | 254/255 |
| Kompan Group Swing | Replace 3no plastic caps | 237/238 |
|  | Replace damage matting under seats (10no) | 239 to 243 |
| Sik Holtz Timber Boat Theme | Re-secure very loose steering wheel | 244/245 |
|  | Replace missing sand scoop component | 247/248/249 |
| Richter Cantilever Tyre Swing | Re-secure top bracket, inspect top chain links and serviced as required, lift and remove worn Technicks matting, remove from site to appropriate dump, excavate new sub-base, supply 100mm of DOT type material to base and consolidate, install kiln dried treated timber to edging 150 x 35mm, supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 70sq m to BSEN 1176 standards, lay approximately 70sq.m of artificial grass safer surface | 250/251 |
|  |  | 250/251 |
|  |  | 252/253 |
| Springfield Park Trim Trail |  |  |
| Overhead Ladder | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area |  |
| Stoke Newington Common |  |  |
| Single Gate | Re-tension and realign gate to close at the correct speed and hit rubber buffer | 254/255 |
|  | Break-out and reset 1no loose gate post | 254 |
| Richter Totter Beam | Re-secure loose brackets to spring under main beam | 257/258 |
| Richter Climbing Beams | Break-out and replace loose post | 260/261 |
|  | investigate loose post at either end of unit and recommend action and cost |  |
| Richter Hut & Slide | Repair 3no holes in green wet pour safer surface measuring approximately .25sq.m | 262/263/264 |
| Richter Cradle Nest Swing | Repair hole in green wet pour safer surface of approximately 1 sq. m |  |
| Richter 1 Bay 2 Cradle Swing Seat | Replace 1 Cradle seat |  |
|  | Replace chain/shackle |  |
| 2 no Richter Springer Mobiles | re-secure 1no loose spring to base clamp | 267 |
| Double Gate | Backfill and level off trip point at edge of dog grid |  |
| Huck Trampoline | Replace trampoline as required |  |
|  | Backfill around edges of item with top soil |  |
| Timber Decking Stage and Stepping Stump | Inspect and provide recommended cost and works as required. |  |
|  | Knock flush all nail heads |  |
| 2no Hammocks | Inspect uprights to for splits and provide recommendations and cost to address |  |
|  | Make safe raised edges in uprights where required and cover over |  |
|  | Replace shackles |  |
| 2no Nest Pod Swings | Remove Graffiti from upright |  |
|  | Adjust 1no nest of seat to correct height by removing unwanted links |  |
|  | replace seat |  |
|  | Repair holes in Tiger mulch under seat measuring .5Sqm |  |
| Richter Totter Beam | Inspect badly damaged equipment and provide recommended works and cost |  |
| Wells Street Common |  |  |
| Overhead ladder | Replace 2no snapped/missing timber hand-grab | 148 |
|  | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area 35sq.m | 148 |
| 4 way Climbing Frame | Replace badly chewed Suspended tyre | 149/150 |
| Playdale 2.4m 2 Bay Cradle Seat, 1 flat seat and 1 pod seat. | Replace Cradle seat with flat seat to comply with relevant standard | 151 |
|  | Replace chain |  |
|  | Grind back projecting bolts under pod seat | 152 |
| Chinning Bar | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area 42sq.m | 151 |

- Site Visits

Site visits are available if required by contacting Area Parks Manager Michael Dixon whose contact details can be found in section 12.

- RFQ Returns

All RFQ Responses should be submitted by email quoting reference no 2015/HPS/045/HCS by no later than midday 20th March 2015 to the Project Manager at the following email address: Michael.dixon@hackney.gov.uk

Responses must include:

- A completed pricing schedule with separate costings for each line item (See Appendix A).

- A detailed Method Statement to include details of the following:

- Project resources – details of key personnel including qualifications, their specialisms and the roles that they will play.

- Site set up and security

- Outline Programme for carrying out the works within the required timescales.

- Previous experience – please provide 3 examples of similar commissions undertaken in the past three years and associated referees

- Health and Safety Policies and Procedures to include:

- H&S Policy Statement.

- Health and safety training and qualifications of those carrying out the work.

- Example of a Risk Assessment and Method Statement for a similar contract.

- Details of any membership of a health & safety professional body or organisation (e.g. IOSH, British Safety Council, Institute of Risk Management).  Please provide a copy of your certification.

- If the contractor has accreditation with the CHAS procurement safety scheme please provide a copy of your current certificate.

- State the name, position, and contact details of the person who acts as your organisation’s Appointed Person for Health and Safety as required in the Management of Health and Safety at Work Regulations.

- Evaluation of RFQ Responses

### The Council is committed to providing high quality, value for money services to the people who live and work in the borough.  Responses will therefore be evaluated against the criteria set out below.

The following weightings will be applied to the evaluation of the responses:

- Quality: 30% of the final score

- Price: 70% of the final score

Quality 30%

The following scoring mechanism will be used to score the qualitative responses:

| Score | Rationale | Definition |
| --- | --- | --- |
| 0 | Response contains insufficient information to make a judgement, or is otherwise wholly unsatisfactory | Wholly unsatisfactory |
| 1 | Response contains significant omissions, weaknesses or concerns, or is otherwise unsatisfactory | Unsatisfactory |
| 2 | Response contains some omissions, weaknesses or concerns | Cause for concern |
| 3 | Indicates an acceptable response | Acceptable |
| 4 | Indicates a very good response | Very good |
| 5 | Indicates an excellent response | Outstanding |

The following detailed criteria and weightings will be used to assess the quality element of the proposals:

| Quality – Method Statement | 30% |
| --- | --- |
| Project Resources Site setup and security Programme Previous experience Health and Safety | 10% 5% 5% 10% Pass/Fail |

In order to ensure that the successful Tenderer has met minimum quality standards, any Tenderer whose score includes two or more answers that are awarded a score of 2 or less, or any awarded a score of 0, will be deemed to have failed minimum quality standards and will be rejected.

Pricing (70%)

## Pricing is worth 70% of the overall marks and shall be scored on the basis that the cheapest bid will score the highest.  Other bids will be scored in relation to how their price compares with the cheapest bid.

Please see below an example to illustrate how the pricing evaluation works based on the price being weighted at 70% in the evaluation criteria:

|  | Bid A | Bid B | Bid C | Bid D |
| --- | --- | --- | --- | --- |
| Cost | 15,849 | 17,094 | 25,497 | 31,246 |
| Score | 70% | 64.9% | 43.5% | 35.5% |

A = 15,849 and gets 70%

B divided by A = X     17,094

= 1.0785

15,849

Proportional score        70

= 64.9%

1.0785

- Payment Terms

Payment shall be made to the contractor upon completion of the project and the Council’s approval of the works.

- Timescale

Below is a table of indicative timescales for the RFQ process. Please note that these dates may be subject to change.

| Task | Date |
| --- | --- |
| RFQs provided to bidders | 5th March 2015 |
| Site Visits | TBA as required |
| RFQ responses due | noon, 20th March 2015 |
| Evaluation of responses | 23rd March 2015 |
| Preferred bidder decided and business case approval | By 30th March 2015 |
| Contract award | wk/c 30th March 2015 |
| Contract commences | 13th April 2015 |
| Works completed | End May 2015 |

- Terms and Conditions

The terms and conditions under which the work will be carried out shall be the JCT Minor Works Building Contract 2011.

- Contact Details

All enquiries with regard to this brief should be directed to:

Name:		Michael Dixon

Job title:		Area Parks Manager

Address:	The London Borough of Hackney, Hackney Service Centre (2nd Floor) 1 Hillman Street, Hackney E8 1DY

##### Tel:		020 8356 5772

Email:		Michael.Dixon@hackney.gov.uk

Appendix A

Pricing Schedule

1.	Please insert costs for each line item and confirm the total contract sum in the table below.

- 2.	The prices and rates quoted shall be the fully inclusive value of the services, excluding VAT, described under the relevant headings, including all costs and expenses which may be required to provide complete services together with all general risks, liabilities and obligations set out in or implied necessary to comply with the conditions of tendering, the conditions of contract, the specification, the  method statements, all schedules and appendices thereto and all other documents forming part of the Contract.

- 3.	All prices should be in Pounds Sterling at current value.

- 4.	All pricing must be submitted in the form prescribed within this document. Additional pricing breakdowns can be added to the table. Any bids not submitted in the prescribed form may result in the bid being rejected.

| Equipment | Works Required | Photo | Cost |
| --- | --- | --- | --- |
| Allen Gardens |  | 282 |  |
| Fence - Steel Flat Top | Replace and re-weld 3 No steel bars to panel near gate | 283 |  |
|  | Re-weld panel next to Pro-safe gate |  |  |
| Single Gate No 2 | Re-align each gate correctly to prevent shear hazard when closed |  |  |
| Picnic Tables No3 | Smooth off burnt edges of table | 285 |  |
| Richter Timber 2 Cradle Seat Swing | Replace 2 shackles |  |  |
|  | Patch repair area of .5sq.m hole in wear pad | 287 |  |
| Wicksteed Hen | Secure loose retaining plate | 288 |  |
| Adventure Trail by Hand Made Places |  |  |  |
| Static Timber Dragon | Reset I no loose wing section into the ground | 290 |  |
| Rope Walk | Replace rope | 280 |  |
| Zig Zag Rope Walk | Replace worn bottom rope 3No | 281 |  |
| Aske Gardens |  |  |  |
| Wicksteed Leisure/Football/Basketball Combination | Treat and repaint section of rust by removing flakes, sand and apply I x primer coat, 1 x undercoat and 1 x gloss | 40 |  |
|  | Replace 4 no missing bolts to panel and re-secure |  |  |
|  | Cut back protruding bolts threads to rear of 2 signs to the side panels | 36 |  |
| Broadway Market |  |  |  |
| Sutcliff 2 bay 4 Flat seat swing | replace 1 no flat seat |  |  |
| RSS Pathfinder Commanche Multi Play Unit | Pattern and replace 2 no worn climbing Rope | 96 |  |
| Richter Cantilever Tyre Swing | Inspect top chin and service as required | 98 to 102 |  |
| Unknown Burma Bridge and Platform | Break out and reset 2no post and secure to the platform | 103 |  |
| Butterfield Green 1 |  |  |  |
| Single Gate No 2 | Weld lug on 1 gate hinge post to remove crush hazard | 83 |  |
| Wicksteed 8ft 1 Bay 2 Flat Seat Swing | Replace 2 flat seat | 87/88 |  |
| Wicksteed 3 Tower & Slide Rainbow Multi Unit | Replace 6 no missing caps to upright | 90 |  |
| Ping Pong Table | Remove Technicks tiles, de-compact ground, level and relay new Technicks tiles 12sq.m | 91/92 |  |
| Butterfield Green 2 |  |  |  |
| RSS Plus-Play Mushroom | Remove current item and replace with similar type of play equipment. |  |  |
| Clapton Square |  |  |  |
| Kompan Moments Billy Goat Gruff | Replace 1 loose slat and provide costing to replace each additional slats | 169/170 |  |
| Kompan Moments See-Saw | Replace 1 plastic handle | 168 |  |
|  | Re-secure 2no loose section Head | 168 |  |
|  | Replace 1 no Plastic cap | 167 |  |
| Kompan Elements Multi Unit MQA 201000 | Inspect Dented slide run out and recommend required works and provide cost to rectify | 163 |  |
| Kompan Elements 8ft 2 Flat Seat Swing | Replace missing end cap | 171/172 |  |
|  | Replace 1no Flat seat |  |  |
| Kompan Elements 8ft 2 Cradle Seat Swing | Replace 2 cradle seats | 173/174 |  |
| Safer Surface | Jet wash approximately 170sq.m of wet pour to remove slipping hazard | 178 |  |
| Richter Cantilever Tyre Swing | Inspect top chain links and serviced as required |  |  |
|  | Lift and remove worn Technicks matting, remove from site to appropriate dump excavate new sub-base, supply 100mm of DOT type material to base and consolidate, install kiln dried treated timber to edging 150 x 35mm, supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 70sq.m to BSEN 1176 standards, lay approximately 70sq.m of artificial grass safer surface | 176/177 |  |
| Clapton Common |  |  |  |
| 2 Bay Swing | Remove 75sq.m of Technicks tile from underneath swing remove from site to appropriate dump. Check sub-base and if required supply excavate new sub-base 100mm DOT type material to base and consolidate. Supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 75sq.m to BSEN 1176 standards. Lay approximately 75sq.m of artificial grass safer surface. Tighten all loose fitting to swing Supply and fit rubber hydraulic hose to chain to make equipment compliant. Sand smooth any rough edges on timber frame | 226/227 |  |
| Large Wooden Multi Play Unit | Remove 30sq.m of Technicks tile from underneath slide run out. Remove from site to appropriate dump. Check sub-base and if required excavate new sub-base, supply 100mm DOT type material to base and consolidate. Supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 30sq.m to BSEN 1176 standards. Lay approximately 30sq.m of artificial grass safer surface Remove 9sq.m of Technicks tile from base of Fireman Pole Remove from site to appropriate dump Check sub-base and if required excavate new sub-base, supply 100mm DOT type material to base and consolidate. Supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 9sq.m to BSEN 1176 standards Lay approximately 9sq.m of artificial grass safer surface | 219/220 |  |
| Bench/Seating Area | Supply and lay approx. 40sq.m of Technicks grass mat tiles on the Right hand side | 223/224/225 |  |
|  | Supply and lay approx. 60sq.m of Technicks grass mat tiles on the left hand side | 223/224/225 |  |
| Daubney Fields |  |  |  |
| Sutcliff Toddler Zone Multi Play Unit | Replace 2no black bolt cover to panel | 180/181 |  |
|  | File off edge and replace damaged rubber to slide deck | 184 |  |
| Kompan Blazer | Replace rubber grips x 2 | 186 |  |
| Kompan 1 Bay 2 Cradle Swing | Replace worn top u hook hangers | 197 |  |
|  | Replace 1 end cap to top bar | 198 |  |
| Kompan Galaxy Sarin Multi Unit | Replace 1 no missing plastic cap to half round handle | 200 |  |
|  | Replace the dumbbell grips to the item | 199 |  |
| Kompan Super Nova | Replace missing rubber seal under item | 192/194 |  |
|  | Patch hole in blue wet pour of approximately .2sq.m | 192 |  |
| DeBeauvoir Square |  |  |  |
| General Surface | Grub out and refill gaps in Safer surface | 131 to 136 |  |
| RSS Playmaker Charokee Multi Play Unit | Replace pull up rope | 137/138 |  |
|  | Re-secure screw to cargo net end fitting |  |  |
|  | Cut back safer surface where it is lifting, remove from site to appropriate dump | 139/140/141 |  |
|  | Relay safer surface material in gap as required |  |  |
| RSS Playmaker 1 bay 2 Cradle Seat Swing | Replace 1 cradle seat | 142 |  |
|  | Replace top corroded end bar bracket | 145 |  |
|  | Cut back safer surface where it is lifting, remove from site to appropriate dump | 146 |  |
|  | Relay safer surface material in gap as required |  |  |
| Hackney Downs |  |  |  |
| Park Leisure Roundabout | Inspect shaft and replace | 204 |  |
|  | Patch holes in blue wet pour safer surface measuring approximately 0.1sq.m | 205 |  |
| Unknown Large timber Multi Play Unit with Spiral Slide | Replace 2 burn deck slats to main deck with treated timber |  |  |
|  | Knock flush nail heads on deck slats |  |  |
|  | Re-weld and fettle off splits on rolled edge of 1 no small slide run out section (under deck) | 211 |  |
| Hackney Downs Trim Trail |  |  |  |
| Press Up | Replace main horizontal log and small upright | 216 |  |
| Over Head Ladder | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area 30m2 | 214 |  |
| Haggerston Park |  |  |  |
| Fence | Grind back projecting bolt thread to the rear of 1 no sign | 49 |  |
| Gate | Adjust 1 closer spring | 48 |  |
| Eibe Spring See Saw 3 Way | Backfill with top soil, level, turf area and lay rubber Technics matting | 50 |  |
| Richter Cradle Seat Swing | inspect splits and recommend the cost for corrective action to prevent further splitting | 51/52/53/55 |  |
|  | Replace 2no cradle seats | 54 |  |
|  | Inspect chain and provide cost to replace if required | 58 |  |
| Richter 1 Bay Nest Seat Swing | Smooth off rough Edges to uprights | 69 |  |
|  | re-secure loose top shackles fitting to top bar | 68 |  |
| Embankment Slide | Re-secure raised tile edges at slide run out+B99 | 63 |  |
| Park Leisure 4 Way Climbing Frame | Replace rope to unit | 64/65/66 |  |
| Park Leisure Daisy Wheel Roundabout | Inspect internal working mechanism in accordance with manufacturer’s specification and recommend action and cost. | 59 |  |
| Haggerston Park Trim Trail |  |  |  |
| 3 No Step on logs | Reinstall log correctly | 77 |  |
| Double Balance Beam | Replace 2no small rotten upright and 1no missing horizontal log | 80 |  |
| Leap Frog | Replace 1no missing log | 78 |  |
| Balance Beam | Replace 1no burn horizontal log | 81 |  |
| London Fields 1 |  |  |  |
| Wicksteed 2Bay 4 Cradle Seat Swing | Replace 1 cradle seat | 112 |  |
| Wicksteed 2 Bay 4 Flat Seat Swing | Replace 2 flat seats | 113/114/115 |  |
|  | Inspect bushes and recommend works and provide cost for any repairs |  |  |
| Balance Beam Walk | Re-secure 1no end balance beam to post | 116 |  |
|  | Jet wash to remove slipping hazard | 116 |  |
| FSC Swing Alley | Replace missing plastic cap | 117 |  |
| London Fields 2 |  |  |  |
| Wicksteed 2 Bay 4 Flat Seat Swing | Replace 4 set of chains and shackles | 126 |  |
| Park Leisure outbound unit | Replace missing 3no egg fixing to cargo net | 123/124/125 |  |
|  | Jet wash approximately 424sq.m wet pour to remove slipping hazard | 128 |  |
| Millfields Trim Trail |  |  |  |
| Ladder Walk | Lift Technicks matting, de-compact ground, level, lay new turves and new Technicks matting |  |  |
| Chinning Bars | Lift Technicks matting, de-compact ground, level, lay new turves and new Technicks matting |  |  |
| 3 no Leaping Frog | Smooth off rough edges to 1no upright |  |  |
| Rowley Gardens |  |  |  |
| Proludic Vivarea Toddler Multi Play unit | Replace missing cap |  |  |
| St Johns Churchyard |  |  |  |
| Sutcliff Toddler Zoner Secret House Multi Play Unit | Replace 1no damage rubber edging to the top of panel | 180 |  |
| Shepherdess Walk |  |  |  |
| 3no Eibe Sheep Springer | Replace missing front section | 004/005 |  |
| Shoreditch Park |  |  |  |
| Kompan Bellatrix Multi Unit | Replace plastic section to green spiral seat | 16 |  |
|  | Replace rope to equipment | 006/008/009 |  |
|  | Replace 1no damaged triangle | 16 |  |
| Kompan Supernova | Replace 7no damaged plastic segments | 10 |  |
|  | Replace rubber seal to the underside of item on one side. | 14 |  |
| Kompan TR20122 Tower & Slide Multi Unit | Replace rope to equipment | 15 |  |
| Kompan Hammock | Replace damage Hammock | 22/23 |  |
| Kompan 1 Bay 2 Flat Seat Swing | Replace 1no plastic end cap to framework | 28 |  |
|  | Replace 1no flat seat | 27 |  |
|  | Repair approximately 0.5sq.m black wet pour under seat | 29 |  |
| Kompan 1 Bay 2 Cradle Seat Swing | Replace 1 no cradle seat and associated chain/shackles | 25 |  |
| Richter Cantilever Nest Swing | Inspect all mechanical parts and serviced as required | 18/19 |  |
|  | Replace all chains/shackles to the basket | 19 |  |
|  | Back fill over exposed foundations around one side of the base | 21 |  |
| 5no Cargo Net Climber | Straighten and re-install 1no upright correctly | 31 |  |
| Springfield Park |  |  |  |
| Single Gate | Weld Lug to 1no gate post near equipment to remove potential crush hazard | 254/255 |  |
| Kompan Group Swing | Replace 3no plastic caps | 237/238 |  |
|  | Replace damage matting under seats (10no) | 239 to 243 |  |
| Sik Holtz Timber Boat Theme | Re-secure very loose steering wheel | 244/245 |  |
|  | Replace missing sand scoop component | 247/248/249 |  |
| Richter Cantilever Tyre Swing | Re-secure top bracket, inspect top chain links and serviced as required, lift and remove worn Technicks matting, remove from site to appropriate dump, excavate new sub-base, supply 100mm of DOT type material to base and consolidate, install kiln dried treated timber to edging 150 x 35mm, supply and lay shock pad appropriate for artificial grass type safer surface to cover an area of 70sq m to BSEN 1176 standards, lay approximately 70sq.m of artificial grass safer surface | 250/251 |  |
|  |  | 250/251 |  |
|  |  | 252/253 |  |
| Springfield Park Trim Trail |  |  |  |
| Overhead Ladder | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area |  |  |
| Stoke Newington Common |  |  |  |
| Single Gate | Re-tension and realign gate to close at the correct speed and hit rubber buffer | 254/255 |  |
|  | Break-out and reset 1no loose gate post | 254 |  |
| Richter Totter Beam | Re-secure loose brackets to spring under main beam | 257/258 |  |
| Richter Climbing Beams | Break-out and replace loose post | 260/261 |  |
|  | investigate loose post at either end of unit and recommend action and cost |  |  |
| Richter Hut & Slide | Repair 3no holes in green wet pour safer surface measuring approximately .25sq.m | 262/263/264 |  |
| Richter Cradle Nest Swing | Repair hole in green wet pour safer surface of approx.1sq.m |  |  |
| Richter 1 Bay 2 Cradle Swing Seat | Replace 1 Cradle seat |  |  |
|  | Replace chain/shackle |  |  |
| 2 no Richter Springer Mobiles | re-secure 1no loose spring to base clamp | 267 |  |
| Double Gate | Backfill and level off trip point at edge of dog grid |  |  |
| Huck Trampoline | Replace trampoline as required |  |  |
|  | Backfill around edges of item with top soil |  |  |
| Timber Decking Stage and Stepping Stump | Inspect and provide recommended cost and works as required. |  |  |
|  | Knock flush all nail heads |  |  |
| 2no Hammocks | Inspect uprights to for splits and provide recommendations and cost to address |  |  |
|  | Make safe raised edges in uprights where required and cover over |  |  |
|  | Replace shackles |  |  |
| 2no Nest Pod Swings | Remove Graffiti from upright |  |  |
|  | Adjust 1no nest of seat to correct height by removing unwanted links |  |  |
|  | replace seat |  |  |
|  | Repair holes in Tiger mulch under seat measuring .5Sqm |  |  |
| Richter Totter Beam | Inspect badly damaged equipment and provide recommended works and cost |  |  |
| Wells Street Common |  |  |  |
| Overhead ladder | Replace 2no snapped/missing timber hand-grab | 148 |  |
|  | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area 35sq.m | 148 |  |
| 4 way Climbing Frame | Replace badly chewed Suspended tyre | 149/150 |  |
| Playdale 2.4m 2 Bay Cradle Seat, 1 flat seat and 1 pod seat. | Replace Cradle seat with flat seat to comply with relevant standard | 151 |  |
|  | Replace chain |  |  |
|  | Grind back projecting bolts under pod seat | 152 |  |
| Chinning Bar | Lift technics matting, de-compact ground, level with new imported top soil, lay new turves and lay new technics matting to area 42sq.m | 151 |  |

Signed for the Bidder: ……………………………………………..

Name: ………………………………………………………………….

Title: ……………………………………………………………………

Company: …………………………………………………………….

Date: …………………………………………………………………..

Appendix B

Park Sites and Addresses

- Allen Gardens, Bethune Road N16

- Aske Gardens, Pitfield Street N1 6LE

- Broadway Market Green, Jackman Street E8

- Butterfield Green, Shakespeare Walk N16

- Clapton Square, Clapton Square E5 8NP

- Daubney Fields, Kingsmede Way E9 5PP

- DeBeauvoir Square, DeBeauvoir Square N1 $LG

- Hackney Downs, Downs Park Road E5 8NP

- Haggerston Park, Dunloe Street E2

- London Fields 1, Eleanor Road E8

- London Fields 2, Exmouth Place E8

- Rowley Gardens, Rowley Gardens N4

- St Johns Churchyard, Lower Clapton Road E5 0DP

- Shepherdess Walk, Shepherdess Walk N1 7JN

- Shoreditch Park, New North Road N1 6TA

- Springfield Park, Springfield E5 9EF

- Stoke Newington Common Northwold Road N16

- Stonebridge Gardens Haggerston Road E8

- Wells Street Common, Gascogyne Road E9

- Woodberry Downs, Goodchild Road N4 2SH