Schedule 3

Thurrock Council

HWRC Haulage and Associated Services

Lot 1 Green Waste Specification

- Specification	3

  - Definitions and interpretation	3

  - The Authority's Service requirements	5

  - Deliveries of Green Waste	7

  - Removal of Green Waste…….…………………………………………………..8

  - Quantities of Green Waste	8

  - Operation of the Reception Point and Composting at the Facility	8

  - Operating Hours	9

  - Weight recording	9

  - Vehicle weight recording…………………………………………………….10

  - Contamination of Green Waste……………………………………………..11

  - Processing I reprocessing of Green Waste	13

  - Ownership of Green Waste	13

  - Operatives	14

  - Health and safety	14

  - Records	14

  - Performance Failure and periods for remedy	15

  - Payments	15

  - Monthly Meetings………………………………………………………………...15

Annex 1 – Load Acceptance Procedure

Annex 2 - Annex 3 Method Statements (note these will be attached to the submission and added as an Annex prior to Commencement of Service).

## Note: Contractor’s Method Statements to include for the following:

The Contractor will agree a Method Statement for the receipt and processing of Green Waste. As a minimum, the Method Statement shall provide details and a description of how the Contractor will deal with the following; -

- Managing the contract and contract monitoring arrangements

- Management Systems including mobilisation plans

- Liaison with Authority

- Reception Point location / details

- Load Acceptance Procedure

- Certified Procedures including weighbridge procedures

- Contingency and emergency planning

- Operating and Composting at the Facility

- Residual waste management

- Product management

# Specification

## Definitions and interpretation

    - In this Specification defined terms have the same meaning as those set out in the Contract and, in addition, the following terms shall have the meanings described below:

“Accepted” shall have the meaning as described in section 1.10 and 1.12 of this Specification and upon the issue of a weighbridge ticket the Contractor shall be deemed to have ownership of the materials delivered by the Authority. Unaccepted shall be construed accordingly;

"Authority” means the Council as defined in the Councils Standard Terms and Conditions issued with this Specification;

“Authority Vehicles" means vehicles operating for the Authority or the Authority's civic amenity site operator or any other vehicle undertaking services on behalf of the Authority and bringing Green Waste to the Reception Point on the Authority's behalf and with the approval of the Authorised Officer;

“Available” means the Facility or Reception Point is available to the receive Authority Vehicles to deliver Green Waste during the agreed Operating Hours. Unavailable and Unavailability shall be construed accordingly;

“Certified System” means a management system that has been certified and externally audited by a certification body which has been assessed against internationally agreed standards by the United Kingdom accreditation service;

“Commencement Date” means from 07:00 hours on 11TH December 2017;

"Compost" means solid particulate material that is the result of Composting, which has been sanitised and stabilised, and which confers beneficial effects when added to soil, used as a component of growing media or used in another way in conjunction with plants;

“Composting” -  means the controlled biological decomposition and stabilisation of organic substrates, under conditions that are permanently aerobic and that allow the development of thermophilic temperatures as a result of biologically produced heat. It results in a final product that has been sanitised and stabilised, is high in humic substances and can be used as a soil improver, as an ingredient in growing media, or blended to produce a top soil that will meet British Standard BS 3882, incorporating amendment No 1. Composting may be carried out as part of the final product stabilisation from the Organic Waste Treatment process;

"Contaminated Material" means items that may contaminate the Green Waste to be transferred for Composting and result in a reduction in the amount of Green Waste that can be recovered for transfer to the Facility. Contaminated Material will require disposal to a suitably authorised waste management facility for treatment or end disposal. For the purposes of this Specification householders are advised that Green Waste is as defined in this Specification. Contaminated material shall also include Prohibited Material which is defined in this Specification;

“Contingency Arrangements” means the Contractor’s pre-determined plans for dealing with provision of the Services during those times when there is an emergency or other operational issue that prevents the Contractor from providing the Services at the designated Reception Point;

"Contingency Facility" means a facility provided by the Contractor in accordance with its Contingency Arrangements;

"Council " means the administrative area of the Authority, as may be varied from time to time pursuant to any boundary or other change;

"Daywork Rates" means rates for labour, plant, vehicles or any other resource that may be used in circumstances or services not outlined in the Specification;

“Facility” means a place where the materials are received, handled, processed (i.e. Composted) or deposited in line with the provision of the Service and operated in accordance with Good Industry Practice;

"Food Waste" means waste derived from food materials, typically vegetable peelings, meat and fish scraps, excess or spoiled prepared food and other discards from household kitchens;

"Green Waste" means

Bark

Branches up to 100 mm diameter

Flowers and plants

Grass cuttings

Hedge and tree cuttings and similar clippings and prunings

Soil entrained in roots from plants being deposited for recycling

Weeds

Green waste shall not include kitchen waste or any animal derived material or Prohibited Material [for example clinical waste];

"HWRC" means the Linford Household Waste Recycling Centre (also known as Civic Amenity Site – CA Site) owned by the Authority and located at Buckingham Hill Road, Thurrock, the opening times at the HWRC are: -

1 March – 31 October:- 08:00 hours – 17:00 hours Monday to Sunday; and

1 November – 28/29 February:-  08:00 hours – 16:00 hours Monday to Sunday.

The following times apply on:-

Christmas Eve: – 08:00 hours – 13:00 hours

Christmas Day -  Closed

Boxing day - Closed

New Year’s Day - Closed;

“Load Acceptance Procedure” means the procedure agreed between the Authority and the Contractor for receipt or rejection of Green Waste delivered in line with the contract Specification as set out in the “Load Acceptance Procedure” flow chart;

“Monthly Reports” means the reporting regime agreed between the Authority and the Contractor prior to the commencement of Service;

"Necessary Consents" means all permits, licences, permissions, consents approvals, certificates and authorisations (whether statutory or otherwise) which are required for Green Waste Composting and the performance of the Services or which are required in order to comply with legislation;

"Operating Hours" means the hours at which the Reception Point may receive Green Waste which, as a minimum, shall be between:

07:00 hours and 17:00 hours Monday to Friday

07:00 and 12:00 on Saturdays.

See section 1.7 Operating Hours for other times of interest to the Authority;

"Operative" means an employee or agent of or other person for the time being engaged by the Contractor or any sub-contractor of the Contractor deployed in connection with the provision of the Services;

“Products” means those materials that have been recovered or recycled as a result of the Service that are not considered wastes and can be marketed in line with Good Industry Practice;

“Prohibited Materials” are contaminates that cannot be handled at the Facility and may, by their very presence mean that the load is rejected in line with the Load Acceptance Procedure. The list of Prohibited Materials shall include [ e.g. Clinical Waste];

"Reception Point" means the location identified in the Method Statement at which Green Waste is to be delivered to the Contractor by or on behalf of the Authority, and which may be either the Facility or a Waste Transfer Station and the locations shall be agreed at the commencement of the Service but which shall in any event be:

- no more than 15 miles outside of the Council,

- north of the River Thames;

"Services" means the provision of processing and reprocessing capacity for defined fractions of the household and municipal waste stream as more particularly described within this Specification;

"Task" means one particular type or item of work as may be specified in the Specification and "Tasks" shall mean more than one Task;

"Waste Transfer Station" means (where applicable) a facility identified in the Method Statement at which Green Waste is delivered to the Contractor by or on behalf of the Authority for onward transfer (by the Contractor) to the Facility;

"Week" means a period of seven (7) days commencing at 00.01 hours on Monday;

    - Any reference to Employees of the Contractor will be deemed to also include the Contractor's agents and sub-contractors unless the context otherwise requires.

    - Any reference to time will be construed, during the period of summertime, to be British Summer Time and otherwise to be Greenwich Mean Time.

    - In the event of any contradiction or discrepancy between this Specification and the Contract, the terms of the Contract shall prevail.

    - The headings in this Specification do not form part of and shall not affect the construction of this Specification.

    - Words importing the singular only shall also include the plural and vice versa where the context requires.

## The Authority's Service requirements

    - The Authority is seeking to procure a 3 -year contract plus up to 24 months (subject to agreement from both parties) from the Commencement Date for the Composting of Green Waste:

      - deposited at the Authority’s HWRC,

      - potentially other Authority sources.

    - The Authority is seeking a contractor to:

      - receive and Accept Green Waste arising from the HWRC (and potentially other Authority sources) delivered by single and/or bulk loads as required (and as determined by the Authority) in the Authority's Vehicles either directly to a Facility or to a Waste Transfer Station (for onward transport to the Facility by the Contractor) or, in the event of Unavailability, to the Contingency Facility provided in the Contingency Arrangements;

Or aa)

Remove and Accept the Green Waste stored in the Authority’s 30.6 cubic metres (40 yard) containers directly from the HWRC and haul directly to a Facility or Reception Point, or in the event of Unavailability, to the Contingency Facility. (Note, the Authority will, when evaluating the bids, take into account both the haulage and treatment costs).  In the case where the Contractor Removes and Accepts the Green Waste from the HWRC, the requirements set down in the Haulage Specification (Lot 2) shall be included as part of the Specification for the combined Service. Where there is any duplication of the Service requirements required for the Green Waste Service and the Haulage Service, then the conditions of the treatment Specification shall take precedence. And

Receive and accept Green Waste from other authority sources eg Grounds/Estates/Highways – c.100 tonnes per annum (tpa).delivered by single and/or bulk loads as required. (and as determined by the authority) in the authority's vehicles either directly to a facility or to a waste transfer station (for onward transport to the facility by the contractor). The locations of the facilities for direct local authority access shall in any event be:

        - no more than 15 miles outside of the Council,

        - north of the River Thames.

The costs incurred by the authority will be calculated on the distance and travelling time of the Contractors facility or waste transfer station, where appropriate, to the authority’s depot based at Oliver Close, Thurrock.

      - make arrangements for the treatment of the Green Waste by Composting at the Facility and ensure delivery to identified processors of the materials to ensure that the materials can be included in the figures that represent the Authority's overall achievement of recycling targets; and

      - manage all haulage, treatment and disposal for any rejected and Contaminated Materials, in accordance with an agreed protocol and associated Schedule of Rates.

      - Facilitate elected member visits to the operational Facility (in particular the Facility used for Composting) as part of the Authority’s member education programme. Such visits would be undertaken with a senior Authority representative and the Contractor. A key aim of the visit would be to understand the Composting process and the Products as well as removal of unwanted residues

The Authority is seeking to commence delivering Green Waste to the Reception

h

h

Point from the Commencement Date.

    - The Contractor will demonstrate to the Authority that the Reception Point and/or the Facility receiving the Green Waste has all the Necessary Consents in place and is operated in line with Good Industry Practice.

    - The Contractor shall be responsible for all liability and costs associated with the necessary Consents associated with the operation of Reception Points, Facilities and those Reception Points or Facilities used in line with agreed Contingency arrangements.

    - The Contractor shall operate and maintain a Certified System for recording weights of Green Waste delivered to the Reception Point and details of the outputs as may be required by the Authority.

    - A weighbridge ticket shall be issued for every movement of the Green Waste delivered to the Reception Point. Details of the information required are as stated in section 1.9.

    - The Authorised Officer reserves the right to require a more frequent reporting of weights and associated information than is outlined in Sections 1.8 and 1.9.

## Deliveries of Green Waste

    - The Authorised Officer shall notify the Contractor of the registration number and description of each and any Authority Vehicle that will deliver Green Waste under the Contract together with its maximum gross weight.   In urgent cases, such notification may be initially by telephone to the Reception Point but will be confirmed by the Authorised Officer in writing, by e-mail or by facsimile within two Business Days. If a vehicle, which has not been notified to the Contractor, attempts to deliver Green Waste under the Contract the Contractor shall not accept any such vehicle and shall inform the Authorised Officer immediately of each incident.

    - For the avoidance of doubt a notification complying with section 1.3.1 shall apply to all future deliveries by the vehicle so notified and notification shall not be required for each delivery by that vehicle.

    - Green Waste will be collected from the HWRCs in 30.6 cubic metre (40 cubic yard) containers. All containers are collected and transported by 'roll on-roll off' (ro-ro) vehicles, rear end loaders or smaller vehicles and delivered to the Reception Point by or on behalf of the Authority.

    - The Contractor shall ensure:

      - that Authority Vehicles delivering Green Waste for the purposes of the Contract are not unduly delayed at the Reception Point;

      - the turnaround time taken at a Facility or Reception point, for any Authority Vehicle to deliver Green Waste, shall not exceed 20 minutes. The time will be measured from the point of entry to the Facility to the point of departure from the Facility. No Authority Vehicle shall be expected to queue to obtain entry to the Facility. If an Authority Vehicle is unable to gain direct access to a Facility and has to queue outside the entrance to the Facility, then the Facility shall not be deemed to be Available; and

      - that any Authority Vehicle shall be able to access the Reception Point, unload and leave the Reception Point without obstruction or interference.

    - The Contractor shall, if required by the Authority, provide assistance and use suitable equipment to remove any immobilised Authority Vehicle to a location at the Reception Point, where it can be repaired or removed by a breakdown vehicle.  If the driver of the Authority Vehicle does not require such assistance from the Contractor, then the Contractor shall allow recovery vehicles access to the Reception Point during Operating Hours to recover the Authority Vehicle. If the Authority Vehicle is obstructing an operational area within the Reception Point and is removed by the Contractor without permission from the Authorised Officer then the Contractor shall be solely liable for any loss, damage or injury arising as a result thereof and shall fully indemnify the Authority against the same.

    - The Contractor shall ensure that the internal access roads and maneuvering areas within the Reception Point are maintained in a suitable condition to allow all Authority Vehicles delivering Green Waste to safely reach the point of discharge without suffering undue wear and tear and that all Authority Vehicles can be unloaded promptly and safely and that their movement into and out of the Reception Point from the public highway is expedited.

    - The Contractor shall allow employees of the Authority and its contractors who are involved with the delivery of Green waste, reasonable access (in line with Good Industry Practice) to amenities provided at the Reception Point. This access shall also apply to any other authorised authorities or statutory bodies who may be employed on the delivery of Green Waste to the Contractor and/or may be employed on services which directly relate to the provision of the Services.

## Removal of Green Waste

## Where the Contractor is providing the Removal of Green Waste to the Facility or Reception Point, the applicable sections of the Haulage Specification (LOT 2) are to be strictly adhered to as set out in section 1.2 of this Specification.

## Quantities of Green Waste

    - The quantities of Green Waste delivered by the Authority will vary on a daily, weekly, monthly and annual basis. No guarantee can or will be offered by the Authority in this regard and the Contractor shall make due allowance to deal with annual and seasonal variations and surges in delivery by having adequate arrangements for the Acceptance of Green Waste at peak periods.

    - The following tonnages reflect 2015/16 throughput and no warranty is provided with regards future tonnages:

      - Linford HWRC: c.900 tonnes p.a.

      - Other Authority Sources: c.100 tonnes p.a

## Operation of the Reception Point and Composting at the Facility

    - The Contractor shall execute all operations necessary for the reception and Composting of the Green Waste delivered to it by or on behalf of the Authority and shall operate in compliance with all applicable Necessary Consents and permits.

    - If for any reason the Reception Point and/or the Facility becomes, or is likely to become, Unavailable or the Acceptance of   Green Waste the Contractor will immediately notify the Authorised Officer.

    - The Contractor shall also immediately inform the Authorised Officer of the reason for Unavailability of the Reception Point and/or the Facility, the steps being taken to make it Available again and the date and time when it is expected to be Available to receive and/or (as applicable) recommence Composting Green Waste.

    - The Contractor will be required to have in place suitable alternative arrangements ("Contingency Arrangements") to enable continuity of Services in the event of disruption. As a minimum, the Contingency Arrangements shall cover: -

      - full or partial closure of the Reception Point and/or Facility;

      - closure of a weighbridge;

      - plant failure;

      - loss of haulage arrangements (where applicable).

    - Should the named Reception Point or the Facility become temporarily Unavailable, the named Contingency Facility shall be able to meet the following minimum requirements:

    - Reception of the Authority's Green Waste delivered by single and/or bulk loads as required (and as determined by the Authority) directly to any Contingency Facility within a 10-mile radius of the Council;

    - The Contractor shall operate and maintain a Certified System for recording weights of Green Waste delivered to the Contingency Facility; and

    - The Contractor will demonstrate to the Authorised Officer that the Contingency Facility receiving the Green Waste has all the Necessary Consents in place.

    - In the event that a Contingency Facility is utilised by the Contractor, the Contractor shall meet any reasonable costs incurred by the Authority, as set out in the Payment Mechanism. The Contingency Facility proposed by the Contractor shall be agreed with the Authority prior to the commencement of the Service.

    - The Contingency Facility shall not be used for a period in excess of 28 days in any rolling twelve-month period without the agreement of the Authority. Where the Contingency Facility is used for periods greater than 28 days in any rolling 12 month period, the Authority shall consider the Contractor to be in breach is his obligations.

    - Prior to the commencement of the Service, the Contractor shall agree with the Authority a mechanism to ensure that account is taken of any shrinkage of materials (caused by water loss) and how this will be reflected in the data reports submitted to the Authority.

## Operating Hours

    - The Authority requires that, as a minimum, the Reception Point shall be open to admit Authority Vehicles during the defined Operating Hours.

    - At times when adverse weather conditions delay or prevent deliveries to the Reception Point, the Contractor shall afford assistance in terms of extended or varied hours of access to the Reception Point to allow the HWRC operator to catch-up within as short a period as possible.

    - The Reception Point may need to be open from 05:30 to until 20.30 hours Monday to Friday and from 05.30 to 18.00 on Saturdays, Sundays and Bank Holidays and any hours outside of the normal Operating Hours, where available and required, will be paid for in accordance with the rates contained in the Schedule of Rates.

## Weight recording

    - Throughout the Contract Period the Contractor shall provide and maintain a Certified System for recording the weights of Green Waste, net of vehicle crew, delivered to the Reception Point.

    - The Contractor will be responsible for the cost of regular weighbridge testing and record keeping and make the results available to the Authorised Officer as required.

    - The Contractor shall maintain at the Reception Point entrance/exit a weighbridge, which records and issues vehicle weight tickets/information and:

      - this weighbridge must be Certified as being in accordance with Section 11 of the Weights and Measures Act 1985 and which complies with the following provisions of this paragraph 1.8.3;

      - the weighbridge shall comply with all statutory requirements and any local regulations. Weighbridges must be tested and passed as fit for trade by a Trading Standards Officer prior to use; and

      - the Contractor shall ensure that any weighbridge malfunction or breakdown is covered by the Contingency Arrangements to ensure continuity of the Services.

## Vehicle weight recording

    - Except with the written agreement of the Authorised Officer, the Contractor shall use the weighbridge to measure the weight of each Authority Vehicle on entry to the Reception Point each Authority Vehicle delivering Green Waste, net of vehicle crew.

    - Except with the written agreement of the Authorised Officer the Contractor shall use the weighbridge to measure the weight of each Authority Vehicle that is leaving the Reception Point, having delivered Green Waste, nett of vehicle crew (i.e a tare weight). The weighbridge shall be linked to a computerised data handling system and shall generate weights of material without the need for manual input. A copy of the electronic weight data shall be forwarded to the Authority on a monthly basis or as and when reasonably requested by the Authorised Officer. Prior to the commencement of the Service, the Contractor shall agree with the Authority the format in which the electronic data is recorded and presented to the Authority.

    - The Contractor shall immediately inform the Authorised Officer of any circumstances, which prevent it from measuring the weight of Green Waste in accordance with Sections 1.8 and 1.9.

    - A weighbridge ticket shall be issued for each delivery of Green Waste and a copy of the ticket supplied to the driver of the Authority Vehicle before leaving the Reception Point or passed to such other representative of the Authority as it may require. Each ticket shall carry a unique serial number and clearly state the following information:

      - name of site;

      - date of delivery;

      - time of entry;

      - time of exit;

      - gross vehicle weight on entry;

      - gross vehicle weight on exit;

      - vehicle registration number and type;

      - Round number (where applicable);

      - name of weighbridge operator;

      - net weight discharged.

      - Description of material

      - Point of waste arising

      - Registered waste carrier number and expiry date

    - A second copy of each weighbridge ticket shall be retained by the Contractor. A summary sheet showing the weighbridge tickets issued for each delivery on each Business Day (or other times as agreed between the Authority and the Contractor) shall be prepared by the Contractor and passed to the Authorised Officer at the end of each Week. The format of the summary sheet shall state:

      - ticket number and date issued;

      - gross, unladen and net weight of each load;

      - total weight of Green Waste deliveries in that period;

      - be signed by a senior representative of the Contractor as a true record of all Green Waste delivered to the Reception Point; and

      - total weight of any Contaminated Materials removed from the Green Waste in that period and in accordance with Section 1.8 and 1.9. The summary sheet shall clearly state the source of the Contaminated Material, the date of delivery and the registration number of the vehicle making the delivery.

    - In the event of any weighbridge or equipment defect the Contractor shall be responsible for providing an alternative site, approved by the Authorised Officer, to weigh off the Authority Vehicles and all costs associated therewith shall be the responsibility of the Contractor. The Contractor shall be responsible for the payment of any additional costs incurred by the Authority through the use of an alternative weighbridge.

## Contamination of Green Waste

    - It is recognised that some contamination of Green Waste will occur.  The Authority will endeavor to minimise the contamination by:

      - working with residents and users of the HWRC and residents using the kerbside scheme to educate and inform them as to the types of Green Waste that are acceptable and not acceptable at the HWRC or for collection; and

      - working with the Authority's HWRC staff (or, in circumstances where the HWRC is not operated by the Authority, the contractor operating the HWRC) and the Authority’s kerbside collection team in the training of relevant staff to recognise and prevent contamination of Green Waste at the HWRC.

    - The Contractor shall follow the agreed Load Acceptance Procedure for each load of Green Wastes. The Load Acceptance Procedure is set down in a flow diagram form in Annex 1. Any rejection of materials shall also follow this procedure.

    - A load shall not be rejected in its entirety by the Contractor at the Reception Point where Contaminated Material can be easily removed or where the level of contamination can be brought to an acceptable level by minimal and safe hand sorting or picking.

    - For the purposes of this Contract the Contractor will accept up to 5% by weight or by volume, whichever the greater, of Contaminated Material in any individual load received at the Reception Point. Any Contaminated Materials found in individual loads up to this level will be disposed of by the Contractor at its own cost.

    - Where there is any disagreement between the Contractor or the Authority concerning the extent of any contamination, each party shall have the right to have the load analysed as set down in the Load Acceptance Procedure. The cost of the analysis shall be borne by the party who was proved to be at fault.

    - The Contractor will make its own arrangements for the temporary and separate storage of rejected loads at the Reception Point for inspection and acceptance by the Authorised Officer, prior to their disposal. Except in cases of emergency or regulatory restriction, the rejected materials will be stored separately at the Reception Point for up to one Business Day.

    - The Contractor shall be responsible for a system of quality control to ensure that loads of Green Waste delivered to the Reception Point are checked for suitability of content upon tipping. The Contractor shall reserve an area within the Reception Point where loads considered to contain an unacceptable amount of Contaminated Material can be tipped prior to removing the contamination or for inspection prior to disposal.

    - Any suspected load containing Contaminated Material must be inspected by trained Operatives of the Contractor. Where the load is deemed, wholly or partly, to be contaminated then the Contractor shall remove this load to the containment bay. The Contractor shall follow the Load Acceptance Procedure and advise the Authority accordingly.

    - The Contractor will immediately supply the Authorised Officer with details of the Authority Vehicle that delivered the contaminated load. This should include:

      - the estimated level of contamination;

      - a broad description of the nature of the Contaminated Material;

      - the vehicle registration number; and

      - its arrival time at the Reception Point and whether the Contractor is able to separate and remove the Contaminated Material.

    - Any cleaning of the load will be undertaken by the Contractor and he will endeavor to isolate the Contaminated Material so that the whole load is not rejected.

    - Should the load, or any part of it, need to be rejected this will be arranged by the Contractor and charged to the Authority. Any charge will be at a reasonable cost and assessed based on the Dayworks Schedule of Rates to be agreed with the Authorised Officer. The costs associated with cleaning of loads and disposal of Contaminated Materials will form part of a Schedule of Rates item.

    - The Contractor will supply, within one Business Day of the occurrence, full details of any incident involving Contaminated Material where the load is completely rejected. This will be in a form to be agreed with the Authorised Officer and will include, but not be limited to:

      - details of the Authority Vehicle that delivered of the Contaminated Material;

      - type and quantity of the load;

      - colour photographs of the Contaminated Material;

      - amount of Contaminated Material cleaned for recovery;

      - amount of Contaminated Material disposed of to a licensed or permitted waste disposal site;

      - chargeable cost of cleaning the load containing the Contaminated Material;

      - chargeable cost of transport;

      - chargeable cost of disposal, if any; and

      - in the event that the contamination causes the Reception Point to be closed, any loss of operating time to the Reception Point as a consequence of the incident.

1.10.13	Where a load has been Accepted at the Reception Point and later determined to have not been acceptable, the responsibility for having Accepted the load will lie with the Contractor, who shall deal with the load at its own expense.

## Processing I reprocessing of Green Waste

    - The Contractor must supply full details of the Reception Point and Facility including site name, full address and details of the Necessary Consents. Any amendment or change to the Reception Point or Facility identified in the Method Statement must be approved by the Authorised Officer, prior to the diversion of Green Waste, including in the event of any diversion of Green Waste to a Contingency Facility.

    - The Contractor must process 100% of the Green Waste that meets the required input specification.

    - The Facility must produce an output product that is capable of use as compost that meets British Standard 3882 (or subsequent equivalent standard), incorporating amendment No. 1 as a minimum requirement. The Authority encourages the production of a product that has an equivalent standard of BS PAS 100 or equivalent recognised standard.

    - The Contractor must ensure that the Products are utilised in line with Good Industry Practice. The Contractor must ensure that the process used at the Facility conforms to the definition of Composting and contributes to the reduction of the amount of bio-degradable municipal waste landfilled by the Authority.

    - The Contractor must ensure that the product (compost) is only utilised at facilities authorised for such use.

    - The Contractor must take all reasonable steps to ensure that no product from the Composting of the Green Waste is sent to landfill for disposal or used in a manner that would be contrary to any landfill avoidance programme adopted by the Authority.

    - The Contractor will assist the Authority in arranging for some compost to be bagged and delivered to the Authority’s HWRC for sale to the public. The Contractor shall provide a detailed cost for this prior to the commencement of the Service.

## Ownership of Green Waste

    - Once the Contractor has received the Green Waste at the Reception Point weighbridge and issued a weighbridge ticket to the driver of the relevant delivery vehicle the Green Waste will be deemed to be Accepted by the Contractor and shall be deemed to be acquired by and in the ownership of and at the risk of the Contractor who shall take full responsibility for it and shall handle and dispose of such Green Waste in accordance with the terms of the Contract.  All Products produced as a result of this Contracts shall be used in line with Good Industry Practice.

    - The Contractor must treat and dispose of the Green Waste in accordance with the contract and adhere to the Contractor’s statutory duty of care regarding “waste” under s.34 of the Environmental Protection Act 1990. The Contractor shall at all times comply with its duty of care under section 34 of the Environmental Protection Act 1990 (EPA) and shall procure the compliance of all Employees involved in the delivery of the Services with section 34.

    - The Contractor shall have a duty to inform the Authority if any Employee or Authority party may in the reasonable opinion of or to the knowledge of the Contractor be in breach of the duty of care under section 34 of the EPA.”

## Operatives

    - The Contractor must ensure that every Operative deployed by the Contractor in

and about the provision of the Services at the Reception Point and/or the Facility is at all times properly and sufficiently skilled and/or instructed with regard to:

- the Task or Tasks that such Operative has to perform;

- any relevant provisions of the Contract;

- all rules, procedures and standards referred to in the Contract and this Specification relevant to the work that the Operative is deployed to perform;

- all relevant rules, procedures and statutory requirements concerning health and safety at work;

- fire risks and fire precautions and emergency action including evacuation procedures;

- the need to maintain the highest standards   of hygiene, courtesy and consideration;

- the need to recognise situations which may involve any actual or potential risk of personal injury to any person (including members of the public), and the need to make such situations safe; and

- the need in the event of any emergency or situation requiring implementation of the Contingency Arrangements immediately to contact the Contract Manager (and the Contractor shall ensure that the Contract Manager shall forthwith notify the Authorised Officer).

## Health and safety

    - In providing the Services, the Contractor shall be responsible for all health and safety matters and must adopt safe methods of work and comply at all times with the requirements of the Health and Safety at Work etc. Act 1974 and of any other legislation, regulations or Orders pertaining to the health and safety of the public, Operatives, sub-contractors or staff of the Authority and all other persons. The Contractor shall provide and maintain a Certified System to manage Health and Safety at the Facility. The Contractor shall provide a monthly report to the Authority that provides details of any accidents relating to the provision of the Service and the corrective and preventative action taken by the Contractor in relation to these accidents. The Contractor shall agree with the Authority the reporting mechanisim and the details to be provided in the reports to the Authority prior to the commencement of the Service.

## Records

    - The Contractor shall maintain, in the format set out in Section 1.9, accurate records   of   all      Authority   Vehicle   movements weighed in and out at the weighbridge, and shall provide the Authorised Officer with copies of those records each month within one Business Day of the end of the month concerned, or on demand.

    - The Contractor shall provide any relevant management information required by the Authorised Officer.

    - The Contractor shall maintain all other records required by the Authorised Officer, the Environment Agency or other controlling authority.

## Performance Failure and periods for remedy

    - The Contractor shall carry out the Services in accordance with the Contract and is specifically referred to the Performance Framework to be followed by the Contractor.

    - The Contractor shall, upon receiving a Rectification Notice from the Authorised Officer or his representatives, rectify a Performance Standard Failure within the period set out in the Rectification Notice and as, where relevant, referenced in the Performance Framework.

    - Failure by the Contractor to remedy a Performance Standard Failure in line with the Rectification Notice shall be dealt with by the Authority in accordance with the Performance Framework and where applicable, Standard Performance Deductions or Ratcheted Performance Deductions made and a Warning Notice and / or Persistent Failure Notice issued in accordance with the Performance Framework.

## Payments

    - Payment for providing the Services, shall be at the rates set out in the Schedule 7  (Schedule of Rates). Any adjustment to the Schedule of Rates for indexation shall be made in line with Schedule 9 (Payment Mechanism).

    - Payments shall be made monthly in arrears and subject to the specified contract management information as set out in section 1.18. For the avoidance of doubt, the Contractor will be responsible for preparing and submitting all the required management information to the client in the specified management reports

## Monthly Meetings

    - There shall be monthly meetings between the Contractor and the Authority. The Contractor will attend such meetings convened by the Authority upon request, at which the operation of the Service shall be the reason for the meeting. At the meetings, the Contractor's representative shall be the Contract Manager (or person agreed between the Authority and the Contractor who shall be at an appropriate level in the company).

    - All information required pursuant to the Contract shall be submitted to the Authority 3 days before each meeting provided that the request for information is submitted to the Contractor within a reasonable time in advance of such meeting. Prior to the commencement of the Service, a method of general communications and special communication procedures for dealing with emergencies shall be agreed with the Authority.

    - All monthly reports required by the Authority shall be submitted by the Contractor to the Authority within one week from the end of each calendar month.

    - All Monthly Reports required by the Authority shall be submitted by the Contractor to the Authority within one week from the end of each calendar month. The Monthly Reports shall provide details of: -

- Authorised Waste tonnages and deliveries

- Health and Safety

- Contract monitoring – service delivery

- Payment Mechanism requirements.

The precise details of the reports and proformas used to submit data shall be agreed between the Authority and the Contractor prior to the commencement of the Service. The Contractor shall submit his proposed reporting system as part of the Contractor’s Method Statements.