Client’s Information Requirements

Client: Science and Technology Facilities Council (STFC)

Project Name: National Quantum Computing Centre

Supplier: XXX

Version Control

| Document Number | Revision | Date | Author | Amendment | Status |
| --- | --- | --- | --- | --- | --- |
| STFC Information Requirements | P01 | 10.09.2020 | MG | STFC/SP | ITT |
|  | V02 | 22.09.2020 | MG | Formatted landscape | ITT |
|  | V03 | 26/10/2020 | MG | Formatted tables & CAFM requirements | ITT |

Note: The following document is based on the Clients Information Requirement Core Content and Guidance Notes Template: Version 07 28.02.13 issued by the Building Information Modelling (BIM) Task Group with amendments taken from the building Smart UK CIR Template 2015.

Contents

- Client’s Information Requirements

1.1	Project Brief

1.1.1	Project Description

1.1.2	Unique Project Challenges Relating to BIM

1.2	Technical

1.2.1	Software Platforms

1.2.2	Data and Exchange format

1.2.3	Coordinates

1.2.4	Level of Detail

1.2.5 	Model Delivery Production Table

1.2.6	Training

1.3	Management

1.3.1	BIM Standards

1.3.2	Roles and Responsibilities

1.3.3	Planning the Work and Data Segregation

1.3.4	Security

1.3.5	Coordination and Clash Detection

1.3.6	Collaboration Process

1.3.7	Health and Safety / Construction Design Management

1.3.8	Systems Performance

1.3.9	Compliance Plan

1.3.10	Delivery Strategy for Asset Information

1.4	Commercial

1.4.1	Data Drops and Project Deliverables

1.4.2	Client’s Strategic Purpose

1.5	BIM Competence Requirements

- Other Tender Documents

- Acronyms / References Used In This Document

- Client’s Information Requirements.

This Client’s Information Requirements (CIR) document is prepared as part of the Project BIM Implementation Strategy for the project known as National Quantum Computing Centre (NQCC) and is to be used to set out clearly to the supplier what models / data are required and what the purposes of the models / data will be used for.

This document is structured to follow the primary core contents sections of Technical, Management, Commercial as broadly defined in the Clients Information Requirement Core Content and Guidance Notes Version 07 28.02.13 issued by Building Information Modelling (BIM) Task Group with amendments taken from the Building Smart UK CIR Template 2015. The previous version of this document has been updated to reflect the changes in the British Standard, in particular the replacement of BS 1192 and PAS 1192‑2 with BS EN ISO 19650‑1 and BS EN ISO 19650‑2.

It is the intention that the requirements set out here will in turn be responded to by the supplier in the form of a Project Pre Contract BIM Execution Plan (BEP). The Project Pre Contract BIM Execution Plan should clearly set out the methods of achieving the outputs defined in this CIR. The Pre Contract BEP should demonstrate the suppliers approach to delivery of the outputs defined in this CIR and it is expected that this Pre Contract BEP will be replaced by a Post Contract BEP once the scheme outputs have been reviewed and agreed with the supplier / Client.

The Client requires that the principles of Building Information Modelling (BIM) as defined by BS EN ISO 19650 be implemented for the design and construction of the NQCC project. The structure of the processes to be followed in delivering the outputs defined in this Clients Information Requirements are to be those defined in the following documents:

- BS EN ISO 19650-1

- BS EN ISO 19650‑2

- PAS 1192-3:2014

- PAS 1192-4:2014

- PAS 1192-5:2015

Specific Notes:

- Unless specified otherwise, Aconex will be used as the project common data environment (CDE) for the project. This is a secured system, structured with access management protocols and suitable system training and support, to ensure the project information and data are kept secure. The CDE will be structured to categorise, label and manage information and data in accordance with data structuring principles set out in BS EN ISO 19650‑2: Information management using building information modelling

- Object based design and construction models (referred to as Building Information Models (BIMs)) should be created to satisfy the requirements of this CIRs to drive efficiency and predictability throughout the project lifecycle. BIMs should be interoperable and available for analysis using associated/complimentary software (see section 1.2.1 for further details);

- The Client requires suppliers to provide evidence of their own and their supply chain evaluation approach for the assessment of BIM capability and capacity of each supplier and/or sub-suppliers ability to deliver the required information in the required format. Unless specified elsewhere, the supplier should (as a minimum) provide evidence to meet the requirements defined in clause 5.3.3 of EN ISO 19650‑2

- The supplier should note that the Client will require a client defined asset data structure to be implemented as one of the information exchange formats

- It is noted that the supplier’s BEP is a live document that will develop during the design and construction model work stages and the supplier should allow for the process of developing the BEP to continue to meet the outputs defined in this CIR .

- The supplier is to ensure that they (or their sub-suppliers) appoint a Project Information Manager to carry out the role of managing and checking the information within the project team and to ensure that their data transactions are carried out in accordance to BS EN ISO 19650 (See Table 3 data drops). The information outputs will be validated against the CIR / BEP by the suppliers Project Information Manager. This information will be shared via the CDE to the Client’s project manager, who will be responsible for validating that the information is aligned to the outputs agreed within this CIR.

- The Client requires that a Government Soft Landing strategy be defined in accordance to BSRIA principles and processes.

The responsibility for the production, development and implementation of the BEP lies with the Supplier. The Supplier should ensure that the contents of this document, and those documents referenced within it, are shared with their supply chain, in order to enable all parties to meet these requirements.

The Supplier should include a comprehensive, initial Pre Contract BEP with their Bid Response to demonstrate their proposed approach, capability, capacity and competence to meet the CIRs.

In addition to information requested as part of the tender process, once agreed, the Post Contract BEP will include the following additional content over and above the requirements of the CIR. This should convey information management capability: In addition to information requested as part of the tender process, the BEP will include the following content:

- Project Implementation Plan.  This should convey information management capability

- Project goals for collaboration and information modelling;

- Major project milestones consistent with the project programme; and

- Project Information Model (PIM) deliverable strategy.

A compliant BEP will demonstrate how the requirements outlined in the CIRs will be met; the CIRs will therefore be used as the basis to review the contents of the Supplier’s BEP. Upon receipt of the pre-contract BEP any changes to necessary to align the CIR will be made in collaboration with the supplier, as a response to the Pre-Contract BEP received with the supplier’s tender response.

The appointed Supplier will be required to develop the BEP o ensure:

- That all relevant parties have agreed and committed to the BEP;

- There is supply chain capability, capacity and competence to meet the CIRs;

- Information exchange can be achieved with plain language questions addressed;

- Efficiency is optimised with waste minimised.

Note: As the BEP develops through the lifecycle of the project the supplier should demonstrate how the selection of further sub-suppliers and/or specialists is to be undertaken and the process for assessing the capability, capacity and competence of the Supplier’s supply chain. Members of the supply chain should be detailed in the BEP as they are appointed to the project.

Reference is made through the CIRs to the Project Information Model (PIM). For clarity the PIM is defined as Information model developed during the design and production and construction phase of a project, consisting of documentation, non- graphical information and graphical information defining the delivered project. The PIM will consist of information communicated through issue of model data (both graphical 3D and 2D), and non-graphical data, linked documents and information indexed through from other relational databases, (e.g. schedules, certificates etc.) and data. PIM is defined by the superseded PAS 1192-2. In this phase of transition to BS EN ISO 19650, the Client decided to use this definition to refer to the collection of graphical and non-graphical information collected by the designers throughout the design and construction phase.

For clarity the Client requires the supplier to retain full control and be responsible for updating the PIM until the information has be accepted by the Client as suitable for incorporation in the Asset Information Model (AIM). At this point of acceptance by the Client of the information as suitable for inclusion into the AIM, the responsibility for updating the information contained in the AIM (post project completion) passes to the Client or their nominated representatives.

Data drops represent information exchange points from the suppliers and are set in order to support key Client decision stages and/or milestones in the project.

The Supplier will be required to submit data to support the information requirements and key activities set out in the data drops structure. Information change control procedures capturing changes to all data source (graphical and documents) should be detailed in the suppliers BEP.

The following standards and protocols are mandated for use on this project:

- PD 19650-0:2019

- BS EN ISO 19650-1:2018

- BS EN ISO 19650-2:2018

- PAS 1192-3:2014 (Asset information)

- BS 1192-4:2014 (COBie/CAFM)

- PAS 1192-5:2015 (Information security)

- CIC BIM Protocol

- CIC Role of the Information Manager

- BIM Level 2

The following classification systems are mandated for use on this project:

- Uniclass 2015

- NRM 1

- NRM 3

- AEC (UK) BIM Protocols v2.0

- CPIx Protocol Documents (www.cpic.org.uk)

- CIC BIM Protocol: 2013

  - Project Brief

    - Project Description

The NQCC facility is a new purpose-designed building that will house the equipment required for a quantum computing development and production.

    - Unique Project Challenges Relating to BIM

Potential project specific challenges which may relate to BIM include:

- Site access issues

- Security restrictions

- Flexible use laboratories

- M&E services provision

  - Technical

This section establishes technical information requirements, including the software, data drop contents and Level of Definitions (which is the combined Level of Detail LOD and Level of Information LOI) requirements.

| Refer | Item | Description | Response |
| --- | --- | --- | --- |
| 1.2.1 | Software Platforms | This defines the platform for the Building Information Model as well as other software platforms to be used. The purpose of this section is to communicate software platforms and versions where these are known and where they might influence the preparation of a bid. Section 1.2.1 also defines content required in the BEP. | Platforms and versions used required by STFC across the project include the following: - The Client requires the use of a common data environment for managing project data. The contractor shall provide a licenced system for the project team and client. - Asset list to be issued as an Excel spreadsheet in the STFC COBie and CAFM file template formats shared by the client with the contractor as an attachment to the present CIR. - The following Computer Aided Facilities Management System (CaFM) will be used by the Client to manage the facility after handover: Pirana CMMS and CAFM Asset Data extracted from the federated model to be shared in CSV format with the client at handover. - The federated 3D model will be developed and issued to the client at handover in the following format: Autodesk Revit model. All the consultants working on the federated model must have the same version of Autodesk Revit (2018). - Supplier coordination during the construction phase must be facilitated by the use of Autodesk BIM360 Field. Licences shall be provided to the Client team. - Information from O&M manuals must be compiled in Autodesk BIM360 Field before handover The ability of the suppliers to work with these platforms and to meet information exchange should be assessed with the CPIx - IT Assessment Form (https://www.cpic.org.uk/wp-content/uploads/2013/06/cpix_-_it_assessment_form_ver_1.0.pdf). The BEP should identify specific software (plus versions) used for design, analysis and simulation of the following BIM types: - Discipline specific and specialist designers’ models setting out the design for (as a minimum) structures, civils, architecture and building services; - Federated model(s) to support model co-ordination and collaboration; |
| 1.2.2 | Data and Exchange Format | This section defines the formats used to deliver data at data drops. | For each of the data drops, information will be required in the following typical formats: - Native file format for design and analysis; - Federated model in federation software format (Navisworks or Revit) - PDF 2D files – no older than version 7.0 (to be extracted from the model to provide a drawing package defined in the drawing schedule contained in and agreed appendix to be produced by the supplier). |
| 1.2.3 | Coordinates | This section stipulates the adoption a common coordinate system for all BIM data with consistent adoption for all models. It defines the requirements for the common coordinate system for all BIM data. | A master grid file should be created with a setting out point of origin notated with ordnance survey co-ordinates, grid intersection points, ground floor level, project north and units of measurement as a minimum. This will set the common co-ordinate system for all BIM data with consistent adoption for all models. There should be no deviation from the data contained in the master grid file. The project base point coordinates shall be defined and specified in the BIM Execution Plan. - BIM projects shall: - Comply with Level 2 requirements - Use real world co-ordinate systems: - Be produced to true height above project datum. - Adopt the established project shared coordinate system across all BIM data files to allow them to be referenced without modification. - Have a project information and BIM manager appointed by the Contractor. - Models shall use consistent units and measurement across the project. Default project units shall be millimetres with two decimal places in order to display accuracy in the temporary dimensions. Dimension styles in the accompanying templates utilise defined units which override project settings, so whilst the temporary dimension might read 3000.00 (project settings), the permanent dimension will read 3000 (dimension style in template). - 2D input/output files shall conform to the unit and measurement protocols designated for specific drawing types. - Site layout drawings relating to the project coordinate system to an accuracy of 3 decimal places. - 1 unit = 1 millimetre CAD data modelled more than 1 mile from the origin (in any plane) shall be shifted to 0.0.0. Prior to importing into (modelling software) to avoid accuracy issues. The shift shall be agreed, consistent and as identified in the BIM Execution Plan. The supplier is required to generate a Master Information Delivery Plan (MIDP) for inclusion in the BEP. The MIDP is the primary plan for when and what project information is to be prepared by the supplier and their supply chain. |
| 1.2.4 | Level of Definitions | The section defines the requirements for information submissions/data drops at project stages. This information is used to develop the Master Information Delivery Table (MIDT) and also defines, for the purposes of the contract, the levels of detail used for the various phases of the project. | Table 1 NBS BIM Toolkit - Levels of Definitions; Model Content Criteria. Symbolic; LOD1: Symbolic place-holder representing an object which may or may not be to scale or have any dimensional values; LOD 1 model consists of overall building massing designed to perform whole building type analysis including building orientation, square meter costs. LOD 1 also pertains to 2D representation of elements as required by the 2D which may include drawings, narratives and hand-built models. LOI1: May inform narratives or provide very approximate information. Conceptual; LOD2: To provide a visual indication of proposals at a Concept stage identifying key requirements such as access and maintenance zones etc. Information to be suitable for zonal spatial coordination of primary systems / elements; The LOD 2 model consists of generalized systems including approximate quantities, sizes, shapes, location and orientation. The LOD 2 model(s) are used for analysis of defined systems and general performance objectives. LOD 2 model(s) include attributes and parameters defined by the owner in the CIR document and BIM Execution Plan. LOI2: A simple description outlining design intent. Developed Design; LOD3: To provide a visual representation of proposals, confirming brief for technical Design stage and to allow general spatial coordination. Can form a brief for a specialist sub-contractor or fabricator to progress with their technical design, fabrication and installation; Model will include elements equivalent to traditional construction documents and shop drawings. LOD 3 models are well suited for estimating as well construction coordination for clash detection, scheduling, and visualization purposes. LOD 3 model(s) include attributes and parameters defined by the owner in the CIR document and BIM Execution Plan. LOI3: The specified overall performance of the deliverable. Technical Design; LOD4: To provide a visual representation of proposals at a Technical Design stage supporting full spatial coordination. Graphical representation of system, dimensionally accurate indicating primary performance characteristics; Model elements are modelled as specific assemblies, which are accurate in terms of quantity, size, shape, location, and orientation. LOD 4 model(s) are virtual representations of the proposed elements and considered to be suitable for construction, fabrication and assembly. This LOD is most likely used by specialty trade contractors and fabricators to build and fabricate project components including ME systems.. LO4I: The prescribed specific products that meet the desired overall performance requirements. Construction; LOD5: To provide full information to support construction / installation of the appropriate systems / products; LOD 5: Model elements represent the project as it has been constructed, including as-built conditions. The model is configured to be the central data storage for integration into the building maintenance and operations system(s). LOD 5 Models(s) will include completed parameters and attributes specified in the Owner CIR document and BIM Plan at the completion of construction, the BIM model(s) will be finalized, linked and cross referenced.. LOI5: The prescribed manufacturer products that that meet the generic product specification. Handover and close out; LOD6: Provide the information specific to the installed deliverable that is required for operation and maintenance. Information covering the detailed maintenance should also be provided in the associated PDF manuals; LOD 6 model will contain LOD 1, LOD 2, LOD 3, LOD 4 facility and geometry data respectively and will be configured to contain the Operations and Maintenance manuals, warranty information, submittal information, and/or any other documents as applicable or required.. LO6I: The prescribed manufacturer products that including handover operational and maintenance documentation. In use; Not Applicable; Owner reserved, LOD 7 model elements will not be generated during planning, design or construction phase. Not Applicable. Note; The NBS BIM Toolkit allows the user to define a different level of graphical output (known as Level of Detail - LOD) and non-graphical out (known as level of information- LOI) – The above definitions are used to define the combined output known as Level of Definition. (Unless otherwise agreed) The project will used the following listed work stages under which both the graphical and non-graphical information would need to be provided. These stages are defined against the RIBA Plan of Work 2013 (for clarity as table 2 below). Table 2 : Unless otherwise specified the work stages are to follow those set out in the RIBA Plan of Work 2013. 0; Strategic definition. 1; Preparation and brief. 2; Concept design. 3; Developed design. 4; Technical design. 5; Construction. 6; Handover and close out. 7; In use. Information should be generated and exchanged at each stage to support specific purposes/activities as defined in the data drop process. The BEP should provide a process for explaining each data drop and this should be linked to the supplier’s deliverables. |
|  |  |  | The following table defines the primary data drops and who is the primary responsible party. Table 3 : Outline Data Drops. Stage; Description; Drop; Purpose; Primary Responsibility for Data Drop. Client’s Representative; Supplier (Project Information Manager). 0; Strategic Definition; N/A. 1; Preparation and Brief; 1; Feasibility study. 2; Concept Design; 2; Bid response. 3; Developed Design; 3; Planning application. 4; Technical Design; 4; Final business case + contract documentation. 5; Construction; 5; Construction. 6; Handover and Close out; 6; Practical completion. 7; In Use; 7; Performance in use; # See note below. # Supplier to note; Data drop 7 will be carried out by verification of the AIM and by Facilities Management Team monitoring and updating the Model Data in Use. |

The following table forms the primary Model Production Delivery Table (MPDT) for the project. This is to be reviewed and responded to by supplier in their BEP.

Note the supplier is not required to go beyond data drop 6 however the Client may require the suppliers’ support in transferring data into the Client’s Asset Information Model (AIM) and be available to comply with any Government Soft Landings approach defined elsewhere in the contract documents.

| 1.2.5 Model Production Delivery Table (MPDT) For LOD definitions see clause 1.2.4 Table 1 | Data Drops; Drop 1; Drop 2; Drop 3; Drop 4; Drop 5; Drop 6; Drop 7. RIBA Stage; 1; 2; 3; 4; 5; 6; 7. Model Author; LOD; LOI; Model Author; LOD; LOI; Model Author; LOD; LOI; Model Author; LOD; LOI; Model Author; LOD; LOI; Model Author; LOD; LOI; Model Author; LOD; LOI. Overall Form & Content. Space Planning; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Site and context; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Site Surveys, e.g. Stats, topography etc.; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A; N/A. External form and appearance; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Building and site sections; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Internal layouts; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Design Strategies. Fire; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; N/A; N/A; N/A; Client FM Team; 7; 7. Acoustics; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; N/A; N/A; N/A; Client FM Team; 7; 7. DDA Compliance; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Maintenance Strategy; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. BREEAM; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Others; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Building Performance. Building; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Structural; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. MEP Services; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Regulation compliance analysis; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Thermal Simulation; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Sustainability Analysis; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Acoustic analysis; Contractor led design; 1; 1; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Services Commissioning; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Elements, Materials, Components. Buildings (all); N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Specifications (all); N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. MEP Systems (all); N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. Construction Proposals. Phasing; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; N/A; N/A; N/A. Site Access; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; N/A; N/A; N/A; N/A; N/A; N/A. Site Set Up & Logistics; N/A; N/A; N/A; N/A; N/A; N/A; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; N/A; N/A; N/A; N/A; N/A; N/A. Health & Safety. Design; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; N/A; N/A; N/A. Construction; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; N/A; N/A; N/A. Operation; N/A; N/A; N/A; Contractor led design; 2; 2; Contractor led design; 3; 3; Contractor led design; 4; 4; Contractor led design; 5; 5; Contractor led design; 6; 6; Client FM Team; 7; 7. (# Note: The specific responsibilities for elements of the design should be detailed in the authorship matrix / MIDP contained in the BEP). |
| --- | --- |
| 1.2.6 | Training |  | The suppliers will be required to provide the following training to the Clients’ project team: - Training for the use of agreed CDE platform by the client or their representatives - Maintenance/updating of the agreed client defined dataset - Allow for soft landing protocols in accordance to BSRIA Soft Landings - Navigation around BIMs using freely available software (it is not a requirement to provide model authoring software training to the Client’s team) - Training for access the AIM (BIM360 Field) just before handover Training should be delivered periodically throughout the design and construction period as required. Responsibility for training rests (with the Client / Supplier). |

  - Management

This section deals with setting the standards to be used for the definition and delivery of the project, along with how the co-ordination and review processes will be managed.

| Refer | Item | Description | Response |
| --- | --- | --- | --- |
| 1.3.1 | BIM Standards | This section defines the BIM Standards that are incorporated into the Information Requirements in Appendix 2 of the CIC BIM Protocol. | The fundamental principles of Government BIM Level 2 information management are defined PAS 1192:2 2013. The core documents and standards that are mandated on this project are: - PD 19650-0:2019 - BS EN ISO 19650-1:2018 - BS EN ISO 19650-2:2018 - PAS 1192-3:2014 (Asset information) - BS 1192-4:2014 (COBie/CAFM) - PAS 1192-5:2015 (Information security) - CIC BIM Protocol - CIC Role of the Information Manager - BIM Level 2 The above PAS & BS standards contain references to other standards / codes of practice etc. The supplier is deemed to have taken these “other” requirements into account in the delivery of their BIM obligations. |
| 1.3.2 | Roles and Responsibilities | The purpose of this section is to bring to the attention of the project team the allocation of roles associated with the management of the model and project information. | It is a client requirement that the supplier will ensure that the requirements of this CIR and the responding suppliers BEP will be incorporated into the sub-suppliers / specialists contracts / appointments. All roles and responsibilities should be compiled by the Supplier in a matrix that includes roles and responsibilities of all the existent members of the supply chain. Master Information Delivery Plan (see attached template example) related to the specific Task Information Delivery Plan provided by suppliers. Include Supply Chain Capability Summary (SCCS): a summary of the assessment of suppliers capabilities. Use of CPIC procedure is advised. CPIx Templates are available for: - Supply chain BIM assessment - Supply Chain Resource assessment - Supply chain IT assessment |
| 1.3.3 | Planning the Work and Data Segregation | The purpose of this section is to set out requirements for the bidder’s proposals for the management of the modelling process | The CDE should be managed in accordance with the standards specified in clause 1.3.1 of this CIR. Model Management The supplier is also required to manage model area, volume and zoning strategies. All models and data should hold consistent reference to blocks, zones, rooms and spaces as a means of identifying these elements across the project to provide a common standard. Any change to block, zone, and room or space references should be managed through a formal change control process and can only be made with the express agreement of the Client. Approval of Information Schedule to be included in BEP showing process for approval of drawings and spreadsheet extractions and responsible people for their approval. Naming Conventions The room naming convention and the asset codes naming convention are described in the Client’s Computer-Aided Facilities Management (CaFM) system and document COBie_STFC_Convention_v2, which will be distributed by the Client to the contractor along with these CIRs. The supplier is to ensure that these room naming conventions are incorporated into the supplier’s models and then transferred into the AIM. Data outputs upon appointment and be referenced in the Post Contract BEP and any of their sub suppliers models or non-graphical data. ## For documents and files there should be a single naming convention which is adopted project wide with all information managed in accordance with the processes described in the standards referenced in this CIR. Fully documented procedures are required in the BIM Execution plan. This should include a brief explanation of how to interpret symbols, annotations, and dimensions. |
| 1.3.4 | Security | The purpose of this section is to communicate client specific security measures required in order to secure the data | All information assets must comply with STFC data security strategy. Note the supplier and their supply chain has a duty of confidentiality and a responsibility to safeguard any information or data that they generate and/or access irrespective of whether it is marked or not. The security measures to be adopted and the means by which compliance is monitored and managed shall be set out in the BEP. The BEP should also address Intellectual Property with consideration given to the following as a minimum (note that this list is not exhaustive). - Compliance with Harwell Development Programme security requirements; - Encryption requirements in dealing with sensitive or confidential information; - Back-up and recovery of information held within the CDE; - Anti-virus and firewall protection; - Detection and management of security events and incidents; - How information is used and stored so that its integrity remains intact; - CDE access management; - CDE structure to deal with drafting, publication and archiving plus associated verification states. For the purposes of this project the supplier is to use the security levels as defined by the latest Government Security Classifications. |
| 1.3.5 | Coordination and Clash Detection | The purpose of this section is to define the required coordination process, together with requirements for quality control | Design co-ordination and clash detection check reviews should be carried out periodically as required in order to satisfy design/construction development and to minimise risk and waste. The suppliers BEP should define the clash detection process and workflows and also define the tolerances to which each set of clash detection workflows will be managed. The clash detection process should deal with both hard and soft clashes. For clarity Hard Clashes, this is where the object is tested to verify that the object will not physically clash with another object within the volume / space allocated to it. This can also be used to verify working space for installation purposes. The objects will be further tested for any soft clashes. This is where the space around the object is tested to ensure that the object can be maintained or replaced without a physical restriction caused by another object with the volume allocated to it. The soft clashes regime is to be aligned with the overall maintenance strategy and should provide evidence of adequate access being provided for the safe maintenance / removal / replacement strategy in accordance with the CDM Regulations 2015 and good industry practice. Models should ensure that all rooms are enclosed by room bounding surfaces (wall: wall, wall: ceiling, wall: floor) and that wall edges align to ensure that clashes can be fully exposed and to maximise model potential. Clash avoidance/checking should continue throughout the design process and into the specialist design through to development of the virtual construction models with particular focus on soft clashes and proximity checks. Checks should also be made for health and safety issues. Clash avoidance/checking should continue during construction as the models are updated with as constructed information. These should be checked against the tolerance strategy. The BEP should identify - Details of the clash detection process including: - Software to be used for model federation and clash detection/management - The clash detection/management/resolution process - Responsibilities - Outputs - Technical query workflow - Tolerance strategy - Clash resolution process |
| 1.3.6 | Collaboration Process | The purpose of this section is to define how, where and when project information will be shared | The BEP should detail the agreed collaboration process in order to demonstrate competence and capability. Detailed consideration should be given (as a minimum) to: - Form of sharing – how is model data going to be shared. - Extent of model – what is going to be modelled and to what tolerance / at what stage. - Model workflow diagrams - Frequency of collaboration and information exchange. - The format and attendance at model review workshops and any other associated collaborative working practices. |
| 1.3.7 | Health and Safety / Construction Design Management | The purpose of this section is to enable the Client to define how BIM based working will support H&S/CDM monitoring aligned with the work stages. Data and records capture processes also need to be documented. | The BEP should detail how BIM enabled processes will be used to manage the Harwell Development Programme and supplier H&S/CDM obligations, sufficient to demonstrate competence and capability and to comply with the Clients requirements. Consideration should be given to deliverables at each project stage, requirements for disaster planning and the approach to design authoring. The supplier is to: - Provide an outline project specific construction phase health and safety plan, including but not limited to: - Project overview / description of works; - Project’s site specific risks for contractor’s design; - Contractor design team details; - Safe systems of work and person responsible for their coordination; - Site compound location and arrangements (e.g. Stacking of site huts; - Traffic management plan identifying access arrangements (refer to other section if included elsewhere); and - Construction plant or equipment to be used in project; - Provide project construction phase health and safety plan on their appointment; - Supply relevant information to update F10 – Notification of Construction Project; - Collate information for operation and maintenance manuals throughout the construction period; - Provide completed operation and maintenance manuals in the timescale dictated by handover procedures; - Provide information in accordance with prescribed format; - Provide the Principle Designer with relevant information; - Ensure that all relevant documents including commissioning certificates and record drawings have been provided and indexed within the building manual. |
| 1.3.8 | Systems Performance | The purpose of this section is to communicate to bidders any constraints in the Client’s systems or specific IT requirements which may need additional resources or nonstandard solutions. | To ensure that all information is accessible to all parties: - Individual model files should not exceed 100mb. If model files become slow or unmanageable they should be segregated. Methods for data segregation shall be detailed in the BEP; - Federated models should not exceed 300mb; - A version of all models should be delivered in native format with an associated IFC file; the content of this will be agreed with the supplier with the post contract BEP and associated modelling standards - All outputs from models should be delivered in open file format; - All outputs from models should be delivered in PDF if required; - Access and use of free viewers must not be limited or restricted as a result of model file formats. The supplier should work with STFC to identify the Client IT requirements and constraints to ensure that the (insert project name here) can access all delivered information. These requirements should be addressed in the BEP. |
| 1.3.9 | Compliance Plan | The purpose of this section is to enable the supplier to communicate how the integrity of the model and other data sources will be maintained | Consideration must be given to: - Quality assurance/control procedures; - Software used to support quality control procedures; - The level of assurance that can be given; - Retaining data integrity/accuracy in model format and its 2D output; - Managing models and data during the period of aftercare. |
| 1.3.10 | Delivery Strategy for Asset Information | This section defines the information exchange standard for asset information and enables the Client to obtain proposals with regards to asset information delivery into the Client’s FM environment | The asset information data should be delivered as defined in the Client’s Computer-Aided Facilities Management (CaFM) system and COBie_STFC_Convention_v2. This document contains a list of type of assets that need to be included in the document, the information required associated with each asset, and the naming convention to be adopted in the model. As a general rule all maintainable MEP assets or single assets with a value over £5,000 must be included in the asset list COBie_STFC_Convention_v2. All the information included in the Client’s Computer-Aided Facilities Management (CaFM) system and table “Asset Template2 in COBie_STFC_Convention_v2, must be provided at handover. The BEP should set out proposals for how best to deliver information to the client to allow a coordinated approach. The Client will work with the supplier to define the contents of their Asset Information Model (AIM) and the way this AIM will be interfaced by the Client’s facilities manager into the Client’s Computer-Aided Facilities Management (CaFM) system. |

Commercial

This section looks at the information requirements, defines purposes for data and the content of key deliverables.

# Note: It may be prudent for the client to insert specific PLQs (based on the PLQs taken from the BIM Task Group website) into the CIR, where it makes it easier for the supplier to understand the client overall strategic objectives, sample questions have been provided below – remove this note for final document).

| Ref | Item | Description | Response |
| --- | --- | --- | --- |
| 1.4.1 | Data Drops and Project Deliverables | The purpose of this section is to communicate the content of data drops and how data drops are aligned to work stages. Section 1.3.1 will also explain how data drops relate to the selected procurement process, as well as the purpose and key contents. | CIRs 1.2.4 sets out the arrangement for data drops and LOD / LOI. The Supplier will be required to generate data to support data drops 2, 3, 4, 5, 6 and 7 as indicated in the Model Production Delivery Table, as clause 1.2.4 of this CIR. However the data provided at each of the drops should reflect the Level of Definition required (and as set out in the Model Information Delivery Table). For each Information exchange, information will be required in the following formats: - Native model files - COBie – BS1192-4:2014 - PDF Files - CAD DWGs (floor plans only) - STFC CAFM requirements |
| 1.4.2 | Clients Strategic Purposes | The purpose of this section is to describe the expected purposes of the information provided using the COBie data exchange format. The CIC BIM Protocol does not specifically state the purposes for which models will be used. Setting out proposed purposes in the CIRs informs the scope of the licences defined in the Protocol | Additionally STFC adoption of BIM seeks to achieve maximum client value by increasing benefits at no extra cost. The Client sees the following principal benefits arising from the use of BIM: - Better Cost Certainty - Better Programme Certainty - Improve Health And Safety During Construction - Improve Maintenance and Health and Safety in Use - Improved Sales Areas and Better Shopping Experience It is anticipated that the primary use of the data will be for the following purposes: - To support decision making / programme gateway progression; - Stakeholder Engagement; - Construction Supplier Selection; - Support the construction delivery process - Project Governance and Technical Assurance; - Operation of facility (Asset Management) - Asset Maintenance (Asset Management.) - Data may also be required to be used as a reference for future reconfiguration, extension and/or refurbishment purposes. |

1.5. BIM Competence Requirements

A supplier BIM assessment form should be completed by all appropriate organisations within the supply chain as PAS 91:2013: Construction Prequalification Questionnaire’s, so that they can demonstrate their competence and understanding of BIM. Supplier assessment should consider:

- Standards, processes, procedures and engagement;

- BIM project experience;

- BIM capability;

- Organisation, team and personnel experience;

- Project information production and distribution policies;

- Drawing and model management;

- Document numbering systems;

- Web-based project tools;

- PI limitation;

- Email distribution and internet policies;

- IT infrastructure;

- Operating systems and software;

- Information distribution capability;

- Disaster recovery, security and archiving systems.

Assessment templates are available as follows:

CPIx BIM Assessment Form: http://www.cpic.org.uk/wp-content/uploads/2013/06/cpix_-_bim_assessment_form_ver_1.0.pdf

CPIx IT Assessment Form: http://www.cpic.org.uk/wp-content/uploads/2013/06/cpix_-_it_assessment_form_ver_1.0.pdf

CPIx Resource Assessment Form: http://www.cpic.org.uk/wp-content/uploads/2013/06/cpix_-_resource_assessment_form_ver2.0.pdf

- Other Tender Documents

Note, the following is for advice only, this section should be used to inform the supplier of any additional information that needs to be considered when tendering the works.

- The CIR should contain a list of the contract tender documents that this CIR should be read in conjuction with (and if available) any primary design work / surveys etc that may have already been carried out

- This CIR should make reference to who the primary design team are and in what capacity there are employed (or intended to be employed) , e.g. who is to employ them / novation / work stage this is expected to happen (if applicable)(# Note: if the client has yet to appoint a full team, the client should make reference to which designers are employed and what disciplines they intend to employ and when this is intending to happen).

- The CIR should contain  a reference to a detailed Government soft landings document (in particular and requirements for Post Occupancy Evaluation)  which specifies the Clients requirements and must be issued with this CIR as part of the tender information to the supplier

- Reference should be made to the type of main contract and any specific amendments which effect the provision of BIM to the project

- The Client should ensure that any contracts are aligned to working in a collaborative BIM environment

- The Client should specify if any data sets / COBie are contract completion requirements or linked to the release of retention.

- Acronyms / References used in this document

BEP	BIM Execution Plan

BIM	Building information model

CAFM	Computer aided facilities management

CBDM	Climate-based daylight modelling

CDE	Common data environment

COBie	Construction operation building information exchange

CIR	Client’s Information Requirements

LOD	Level of Detail

LOI 	Level of Information

MIDP	Master Information delivery plan

MPDT	Model production and delivery table

PIM	Project information model

CDM 	Construction Design Management

AIM 	Asset Information Model

FM 	Facilities Management

IFC 	Industry Foundation Classes