Rawcliffe Recreation Pavilion Extension and Refurbishment

Scope of works

External:

- Construct new vestibule as per drawings

- Allow for new UPVC entrance doors and windows.

- Allow for re-routing of footpath and drainage works

- Remove rear entrance doors and reduce opening to approximately one meter

- Supply and fit new single aluminium door

- Make good internally and allow for all decoration.

Internal:

Male and Female Toilets:

- Remove cubicles, vanity unit and all sanitary wear

- Remove existing wall tiles and make good

- Remove window

- Remove entrance door

Building fit outs:

- Alter position of drain for W.C. pans as per drawing; cap-off any redundant drains

- Supply and fit new PVC window

- Supply and fit new oak faced door with ironmongery

- Supply and fit new cubicles and vanity units

- Supply and fit sanitary ware

- Supply and fit tile splashback to urinal area and vanity units.

- Allow for all decoration works

Old changing rooms:

- Remove existing shower cubicles and make good to ceiling and floors

- Remove existing doors and frames

- Remove 4 windows and enlarge openings to suit new windows

- Form 2 new window openings and make good

- Remove all wall tiles and make good all plasterwork

Building fit outs:

- Supply and fit 6 UPVC windows

- Supply and fit new soft oak faced door with top light

- Supply and fit new soft oak faced door complete with ironmongery

- Supply and fit new soft oak architraves and skirting.

- Supply and fit kitchen base units and worktops to form servery as per drawings, allow for entrance flap.

- Supply and fit inset sink to servery

- Supply and fit hand wash to servery.

- Allow for all decoration works

Disabled toilet:

- Remove all sanitaryware

- Remove door

Building fit outs:

- Supply and fit Doc M pack sanitaryware

- Supply and fit new soft oak faced door complete with ironmongery

- Allow for decoration

Hallway and Rawcliffe Parish Council Office:

- Remove internal double doors and door frame – make good

- Construct stud partition with soft wood door frame and oak faced door complete with ironmongery.

- Wall to be plaster finish and fitted with sound quilt.

- Supply and fit soft wood skirting and architrave to match existing

- Supply and fit 3 oak faced doors complete with ironmongery

- Supply and fir 1 pair of oak faced doors to match main hall entrance, complete with ironmongery.

- Allow for decoration to all areas

Electrical works:

- To be completed as per drawings

- To make good all areas affected by electrical works

- To allow for decoration of all areas affected by electrical works.

Decoration:

- Walls – Dulux Diamond range or equivalent

- Ceilings – matt emulsion

- Existing wood work – undercoat and gloss

- New wood work – primer, undercoat and gloss

Ancillary:

Contractor to be responsible for the management and removal of all rubbish and waste materials.

April 2018