THETFORD TOWN COUNCIL

Provision of Stock Fencing for Castle Park

TTC/20/002

Quotation Requirements:

- Written itemised quotation price not to include VAT for softwood posts on stock fencing

- And an alternative written itemised quotation price not to include VAT for chestnut posts on stock fencing.

- Copy of public liability insurance

- Copy of company Health and Safety Policy

- Sample Method Statements

- Sample Risk Assessments

- The council has an expectation that contractors will include a copy of their environmental strategy/policy which explains how your company takes care of the environment. It may include

recycling, energy use, and ways of improving biodiversity.

Failure to supply any of the above will result in the Tender being rejected.

Contact name and number:  Tina Cunnell, Town Clerk, (01842) 754247. Quotations to be received by Friday the 28th of August 2020 the winning contractor will be contacted by September 30th 2020.

Sealed inner envelopes to be clearly marked ‘Tender – Provision of Stock Fencing for Castle Park

For the Attention of the Town Clerk, Thetford Town Council.

Postal Address: The Carnegie, Cage Lane, Thetford, Norfolk, IP24 2DS. Tel: 01842 754247.

Details of the winning contract will be published IAW Local Government Transparency Code 2014.

Tina Cunnell

Thetford Town Clerk

General Notes for Contractors:

- Please itemise the quotation per section using the attached tables

- It is anticipated that the works will be completed autumn 2020.

| Section A - Organisation Details |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full name of organisation tendering |  |  |  |  |  |  |  |
|  | Registered office address |  |  |  |  |  |  |  |
|  | Named contact for this contract: |  |  |  |  |  |  |  |
|  | Email |  |  |  |  |  |  |  |
|  | Phone |  |  |  |  |  |  |  |
| - 6.3 | Company or charity registration number. |  |  |  |  |  |  |  |
| - 6.4 | VAT Registration number. |  |  |  |  |  |  |  |
| - 6.5 | Type of organisation e.g limited company/sole trader |  |  |  |  |  |  |  |
| Section B - Health & Safety |  |  |  |  |  |  |  |  |
| Please provide a copy of your health and safety policy |  |  |  | Yes |  | No |  |  |
| Note: if your organisation has less than 5 employees, the Thetford Town Council still requires you to have a written Health and Safety Policy. |  |  |  |  |  |  |  |  |
| Note: Please enclose Sample Method Statements, Sample Risk Assessments for the types of works included in this contract. |  |  |  |  |  |  |  |  |
| In no more than 200 words can you please state how your company ensures the well being of your employees and the public. Listing what measure you use to comply with health and safety legislation. |  |  |  |  |  |  |  |  |
| Please provide details of the health and safety training you provide to employees, relevant to this contract in the box below eg herbicide use/machinery use. If from your answer we deem that adequate training is not/or has not been carried out, we will reject your bid in full. |  |  |  |  |  |  |  |  |
| Section C - Insurance |  |  |  |  |  |  |  |  |
| You must either confirm that you have the following levels of insurance in place for each and every claim rather than on an aggregate basis or, alternatively, undertake that should you be awarded a contract under this procurement such levels of insurance will be available to you and that you undertake to maintain these levels of insurance for the duration of the contract. |  |  |  |  |  |  |  |  |
| Insurance Policy |  | Indemnity Value (£) |  |  | Yes |  | No | Will secure if successful |
| Employers Liability |  | Min £5m per claim |  |  |  |  |  |  |
| Public Liability |  | Min £10m per claim |  |  |  |  |  |  |
| Section D – Environmental Responsibility |  |  |  |  |  |  |  |  |
| The council has an expectation that contractors will include a copy of their environmental strategy/policy which explains how your company takes care of the environment. It may include recycling, energy use, and ways of improving biodiversity. |  |  |  |  |  |  |  |  |
| In less than 150 words please give examples of how you have put your environmental policy/strategy in place. |  |  |  |  |  |  |  |  |
| The Castle park is a scheduled monument and any works on the Mound will be supervised by archaeologists employed by the council. Could you please provide evidence that you have experience of working with experts on a protected site or provide a statement of your willingness to follow the direction of the Archaeologists. . |  |  |  |  |  |  |  |  |
| Section E – Partnership and flexible working |  |  |  |  |  |  |  |  |
| The Town Council want to have a very positive and constructive relationship with their contractors. Whilst we have tried to specify what we need in the contract we realise that this may change we expect contractors to be flexible. |  |  |  |  |  |  |  |  |
| In no more than 200 words can you please state how your company would work with the Town Management Team to ensure the resources that we have for site maintenance are used wisely ensuring best value for money for the council. |  |  |  |  |  |  |  |  |

Fencing specification for Castle Hill Mound and Ramparts

Castle Hill Perimeter Fencing to allow grazing

Purpose

To install a perimeter fence and gates of approximately 650m and 340m compartments around the Castle Mound and Ramparts that is stock proof for; ‘new-born native breed lambs’, sheep, ponies and cattle. Including, as necessary tree and scrub removal as part of line clearance works. Sections of existing chain-link fencing are to be removed and replaced presented as an option outlined below. Installation of a section of bow-topped fencing is also to be included as an option below. All operations to be undertaken in a manner that minimises impact to the site with materials and installation in compliance with BS EN 10223-1:2012 (was BS 4102)

As this site is a Scheduled ancient Monument all Excavation of Strainer and Gate posts will be carried out under a watching brief from a Historic England approved archaeologist arranged by Thetford Town Council. Therefore we suggest that installation of all gates and strainers is carried out as a concise element of these works for which the watching brief is necessary.

Specification

Delivery of materials: If the contractor is in any doubt as to whether materials delivered to site meet the specifications laid out in the contract, the contractor is to contact the Council’s Conservation Land Manager before delivery to site

The work consists of the following main elements:

- Where necessary to clear line to allow installation of new fence as outlined on attached Maps of Castle Hill Mound and Ramparts Gates and Fencing.  All cut material will be chipped, removed from site or neatly stacked under adjacent standing scrub.

- To install fencing to the following specification;

  - Option One

    - Stock fencing for whole perimeter with field and kissing gates erected at locations highlighted on attached Map 1

  - Option Two

    - Chain link fencing to be removed and replaced along the western perimeter section approximately 200metres. Stock fencing and gates installed on remaining perimeter as Option One

  - Option Three

    - Additional bow-topped fencing to be installed along existing on north western perimeter wall top approximately 35 meters

Gates and Fencing Materials

All types of fencing should be erected in accordance with British Standard 1722. All timber to be softwood pressure treated to British Standard and HC4 (hazard class 4), any cut surface to be treated with recommended sealant – no post tops to be cut without follow up sealant application.

All round posts to be peeled and pointed. The diameter measurements of round posts are a minimum along the entire length of the post.

All timber to be FSC (Forest Stewardship Council) and from a sustainable British source.

All steel to be galvanised to British Standard. All fixings should be in double galvanised or stainless steel. No aluminium fixings should be used.

All strainer posts, turning posts, intermediate posts, breast plates, fence posts and horizontal bars and weather topped ‘post and rail’ shall be impregnated with a copper/organic biocide and industrial preservative in accordance with BS 8417 for a BS EN 335 Use Class 4 application, desired service life >15 years.  Strainer, turning and fence posts to be purchased with minimum 25yr guarantee.  Copies of fence purchase invoices to be supplied to Thetford Town Council on completion of contract.

All components shall be machined to their final dimensions and have a moisture content not exceeding 27% prior to treatment. All ‘post and rail’ rails shall be similarly treated but for a Use Class 3b application.

No notching, cross cutting or boring shall be permitted unless all exposed timber is given two generous brush applied coats of a compatible end grain sealer/preservative. No cut ends should be placed in ground or water contact.

Variable options to be agreed between contractor and Thetford Town Council

Fencing

All posts that are in contact with the ground to be Jakcured or similar 25 year treatment(strainer, turning posts, intermediates)

All other rails can be purchased through local supplier.

Stock Fencing (Standard)

Overall fence (post-top) height 1100mm.

Strainers, box assembly, vertical posts 1x2.4mx150-200mm Ø, 1x2.4mx150-200mm Ø, with 900mmx125mmx75mm breast plates, horizontal bar 2.4mx125-150mm Ø, max 400m apart, minimum of 1.30m into the ground. Multiple boxes to be used as ground conditions dictate, at the contractor’s discretion. Horizontal bars to be rebated max 10mm, fixed by galvanised 12mm Ø, 300mm steel pins, at a height that brings it between the 2 top wires, pins should be driven into a pre-drilled hole of 10mm to prevent moisture from entering the timber. 3.15mm HT plain wire strained around assembly and joined with crimp sleeves. The 12mm Ø pin on the strutting post should be left with 25mm showing to put the diagonal wire over. Traditional strutted strainers may be used without HT top and bottom wires

Turning posts, 2.4mx150-200mm Ø with 900mmx125mmx75mm breast plate, at every change of direction, less than 30 degrees. Minimum of 1.30m in the ground. Strainer assembly to be used at turns exceeding 30 degrees and as ground conditions dictate, at the contractor's discretion to ensure no rising strainers, posts or fencing due to lifting stresses.

Intermediate posts, 1.85mx75-100mm Ø, max 3.5m apart. Minimum 0.75m into the ground.

Tie downs to be used on all types of post as necessary. An intermediate post should be installed at 45º to the vertical post to a depth of at least 0.9m. Tie down post should be fixed with a 12mm galvanised pin using a pre-drilled 10mm hole, between 100mm and 200mm above ground level.

2 plain top wires, 4.00mm mild steel or equivalent as stated in BS 4102 (now BS EN 10223-1:2012), lower wire approx. 25mm above netting, top wire 1050mm above ground level (therefore 50mm below top of post).

Stock netting, HT 8/80/15, green, approx. 25mm above bottom wire.

Plain bottom wire, 4.00mm or equivalent high tensile in BS EN 10223-1:2012, approx. 25mm above ground level

Staples, 40mm x 3.15mm galvanised, barbed, angled across the grain of the timber, driven to a depth that does not crush the galvanising, does allow wire to move as it expands and contracts (or gets pressured by stock), but does not allow enough room for wire to rattle in the wind, which can damage the galvanising.

Wires should be joined by tying to the manufacturers approved specification, or by using a crimp sleeve designed for the task, 2.5 / 3.15 sleeve or 3-4 sleeve and crimped to the manufacturers specification. We don’t consider Gripples to be appropriate.

Gates

Field Gates

Quote to allow for Wooden Field gates to be installed at locations shown on attached Map below. Gates and posts must comply with relevant British Standards for manufacture and installation. All hanging and closing posts to be set in concrete.

Kissing Gates

Quote to allow for wooden or metal kissing gates to be installed at locations shown on attached Map below. Gates and posts must comply with relevant British Standards for manufacture and installation. Compliance with Section 38 requirement for a 1 metre sphere to pass through the gate is also necessary to comply with the relevant Disability Discrimination legislation. Use of Jackson C Gate Metal Kissing gate (or similar) is required for single gate locations and those adjoining metal field gate locations. All hanging and closing posts to be set in concrete.

Bow Top (Loop Top) Fencing

Specifications for Bow Top fencing to match those of recently installed fencing on eastern boundary:

- Materials must meet British Standard BS1722 part 9

- Installation is to along the top surface of existing coping brick topped wall approximately 35 meters

- Finished height to be 1.2 meters above wall top surface

- Black finish required for all bow topped fencing materials

- Anchors used on the wall must not damage the coping bricks with chemical anchors preferred to compression anchors

- No gaps on meeting stock or other fencing or gates to be more than 10cms

Chain Link Fencing

- Installed to the requirements of BS1722 pt 1 & 10

- 1.8 metre high galvanised or plastic coated chain link fencing,

- set on galvanised metal posts or concrete posts

- at least 3 line wires of 3-4.5mmgalv/plastic coated line wires that are tensioned by hand at least at every 25 metres or change in direction

- Steel posts are made from either RSA (Rolled Steel Angleiron) or SHS (Square Hollow Section).

- Posts to be hot dip galvanised or polyester coated with colour agreed with TTC

- Chain link to be fixed to cross wires using plastic coated or galvanised r