Estates Services Service Level Agreement:

I-TAC MICRO AND NAO TECHNOLOGY FACILITY

1.5 X full time cleaner with clean room compatible cleaning materials for (appropriate trained cleaners are needed to provide cover when required). 62 hours per week.

All cleaning is to be carried out according to standard Clean Room practices in line with ISO standards.   Suitable vacuum cleaners and clean room specific equipment and consumables will be provided by MNTC.

The staff that carry out the tasks listed below will require special training. Staff working in these areas should not be suffering from any chronic skin complaint.

# Cleaning Schedule for I-TAC Micro and NanoTechnology Facility

| Frequency | Area | Detail |
| --- | --- | --- |
| Daily | All Clean room areas, rota as table 2 | Vacuum and wet mop/wipe floor, flat surfaces, ledges and windows, in all areas of the clean rooms in rotation so that all areas are cleaned during each week Empty bins Check supplies of Clean room wipes, masks and gloves |
|  | Change Room | Clean and vacuum, wash floors daily empty bins |
| Weekly | R12 Labs | Empty bins |
|  | Display Area | Vacuum clean and dust |
|  | Laundry | Laundry is normally to be changed weekly. Check and record all outgoing garments, note need for any repairs. Check and record all returned garments and sort into lockers. Issue garments for visitors |
| Monthly | Mats in R1 entrance areas | Change fortnightly |
|  | R1 service areas | Vacuum and clean, wet mop floor |
|  | R12 labs | Vacuum and clean, wet mop floor |
|  | R1 Bio lab | Empty bins wet mop floor only |
| Ad hoc | Change Room | Peel off and change tacky mats as required Clean windows as required |
|  | Stock Keeping | Maintain stocks of clean room wipes, masks and gloves and keep supplies available in the Clean rooms. Inform purchasing officer when stocks are low |
|  | Extra Cleaning | Extra cleaning in the Clean rooms (e.g. when equipment is removed or installed or the integrity of the Clean rooms has been compromised) will be required |

# Areas to be cleaned

| Day | Detail | Day | Detail |
| --- | --- | --- | --- |
| Monday | Changing room Wet processing area General processing area Corridors Laundry changeover R1 north clean room area | Tuesday | Changing room Minifab SEM room Entry shower Entry Corridor |
| Wednesday | Changing room Metrology room Lithography rooms Evaporator corridor Evaporator room | Thursday | Changing room Entry shower Entry corridor VB6 room R & D lab MA6 room Etch area |
| Friday | Changing room E Beam room ( EBL300) R12 and R1 Biolab labs rubbish R12 labs and R1 Biolab – clean in rotation during month |  |  |

Reporting:  All issues, non-attendance, compliments should be directed through the Estates help desk or directly to Sally Rutty or Liz Keefe for resolution.  As discussed we are refining the management of SOL cleaning site wide in collaboration with all departments who use SOL and will provide further guidance and processes once agreed.