Call-Off Schedule 20 (Call-Off Specification)

Clarification Q&A

Bid Pack

Attachment 3 – Statement of Requirements

Contract Reference: CCTE24A10

Procurement Title: Provision of Initiation Protocol (IP) Telephony Service across the GPA Estate

CONTENTS

1.	PURPOSE AND OVERVIEW OF THE REQUIREMENT	4

2.	BACKGROUND TO THE BUYER	4

3.	BACKGROUND TO THE REQUIREMENT	4

4.	DEFINITIONS	5

5.	SCOPE OF THE REQUIREMENT	5

6.	MANDATORY DELIVERABLES (SUPPLIER RESPONSIBILITIES)	6

7.	OPTIONAL DELIVERABLES (SUPPLIER RESPONSIBILITIES)	22

8.	POTENTIAL, ACTUAL OR EXPECTED DELIVERY VOLUMES (IF APPLICABLE)	25

9.	LOCATION	26

10.	BUYER RESPONSIBILITIES	26

11.	KEY MILESTONES AND DELIVERABLES	26

12.	Quality	27

13.	SOCIAL VALUE	27

14.	STAFF AND CUSTOMER SERVICE	28

15.	CONTINUOUS IMPROVEMENT	29

16.	SECURITY AND CONFIDENTIALITY REQUIREMENTS	29

17.	PRICING MECHANISM	35

18.	PAYMENT AND INVOICING	37

19.	CONTRACT MANAGEMENT AND KEY PERFORMANCE INDICATORS	37

# PURPOSE AND OVERVIEW OF THE REQUIREMENT

  - The Supplier shall provide IP Telephony based services to the Buyer via the Crown Commercial Services Network 3 Services Framework-Reference RM 6116-Lot 4b-Digital Communication Services (Unified Communications).

  - The purpose of this Call Off Contract is for the provision of IP telephony-based services across the Buyer’s managed building estate. The buildings are located nationwide, as listed in Section 19 Location, of this Statement of Requirements. The Supplier shall provide the Services in accordance with the Service Levels.

  - The Minister for the Cabinet Office on behalf of the Crown represented by the Government Property Agency (GPA) may be referred to as ‘The Buyer’ hereafter.

# BACKGROUND TO THE BUYER

  - The Buyer operates government buildings across the UK and delivers property and workplace solutions across government, managing central government property as a strategic asset. To do so effectively, the buyer requires telephony services within these buildings to provide core functions such as reception, security, hospitality, and workplace service teams. The contract is intended to supersede existing tactical contracts that the Buyer has either inherited as part of building services or awarded for a smaller subset of locations.

  - Without these services, the buyer would not be able to operate buildings in a safe and compliant manner. Telephony is an essential part of the Buyer’s communications structure.

# BACKGROUND TO THE REQUIREMENT

  - This document outlines the requirements for a robust, Cloud-based Session Initiation Protocol (SIP) telephony service to enhance communication capabilities within the Buyer’s managed building network.

## The requirement is to provide a reliable and scalable telephony solution that enhances both internal and external communication.

  - This requirement is to support both office and remote working capabilities and reduce telecommunication costs.

  - The new service will be able to replace existing analogue legacy services (PSTN and others) and potentially replace existing SIP telephony services where appropriate.

# DEFINITIONS

| Expression or Acronym | Definition |
| --- | --- |
| IP | Internet Protocol. Also known as TCP/IP (transmission control protocol/ internet protocol. A digital communications protocol used by modern telephony systems replacing older analogue styles of connection |
| SIP | Session Initiation Protocol. TCP/IP-based network protocol which can be used to establish and control communication connections of several subscribers. |
| Cloud | A remotely based system or service which telephone handsets will connect to in order to function |
| IVR | Interactive Voice Response |
| QoS | Quality of Service – prioritisation of voice and video traffic |
| MoS | Mean opinion Score |

# SCOPE OF THE REQUIREMENT

## The following areas are in scope:

    - The solution shall provide wired and Wi-Fi-enabled IP handsets and softphones, ensuring seamless organisational communication. It shall support the porting of existing telephone numbers, facilitating a smooth transition to the new contract.

    - The service shall fully comply with SIP standards and the Buyer’s data protection regulations (GDPR) regarding the handling of user data. Essential call management features include, but are not limited to, forwarding, voicemail, call recording, and transfer, as well as a user management portal for administration.

    - Additional requirements shall include but are not limited to, providing Direct Dial-In (DDI) numbers, critical phone support with emergency backup, and robust security measures, such as encryption protocols and device authentication. The solution must also support Quality of Service (QoS) to prioritise voice and video traffic and maintain optimal communication quality.

    - The service provider shall deliver a comprehensive service wrap, including Service Level Agreements (SLAs), to ensure performance, reliability, and ongoing support.

  - The following areas are out of scope for this procurement:

    - The contract does not include wired or wireless networking infrastructure, which will either be pre-existing or provided through separate projects.

    - This telephony service will be utilised solely for the Buyer’s essential internal building management.

    - The supplier shall not be responsible for providing or maintaining a dedicated on-site support service for IP telephony systems; all support shall be provided remotely or through scheduled visits as agreed with the Buyer.

# MANDATORY DELIVERABLES (SUPPLIER RESPONSIBILITIES)

## IP Telephony Core Services

    - The primary elements of a telephony solution shall include but not be limited to:

      - Number porting

      - Call Setup

      - Call management

      - Call transfer

      - Call termination

    - IP Telephony Platform & Software Requirement

      - The Supplier shall ensure that the IP Telephony Platform shall deliver a complete, software-driven solution for managing voice communications over an IP network. It shall support a range of telephony features and integrations for enhanced connectivity.

      - The Supplier shall: -

| IPTS_001001 | provide a cloud-hosted, IP-based telephony solution. |
| --- | --- |
| IPTS_001002 | ensure platform compliance with Session Initiation Protocol (SIP) standards. |
| IPTS_001003 | offer integration with softphones and hardware handsets. |
| IPTS_001004 | ensure softphone compatibility across operating systems. |
| IPTS_001005 | ensure compatibility with the Buyer’s current IP Telephony devices, to enable their continued use with the new service. (For information: The buyer has circa 50 IP telephone units in service, all Cisco UC Phone 7841) |
| IPTS_001006 | ensure their solution aligns with the principles and objectives of the UK Government's Wi-Fi First policy. |

    - Call Management and Routing Requirement

      - The Supplier shall ensure effective handling and direction of incoming and outgoing calls within the IP telephony system. Features shall include but are not limited to call forwarding, automated routing, voicemail, and seamless call transfers.

      - The Supplier shall: -

| IPTS_002001 | provide support for features including, but not limited to, call forwarding, hold, transfer, and voicemail features. |
| --- | --- |
| IPTS_002002 | provide configurable automated call routing for departments and sites. |
| IPTS_002003 | provide real-time call monitoring and recording options for compliance, including but not limited, to covert recording, in accordance with the Buyer’s Data Protection policies. |

    - User Administration & Account Management Requirement.

      - The Supplier shall enable centralised control over user profiles, permissions, and settings. This allows efficient management of accounts, access levels, and individual preferences, streamlining system operation and enhancing security.

      - The Supplier shall: -

| IPTS_003001 | provide a secure, role-based administrative portal including, but not limited to, account creation, deletion, and modification. |
| --- | --- |
| IPTS_003002 | provide API access to the same data that is available through the portal. |
| IPTS_003003 | provide detailed reporting on user activity and call metrics. |
| IPTS_003004 | provide provisioning for spare Direct Dial-In (DDI) numbers. |
| IPTS_003005 | bar access to all premium-rate numbers, on all devices covered by this contract by default, to prevent unauthorised or unintended usage. (Premium-rate numbers include, but are not limited to, international premium services, directory assistance, and high-cost service numbers as defined by regulatory authorities). |
| IPTS_003006 | provide an administrative control feature allowing authorised administrators to remove premium number barring on a per-user basis as specified by the Buyer. |
| IPTS_003007 | ensure the process for lifting restrictions is role-based and configurable, limiting access to designated users or groups. |
| IPTS_003008 | enable management and monitoring of premium number access settings via the administrative portal. |
| IPTS_003009 | generate usage reports and alerts for calls to premium numbers, ensuring transparency and cost control. |
| IPTS_003010 | maintain audit logs tracking changes to premium number access settings, including date, time, and administrator details. |
| IPTS_003011 | enforce premium number access control settings across all IP telephony devices, including desk phones, softphones, and mobile clients. |
| IPTS_003012 | ensure compliance with Buyer’s security policies to mitigate the risk of unauthorised premium-rate calls. |

    - Security and Compliance Requirement.

      - The Supplier shall ensure that communication protocols, data handling, and access controls adhere to strict standards, thereby protecting sensitive information and fulfilling regulatory requirements, including GDPR.

      - The Supplier shall: -

| IPTS_004001 | implement encryption protocols including, but not limited to, TLS (v1.2 or better) to ensure secure voice communications. |
| --- | --- |
| IPTS_004002 | ensure compliance with data protection standards, including but not limited to GDPR. |
| IPTS_004003 | perform regular security audits and updates across the whole service provision including but not limited to, deployed phones, supporting cloud service. |

    - Emergency Service Support.

      - The Supplier shall ensure that the Buyer has prioritised access to emergency services at all times, ensuring accurate location data and connectivity during critical situations in compliance with regulatory standards for emergency call handling.

      - The Supplier shall: -

| IPTS_005001 | provide functionality to support calls to emergency services. |
| --- | --- |
| IPTS_005002 | provide location-based services for identifying the caller’s location in emergencies. |
| IPTS_005003 | provide redundant paths for high availability during emergencies. |

    - Quality of Service Level Agreements

      - The Supplier shall establish performance benchmarks, defining metrics including call quality, uptime, and response times to ensure a reliable, high-quality user experience and accountability in service delivery.

      - The Supplier shall: -

| IPTS_006001 | prioritise voice traffic to ensure clear, uninterrupted calls. |
| --- | --- |
| IPTS_006002 | ensure minimum acceptable latency and call quality. |
| IPTS_006003 | ensure 99.99% monthly uptime for IP telephony handsets, with monitoring and reporting. |

## Transition and Migration Services

The Supplier shall facilitate the shift from the existing telephony system to the new IP-based solution. This includes planning, data migration, system testing, and user training to ensure minimal disruption and a smooth transition for all Buyer stakeholders.

    - Number Porting and Migration

      - The Supplier shall enable the transfer of existing phone numbers from the legacy system to the new IP telephony platform. This process ensures continuity by retaining current contact numbers, minimising disruptions, and supporting a smooth transition for the Buyer’s users and external contacts.

      - The Supplier shall: -

| IPTS_007001 | port existing telephone numbers from the current estate to the new telephony system, where required by the Buyer, with minimal service interruption. |
| --- | --- |
| IPTS_007002 | provide full support for the migration of connections, ensuring seamless transition to the new contract. |
| IPTS_007003 | comply with regulatory requirements for number portability and user notifications. |

    - Service Continuity

      - The Supplier shall ensure that IP telephony service remains available during unexpected disruptions, such as technical failures or natural disasters.

      - The Supplier shall: -

| IPTS_008001 | provide detailed transition planning to prevent downtime. |
| --- | --- |
| IPTS_008002 | coordinate with legacy providers and internal stakeholders to manage cutover timelines. |
| IPTS_008003 | provide post-migration support to resolve any immediate connectivity issues. |

    - Documentation and Training

      - The Supplier shall provide essential resources and guidance for the IP telephony system users and administrators. This will include comprehensive manuals, quick-reference guides, and structured training sessions to ensure users understand system functionalities, maintenance procedures, and troubleshooting steps for efficient and effective usage.

      - The Supplier shall: -

| IPTS_009001 | maintain comprehensive documentation of the migration process and timeline. |
| --- | --- |
| IPTS_009002 | provide end-user training to ensure familiarity with the new system post-migration. |

## Network and Connectivity Service Level Agreements

The Supplier shall ensure reliable, high-speed connections between the building and external networks for the IP telephony system. This will include providing robust, low-latency network routing, secure connectivity options, bandwidth management, and monitoring to ensure voice quality.

    - Network Configuration and Integration

      - The Supplier shall set up and align network components to seamlessly support the IP telephony system within the existing IT infrastructure. This will include configuring routers, switches, and firewalls, ensuring compatibility with existing systems, and implementing Quality of Service (QoS) settings to prioritise voice and traffic.

      - The Supplier shall: -

| IPTS_010001 | provide integration with the Buyer’s existing network infrastructure, including wired and wireless. |
| --- | --- |
| IPTS_010002 | ensure compatibility with Quality of Service (QoS) and network prioritisation settings. |
| IPTS_010003 | collaborate with the Buyer’s IT functions to ensure low-latency connections. |

    - Firewall and Security Policies

      - The Supplier shall ensure the protection of the IP telephony system at all times by defining and enforcing rules to control incoming and outgoing network traffic. This includes configuring firewalls to allow only authorised communication, implementing encryption protocols for data security, and establishing access controls to safeguard against unauthorised access.

      - The Supplier shall: -

| IPTS_011001 | ensure secure access through managed VPN or other secure access protocols. |
| --- | --- |
| IPTS_011002 | perform regular security updates to protect against cyber threats. |
| IPTS_011003 | deliver a comprehensive set of Least Privilege firewall rules, detailed and tailored to the buyer's shared network environment, for all traffic associated with the IP telephony service. |
| IPTS_011004 | notify the buyer in advance of any planned changes to these rules to ensure proper evaluation and minimise potential network disruption. |
| IPTS_011005 | perform regular reviews to ensure outdated or unnecessary access is removed. |

  - Disaster Recovery and Resilience

The Supplier shall ensure that the IP telephony system remains operational during unexpected disruptions. This includes backup and failover mechanisms, redundancy in network infrastructure, and recovery plans for critical components to minimise downtime and service impact.

    - High Availability Architecture

      - The Supplier shall ensure that the IP telephony system is designed with redundancy and failover capabilities to guarantee continuous operation and minimise downtime. By implementing multiple servers, load balancing, and backup systems, the architecture provides resilience against component failures.

      - The Supplier shall: -

| IPTS_012001 | ensure redundancy across critical components, to prevent single points of failure. |
| --- | --- |
| IPTS_012002 | ensure the solution supports automatic failover to provide service continuity in case of primary system failure. |

    - Data Backup and Recovery

      - The Supplier shall provide a structured approach to regularly saving and securing critical telephony data. This process involves automated, scheduled backups and mechanisms to quickly restore data in case of system failure, data corruption, or cyber incidents.

      - The Supplier shall: -

| IPTS_013001 | implement a secure, encrypted backup system for all critical data, with daily backups stored in multiple locations. |
| --- | --- |
| IPTS_013002 | define recovery time objectives (RTO) and recovery point objectives (RPO) that meet the Buyer's requirements. |

    - Emergency Power Backup

      - The Supplier shall ensure that critical telephony systems remain operational during power outages. This involves using UPS (Uninterruptible Power Supply) systems or backup generators to maintain power to essential IP telephony infrastructure, preventing disruptions to communication services during emergencies and supporting continuous availability for critical functions.

      - The Supplier shall: -

| IPTS_014001 | ensure emergency power backup for critical telephony systems, including essential handsets, routers, and switching equipment. |
| --- | --- |
| IPTS_014002 | ensure critical phones should operate with independent battery support to ensure functionality during power outages. |

    - Resilient Connectivity

      - The Supplier shall provide robust, uninterrupted network links for IP telephony systems, incorporating redundancy through multiple internet and network pathways.

      - The Supplier shall: -

| IPTS_015001 | maintain multiple internet connections or network paths to provide redundancy in case of primary connection failure. |
| --- | --- |
| IPTS_015002 | ensure the system automatically switches to backup connections without interrupting service. |

    - Testing and Regular Drills

      - The Supplier shall schedule assessments of the IP telephony system’s emergency protocols and failover capabilities. This will include simulating outages, conducting system recovery exercises, and verifying the functionality of backup solutions to ensure preparedness for real-world disruptions.

      - The Supplier shall: -

| IPTS_017001 | conduct regular disaster recovery testing, including but not limited to failover drills and data recovery tests, to verify the resilience plan's effectiveness under real-time conditions. |
| --- | --- |
| IPTS_017002 | provide the Buyer with scheduled testing reports and results to ensure adherence to the resilience strategy. |

    - Emergency Procedure Documentation

      - The Supplier shall ensure all emergency protocols, failover steps, and recovery actions for the IP telephony system are thoroughly documented.

      - The Supplier shall: -

| IPTS_018001 | maintain thorough documentation of emergency procedures for rapid deployment and service continuity. |
| --- | --- |
| IPTS_018002 | provide the Buyer with scheduled testing reports and results to ensure adherence to the resilience strategy. |

    - Technical Design Documentation

      - The Supplier shall ensure that the IP Telephony Technical Design Documentation outlines the detailed architecture, configuration, and operational guidelines for implementing and managing the telephony system, ensuring alignment with the Buyer’s standards.

      - The Supplier shall: -

| IPTS_019001 | develop comprehensive technical design documentation outlining the IP telephony platform’s architecture, configuration, and operation, ensuring it aligns with the Buyer’s strategic objectives. |
| --- | --- |
| IPTS_019002 | Ensure all technical design documents adhere to the Buyer’s formatting, branding, and governance standards, including, but not limited to, version control and metadata tagging. |
| IPTS_019003 | update design documentation, within one Month, to reflect changes in system architecture, features, or configurations, with a detailed change log for tracking purposes. |
| IPTS_019004 | include detailed diagrams, flowcharts, and schematics in documentation to represent the telephony system design. |
| IPTS_019005 | collaborate with Buyer stakeholders to review and validate technical documentation, incorporating their feedback before finalising. |
| IPTS_019006 | ensure design documentation includes compliance checks for all relevant technical standards (e.g., SIP, QoS) and regulatory requirements (e.g., GDPR). |
| IPTS_019007 | submit updated design documentation for formal Buyer approval through the defined governance process within an agreed timeframe. |
| IPTS_019008 | maintain a repository of all design documents accessible to Buyer-authorised personnel, with secure access controls to prevent unauthorised modifications. |
| IPTS_019009 | include disaster recovery, resilience strategies, and failover mechanisms in the design documentation as critical components for system robustness. |
| IPTS_019010 | conduct annual reviews of the design documentation to ensure it remains relevant and accurate, with updates submitted for Buyer re-approval as required. |

    - Staff Training

      - The supplier shall train all relevant personnel on the IP telephony system’s operational, security, and emergency procedures.

      - The Supplier shall: -

| IPTS_119001 | ensure all relevant personnel, as defined by the Buyer, are trained on emergency recovery protocols for swift response during disruptions. |
| --- | --- |

  - Support and Service Management

The Supplier shall ensure continuous assistance and oversight of the IP telephony system. This will include, but not be limited to, helpdesk support, troubleshooting, maintenance, and proactive monitoring to meet service standards, quickly resolve issues, and maintain optimal system performance.

    - Helpdesk and Technical Support

      - The Supplier shall provide users with immediate assistance for troubleshooting and resolving issues with the IP telephony system.

      - The Supplier shall: -

| IPTS_020001 | provide a dedicated 24/7 support line for IP telephony issues. |
| --- | --- |
| IPTS_020002 | provide real-time monitoring of telephony services for prompt issue resolution. |
| IPTS_020003 | provide comprehensive support documentation and user guides. |

    - Incident Management and Reporting

      - The Supplier shall systematically identify, track, and resolve issues within the IP telephony system. This process shall ensure swift incident response, minimising service disruption through defined escalation protocols and root cause analysis.

      - The Supplier shall: -

| IPTS_021001 | define processes for logging, tracking, and resolving incidents. |
| --- | --- |
| IPTS_021002 | produce monthly service reports, including but not limited to uptime, incident history, and SLA performance, in accordance with section 15 Service Levels and Performance of this Statement of Requirements. |
| IPTS_021003 | provide incident escalation paths, including critical support for outages. |

    - Routine Maintenance and Updates

      - The Supplier shall ensure regular system checks, software patches, and performance optimisations to ensure that the IP telephony system remains secure, efficient, and up to date.

      - The Supplier shall: -

| IPTS_022001 | schedule regular software updates and patching. |
| --- | --- |
| IPTS_022002 | complete routine device firmware updates to ensure compatibility. |
| IPTS_022003 | conduct proactive monitoring for device and network health. |
| IPTS_022004 | implement a monitoring system to track the usage of all IP telephony handsets and devices connected to the network. |
| IPTS_022005 | flag devices that remain unused for a continuous period of three months or more for review. |
| IPTS_022006 | investigate flagged devices to determine their reason for inactivity and ensure they are not assigned to critical or reserved roles. |
| IPTS_022007 | regularly report unused devices to the Buyer, allowing validation for deactivation or reassignment. |
| IPTS_022008 | decommission and remove unused devices from the system only after receiving explicit approval from the Buyer. |
| IPTS_022009 | reassign deactivated devices to new users or locations as specified by the Buyer to optimise resource allocation. |
| IPTS_022010 | make this information available via the portal, where authorised personnel can view device activity logs, identify unused devices, and approve actions. |
| IPTS_022011 | generate detailed audit logs tracking device status changes, reassignment approvals, and any administrative actions taken. |

  - Training and User Support

The Supplier shall provide users with the knowledge and assistance required to operate and manage the IP telephony system effectively.

    - End-User Training

      - The Supplier shall ensure all users receive practical, role-specific guidance on operating the IP telephony system.

      - The Supplier shall: -

| IPTS_023001 | provide customised training sessions for the Buyer’s staff on IP telephony features. |
| --- | --- |
| IPTS_023002 | provide training for troubleshooting common issues and device usage. |
| IPTS_023003 | provide onboarding sessions for new users. |

    - Documentation and Self-Help Resources

      - The Supplier shall ensure that the documentation and Self-Help Resources provide comprehensive, accessible guides and FAQs for users, covering system features, troubleshooting steps, and best practices.

      - The Supplier shall: -

| IPTS_024001 | produce user manuals and quick start guides for telephony devices and softphones. |
| --- | --- |
| IPTS_024002 | provide an online knowledge base with FAQs and troubleshooting guides. |
| IPTS_024003 | provide access to live or recorded training webinars for refresher sessions. |

  - Integration and Customisation Services

The Supplier shall provide users with the knowledge and assistance required to operate and manage the IP telephony system effectively.

    - CRM and IT System Integration

      - The Supplier shall ensure seamless connectivity between the IP telephony platform and other essential business systems, such as Customer Relationship Management (CRM) tools and other systems.

      - The Supplier shall: -

| IPTS_025001 | facilitate API access for customising telephony interactions with the Buyer’s digital infrastructure. |
| --- | --- |
| IPTS_025002 | provide a configurable interface for personalisation based on departmental needs. |

    - Futureproofing and Scalability

      - The Supplier shall ensure that the IP telephony solution will adapt to evolving technology and business needs.

      - The Supplier shall: -

| IPTS_026001 | provide flexible infrastructure to support future expansion of the Buyer's telephony needs. |
| --- | --- |
| IPTS_026002 | provide options for additional functionality as technology or requirements evolve. |
| IPTS_026003 | ensure scalability to accommodate increased user capacity or new locations. |

# OPTIONAL DELIVERABLES (SUPPLIER RESPONSIBILITIES)

  - Device Provision and Maintenance Services

    - The Supplier shall provide Service Catalogue items to meet the following requirements, including the associated costs and provisioning times.

    - These requirements are not part of the core service but are items the Buyer may occasionally require the Supplier to provide. (Optional Deliverables are provided as chargeable Service Catalogue items.)

    - The Supplier shall ensure the provision and setup of the physical IP telephony devices for telephony operations.

    - The Supplier shall: -

| IPTS_027001 | Supply IP handsets compatible with wired, Wi-Fi and cellular connections. |
| --- | --- |
| IPTS_027002 | make IP telephony devices available through the Buyer’s service catalogue for users to order. |
| IPTS_027003 | regularly collaborate with the Buyer to ensure the service catalogue remains up-to-date and accurately reflects the available IP telephony devices. |
| IPTS_027004 | provide hardware replacement options, including but not limited to, next-day delivery. |
| IPTS_027005 | supply critical telephony devices, as designated by the Buyer, with integrated backup power solutions to ensure uninterrupted functionality during power outages. |
| IPTS_027006 | supply critical phones, as designated by the Buyer, with internal SIM cards to allow operation on mobile networks if the primary network is unavailable. |
| IPTS_027007 | provide a solution for connecting analogue Plain Old Telephone Service (POTS) devices that cannot be replaced with digital IP alternatives to the IP Telephony system. This service enables continued communication and functionality without requiring device replacement until a suitable IP-based, digital replacement solution becomes available. |

  - Device Authentication and PKI Compliance

    - All telephony devices (wired and wireless) connecting to the Buyer’s shared network must support Public Key Infrastructure (PKI) authentication to ensure secure and seamless integration. Suppliers must provide a PKI-based authentication mechanism that enables devices to connect to the network securely while adhering to Buyer security policies.

    - The Supplier shall: -

| IPTS_028001 | ensure all devices authenticate using device-specific PKI certificates issued by a recognised Certificate Buyer (CA). |
| --- | --- |
| IPTS_028002 | manage the PKI certificate lifecycle, including issuance, renewal, and revocation. |
| IPTS_028003 | ensure Certificates are valid for one year (13 Months) before renewal is required. (However, the ability of reducing the validity period to six months should be considered). |
| IPTS_028004 | ensure PKI authentication aligns with the Buyer's network security framework to prevent unauthorised access. |
| IPTS_028005 | ensure all devices comply with UK Government-mandated security policies and standards for secure communications. |
| IPTS_028006 | provide the Buyer with the root certificate for integration with the network’s authentication system. |
| IPTS_028007 | shall ensure for WiFi-enabled devices, 802.1X authentication and encrypted WPA2/WPA3 protocols are supported in addition to PKI. |
| IPTS_028008 | provide comprehensive documentation detailing PKI implementation, certificate renewal processes, and Buyer security standards compliance. |

  - Device Configuration and Deployment

    - The Supplier shall set up and customise IP telephony devices according to user and departmental requirements, followed by their efficient deployment across various locations.

    - The Supplier shall: -

| IPTS_029001 | provide device pre-configuration based on user and departmental requirements. |
| --- | --- |
| IPTS_029002 | provide on-site deployment and installation for initial setup. |
| IPTS_029003 | provide remote setup support for devices in distributed locations. |

  - Device Exchange and Support Services

    - The Supplier shall provide seamless replacement and troubleshooting for IP telephony devices, ensuring minimal disruption in case of device failure or user issues.

    - The Supplier shall: -

| IPTS_030001 | manage device exchanges for IP phones in designated Buyer buildings. |
| --- | --- |
| IPTS_030002 | ensure availability of replacement devices and provide remote support. |
| IPTS_030003 | provide direct end-user support via a dedicated helpdesk for device-related issues. |

# POTENTIAL, ACTUAL OR EXPECTED DELIVERY VOLUMES (IF APPLICABLE)

  - The following estimates provide an indicative overview of the expected telephony requirements across Buyer-managed buildings. While these figures serve as a procurement and implementation planning baseline, they are subject to change based on operational needs, site expansions, and future service adjustments. Suppliers should ensure flexibility in their solutions to accommodate potential variations in service and hardware volumes.

## Buyer-Managed Buildings: Approximately 93 sites requiring telephony services

## Discrete Lines (Reception, Access Control, Security): Estimated at 1,700 across all locations.

  - Desk Phones: Around 1,190 units deployed for general workspace communication.

## Resilient Critical Phones: Approximately 510 devices with emergency SIM and battery backup for essential services.

  - Additional Services Requiring Telephony Capability: Estimated at 510 instances supporting building operations.

  - Suppliers must ensure their solutions can scale to meet these requirements while maintaining service reliability, security, and performance in line with the overall contract scope.

# LOCATION

  - The Buyer anticipates that most support interactions and service provisions will be conducted remotely, with on-site visits restricted to those considered necessary for the efficient delivery of the Services.

  - A complete list of current locations where services may be required to be delivered is provided in Annex A: Locations of this Statement of Requirements. Consistent service levels, support, and compliance must be maintained across all sites in accordance with the agreed SLAs.

# BUYER RESPONSIBILITIES

  - The Buyer will provide prompt approvals to proceed at each key stage of the contract lifecycle.

  - The Buyer will provide access to all necessary buildings and facilities for the installation, configuration, and testing of IP telephony equipment.

  - The Buyer will make any relevant existing technology infrastructure and network schematics available to support effective integration.

  - Where applicable, the Buyer will provide the Supplier with any necessary Buyer’s Assets, including but not limited to racks, power supplies, and connectivity elements, for use in connection with the provision of the Deliverables.

  - The Buyer will designate a Buyer Authorised Representative to coordinate access, schedule site visits, and quickly respond to supplier inquiries.

  - The Buyer will assure the Supplier’s compliance with all internal governance and procurement policies during the implementation of the IP telephony solution.

# KEY MILESTONES AND DELIVERABLES

## The following Contract milestones/deliverables shall apply:

| Milestone / Deliverable | Description | Timeframe or Delivery Date |
| --- | --- | --- |
| 1 | Planning - Detailed Project plan | Within 10 working days of PO receipt for each building |
| 2 | Service Definition/Design - a design for the support model and processes (of the services set stated in the SOR) and Integration Design - Design of the Tool-set integration (Occupiers and FM providers, Other Suppliers) | Within two (2) weeks of Planning agreed |
| 3 | Set-up and Build - Configuration of systems, integration with other helpdesks, process build and documentation etc) | Within four (4) weeks of Design |
| 4 | Testing - this to include testing the various support scenarios and ensuring processes and roles and responsibilities are clearly defined and agreed. Supporting documentation to be agreed and published. | Within two (2) weeks of Set-up and Build |
| 5 | Service Go-Live | On completion of testing and Approval of Service Readiness |

# Quality

## The chosen equipment manufacturers’ design standards must be referenced within the proposed service.

## The supplier shall be accredited to ISO9000 family standards or equivalent.

## The supplier shall, at minimum, be accredited to ISO14001 standards or equivalent.

## The supplier shall be accredited to Cyber Essentials Plus and have ISO27001 Certification with supporting Statement of Requirements (SoR) for the service offered. The supplier's operations under this contract must come within the scope of that certification and accreditation, and the expectation that this will be maintained for the contract's lifecycle.

# SOCIAL VALUE

  - Themes and Model Award Criteria (MAC)

    - The Themes and Model Award Criteria (MAC) selected from the Social Value Model for this procurement are: 2. Tackling Economic Inequality, and 3. Fighting Climate Change.

    - The following MAC sub-criteria shall be used:

| ## Theme Number and Description | ## Model Award Criteria and Sub-criteria Description |
| --- | --- |
| ## 2. Tackling Economic Inequality | ## 3.2 Innovation and Disruptive Technologies |
| ## 3. Fighting Climate Change | ## 4.1 Additional Environmental Benefits |

    - If you feel you need a better understanding of social value you may wish to:

    - Read the Procurement Policy Note (PPN) 6/20 – Taking Account of Social Value in the Award of Central Government Contracts.  The PPN states that social value should be explicitly evaluated in all central government procurement, where the requirements are related and proportionate to the subject-matter of the contract, rather than just ‘considered’

    - Explore our social value website which has lots of useful information or read our Procurement Essentials article

    - Complete a Government Commercial College e-learning course - ‘Social Value for Commercial Success’.  It takes less than one hour to complete and will help you to better understand what social value is, why it is important and how to implement it.

# STAFF AND CUSTOMER SERVICE

## The Supplier shall provide a sufficient level of resource throughout the duration of the Contract in order to consistently deliver a quality service.

## The Supplier’s staff assigned to the Contract shall have the relevant qualifications and experience (as detailed in the Mandatory Requirements Section) to deliver the Contract to the required standard.

## The Supplier shall ensure that staff understand the Buyer’s vision and objectives and will provide excellent customer service to the Buyer throughout the duration of the Contract.

# CONTINUOUS IMPROVEMENT

## The Supplier will be expected to continually improve the way in which the required services are to be delivered throughout the Contract duration.

## The Supplier should present new ways of working to the Buyer during Contract review meetings as detailed in section 19.

## Changes to the way in which the Services are to be delivered must be brought to the Buyer’s attention and agreed prior to any changes being implemented.

# SECURITY AND CONFIDENTIALITY REQUIREMENTS

  - Call Off Schedule 9 (Security) Long Form terms shall apply.

  - The Buyer requires the Supplier to implement additional security measures that exceed the standard security terms outlined in the framework agreement. These measures aim to enhance protection, ensure compliance, and mitigate risks in line with industry best practices and regulatory expectations.

  - The Supplier shall: -

| IPTS_031001 | ensure that all services are developed in line with recognised security good practice as set out by respected security organisations, including but not limited to: - - the National Cyber Security Centre (NCSC), - the US National Institute of Standards & Technology (NIST), - the Centre for Internet Security (CIS); and - the Open Worldwide Application Security Project (OWASP). |
| --- | --- |
| IPTS_031002 | ensure the services are compliant with the NCSC Cyber Assessment Framework (CAF) Control Objectives and will maintain compliance documentation, shared with the Buyer and subject to the Buyer’s review, which provides comprehensive evidence to demonstrate how they meet each of the CAF Control Objectives. The supplier will provide evidence to facilitate GovAssure assessment of the services. |
| IPTS_031003 | ensure that all services are compliant with all relevant security principles; the Supplier will be expected to provide documentary evidence of this compliance to the Buyer's satisfaction prior to service handover. |
| IPTS_031004 | subject to review and agreement of their services supplied under this agreement as part of the Buyer’s standard Information Assurance process; the Buyer will provide Information Assurance input and feedback to the Supplier throughout the design and build process where reasonably requested, to ensure a secure-by-design approach to service development and delivery. |
| IPTS_031005 | ensure that the service is hardened against a recognised security benchmark (e.g. CIS), as agreed with the Buyer: - - Hardening will be conducted as part of the build for the service prior to technical security testing. - Further hardening will be applied following significant changes to the service. |
| IPTS_031006 | on an ongoing basis, be responsible for working with the Buyer and the Buyer’s other service providers in a collaborative way, to manage the security risks of the service in line with the obligations in the security schedule of the contract. |
| IPTS_031007 | employ measures to continually monitor the service for vulnerabilities: - This will include a security dashboard or regularly updated security view, shared with the Buyer, highlighting the threat management status and security patching status for all devices and other deployed service components. - Vulnerabilities will be patched on a schedule as agreed with the Buyer depending on criticality. - The Supplier will immediately notify the Buyer of any CRITICAL findings and provide regular progress updates on mitigation. |
| IPTS_031008 | employ measures, where the Supplier is responsible for the ongoing management of the service, to continually monitor, prevent, detect and respond to any anomalous activity or indicators of possible compromise on the service. |
| IPTS_031009 | immediately notify the Buyer of any event or activity which requires an immediate response to limit the impact of compromise and provide regular progress updates on mitigation. |
| IPTS_031010 | where the Supplier is responsible for the ongoing management of the service, provide a brief monthly report highlighting any security incidents and anomalous activity for the period and summarising new vulnerabilities, patching progress, trend analysis and other information of security interest. |
| IPTS_031011 | employ measures to prevent physical tampering with equipment. Tamper-proofing measures (such as physical locking mechanisms) are to be preferred over tamper-evident mechanisms. |
| IPTS_031012 | ensure that any device connected to a network port authenticates independently and does not inherit authentication from the host device. |
| IPTS_031013 | disable unnecessary through-ports on IPT telephones to prevent unauthorised access. |
| IPTS_031014 | disable all non-essential IPT functions to enhance security and prevent misuse. |
| IPTS_031015 | implement a service on the Buyer's Layer 2 Network. The Supplier will implement security measures appropriate to protecting: the service; all voice and data traffic handled by the service and related metadata; all other information stored or handled by the service; and the underlying network on which the service sits: - For the avoidance of doubt, the voice and data traffic which the service carries, and the metadata related to the devices that have connected, or attempted to connect to the service are considered sensitive data. - these may include OFFICIAL-SENSITIVE information as well as personal data of a sensitive nature. |
| IPTS_031016 | ensure that the service adheres to the Buyer’s network security model, including but not limited to: - strong network authentication of all handsets and other devices using CA-issued X.509 certificates which are issued by the supplier using their own CA (with the CA certificate to be shared with the Buyer and rotated in good time before this expires) and which are managed by the supplier in line with recognised good practice, - strong (certificate-based) authentication of all traffic flows, - encryption of all network traffic following NCSC recommendations for cryptographic protocols and cypher suite options; and - fully documented data flows with most-restrictive firewall rules. - for the avoidance of doubt, there is an expectation of 802.1x authentication for all wireless handsets and other devices deployed by the Supplier (not PSK) and an expectation of certificate-based authentication for all wired handsets and other devices deployed by the Supplier (not MAB). - All details of secure network configuration will be subject to agreement by the Buyer. |
| IPTS_031017 | provide support for the Buyer’s ITHC and other security testing of the underlying network where relevant and will remediate relevant findings from such testing. |
| IPTS_031018 | restrict access to the service and its data to only those persons with a valid 'need to know': - All access and activities will be attributable to the specific user who initiated them and enable non-repudiation of user actions. - All access will be provided on a 'least privilege basis' so that the minimum level of access is provided to meet the business need. - Access control lists will be regularly reviewed and updated in line with JML processes so that individuals only have access during such time as their role requires it. - The Supplier will retain accounting logs relating to management of the service in a format, and for a period, to be agreed with the buyer. - Sufficient background checks will be conducted on all supplier personnel working on the service. |
| IPTS_031019 | provide a solution which can apply different rules for devices based on grouping by Source IP, device CA and/or device type. |
| IPTS_031020 | ensure that all handsets and other devices delivered as part of this contract meet the Buyer’s hardware security requirements: - Deployed devices must be consistent with NCSC device security guidance. - There must be transparency around where devices are built and full traceability of the origin of all device components. Devices must be manufactured in locations consistent with UK Government policy and explicitly agreed by the Buyer. - The device manufacturer must be able to demonstrate an acceptable baseline for organisational security, evidenced by ISO27001 certification or equivalent as explicitly agreed with the Buyer. - Device configurations must be subject to security testing. |
| IPTS_031021 | secure the Cloud Telephony Service in line with the NCSC Cloud Security Principles and subject to periodic (at least annual) security testing (ITHC) for the duration of the contract: - ITHC will be conducted by a qualified third party under the auspices of the NCSC CHECK scheme. - The ITHC scope will ensure comprehensive coverage of the cloud service and will be shared with the Buyer. - The test report, remediation plans, and all other relevant test artefacts will be shared with the Buyer, along with regular updates on remedial progress. - In addition, the supplier will permit the Buyer to conduct continuous non-destructive external vulnerability scanning of this infrastructure and will remediate the findings of any such scan on a timescale commensurate with the criticality of the finding as determined by the Buyer. |
| IPTS_031022 | ensure the service is implemented in line with recognised security good practices as set out by respected security organisations including but not limited to the National Cyber Security Centre (NCSC) Guidelines. |
| IPTS_031023 | hold ISO27001 Certification and Cyber Essentials Plus Certification. |
| IPTS_031024 | supply a completed Buyer Supplier Security Assurance Questionnaire with the supporting evidence and documentation, at the start of the contract and then, at the beginning of each contract year. This must also be reviewed and resubmitted on an annual basis. |
| IPTS_031025 | accept that access to Buyer and/or HMG premises is subject to the controls and checks that may be in place, and which could change from time to time, by the building landlord, controller, or tenant. These checks could include the search of bags or equipment, and/or a change to the minimum security clearance required to enter the premises unescorted. |
| IPTS_031026 | ensure all supplier (Or subcontractor) staff working on the Buyer’s service shall be based within the UK. |
| IPTS_031027 | ensure all supplier (or subcontractor) staff working on the Buyer’s service shall not offshore production and/or development environment data outside of the UK. |
| IPTS_031028 | ensure all supplier (or subcontractor) staff accessing the Buyer's data service or handling OFFICIAL-SENSITIVE information and/or government assets, must be SC cleared as a minimum. |
| IPTS_031029 | ensure all other employees not included in the requirement above and are providing a service to the Buyer must have a minimum clearance of Baseline Personnel Security Standard (BPSS). |
| IPTS_031030 | ensure all supplier (or subcontractor) staff visiting Buyer Locations must be SC cleared as a minimum. |
| IPTS_031031 | ensure all supplier (or subcontractor) personnel that require access to the Buyer’s data can only do so while holding valid SC clearance. |
| IPTS_031032 | demonstrate how the solution will adhere to: - BIP 0008-1:2004 Code of practice for legal admissibility and evidential weight of information stored electronically. - Data Protection Act 2018 - National Cyber Security Centre (NCSC) Guidelines |
| IPTS_031033 | comply with the Cabinet Office Secure by Design security management requirements. |

# PRICING MECHANISM

## Prices shall be submitted via the e-Sourcing Suite Attachment 4 – Price Schedule excluding VAT and including all other expenses relating to Contract delivery.

  - During the initial contract term (3 years), the contract charges shall be firm and are not eligible for variation. Should the Authority choose to undertake one or more of the optional extension terms then the charges in Attachment 4 - Price Schedule will be increased in line with D7BT - CPI - Consumer Price Index (CPI). The variation of price will be applied on day one of each successive year following the end of the initial term, varying the price compared to the index 12 months prior.

## All prices exclude VAT and include all other expenses relating to Contract delivery.

## All prices are inclusive of expenses, such as travel and subsistence.

## Suppliers are required to submit the following pricing information:

    - Professional Services roles (price per day, price per visit or price per hour) to include but not limited to: -

      - Principal Technical Design Engineer

      - Installation/Configuration Engineer

      - Senior Project Manager

      - Out of Hours Installation Engineer on site (Mon-Fri)

      - Out of Hours Installation Engineer on site (Weekends and Bank Holidays)

      - Floor Walkers / Trainers

    - Core Telephony Infrastructure to underpin local service delivery, to include but not limited to: -

      - VoIP Gateways and Session Border Controllers (SBCs)

      - Telephone Servers & Cloud PBX Solutions

      - Call Routing & Auto-Attendant Systems

      - SIP Trunking

    - Provision of IP Devices, to include but not limited to: -

      - IP Handsets and Devices

      - Hardware Replacement and Warranty Services

    - Licensing & Subscription Costs to include, but not limited to: -

      - Ongoing Support and Maintenance, such as; Helpdesk and support; Software Updates and Security Patching

      - Call Recording and Compliance Features

      - Dedicated Emergency Services Links

      - One Number included within Licensing & Subscription Charge

    - Implementation and Deployment to include, but not limited to: -

      - System Installation & Configuration (including Network & Qos Configuration)

      - Number Porting and Migration Services

# PAYMENT AND INVOICING

## Payment shall only be made against a valid Purchase Order.

## Payment can only be made following satisfactory delivery of pre-agreed certified products and deliverables.

## Before payment can be considered, each invoice must include a detailed elemental breakdown of work completed and the associated costs.

## Amounts due under such invoice shall be payable within thirty (30) days after receipt of such invoice.

## Invoices should be submitted to: Government Property Agency, 23 Stephenson Street, Birmingham B2 4BJ

  - Invoices and credit notes must be sent to the following email: gpaapinvoices@gpa.gov.uk.

## For statements and queries: financeoperations@gpa.gov.uk.

  - You must be in receipt of a valid PO Number before submitting an invoice. To avoid delay in payment it is important that the invoice is compliant and that it includes a valid PO Number in the format of GPA [xxxxx], your full company name and address, clearly addressed to the payee, and a unique invoice number. Invoices must be in PDF or Word format and each invoice should be on a separate attachment. Documents such as JPEG’s or excel do not constitute a valid invoice/credit therefore will be returned. If you are unable to email invoices, please post them to:  Government Property Agency 23 Stephenson St, Birmingham B2 4BJ

## Non-compliant invoices will be returned to you if they are not in the correct format.

# CONTRACT MANAGEMENT AND KEY PERFORMANCE INDICATORS

  - Attendance at Contract Review meetings shall be at the Supplier’s own expense.

  - The Supplier shall appoint a contract manager for the purposes of this contract and who shall manage the day-to-day deliverables of this contract.

    - The Supplier’s contract manager shall be the primary point of contact for receiving and sending communications.

    - The Supplier’s contract manager shall be able to delegate their position to another person; however, they must inform the Buyer immediately.

    - The Supplier shall provide details of the Contract Manager.

  - The Supplier shall ensure that appropriate resources are made available on a regular basis, such that the aims, objectives and specific provisions of this contract can be fully realised.

  - The Supplier and members shall attend face to face or virtual contract review meetings. This meeting shall be the forum for the review by the supplier and the buyer of the progress reports.

  - The Supplier shall ensure that its members shall make reasonable efforts to attend contract review meetings at which the supplier’s attendance is required. If any member is not able to attend a meeting, that person shall use all reasonable endeavours to ensure that a delegate attends on their behalf.

  - The Purpose of the contract review meetings will be to review the Suppliers performance and progress.

  - The Agenda for each meeting shall be set by the Buyer and communicated to the Supplier in advance of that meeting.

  - Contract review meetings shall take place within one week of the progress reports being issued by the Supplier as such location and time (within normal working business hours) as the Buyer shall reasonably require.

  - Contract review meetings shall be fully minute by the Supplier and the minutes shall be circulated to all attendees at the meeting.

## Contract Review meetings shall be Monthly, Quarterly and Annually and as the Buyer shall reasonably require.

  - Contract review meetings shall include the following and is not limited to this list;

    - Discuss and review past performance/progress achievement against service levels, including service credit calculations if applicable

    - Provide assurance that risks are being managed as agreed.

    - Management Information Reports

    - Continuous Improvement agenda items

    - Contractual obligation, including transparency of KPI’s.

  - The following section outlines the core Service Level Agreements (SLAs) the Buyer wishes to implement.   The final Service Level Agreements will be fully documented in the Service Design.

    - Business Hours are Monday to Friday excluding applicable Bank Holidays 08:00 to 18:00

    - Extended Business Hours are Monday to Friday excluding applicable Bank Holidays 07:00 to 19:00

    - The reporting period for all service metrics is one calendar month

## The Buyer will measure the quality of the Supplier’s delivery by:

| KPI/SLA | Service Area | KPI/SLA description | Target |
| --- | --- | --- | --- |
| 1 | Cloud Services Uptime | Cloud Service fully functional 24/7/365. Limited duration and agreed maintenance windows are accepted | 99.99% |
| 2 | Fault Response and Resolution Time | Meet the response and resolution targets for all tickets. First Time Fix Rate report shall be provided for information only | 97% of tickets resolved for P1 97% of tickets resolved for P2 97% of tickets resolved for P3 97% of tickets resolved for P4 |
| 3 | Software/ Firmware updates and Release | Timescale to apply updates, or release client software | General Updates applied within 20 business days of agreed release Security Updates applied within 2 Business Day for Critical and High Severity, and Medium and Low severity vulnerabilities updates applied as agreed, as defined by the Common Vulnerability Scoring System (CVSS) Client software released within 8 weeks of OS or MFD release of update |
| 4 | Billing, Invoicing & MI Reporting | Provision of Invoice Supporting Schedule and MI Reports | 10 business days from period end |
| 5 | Change Requests | Change request timeframe to be agreed for each change request | To be agreed on each change request |
| 6 | Client Deployments | Deployment project timeline to be agreed for each deployment request | To be agreed on each deployment request |
| 7 | Latency and Call Quality | MOS Score | A MOS score of 4 and above |
| 8 | Disaster Recovery | Recovery Time Objective (RTO) | The service should be fully restored and operational within 4 hours following any unplanned outage. |
| 9 | Disaster Recovery | Recovery Point Objective (RPO) | Data and call logs should be recoverable up to the last 15 minutes prior to the incident, ensuring minimal loss of service-related information. |

Response Times

| Priority | Definition | Response | Target Resolution | Hrs. In effect |
| --- | --- | --- | --- | --- |
| P1 | Critical Impact/Urgency Incident (in Operating Hours) | 15mins | 1hr | 24x7 |
| P2 | High Impact/Urgency Incident | 30mins | 4hr | 24x7 |
| P3 | Medium Impact/Urgency Incident | 1hr | 8hr | Operating hours |
| P4 | Low Impact/Urgency Incident | 16hr | 16hr | Operating hours |
| SR | Service Requests | 16hr | 16hr | Operating hours |

### Vulnerability Patching Policy

To ensure the security and integrity of the Buyer’s network and infrastructure, the Supplier shall patch vulnerabilities based on their criticality within specified response times. The following table outlines the required response and remediation times for vulnerabilities:

#### Vulnerability Patching Response Times

| Vulnerability Type | Affected Infrastructure | Response & Resolution Time |
| --- | --- | --- |
| CRITICAL / HIGH | Boundary firewalls or other security-critical infrastructure | Treated as a P1 priority incident; response and resolution times as per the Prioritisation Matrix |
| CRITICAL / HIGH | Other network infrastructure | Addressed through emergency change procedures |
| All other CRITICAL | Any affected system | Addressed through emergency change procedures |
| All other HIGH | Any affected system | Remediated within 14 calendar days |
| MEDIUM | Any affected system | Remediated within 30 calendar days |
| LOW (patchable) | Any affected system | Remediated as part of the normal monthly patch cycle and within 30 calendar days |
| LOW (workaround required) | Any affected system | Workaround/fix within 90 calendar days, unless otherwise agreed by the Buyer in writing |
| Service Hardening (vulnerability remediation) | Any affected system | Implemented within the relevant timescale for the vulnerability |
| Service Hardening (not associated with current vulnerability) | Any affected system | Implemented on a timeline set by the Buyer, and in any case, within 90 calendar days |

Attachment 3 – Statement of Requirements

Provision of Initiation Protocol (IP) Telephony Service

across the GPA Estate

Annex A: Locations

Where requested by the Buyer, the Supplier shall deliver the Services set out in this Statement of Requirements and Order Form at the Buyer’s Locations listed below.

The Supplier shall adhere to the policies and processes of the Buyer’s Locations at all times, which are laid out in the Standards and policies document within the Order Form.

All Supplier Personnel working with the Buyer shall operate from within the UK when conducting any work on or in connection with the Services.

The Buyer reserves the right to change the list of Customer Premises periodically and shall inform the Supplier in writing.

| Property Name | Address Line 1 | Address Line 2 | Town | Postcode |
| --- | --- | --- | --- | --- |
| Crimon Place |  |  | Aberdeen | AB10 1SH |
| International House | Dover Place |  | Ashford | TN23 1HU |
| Priestley House | Priestley Road |  | Basingstoke | RG24 9NW |
| Queensway House | West Precinct | Billingham | Stockton on Tees | TS23 2NF |
| 23 Stephenson Street |  |  | Birmingham | B2 4BH |
| 9 Colmore Row |  |  | Birmingham | B3 2BT |
| Apex House | 3 Embassy Drive | Calthorpe Road | Birmingham | B15 1TR |
| Cannon House | 18 The Priory | Queensway | Birmingham | B4 6FD |
| Colmore Gate | 2-6 Colmore Row |  | Birmingham | B3 2QA |
| Quinton Business Park | 3 Ridgeway | Quinton | Birmingham | B32 1AF |
| Victoria Square House |  |  | Birmingham | B2 4AJ |
| 2 Rivergate | 2 Rivergate |  | Bristol | BS1 6EH |
| Temple Quay House | 2 The Square | Temple Quay | Bristol | BS1 6PN |
| Ty Myrddin | Cambrian Place |  | Carmarthen | SA31 1QG |
| Riding Gate House | 37 Old Dover Road |  | Canterbury | CT1 3JG |
| Capital Tower | Greyfriars Road |  | Cardiff | CF10 3PL |
| Companies House | Crown Way |  | Cardiff | CF14 3UZ |
| 2 Victoria Place |  |  | Carlisle | CA1 1ER |
| Stocklund House | Castle Street |  | Carlisle | CA3 8SY |
| East Midlands Fire Control Centre | Willow Farm Business Park |  | Castle Donington | DE74 2UD |
| Fifty Pembroke Court WS | Pembroke | Chatham Maritime | Chatham | ME4 4EL |
| County House | County House | 100 New London Road | Chelmsford | CM2 0RG |
| Regent House | 65 Rodney Road |  | Cheltenham | GL50 1HX |
| Cheylesmore House | 5 Quinton Road |  | Coventry | CV1 2WT |
| QCA Building | Earlsdon Park |  | Coventry | CV1 3BH |
| 1 Ruskin Square |  |  | Croydon | CR0 2WF |
| 2 Ruskin Square |  |  | Croydon | CR0 2WF |
| Ruskin Square Car Park | Dingwall Road |  | Croydon | CR0 0XH |
| Southern House | Wellesley Grove |  | Croydon | CR0 1XN |
| Bishopsgate House | Feethams |  | Darlington | DL1 5QE |
| Feethams House |  |  | Darlington | DL1 5RD |
| Stephenson House | Alderman Best Way |  | Darlington | DL1 4WB |
| White Rose Way | East of White Rose Way |  | Doncaster | DN4 5ND |
| Abercrombie House | Eaglesham Road |  | East Kilbride | G75 8EA |
| Black Horse House | Leigh Road |  | Eastleigh | SO50 9FH |
| Ash House | Falcon Road | Sowton Industrial Estate | Sowton, Exeter | EX2 7LB |
| Senate Court | Southernhay Gardens |  | Exeter | EX1 1UG |
| One Priory Square | Priory Street |  | Hastings | TN34 1EA |
| High Trees | Hillfield Road |  | Hemel Hempstead | HP2 4AA |
| City Gate | 185 Dyke Road |  | Hove | BN3 1TL |
| Viking Industrial Park |  |  | Jarrow | NE31 1SF |
| Jefferson House | 27 Park Place |  | Leeds | LS1 2SZ |
| Princes Court | 34 York Road |  | Leicester | LE1 5TU |
| Walker House | Exchange Flags |  | Liverpool | L2 3YL |
| One Horse Guards Road |  |  | London | SW1A 2HQ |
| 1-19 Victoria Street |  |  | London | SW1H 0ET |
| 10 - 12 Downing Street |  |  | London | SW1A 2AA |
| 10 South Colonnade | Canary Wharf |  | London | E14 4PU |
| 10-18 Victoria Street |  |  | London | SW1H 0NB |
| 100 Parliament Street |  |  | London | SW1A 2BQ |
| 22-26 Whitehall |  |  | London | SW1A 2EG |
| 3-8A Whitehall Place |  |  | London | SW1A 2A |
| 35 Great Smith Street |  |  | London | SW1P 3BQ |
| 36 Whitehall |  |  | London | SW1A 2AY |
| 55 Whitehall |  |  | London | SW1A 2EY |
| 70-72 Whitehall |  |  | London | SW1A 2AS |
| 8- 10 Great George Street |  |  | London | SW1P 3AE |
| Admiralty Arch | The Mall | St. James's | London | SW1A 2WH |
| Admiralty House | Whitehall |  | London | SW1A 2DY |
| Gwydyr House | Whitehall |  | London | SW1A 2NP |
| King Charles Street | FCDO Office | King Charles Street | London | SW1A 2AH |
| MOD Main Building | Whitehall |  | London | SW1A 2HB |
| Old Admiralty Building | Whitehall |  | London | SW1A 2BL |
| Salisbury Square House | 8 Salisbury Square |  | London | EC4 |
| Sanctuary Buildings | 20 Great Smith Street |  | London | SW1P 3BT |
| 5-8 The Sanctuary | Westminster |  | London | SW1P 3JS |
| The Whitechapel Building | 10 Whitechapel High Street |  | London | E1 8QS |
| Whitehall Systems |  |  | London | CR8 3EE |
| Piccadilly Gate |  |  | Manchester | M1 2WD |
| City Tower | Piccadilly Plaza |  | Manchester | M1 4BT |
| Sunlight House | Quay Street |  | Manchester | M3 3LU |
| 1 Hudson Quay | The Halyard |  | Middlesbrough | TS3 6RT |
| Unit 19 | Mold Business Park |  | Mold | CH7 1XP |
| 9-11 St. Anns Quay | 122 Quayside |  | Newcastle Upon Tyne | NE1 3BD |
| Citygate | Gallowgate |  | Newcastle Upon Tyne | NE1 4WH |
| Civic Centre | Barras Bridge |  | Newcastle Upon Tyne | NE1 8QH |
| Concept House | Cardiff Road |  | Newport | NP10 8QQ |
| Carmelite House | 2 St. James Court |  | Norwich | NR3 1SL |
| Rosebery Court | St Andrews Business Park |  | Norwich | NR7 0HS |
| Agora Building | 6 Cumberland Place |  | Nottingham | NG1 6HJ |
| Apex Court | City Link |  | Nottingham | NG2 4LA |
| Fletton Quays |  |  | Peterborough | PE2 8YY |
| Cobourg House | 32 Mayflower Street |  | Plymouth | PL1 1DJ |
| Crown House | Winston Churchill Avenue |  | Portsmouth | PO1 2PJ |
| Guildhall House | 59-61 Guildhall Street |  | Preston | PR1 3NU |
| Aquis House | 49-51 Blagrave Street |  | Reading | RG1 1PL |
| 2 St Pauls Place | 125 Norfolk Street |  | Sheffield | S1 2LE |
| Greenfield House | 32 Scotland Street |  | Sheffield | S3 7DF |
| Building 3 | Esturia House |  | Stoke on Trent | ST1 5RU |
| Polaris House | North Star Avenue |  | Swindon | SN2 1FL |
| 2 Navigation Walk |  |  | Wakefield | WF1 5RH |
| 34 Clarendon Road |  |  | Watford | WD17 1JJ |
| I9 | Railway Drive |  | Wolverhampton | WV1 1LH |