# STANBRIDGE & HILLVIEW PARKS

# BITMAC EXTENSIONS TO EXISTING MUGA SURFACES

It has been drawn to the attention of Banbury Town Council that any new fencing requirements to replace the existing MUGA goals on both sites may require extensions to the bitmac area rather than just patching as identified in the Contract Specification Documents.  If this is the case, then the following additional specification to the documentation should apply.

However, if the new fencing (including the goal mouths) can be contained within the current MUGA bitmac footprint then all the better.

# BITMAC

  - Excavate as necessary to reduced depth of approximately 210mm below finished level (350mm below ground level) and cart all spoil to an EA approved, off-site tip.

  - Excavate any soft spots in sub-grade as required and dispose.

  - Supply, lay and compact minimum of 150mm of DTp1 crushed stone sub-base material to falls and levels especially where the ground has to be made up. Area to be rolled using 508 kilo compacting roller.

  - Supply, lay and compact 50mm of 20mm dense binder course bitmac, and 25mm of 6mm hardstone open/porous bitmac wearing course to falls and levels.

  - Clear all site of debris and rubble on completion and make good.

  - All Bitmac surfacing to comply with BS 4987.

  - Finished bitmac surface should not exceed surrounding grass surface levels.

# EDGING KERBS

  - Supply and lay on 100mm C7.5 concrete bed and suitably haunch, 915x150x50mm P.C.C. flat top edging kerbs to straight lines and curves.

  - Make good to back of kerb on completion.

  - All kerbs to comply with BS EN 1340.

# OTHER QUERIES

  - Banbury Town Council has been asked whether: -

    - With open goal ends are gates necessary in the goal end fencing?  The answer is no.

    - At both sites, are gates required on both sides of the fencing?   The answer to this is again no, gates are only required on one side.

    - the existing no dog slabs should be retained/ replaced if there are no gates in the goal end fencing?  The decision on this was that in such cases the slabs can be removed with the holes filled with concrete

    - Can rounded corners can be used?   Due to the size of the bitmac pads this will not be possible as it would significantly reduce the playing surface area.