Schedule 2 – Specification

Lot 1 - Aerial Photography and Height Data Supply

(APHDS)

This Schedule sets out what the Buyer wants.

For all Deliverables, the Supplier must help the Buyer comply with any specific applicable Standards of the Buyer.

- Introduction

  - This Specification details the Service Deliverables for orthorectified aerial photography, height data and other delivery services required by public sector users (defined as an ‘Eligible Body’ in the User Licence contained in Schedule 36 (Licences)  (“Users”).

  - Annex A to this Schedule 2 - Specification sets out information regarding the services that the Lot 2 Aerial Photography and Height Web Service Supplier (the “APHWS Supplier”) will provide to the Buyer.

  - The Buyer requires that the Supplier collaborate with the APHWS Supplier to ensure that the requirements of the Users set out in Clause 1.1 above are met.

  - The Supplier shall provide the Services in accordance with this Specification and the terms of the licencing agreement set out in Schedule 36 of the Contract. In summary, all Users will be eligible to access the Services by entering into a ‘free-at-the-point-of-use’ licence with the Supplier.

  - Aside from requirements described below as ‘optional’, the Supplier must commence providing the Services by the applicable date set out in Section 2 below.

- Technical Specification

  - Each technical requirement for the Service is set out in tables A to C below.

  - Within each table, a requirement is graded as follows:

    - Mandatory (“M”). The Supplier must deliver the requirement as part of the Services from the Contract Start Date;

    - Highly Desirable (“M6”).  The Supplier must implement and deliver the requirement as part of the Services within six (6) months of the Contract Start Date. Following implementation, the M6 requirement will be mandatory for the remainder of the Contract Period.

    - Desirable (“M12”). The Supplier must implement and deliver the requirement as part of the Services within twelve (12) months of the Contract Start Date. Following implementation, the M12 requirement will be mandatory for the remainder of the Contract Period.

    - Optional (“O”).  The Supplier can offer the optional requirements however the Buyer reserves the right not to accept the offer in whole or in part.

| #### Table A Specification Requirement – Aerial Photography Products |  | #### Grading |
| --- | --- | --- |
| #### 1. Product / Content |  |  |
| 1.1 | Natural colour orthorectified digital aerial imagery including source photography. The Natural colour orthorectified digital aerial imagery shall contain the red, green and blue bands of the electromagnetic Spectrum. Mean High Water (MHW) Mean High Water Springs in Scotland (MHWS) Line to be provided, if requested by Users. | M |
| 1.2 | False colour orthorectified digital aerial imagery comprising the following 3 colour bands: - Near infrared (ranges from 0.75 micrometers to 1.0 micrometers), visible as red; - Green, visible as blue; and - Red, visible as green. | M |
| #### 2. Data Representation |  |  |
| 2.1 | The digital aerial imagery data provided by the Supplier to each User shall be: - in Raster Format; and - Colour balanced. Additionally, the Supplier should provide the following to Users on request: - Non colour balanced; and - Stereo scans. The Air Height and Spot Height Features provided by the Supplier to each Users shall be provided as a Point Layer. | M M6 M |
| 2.2 | The MHW Line provided by the Supplier to each User shall be: - In Vector Format; - A Line Layer; and - A polygon layer. | M |
| #### 3. Spatial Accuracy |  |  |
| 3.1 | The Supplier shall ensure that the resolution of the digital aerial imagery data supplied to each User is: - Natural Colour orthorectified digital aerial imagery is 25 cm on the ground for the full coverage; - Natural Colour orthorectified digital aerial imagery is 12.5 cm on the ground for all urban areas (urban areas as defined by ONS); - False Colour orthorectified digital aerial imagery supplied to each User is 50 cm on the ground for coverage supplied for full coverage and temporal consistency with the Natural Colour orthorectified digital aerial imagery; - Stereo pairs at capture scale for the full coverage; - Natural Colour orthorectified digital aerial imagery is 12.5 cm on the ground for the full coverage; and - False Colour orthorectified digital aerial imagery supplied to each User is 25 cm on the ground for coverage supplied for full coverage. | M M12 O |
| 3.2 | Where available, the Supplier shall provide higher resolution data to each User. Natural Colour orthorectified digital imagery shall be equivalent to at least 5 cm on ground for urban areas (urban areas as defined by ONS). | O |
| 3.3 | The Supplier shall ensure that the data for Natural orthorectified digital aerial imagery has a Horizontal Positional Accuracy to deliver a confidence level of no less than 95% to deliver +/- 1.25 metre RMSE. The Supplier shall ensure that the data for False Colour orthorectified digital aerial imagery has a Horizontal Positional Accuracy to deliver a confidence level of no less than 95% to deliver +/- 1.25 metre RMSE. The RMSE must be included with the metadata for each image. | M |
| 3.4 | Radiometric Quality - Prior to its compression into the delivery format, both the Natural and False Colour orthorectified digital aerial imagery derived from the photography shall be captured using full 24 bit colour or better with three colour bands (RGB). | M |
| 3.5 | Orthorectification accuracy shall be consistent throughout the imagery data product. | M12 |
| #### 4. Data Quality |  |  |
| 4.1 | The Supplier shall ensure that Natural Orthorecitifed aerial imagery and False Orthorecitified aerial imagery is created using full stereoscopic coverage of the source photography. | M |
| 4.2 | The Supplier shall ensure that the Aerial Photography Data supplied to each User complies with RICS 2001 or better specification for vertical air photography or equivalent. | M |
| 4.3 | The Supplier shall ensure that the Aerial Photography Data product supplied to each User is of a consistently high aesthetic quality across the full extent of the Aerial Photography Data in terms of: a) radiometric consistency; b) Brightness - Rainbow effects are caused by image processing and can be visible at the border of bright areas, this must be kept to a minimum; c) Shadows - Obscuring shadows must be kept to a minimum; d) Contrast - Contrast must be consistent across the image; e) Absence of cut lines; f) Foreign artefacts - Any images with artefacts that obscure objects/features should be rejected; g) Radial displacement of elevated features; h) Colour balance - Colour misregistration and bleeding must be kept to a minimum; i) Absence of snow cover; j) Absence of cloud cover; k) Data capture for Scotland is only attempted between April and September inclusive and complies with all other quality requirements; and l) Trees in leaf. | M M6 O |
| 4.4 | The Supplier shall ensure that the Aerial Photography Data supplied to each User is of a consistently high aesthetic quality across the full extent of the Aerial Photography Data in terms of: a) Shadows - Obscuring shadows must be kept to a minimum, such as in mountainous areas where shadowing must be less than 3% per square km; b) Snow cover must be less than 1% per 5 km by 5 km block, and less than 1% per 1 km image; and c) Cloud cover - Cloud cover must be less than 1% per 5 km by 5 km block, and less than 1% per 1 km image. | M12 M12 M6 |
| 4.5 | The Supplier shall ensure that the source photography used to create the Natural orthorectified digital aerial imagery and False colour orthorectified digital aerial imagery supplied to each User is flown to RICS specifications. | M |
| 4.6 | Natural colour orthorectified digital aerial imagery must cover all areas of the coast out to Mean Low Water, including intertidal areas. | M |
| 4.7 | If requested by a User, and where reasonably practicable, photography covering large intertidal areas such as significant areas of mudflat or large estuaries shall be taken at low tide (within 2 hours either side of low water/tide). | O |
| 4.8 | Compressed Aerial Photography formats shall be sourced from a previously uncompressed TIFF, with a maximum target compression ratio of 10:1. The Aerial Photography Data should also be available in original uncompressed format if required by a User. | M |
| 4.9 | Within one (1) month of the Contract Start Date, the Supplier shall provide a draft plan to the Buyer detailing how they will be quality assuring the data and products. The final plan will be approved by the Buyer. | M |
| #### 5. Supply Options |  |  |
| 5.1 | The Supplier shall provide the aerial photography data with the appropriate geo­referencing files: (a) JPEG; (b) ECW; (c) Uncompressed TIFF for individual frames; and/or (d) GEOTIFF. (e) ERDAS Imagine; and/or (f) JPEG2000. | M |

| #### Table B Specification Requirement – Height Products |  | #### Grading |
| --- | --- | --- |
| - Product / Content |  |  |
| 1.1 | The Contours Data provided by the Supplier shall be at 5 metre intervals in all areas. The Supplier shall ensure that individual Line segments of a given value must exactly join neighbouring Line segments sharing the same height value. The Contours Data provided by the Supplier shall also include the following Features: (a) Air Heights; (b) Spot Heights; (c) an MHW (MHWS in Scotland) Line if requested by Users; and (d) a Unique Identifier for each feature drawing upon Geospatial Commission linked identifier schemes best practice guide. A polygon contour layer shall be provided by the Supplier to each User. | M M12 |
| 1.2 | Each Feature provided by the Supplier to each User shall have associated Attributes, including as a minimum: Contours Product (a) Unique Identifier - drawing upon Geospatial Commission linked identifier schemes best practice guide; (b) Version number; and (c) Elevation; Air Heights (a) Unique Identifier - drawing upon Geospatial Commission linked identifier schemes best practice guide; (b) Version number; (c) Air height elevation; and (d) Coordinates; Spot Heights (a) Unique Identifier - drawing upon Geospatial Commission linked identifier schemes best practice guide; (b) Version number (c) Spot height elevation; and (d) Coordinates; and MHW/MHWS Line a) Unique Identifier - drawing upon Geospatial Commission linked identifier schemes best practice guide; b) Version number; and c) Description. | M |
| 1.3 | The Digital Terrain Model (DTM) / Digital Surface Model (DSM) shall be provided by the Supplier to each User. The Supplier shall ensure that the DTM supplied to each User is a bare earth surface model, with surface objects such as buildings, bridges and vegetation removed but flood defence embankments retained. The Supplier shall ensure that the DSM supplied to each User includes all surface objects such as buildings, bridges, flood defence embankments and vegetation. The DTM/DSM provided by the Supplier to each User as a raster data type shall have associated metadata information either in DTM header depending on DTM format or as a separate file, including: (a) number of columns; (b) number of rows; (c) lower left corner x-coordinate; (d) lower left corner y-coordinate; (e) pixel size; (f) NODATA_value; and (g) elevation. NODATA_value used will be- 9999 by convention and refers to all pixels that do not contain any data (null values). Ground control points used for the DSM/DTM should be made available upon request. | M M6 |
| #### 2. Data Representation |  |  |
| 2.1 | The DTM Data provided by the Supplier to each User shall be in raster Format of a 5m cell size. The DSM Data provided by the Supplier to each User shall be in raster Format of 2m cell size. The DTM Data provided by the Supplier to each User shall be in raster Format of 2m cell size. | M O |
| #### 3. Spatial Accuracy |  |  |
| 3.1 | The Contours Data and DTM/DSM provided by the Supplier shall have: - a Horizontal Positional Accuracy to deliver a confidence level of no less than 98% to deliver a +/- 2.5 RMSE from the sample mean or better; and - a Vertical Positional Accuracy to deliver a confidence level of no less than 98% to deliver a +/- 1.5 RMSE from the sample mean or better. | M |
| 3.2 | The Contours Data and DTM/DSM provided by the Supplier to each User shall be stored to a minimum coordinate precision of a centimetre. | M |
| #### 4. Data Quality |  |  |
| 4.1 | The Supplier shall ensure that the Contours Data and DTM/DSM provided to each User contains each and every land feature in Great Britain. The Contours Data provided by the Supplier to each User shall have no less than 100% of all Attributes/metadata correctly assigned to each Feature. The Supplier shall maintain the Contours Data to this level of accuracy. No less than 100% of the Contours Data provided by the Supplier to each User shall be Topologically Correct. The Supplier shall maintain the Contours Data to this level of accuracy. The Contours Data provided by the Supplier to each User shall originate from a DTM (Digital Terrain Model). The Supplier is to provide details of methodology of how the DTM/DSM data has been and will be generated. | M |
| 4.2 | The DTM/DSM provided by the Supplier to each User shall have no less than 100% of all metadata information correctly assigned to each DTM/DSM and be Morphologically correct. The Supplier shall maintain the DTM/DSM to the same level of accuracy. | M |

| #### Table C Core Requirements for All Products |  |  |  |
| --- | --- | --- | --- |
| 1. Data Capture |  |  |  |
| 1.1 | The Supplier shall ensure that: - the Natural Colour orthorectified digital aerial imagery; - the False Colour orthorectified digital aerial imagery; and - the DTM/DSM all cover the complete land area of Great Britain, including coastal structures such as bridges and piers, and are maintained so that they shall be no more than five (5) years old at the Contract Start Date. The Supplier shall ensure that all imagery is maintained so that it shall be no more than three (3) years old by the start of the second year of the Contract. |  | M12 O |
| 1.2 | The Supplier shall ensure that False Colour orthorectified digital aerial imagery has 100% temporal compatibility with the true colour orthomosaics. |  | M |
| 1.3 | All products must have the same currency within one (1) year of the Contract Start Date. |  | O |
| 1.4 | The Supplier must have in place and maintain a Data Capture Programme. The Data Capture Programme is to be provided to the Contract Manager at the start of the Contract or within one (1) month of the Contract Start Date. The Supplier shall provide the Data Capture Programme for subsequent years, no later than three (3) months prior to the start of the flying year, to the Contract Manager for the Contract. The Supplier will publish progress against the Data Capture Programme on the User website, at least monthly. The Supplier will ensure that a currency layer is included in data downloads/web services products showing the currency of each tile. |  | M |
| 1.5 | The Supplier will prioritise certain areas for capture at any time at the Buyer’s request and ensure that data is made available no later than two (2) months after capture. The Supplier will capture areas at a higher resolution than 12.5cm at the Buyer’s request. |  | M12 O |
| 2. Coverage |  |  |  |
| 2.1 | #### Natural colour orthorectified and false colour orthorectified digital aerial imagery data and DTM/DSM data provided by the Supplier to each User shall cover 100% of Great Britain (England, Wales and Scotland) in terms of land area. |  | #### M |
| #### 3. Reference framework |  |  |  |
| 3.1 | The data for GB provided by the Supplier to each User shall be supplied in the British National Grid (OSGB36®). The Contours Data and DTM/DSM data provided by the Supplier to each User shall be supplied with the vertical datum based on the Mean Sea Level at Newlyn with the exception of Scottish island groups that use their own specific vertical datums. Data will also be provided in WGS84 and other frameworks by the Supplier to each User. |  | M #### M6 |
| #### 4. Supply Options |  |  |  |
| 4.1 |  | The Supplier shall provide data to the APHWS Supplier prior to the data being made available to Users directly to ensure temporal alignment between web services and the Supplier’s own data delivery services. Data should be provided to the APHWS Supplier as per this Specification. | M |
| 4.2 |  | The Supplier shall provide the Data to each User on one or more of the following media: (a) CD and/or DVD; (b) portable hard drive; and/or (c) Download, as the User may specify within 15 working days of receipt of a Data Request from a User. The Supplier shall also provide each User with online access to a history of Data Requests for that User. The Supplier shall at all times have and maintain a strategy for supplying Datasets in any new format which is introduced during the term of the Contract. Additional media to be provided by the Supplier to User on request. Where media requests are inappropriate for the quantity of data being requested by User, the Supplier will engage with the User to suggest alternatives. | M M |
| 4.3 |  | The Supplier shall label each CD, DVD or portable hard drive with: (a) order number; (b) relevant filenames; (c) Data Format; and (d) Date of supply. | M |
| 4.4 |  | The Supplier shall name each download file with: (a) Order number; and (b) Date of capture. | M |
| 4.5 |  | The Supplier shall provide the data to each User: (a) for Great Britain as a whole; (b) for one or more of the following countries: (i) England; (ii) Wales; and/or (iii) Scotland, as the User may specify. | M |
| 4.6 |  | The Supplier shall provide the digital aerial imagery Data, contours data and DTM/DSM to each User according to a specific User Defined Area. The User shall define the User Defined Area as a Tile, Bounding Box or Irregular Shaped Polygon. | M |
| 4.7 |  | The Supplier shall provide the Data to each User as a series of predefined Tiles. The Aerial Imagery Tiles shall be of a 1km x1km size. The Contours Data and DTM/DSM Tiles shall be 5km x 5km size. | M |
| 4.8 |  | Data provided by the Supplier to each User in the form of Tiles shall have each Tile Edge Matched. | M |
| 4.9 |  | The Data provided by the Supplier to each User shall be supplied as individual frames, for project specific areas, as requested by Users. | M |
| 4.10 |  | The User may request a complete annual resupply of Contours, DTM and/or DSM at any time. | M |
| 4.11 |  | The Supplier shall provide each User that has expressed an interest in the GB wide data set with Full Supply of Digital Aerial Photography, DTM, DSM and Contours Data on request The Supplier shall provide each User with Full Supply, or resupply, of Digital Aerial Photography, DTM, DSM and Contours Data for the area specified by the User that they require data and updates for. | M M |
| 4.12 |  | The Supplier shall provide each User with Full Supply, or resupply, of the Data on request. The Supplier shall provide each User with Change Only Updates: (a) continuously throughout the contract as new data becomes available; or (b) at fixed intervals at members reasonable request. | M M12 |
| 4.13 |  | The Supplier shall provide semi processed versions of the products to Users on request. | M |
| 4.14 |  | The Supplier shall provide a processing plan to the Buyer one (1) month prior to the end of the capture season in each Contract year. This plan will be published on the User’s website within two (2) weeks of the start of the processing schedule. The plan will also be made available as a scaled layer within the data products. All data captured in a given calendar year will be made available within six (6) months of the capture date. All data captured in a given calendar year will be made available no later than the end of that calendar year. | M M6 O |
| #### 5. Metadata - All Products |  |  |  |
| 5.1 |  | The Data provided by the Supplier to each User shall be accompanied by the mandatory ‘Elements of Discovery Level Metadata’ conforming to the UK Gemini 2.3 standard (and future updates to this standard). Metadata will conform to the standards required for imagery data to be used for legal reasons. | M |
| 5.2 |  | Additionally, individual metadata shall be supplied to accompany each Tile, and this will be provided as a vector layer within the product. Additionally, metadata shall be supplied in spatial formats (e.g. ESRI shapefile and Mapinfo equivalents) covering the entire dataset. This metadata shall also include the following information: (a) tile reference; (b) flight line; (c) time and date of capture; (d) year of capture; (e) image footprints; (f) process applied to data; (g) Ortho accuracy; and (h) RMSE. The cut lines within Tiles should be supplied within the vector layer with associated metadata. | M |
| #### 6. Data ordering - User Support and Communication with Users |  |  |  |
| 6.1 |  | The supplier will provide a User with Datasets as they may request as per the timescales specified in the Service Levels (Schedule 10). | M |
| 6.2 |  | The Data provided by the Supplier to each User shall be available for ordering online on a website provided by the Supplier. The Supplier shall also provide each User with online access to a history of Data Requests for that User. | M M |
| 6.3 |  | The Supplier shall provide Users with a range of User support options including, but not limited to: - Data Capture Timetable, including progress against the published Timetable; - Geographic areas associated with the capture timetable, and progress against capture timetable, supplied in spatial formats (e.g. ESRI shapefile and Mapinfo equivalents). This must enable Users to view the year when the imagery was captured or will be captured in future. | M |
| 6.4 |  | The Supplier shall provide each User with contact details of such person who shall be the User’s nominated point of contact. The Supplier must have in place the appropriate level of qualified resource to accommodate Users with differing complexities and maturity levels during the Contract. | M |
| 6.5 |  | The Supplier must ensure an effective communication plan is put in place with Users in order to provide the appropriate level of support and guidance required throughout the Contract. | M |
| 6.6 |  | The Supplier must manage and resolve concerns and queries, including a process for managing queries, data error reporting and User service complaints. | M |

- Copyright, Licensing and usage of data

  - Schedule 36 of the Contract contains the licence agreement for use between the Supplier and Users for the User to receive and use the Supplier’s data (the “User Licence”).

  - In summary, the User Licence is a perpetual irrevocable, non-exclusive royalty-free licence for the User to use the data in accordance with the terms of the User Licence.

  - The Supplier will also make data available to the Lot 2 APHWS Supplier under a separate licence that will enable:

    - the Supplier to meet the requirements for the provision of the Services as set out in this Specification; and

    - the APHWS Supplier to meet the requirements for the provisions of the web services as set out in Annex A of this Schedule 2 - Specification.

  - A licence between the Supplier and the APHWS Supplier will be entered into (on terms to be agreed) in accordance with the requirements set out in the Implementation Plan (Schedule 8).

  - The Supplier must notify the Buyer of any proposed changes to the licence between it and the APHWS Supplier.

  - The Supplier must notify the Buyer of any other third-party licensing agreements required to meet or comply with the Specification.

- Providing data to the APHWS Supplier

  - Subject to the Implementation Plan (Schedule 8), the Supplier shall provide an initial dataset within two (2) weeks from the Contract Start Date to the APHWS Supplier who will publish it through its web services in accordance with the process set out in Annex A of this Schedule 2 - Specification.

  - For the duration of the Contract, the Supplier shall continue to provide the APHWS Supplier with data as it becomes available.

  - In complying with its obligations to provide the Services to the Buyer, the Supplier is required to:

    - respond to any reasonable request from the APHWS Supplier relating to;

- the provision of data or metadata for the purposes of updating the web services;

- technical queries and/or error reports associated with either the data or the metadata provided to Users;

- eligibility queries relating to Users access to APHWS Supplier’s services; and,

- User queries received by the APHWS Supplier that relate to the data provided under this Contract rather than the web services.

    - advise the Buyer immediately of any issues with the provision of data to the APHWS Supplier.

- Implementation and On-Going User Support

  - In accordance with the Implementation Plan (Schedule 8),  the Supplier will undertake an implementation exercise to onboard all current Users of the incumbent supplier and this will include:

    - entering into new User Licences for each existing User;

    - ensuring transition of existing User information from the incumbent;

    - ensuring transition of the existing User information to the APHWS Supplier (as applicable);

    - ensuring support for existing User to ensure that they gain immediate access to the data and Services.

  - As part of the Service Deliverables, the Supplier will provide a website for ordering data as described in Table C above which contains, without limitation, the following information, documents and tools which enable the User to self-serve the following:

    - information relating to the Supplier’s digital services, products and other corporate information;

    - tools to enable Users to inform the Supplier of errors and omissions in the data;

    - technical supporting documentation to help Users use the Supplier’s products, including product update release schedules;

    - information on this Contract and on the use cases for the data provided under this Contract; and

    - a log of quality reports for all known data quality issues. These reports will be the mechanism to ensure that all quality issues are recorded, actioned and monitored until rectified to the satisfaction of all parties. All reports to be online within five (5) Working Days.

  - As part of the Service Deliverables, the Supplier will provide support to Users, with the appropriate level of qualified resource to accommodate users with differing complexities and maturity levels during the contract, through:

    - its own User website;

    - a dedicated helpdesk (that will charge local UK call rates) and will operate in Business Hours (09:00 to 17:00 hours) on Working Days;

    - trained helpdesk User support agents with dedicated phone lines and email addresses; and

    - helpdesk support that will include, but is not limited to:

- onboarding of new Users,

- change of account details, Username and password reset;

- licensing enquiries or clarification of terms and conditions;

- guidance on resolving day to day technical issues;

- order status clarification;

- notification of potential errors and omissions;

- supporting the communication and management of impacts as a result of new developments under the Contract, such as new data updates, that may impact on User’s' business; and

- assistance with, online order issues, invoice payments and product developments;

    - a communication plan put in place with Users in order to provide the appropriate level of support and guidance required throughout the Contract.

  - The Supplier will ensure all Users’ queries are responded to and resolved within the times specified in the Service Levels in Schedule 10.

  - Business critical issues with the Services specified under this Contract will be addressed within the timescales specified in the Service Levels in Schedule 10. An issue, complaint or query is deemed to be business critical if the User states that it is business critical.

  - The Supplier will maintain a log of all queries and complaints and these will be shared on request with the Buyer. The log will detail items including but not limited to:

- date and time of query;

- name of User organisation;

- query details;

- target and actual date time of response; and

- query response.

  - The Supplier will operate a complaint’s procedure and will respond to and resolve complaints times specified in the Service Levels in Schedule 10. Complaints include but are not limited to any User expression of dissatisfaction about the Services provided by the Supplier, whether in writing or spoken. Generally, complaints will arise when the User has suffered (or may suffer):

    - material inconvenience;

    - material distress; and/or

    - financial loss.

  - The helpdesk shall log the query and/or complaint and time received and do the following:

    - acknowledge the query/complaint on the same day as it is received, providing a time by which the query will be resolved if it cannot be resolved immediately; and

    - resolve the query/complaint by the time specified.

  - The Supplier will provide information relating to User support on request to the Buyer at monthly contract performance review meetings.

- Contract Management

  - The Authority will nominate an individual to represent the Users in the management of this contract as Contract Manager.

  - The Suppliers will nominate a primary point of contact as a representative for the management of this contract, an Account Manager, who will act as first point of contact for the Authority’s Contract Manager.

  - The duties of the Account Manager include, but are not limited to:

    - operate the contract and report on performance in accordance with the Specification of Requirements and agreed Key Performance Indicators, including but not limited to those outlined in Schedule 10;

    - to assess and manage Requests for Change with the Authority’s Contract Manager  in accordance with Schedule 21 and provide supporting statements if necessary;

    - to review and report on the delivery of services under the contract, including:

    - any delivery services incidents and any further actions required;

    - any planned work, during the previous or next period outcomes and any further actions required;

    - risks to delivery of goods and services;

    - progress on the implementation plan, as specified in Schedule 8 of the Terms and Conditions;

    - compliance with cyber essentials and data protection requirements, as specified in Schedules 19 and 20;

    - to endeavour to identify any other items / risks which may have a material impact on the contract insofar as such other items / risks relate to the remit and responsibilities of the Account Manager and the Authority Contract Manager.

  - The Suppliers’ Performance will be assessed in line with Government Commercial Function strategic contract management tools.

  - The Suppliers will provide performance reports within five (5) Working Days following the end of each month or within five (5) Working Days of a request by the Authority. Reports will measure their performance against the Service Levels (Schedule 10).

  - The Suppliers’ Account Managers will be required to attend a monthly contract performance review meeting at the Authority’s offices, or at a frequency and location specified by the Authority.

  - The Suppliers will be required to attend quarterly Public Sector Geospatial Contracts Assurance Group meetings and support the Authorities engagement with relevant subject matter experts across the public sector for the duration of this contract.

### Annex A:

### Information on services provided by the Lot 2 (APHWS) Supplier

### [Redacted]