| 1.0 | Site preparation |  |
| --- | --- | --- |
| 1.01 | Scope of the Works The works entails the design, manufacture, installation and commissioning of a suitable Dissolved air Filtration System (DAF). |  |
| 1.02 | Drawings This schedule is to be read in conjunction with the following King Welsh drawing numbers and documents: Fabric KW-294-D00 – Proposed location plan |  |
| 1.03 | Site Visit The Contractor must, during their site inspection, make all necessary measurements and allowances to undertake and complete the works. No claims will be entertained that result from the Contractor's failure to undertake a Pre-tender inspection and fully familiarise themselves with the Scope of the Works. |  |
| 1.04 | Existing Health & Safety File In order to view the existing Health & Safety information please contact: Jo Martin Email: jo@pellows.co.uk Tel. 01872 863927 |  |
| 1.05 | Asbestos No record information exists. Please see PCI for asbestos precautions. |  |

| 2.0 | Groundworks |  |
| --- | --- | --- |
| 2.01 | A new suitable, level load bearing base will be provided by the employer prior to works commencing on site. No allowance required for any ground works. We would propose a £2,000 provisional sum for any additional unforeseen groundworks required. | £2,000.00 |
| 3.0 | Dissolved Air Flotation System (DAF Plant) |  |
| 3.01 | Allow to design, manufacture, installation and commissioning of a suitable Dissolved air Filtration System (DAF). The works comprise the following: - New skid mounted DAF Package - Rated to treat 20,000 litres of wastewater per hour. - System to include chemical dosing - Onboard automatic polymer make-up system - To include pH control and relevant dosing pumps - Unit to be pre-piped, wired and tested. In addition the system will need to include - Midi Low-level rotary screen complete with base tank for pre-screening any gross solids down to 2mm as well as a Level Controller for balance tank. - Coagulent dosing pump - Sludge transfer pump - Integral air compressor - Control panel with Siemens PLC and touch screen - Lockable GRP controller housing enclosure. |  |
| 4.0 | Electrical and Water connections |  |
| 4.01 | An existing three phase supply is already installed at Pellows waste. Please make allowance to make your connection to this three phase supply and test all works on completion. Please provide the necessary test certificates as required. The complete installation shall be undertaken by an NICEIC approved and registered contractor. |  |
| 4.02 | A clean water supply is already located in close proximity to the new DAF system. Please make allowance to make your connection to this existing supply and test all works on completion. Please provide the necessary test certificates as required. The complete installation shall be undertaken in accordance with The Water Supply (water Fittings) Regulations 1999. |  |