Intended for Defence Infrastructure Organisation (DIO) Document type Pre-Construction Health & Safety Information Date 04/04/2023

Intended for Defence Infrastructure Organisation (DIO) Document type Pre-Construction Information Date September 2023

Pre-Construction H&S Information Pack PROJECT APOLLO WORKSTREAM C - Episkopi

Pre-Construction H&S Information Pack PROJECT APOLLO WORKSTREAM C - Episkopi

Pre-Construction H&S Information Pack PROJECT APOLLO WORKSTREAM C - Episkopi

This report is produced by Ramboll at the request of the client for the purposes detailed herein. This report and accompanying documents are intended solely for the use and benefit of the client for this purpose only and may not be used by or disclosed to, in whole or in part, any other person without the express written consent of Ramboll. Ramboll neither owes nor accepts any duty to any third party and shall not be liable for any loss, damage or expense of whatsoever nature which is caused by their reliance on the information contained in this report.

| Project name | Project Apollo Workstream C |
| --- | --- |
| Project no. | 1620013960 |
| Recipient | DIO |
| Document type | Pre-Construction Information |
| Version | P03 |
| Date | 25/09/2023 |
| Prepared by | Richard Ducker |
| Checked by | Jonathan Giles |
| Approved by | Jonathan Giles |
| Description | Pre-Construction Information |
| Report Ref | PCI_Project Apollo Workstream C - EPISKOPI |

Version Control Log

| Revision | Date | Made | Checked | Approved | Description |
| --- | --- | --- | --- | --- | --- |
| P01 | 23/02/2023 | RD | JG | JG | First build issue from initial discussions. PCI to cover the initial Ground Investigation works scope only. |
| P02 | 04/04/2023 | RD | JG | JG | Second issue. PCI amended, to cover main works |
| P03 | 25/09/2023 | RD | JG | JG | PCI updated following receipt of asbestos survey information and the ground investigation report. |

Contents

1.	Pre-Construction Information	3

2.	Project Information	4

2.1	Proposed Works	4

2.2	Duty Holders	7

3.	Client Considerations and Requirements	11

3.1	Structure and Organisation	11

3.2	Client’s Safety Goals and Rules	11

3.3	Liaison between Parties	11

3.4	Monitoring and Review	11

3.5	Sub-Contractor Selection	11

3.6	Permits and Authorisations Requirements	12

3.7	Hazardous Energy Isolation	12

3.8	Fire and Emergency	12

3.9	Activities on or Adjacent to the Site during the Works.	13

3.10	Location and Access/Egress	13

3.11	Security Arrangements	13

3.12	Welfare Facilities / Provision	14

4.	Existing On-Site Hazards	15

4.1	Health, Safety and Environmental Hazards	15

5.	Significant Design and Construction Hazards	24

5.1	Principals of the Design	24

5.2	Risk Assessment	24

5.3	Design Development	24

6.	Construction Phase Plan	25

6.1	Requirements	25

7.	The Health and Safety File	26

7.1	Requirements	26

Appendix A

Existing Information

# Pre-Construction Information

The 2015 Construction (Design and Management) Regulations (CDM2015) require that construction clients provide pre-construction information as soon as is practicable to every Designer and Contractor appointed, or being considered for appointment, to the project. Where there is more than one Contractor, the appointed Principal Designer should provide advice and help compile the pre-construction information and provide it to the Designers and Contractors.

CDM2015 defines pre-construction information as 'information in the client’s possession or which is reasonably obtainable by or on behalf of the client, which is relevant to the construction work and is of an appropriate level of detail and proportionate to the risks involved, including information about:

- The project.

- Planning and management of the project.

- Health and safety hazards, including design and construction hazards and how they will be addressed.

- Information in any existing health and safety file'.

This Pre-Construction Information covers the project up to the end of RIBA Stage 2 (Concept Design).

A further update of this document will be required as the design progresses through RIBA stages 3 & 4, prior to the construction phase.

The information detailed herein relates specifically to the Apollo Workstream C project and may be expanded upon as the design progresses and additional relevant information is assembled.

# Project Information

## Proposed Works

The Workstream C project consists of the replacement of the Service Family Accommodation (SFA) housing across three Defence Infrastructure Organisation (DIO) bases; Episkopi, Akrotiri, Dhekelia.

The project is part of the UKStratCom’s APOLLO Programme which addresses long standing concerns over the MOD’s Cyprus estate. Workstream C, is part of a series of projects to address the need for seismically compliant SFA for service personnel and UK civil servants and their families based within the Sovereign Base Areas (SBA) in Cyprus.

Across the aforementioned three bases there will be five areas undergoing construction activities. This report focuses upon the two Episkopi sites; East and West.

2.2 Site Description

Episkopi Cantonment is part of the Western Sovereign Base Area (WSBA) enshrined in the Treaty of Establishment, between the UK and the Republic of Cyprus in 1960. Episkopi Cantonment is the capital of Akrotiri and Dhekelia, administered as a military base.

It is located in the north-western part of the Western Sovereign Base Area (Akrotiri). Although it is not the largest of the British military bases on the island, it is home to both the civilian and military administration headquarters of the Sovereign Base Areas. Episkopi is the current command centre of British Forces Cyprus.

Figure 1: Western Sovereign Base Area (WSBA) showing the location of Episkopi

The sites proposed for development are:

Episkopi East

The proposed development area is located at the western part of Episkopi Cantonment, north of the main east-west road. The area is mainly developed (residential), including the St. John School at the western side and some undeveloped spaces south of the school and to the north of the proposed development area. The residential areas is made up of houses with yards / gardens, some of them with swimming pools.

It is situated around 700 metres south of the monastery of St. George Symboulas, built on the remains of the early Byzantine complex of the 7th century AD.

Episkopi West

Episkopi West is to the west of the “Happy Valley”, south of the main east-west road. It is part of the “South Paramali” residential compound. It is predominantly developed (existing housing) but it includes big areas of undeveloped land, mainly to the south of the proposed development area. It is made up of houses with big yards / gardens; again some of them with swimming pools. It is situated on a hill to the west of the Symboulas stream valley.

West

East

Figure 1: Satellite view the areas proposed for development at Episkopi.

Figure 3:  Episkopi East - Area proposed for development

Figure 4: Episkopi West- Area proposed for development

## Duty Holders

| Duty Holder | Details |
| --- | --- |
| Client | Dominic Hirst Project Manager Defence Infrastructure Organisation (DIO) RAF Akrotiri BFPO57 00357 99617295 Dominic.Hirst107@mod.gov.uk Carly Hayes Senior Commercial Manager – Overseas Projects Defence Infrastructure Organisation Room 1.2.02 – 1.2.21 ¦ Kentigern House 65 Brown Street, Glasgow, G2 8EX 07590 065408 Carly.Hayes112@mod.gov.uk |
| Designer(s) | Ashley Park Structural Design Lead Ramboll 2nd Floor, The Exchange, St John Street Chester CH1 1DA United Kingdom +44 7870 806908 ashley.park@ramboll.co.uk Paul Walker Mechanical Design (MEP) Ramboll 2nd Floor, The Exchange, St John Street Chester CH1 1DA United Kingdom +44 7790 343101 paul.walker@ramboll.co.uk Stewart Lund Electrical Design (MEP) Ramboll 2nd Floor, The Exchange, St John Street Chester CH1 1DA United Kingdom stewart.lunt@ramboll.co.uk +44 7976 620479 Rhiannon Pierce Landscape Design Ramboll Cornerblock, Two Cornwall Street Birmingham B3 3DX United Kingdom rhiannon.pierce@ramboll.co.uk +44 7929 864057 Robert Ankers Drainage & Highways Director Betts Associates Old Marsh Farm Barns Welsh Road, Sealand Flintshire CH5 2LY 01244 288178 robankers@betts-associates.co.uk Carl Brookes Seismic / Blast Design Lead Ramboll 240 Blackfriars Road London SE1 8NW United Kingdom carl.brookes@ramboll.co.uk Carl Bailey Environmental - Principal Consultant Ramboll 2nd Floor, The Exchange, St John Street Chester CH1 1DA United Kingdom 07870 809693 carl.bailey@ramboll.co.uk Simon Miller Geotechnical Director Ramboll 2nd Floor, The Exchange, St John Street Chester CH1 1DA United Kingdom 07918 695072 simon.miller@ramboll.co.uk Rob Skeet Architect HLM Architects 2, Ruskin Buildings Tudor Square Sheffield City Centre Sheffield S1 2LA 0114 263 9600 rob.skeet@hlmarchitects.com |
| Principal Contractor | To be confirmed. |
| Principal Designer | Richard Ducker Ramboll 1 Broad Gate The Headrow Leeds LS1 8EQ M +44 7977173570 richard.ducker@ramboll.com |

# Client Considerations and Requirements

## Structure and Organisation

The Principal Contractor is to produce within the Construction Phase Plan an organisational chart illustrating the project health and safety management structure and named Project Health and Safety Manager, First Aider and Fire Wardens.

All construction activities will be co-ordinated with the site team and as such the organogram must confirm links to Client site management including health and safety team.

## Client’s Safety Goals and Rules

The aim of the project is to comply with all site-specific rules, applicable legislation and to apply good practice principles, providing a safe and healthy working environment.  This approach will in turn minimise hazards and potential accidents to project personnel and others involved directly or indirectly with the work and minimise the likelihood of harmful environmental incidents.

The Principal Contractor’s Safety, Health and Environmental (SHE) objectives are to be displayed in the site office.

The Construction Phase Plan must outline the co-ordinated approach between the client and the site management team, in relation to; site rules, the RAMS approval process, permit procedures, incident and accident reporting etc.

## Liaison between Parties

The Principal Contractor is to detail how they will ensure co-operation between Client stakeholders, staff and sub-contractors on site for health and safety purposes, e.g.:

Pre-Start Meetings

Site induction

Weekly Site Meetings

Formal and Informal Inspections

Toolbox Talks

The Principal Contractor must detail arrangements for management meetings and initiatives by which the Safety, Health and Environmental (SHE) objectives of the project are to be achieved e.g. provide a meeting schedule, defining the type, purpose and frequency of each meeting.

## Monitoring and Review

The Principal Contractor’s Construction Phase Plan (CPP) will detail how site health and safety arrangements will be monitored and reviewed during works.

The Principal Contractor must document in the CPP their arrangements for following up hazardous incidents and observations and the implementation of corrective actions and improvement opportunities.

## Sub-Contractor Selection

The Principal Contractor must ensure that all appointed sub-contractors are competent and have the knowledge and resources to comply with their duties imposed under the CDM 2015 regulations.

## Permits and Authorisations Requirements

In addition to the development of task specific risk assessment and method statements, works deemed higher risk will require specific controls via the implementation of a permit to work procedure.

High risk works include; hot works, lone working, confined space entry, live electrical works, working at height, excavating, working on the fire sprinkler system etc. This list of activities is not exhaustive and additional permits may be required for the planned construction project.

During construction works, the Principal Contractor will be responsible for implementing permit procedures, in accordance with their internal policies and procedures.

The Principal Contractor must document the permit procedure within their CPP and clarify if they will be implementing their own permit procedures and / or working in conjunction with the client, through a co-ordinated approach i.e. will client permits still apply, even within the Principal Contractors construction areas. This is critical, particularly where there are enabling works, temporary works, work on live services, and site boundaries are regularly altering.

The permit procedure must detail the client’s onsite contact (and the third-party facilities management team / Mitie contact), to be kept informed of all works which are subject to a permit.

The contractor must not commence works until they are in receipt of the ‘Statement of Known Hazards’, reference: MOD Form 5050, which should be written in to the site procedure, to support the permit system. The blank template is attached for reference in Appendix A.

The form states;

- ‘The Authority Local Representative (ALR)/Site Establishment Representative (SER)/Host (Host) shall ensure that all parts of the form are completed prior to work being allowed to proceed. Once complete the original form shall be passed to the contractor undertaking the work with a copy to the Contractor’s Site Manager (CSM), Regional Prime Contractor (RPC) or other Contractor as appropriate, and a copy retained by the ALR/SER/Host.’

- ‘This Statement of Known Hazards Form gives information to the Contractor carrying out works; it is not intended as a ‘permit to work’.  Any excavations undertaken on the MoD Establishment are to be in accordance with HSG (47) Avoiding Danger From Underground Services.’

- ‘This form supports the transfer of information between persons planning to undertake excavation works on MoD Establishments.’

## Hazardous Energy Isolation

Should it become applicable (although not foreseen) the Principal Contractor must ensure that suitable arrangement are in place and documented in the CPP to ensure the isolation of hazardous energies, where work involving such energies is required.

Any procedure must detail for the client’s onsite contact to be kept informed and a co-ordinated approach to Lock Out Tag Out procedures (LOTO) requirements.

## Fire and Emergency

The Contractor will be working within areas occupied by the Client and onsite civilians and must ensure that emergency and fire procedures are incorporated in to the Construction Phase Plan.

In the absence of existing onsite / base arrangements, the Contractor is to devise an adequately detailed emergency plan. The following elements should always be included.

- Training and instruction of all operatives;

- Induction of site staff and any visitors;

- Location of the Assembly Point;

- Instruction on what to do in an emergency;

- Identification of firefighting equipment

- Special arrangements for evacuation

The Principal Contractor is to liaise with the Client to develop a Fire Action Plan that considers all works within the CDM 2015 site boundary(s) and the proposed construction activities, inclusive of any temporary welfare accommodation and the consideration of the client activities on site.

The Contractor is to include within the Construction Phase Plan a Site Fire Action Plan that clearly identifies fire preventative measures and incorporates the provision of appropriate firefighting equipment, maintenance of existing escape routes, signage etc.

## Activities on or Adjacent to the Site during the Works.

The Principal Contractor must liaise with the client and their third-party Facilities Management team, Mitie, to identify and coordinate any activities occurring (or proposed) that could affect their site operations. At present, no information has been made available by the client in this regard and hence, the Principal Contractor must undertake a dynamic risk assessment when establishing their site and during site works.

The Principal Contractor must ensure adequate physical separation between construction and neighbouring areas, giving consideration to; security, making work areas safe and secure (particularly if / when left unattended), limiting the spread of construction dusts and dirt, contamination of residential areas, noise levels, construction vehicle movements etc.

The Principal Contractor must consider whether construction activities could be affected by any other work being carried out by other parties in adjacent areas.

The works are to take into consideration other users of the area to include pedestrian movement and children at all times.

## Location and Access/Egress

The site access / egress routes are currently unknown. Close liaison is required with the client prior to works to confirm the access point(s) and the required protocol.

The Principal Contractor must consult with the client to agree construction traffic volumes (materials delivery, lay down areas, waste storage and removal, staff vehicles etc) and arrangements for access, parking, deliveries and exiting the site, causing minimal impact on current traffic and pedestrian management arrangements.

## Security Arrangements

The Principal Contractor is to ensure that unauthorised access to the construction area(s) is prevented for the duration of the works. The Principal Contractor will be required to propose, implement and manage on-site security throughout the duration of construction works. Please refer to HSG 151 Protecting the Public — Your Next Move available from HSE Books.

There is no envisaged requirement for the works contractor’s staff to have higher security clearance than the local baseline standard. The works contractor will be required to comply with the various security procedures required by the Episkopi base and this needs to be reflected in the project planning.

As at any other MOD location, the security threat at Episkopi is constantly reviewed. This may lead to periods of increased security activities and requirements which the contractor will need to comply with.

All Client staff and contractors entering the defined CDM 2015 area(s) must be signed in, appropriately inducted, and escorted where necessary by the Principal Contractor.

## Welfare Facilities / Provision

The Contractor is to ensure that adequate welfare facilities are provided and maintained on site throughout the construction phase in accordance with Construction (Design and Management) Regulations 2015, Regulations 4(2)(b), 13(4)(c), and 15(11).

It is not known at this stage if the contractor will be able to utilise any of the existing onsite provisions.

The welfare requirements must be stipulated in the construction phase plan (based on the scope and contractor numbers) and the suitability of the existing facilities must be assessed.

# Existing On-Site Hazards

## Health, Safety and Environmental Hazards

The following known existing hazard information must be considered by the client, project designers and contractors, to enable them to carry out their duties under CDM2015 and minimise the risk of harm.

Table 4.1 Hazard Information

| Hazard Topic | Description and Existing Information |
| --- | --- |
| Site Utilities / Services | The Principal Contractor will need to consider the presence of existing services and utilities, some of which may need to be isolated, adapted or otherwise negotiated. A Utility Survey is a below surface survey, identifying all underground utilities and service routes including; gas, water, telecommunications, electricity and drainage, as well as voids within a specified area. Accurate information on the services located underground is critical in the development of the site, to ensure that the project complies with health and safety regulations, reduces project planning time and minimise the risk of service strikes. The underground utilities are normally shown on top of a topographical survey plan, to show the above ground features such as buildings and boundaries, in order to provide accurate survey drawings. Topographic survey plans are attached for additional reference, within Appendix A. PAS 128 is the British Standards Institution (BSI) standard that defines underground utility detection. It was first published in 2014 and sets out the information and accuracy to be expected from utility surveys. The PAS 128 standard takes a hierarchical approach to survey types. - PAS 128 Level A – Underground utilities are observed, excavated or exposed. - PAS 128 Level B – Underground utilities are detected by geophysical techniques. For example, Electromagnetic Location (EML) survey and/or Ground Penetrating Radar (GPR). - PAS 128 Level C – Utility records are validated by visual inspection. - PAS 128 Level D – Desktop utility record search. When the contractor(s) start to break ground, there must be either a PAS 128 Level A or B survey appended to this document (to enable the Principal Contractor to comply with HSE Safety Guidance). Before undertaking a Level A or B survey, an up-to-date set of utility searches (PAS 128 Level D survey) are required as a baseline. Note: Mitie are the onsite facilities manager and currently only release the Service location plans to the contractors once they are onsite. The plans are hard copies which need to be returned once the works are completed at each site (as part of a security measure). When breaking ground, a permit to dig must be in place, outlining the need to; - review the provided service drawings and supporting information, - carry out a visual inspection of the works area to identify clues where services may be present such as surface boxes, marker posts, reinstatement, streetlights etc. - conduct a Cable Avoidance Tool (CAT) scan of the area. The scanning process must be continuous as the excavation progresses, ensuring to use the correct setting. - Suitable hazardous energy isolation / Lock out and tag out procedures should be employed (as / where appropriate). Irrespective of the information provided by a utility survey and statutory plans, excavation work should be undertaken with extreme caution and in accordance with HSE Guidelines – HSG47 Avoiding Danger from Underground Services. Where areas could not be surveyed within the survey extents due to overgrowth, restricted access or due to waste & site materials/storage we recommend a re-survey is carried out as soon as areas are cleared and levels reduce. Where a utility has been labelled as unknown or a utility has been taken from plans or shown as an Assumed Route, then Vacuum Excavation or similar is recommended here for verification of Utility type, position and depth of cover. |
| Overhead lines and services | The contractor must take account of all above ground live services i.e. overhead electrical lines and telephone lines. No information has been made available by the client in this regard and hence, the Principal Contractor must undertake a dynamic risk assessment when establishing their site and during ongoing site works. Hardcopy service plans will be made available on site (as aforementioned). All vehicle routes and site plant equipment movements must be planned and co-ordinated accordingly to prevent direct collisions but critically to maintain safe distances i.e. avoid electrical arching and subsequent electrocution. |
| Electricity and other hazardous energies | Safe systems of work must be in place and adhered to for all works with electricity. Please refer to section 3.6 of this document, above. The Principal Contractor must comply with the client’s LOTO procedures, ensuring that appropriate physical (Lockout/Tagout) isolations are made to equipment connected to energy sources, ensuring a ‘zero energy state’ prior to the work taking place. All isolations must be adequately communicated to persons involved in the work, including client employees and contractors. All works to be undertaken in accordance with the Electricity at Work Regulations 1989, using only competent electrical engineering teams. |
| High Voltage Equipment | The project involves work being undertaken within the live electrical substation and onsite plant rooms. Client feedback required: Are there any other inherent hazards to be aware of, such as live machinery and chemical hazards? All such work must be suitably planned with the implementation of permits to work, emergency procedures and hazardous energy management as referred to under Section 3.6 and 3.7. |
| Archaeological discovery | Cyprus has legislation and regulation for the protection of the historic environment, including archaeological remains under the Antiquities Law. Please make reference to the Archaeological Assessment Report (Archaeological Report Episkopi_workstream C_Sept 2022_DN), as referenced within Appendix A. Potential effects on buried archaeological deposits, potentially sited within the proposed development areas Akrotiri East and West have been identified. Mitigation principles are outlined in the survey report, which provide adequate and proportionate measures to address the proposed development’s effects to heritage assets. The Principal Contractor must account for suspected discovery within their Construction Phase plan and incorporate within the site induction process. Report any archaeological finds during construction to the Cypriot Department of Antiquities. |
| Ground Contamination | Please see attached the historic Land Quality Report(s), as per appendix E. As referenced in the Desktop Environmental Assessment (appendix G); - Episkopi East (Z9M9389Y21-RAM-EP-XX-AS-Z-00001) – ‘Potentially contaminative activities have been identified as having taken place on site. It is understood that the residential properties historically had individual fuel tanks for heating. - Episkopi West (Z9M9389Y21-RAM-EP-XX-AS-Z-00002) - The site walkover identified limited potential for on-site sources of contamination, as the site comprised residential properties. However, there is potential to encounter made ground in localised areas of the site. A Phase 1 Land Quality Assessment (LQA)2 for the Episkopi Cantonment was prepared by Entec in 2002 and has been reviewed as part of this assessment, as well as a Phase 2 LQA3 that was undertaken in 2004 following the completion of a ground investigation. The Phase 1 LQA identifies a number of potential contamination sources at the site.’ As recommended in the two aforementioned document(s), an intrusive ground investigation (see appendix F) has been undertaken in 2023. As stated, ‘requirements of both the contamination risk assessment and the geotechnical requirements were combined in a single ground investigation’): Preparation of a Land Quality Assessment (LQA) is now required following completion of the ground investigation and receipt of the chemical test results, so that the Conceptual Site Model (CSM) and risk assessment can be updated, and to allow further recommendations for remediation and / or mitigation measures to be made.’ The LQA remains outstanding. |
| Unforeseen Ground Contamination (including Asbestos in Soils) | It is recommended that a protocol is developed by the principal contractor for the management of suspected contaminated materials and included within the construction phase plan. Suitable controls must be in place to detail the arrangements for the discovery of asbestos in soils (and any instances of fly-tipping / visible debris). If there are ground conditions encountered during the works which the Contractor considers more adverse than could have reasonably been foreseen, and these require a change of method of working or design, works must cease and the client must be notified immediately and supplied with proposals for overcoming the adverse ground conditions. |
| Geological instability and Groundwater | A ground investigation has been undertaken in 2023, (see appendix F), to ‘provide specific and detailed information about the geotechnical properties of the soil in the investigated area, such as the site geology, the bearing capacity and laboratory tests for the purpose to obtain any sufficient information for feasibility and economic studies for any project.’ The Land Quality Assessment (and the interpretation of the Ground Investigation results), remains outstanding. |
| Open Excavations | The law states that you must prevent danger to workers in or near excavations. To maintain the required precautions, a competent person must inspect excavation supports (temporary works) or benching / battering of the excavation, at the start of the working shift and at other specified times. No work should take place until the excavation is safe. Key hazards associated with an open excavation(s), (not exhaustive): - Collapse of the excavation - Falling or dislodging material - Persons falling in to the excavation - Undermining nearby structures - Underground and overhead services - Inflow of ground and surface water - Damage to trees Means of access and egress |
| Ground gases and Radon | As referenced in the Desktop Environmental Assessment (appendix G); - Episkopi East (Z9M9389Y21-RAM-EP-XX-AS-Z-00001) – ‘No areas of landfilling have been identified at the site or within 250m. Whilst ground gas can be present in carbonate soils such as chalk or limestone, in particular carbon dioxide, gas production is generally low. Radon levels for the area are understood to be lower than the 200 becquerels/m3 level recommended by Public Health England where radon levels should be reduced in homes. Ground gas and vapours are more likely to be present where USTs, ASTs and POLs are or have been present, such as in the north eastern part of the site around the former RIB MT facility.’ - Episkopi West (Z9M9389Y21-RAM-EP-XX-AS-Z-00002) - ’No areas of landfilling have been identified at the site or within 250m. Whilst ground gas can be present in carbonate soils such as chalk or limestone, in particular carbon dioxide, gas production is generally low. Radon levels for the area are understood to be lower than the 200 becquerels/m3 level recommended by Public Health England where radon levels should be reduced in homes.’ Potential for ground gas generation from organic materials and / or possible hydrocarbon contamination in site soils. Potential for hydrocarbon vapours arising from contaminated soil/groundwater. |
| Work at Height | The project involves demolition / removal and adaptation of existing buildings, plant equipment and services. Coupled with the construction of new structures. Much of this work will involve work at height and / or areas where there is a risk of falls. Examples including but not limited to: - Working from mobile elevating work platforms; - Scaffolding and tower scaffold usage; - Fragile surfaces (including roofs); - Roof voids; - Vertical ladders and platforms; - Work upon open edges The following are all requirements in law that you need to consider when planning and undertaking work at height. You must: - take account of weather conditions that could compromise worker safety; - check that the place (eg a roof) where work at height is to be undertaken is safe. Each place where people will work at height needs to be checked every time, before use; - stop materials or objects from falling or, if it is not reasonably practicable to prevent objects falling, take suitable and sufficient measures to make sure no one can be injured, e.g. use exclusion zones to keep people away or mesh on scaffold to stop materials such as bricks falling off; - store materials and objects safely so they won’t cause injury if they are disturbed or collapse; - plan for emergencies and rescue, e.g. agree a set procedure for evacuation. Think about foreseeable situations and make sure employees know the emergency procedures. Don’t just rely entirely on the emergency services for rescue in your plan Avoid work at height where it's reasonably practicable to do so. Where work at height cannot be easily avoided, prevent falls using either an existing place of work that is already safe or the right type of equipment. Minimise the distance and consequences of a fall, by using the right type of equipment where the risk cannot be eliminated. Refer the Work at Height Regulations 2005 and associated HSE guidance. |
| Crane Usage | Potential crane usage will require an application through the client’s onsite Project Manager. It is understood that restriction of crane usage is applicable to RAF Akrotiri due to the proximity to the military airways and potential interference with the Air Traffic Control Tower. The location of the crane also must be identified in the Construction Phase Plan and the size of the any crane (temporary or permanent) must be shared with the client, to assess the impact prior to providing a formal acceptance for a crane use. The Contractor must get formal acceptance after engagement with the air authorities (regarding maximum height for the crane use, should it be an option), ensuring that it doesn’t interfere with any aircrafts. |
| Temporary Works | The law says any temporary structure must be designed, installed and maintained to withstand any foreseeable loads which may be imposed on it and that it be only used for the purposes for which it was designed, installed and maintained. Temporary works such as scaffolding and excavations must be inspected by a competent person on a regular basis. Temporary works to provide structural support include: - Shoring for excavations - Propping for walls and floors - Edge protection - Formwork / Falsework for above ground concrete pours - Scaffolds for access - Haul roads and working platforms for plant including cranes - Permanent works used for temporary purposes during construction Temporary works on a construction site also include: - Site fencing, signage and hoarding - Welfare and office units - Earthworks, including stockpiles that will be removed later. NB. Any proposed stockpile locations need to be confirmed with the client. There are no specific legal requirements about how temporary works should be managed; each organisation can choose how they arrange and manage safety on site. An effective way of managing temporary works is to appoint a competent Temporary Works Coordinator (TWC) to ensure suitable designs are prepared, checked and correctly erected on site. The TWC should be formally appointed and have adequate authority to carry out the role, including stopping the work if it is not satisfactory. The TWC may also need to advise on build sequence, systems of work and equipment so that other activity does not damage or compromise the temporary works. On a complex project this person is often a chartered civil or structural engineer with relevant experience and training and based on site. On a simpler job the main contractor may take on this role and seek advice as needed from an engineer. Equipment manufacturers’ instructions must be followed in the selection and use of temporary support. In all bar the simplest situations a design, including calculations, will be needed for the temporary works. In more complex works the design will also need to be checked by an independent party. |
| Lifting and Suspended Loads | The construction project will involve significant lifting operations for construction materials and plant installation. The Principal Contractor must ensure a risk assessment and lifting plan is in place for all lifting activities, ensuring persons operating lifting equipment / crane are suitably competent to do so. |
| Hazardous Substances (DSEAR) | Client feedback required: Are there any dangerous substances stored on site? Liquid Petroleum Gas storage? Fuel lines? Any generator’s onsite? Dangerous Substances and Explosive Atmospheres Regulations 2002. Dangerous substances are any substances used or present at work that could, if not properly controlled, cause harm to people as a result of a fire or explosion (or corrosion of metal). They can be found in nearly all workplaces and include such things as solvents, paints, varnishes, flammable gases, such as liquid petroleum gas (LPG), dusts from machining and sanding operations, dusts from foodstuffs, pressurised gases and substances corrosive to metal. Hazardous areas are defined in DSEAR as "any place in which an explosive atmosphere may occur in quantities such as to require special precautions to protect the safety of workers". Any chemicals in current use on site: Chemical / concentration; Quantity at any one time on site; Location of chemical on site. |
| Substances hazardous to Health (COSHH) | Client feedback required: Is there a COSHH register available for the hazardous substances (substances hazardous to health) on site? Any tanks, chemical stores, silo’s, gas cylinders etc.? |
| Radon | Radon risk in Cyprus is evaluated overall as low risk. General radon levels in Cyprus are at very low levels and within acceptable limits set by The European Union. This is due to the lack of nearby nuclear plants and the EU control measures regarding imported goods. Furthermore, ground investigation works are all outdoors and not confined, so risk is very low / negligible. Source of information; (https://pubmed.ncbi.nlm.nih.gov/12782477/). This is a view to concentrations in groundwater which we do not anticipated encountering and quantified by indoor risk where gas may accumulate. |
| Unexploded Ordnance | The UXO risks are defined as low due to the fact that the sites are existing residential development areas with no previous history of UXO threat i.e. no previous storage or arms use. Therefore, the information within the Ramboll specifications requires no further survey or assessment. Despite the low risk, the contractor should document the UXO hazard in their risk assessment and document a procedure (a clear emergency response plan, with roles and responsibilities outlined) incase there was to be an unforeseen discovery of a UXO. Considered controls should include; do not touch it, extract yourself and others, warn others in the area, first aid provision, method for recording the location and contacting emergency services (giving as much information as possible about the location and nature of the potential UXO (without returning to the site of the UXO to do so)), nearest hospital location, details for contacting the health and safety team, switch off any machinery in current use (e.g. a piling rig) if safe to do so, evacuate the site and surrounding area to the agreed muster point etc. |
| Asbestos Containing Materials | Asbestos containing material (ACM) may be present within; existing buildings including their external fixtures and fittings, within external ducts / trenches, within onsite plant services and equipment, within the grounds of existing buildings, including potential fly-tipping etc. Asbestos management surveys have been provided by the client. Prior to works commencing, asbestos demolition surveys must be undertaken by a competent contractor, within all properties which are due to be demolished. Asbestos refurbishment surveys are required within any areas which are due to be remaining and where the fabric of the building will be disturbed during the proposed works. The Principal Contractor must review all refurbishment and demolition survey’s prior to making structural intrusions and / or disturbing the structure of the building (inclusive of disturbing any fixtures and services i.e. lagging removal), to ensure that the scope of the proposed works has been captured by the ‘scope specific survey’s’. Beware of caveats and exclusions from the reports. Added to that, the contractor’s risk assessment should account for the unexpected disturbance of asbestos containing materials, during works. Where any proven/presumed ACM could be impacted on by the proposed work, these must be abated prior to construction work progressing. Arrangements must be in place to ensure no proven/presumed residual ACM is disturbed during the course of the works, and that no ACM fibres are released. |
| Site Ecology | Ecology ‘Phase 1’ surveys of the sites and surrounding areas are to be undertaken, the output of which will be a Preliminary Ecological Appraisal (PEA). The survey will be undertaken during the appropriate season for botanical survey in Cyprus, which is approximately from December to April inclusive. The survey and PEA will help to inform design constraints subsequent HRA screening. Contractor is to provide a site ecologist to undertake the necessary supervisions works for the duration of the Ground Investigation survey. |
| Seismic Activity | The site is in an area of potential seismic activity. This hazard must be assessed within the Contractors risk assessment, to include emergency considerations in the event of any occurrence. |
| Vehicle and Pedestrian | Vehicle movements are not known in detail but construction traffic to and from the site will need to pass through residential areas across the site. Careful consideration will need to be given to construction traffic to minimise traffic congestion and disturbance on the surrounding roads. Segregation must be in place to protect vehicles, pedestrians and all activities around the vicinity of the construction activities. The Principal Contractor must ensure that those within the garrison are not at risk of vehicle impact, that pedestrian routes are clear, and that warning signage and barriers are employed as appropriate to prevent incident / accidents. A traffic management plan must be devised and included with the Construction Phase Plan. |
| Noise | The Cyprus Government website notes the requirement to comply with EU Directives on the assessment and management of environmental noise. The Principal Contractor must comply with all site requirements relating to noise, including the wearing of hearing protection in designated areas. Adequate hearing protection must be provided to persons exposed to high noise levels. |
| Work in hot environment & UV rays | The project will involve working within a hot climate which presents a risk of sunburn and heat stress. The Principal Contractor should ensure that suitable provisions are in place to manage the exposure to the sun and suitable controls are implemented to prevent any person involved in the project experiencing heat stress (regular rest breaks, welfare provision, emergency arrangements, access to drinking water, PPE etc.) |

# Significant Design and Construction Hazards

## Principals of the Design

Residual risks associated with the design will be recorded in a design risk register and appended to the document during RIBA stages 3 and 4.

## Risk Assessment

Any identified hazards that cannot be designed out must be assessed by the Contractor prior to commencement of work. Safe systems of work must be introduced and communicated to their site workforce.

## Design Development

The Principal Contractor is required to list all those in their team, including any sub-contractors, with any design input, so that the procedure for appraising all design in accordance with CDM2015 can be established.

The Principal Contractor is required to provide to the Client and Principal Designer at least 10 working days prior for implementation, a document setting out his method statement, hazard analysis and risk assessments for elements of the work designed by them.

# Construction Phase Plan

## Requirements

The Construction Phase Plan is a document that records how health and safety will be managed for the construction phase of a project.  It is the basis for communicating to all those involved in the construction phase of the project.

The Principal Contractor must ensure that a project specific Construction Phase Plan is developed in line with the requirements of the Client and CDM2015 which should be submitted to the Client and Principal Designer for review in advance of the planned project commencement date.

Specific to this project, the Construction Phase Plan must contain:

- Health and Safety arrangements for the construction phase;

- Description of the project and scope of works;

- Client considerations

- Site rules;

- Arrangements to ensure cooperation between project team members and coordination of their work;

- Arrangements for worker involvement;

- Site Induction;

- Details of site welfare facilities;

- Fire and emergency procedures

- Details on how significant site risks will be controlled

The Construction Phase Plan should not include documents that get in the way of a clear understanding of what is needed to manage the construction phase, such as generic risk assessments, records of how decisions were reached or detailed safety method statements.

The Principal Contractor must ensure that the Construction Phase Plan is reviewed, updated and revised when required. The plan is a working document and should be reviewed regularly enough to address any significant changes to the risks involved in the work.

# The Health and Safety File

## Requirements

The Health & Safety File should contain information needed to allow future construction work, including cleaning, maintenance, alterations, refurbishment and demolition to be carried out safely. Information in the file should alert those carrying out such work to risks and should help them to decide how to work safely.

Specific to this project, the Health & Safety File must contain:

Detailed scope of work;

Project directory that includes sub-contractor’s details;

Any residual Hazards remaining from the work carried out;

Detailed as-built drawings that clearly identify all areas when demolition, enabling and construction works have taken place, and including all mechanical and electrical installations;

Manufacturers operation and maintenance manuals;

Photographic record of works completed;

Asbestos abatement records and waste consignment notes (if applicable);

Records of all identified live statutory services passing through the site (if any); and

Details of any identified harmful materials or chemicals.

- It is the responsibility of the Principal Designer to collate information for the Health and Safety File. If the Principal Designer’s appointment ceases prior to completion of a project, the Principal Contractor must take on this responsibility and collect and deliver all information relevant to the Health and Safety File to the client.

- The Health & Safety File must be supplied to the client in an electronic format and no more than two weeks after completion of the construction phase.

Appendix A – Existing information

Please refer to the below appendices from the Site Information documents, for any existing information.

Appendix A – Topographical Survey

Appendix B – Archaeological Survey

Appendix C – Weather Data

Appendix D – Asbestos Report

Appendix E – Land Quality Report

Appendix F – Ground Investigation

Appendix G – Desktop Environmental Assessment

Appendix H – Legionella Management Plan

Appendix I – Statement of Known Hazards