- REFURBISHMENT OF ROOMS kw032b & kw036d

CITY COLLEGE PLYMOUTH

king’s road campus

ON BEHALF OF

CITY COLLEGE PLYMOUTH

DEVONPORT

PLYMOUTH

PL1 5QG

## SPW/spw/71609

MAY 2017

CONTENTS

| SECTION |  |  |  | PAGE NO. |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  | The Works | 8 |  |
|  | 1 – WORKS |  |  |  |  |  |
| 1.0 | new ADMIN Hub – ROOMS KW032B & KW036D ONLY |  |  |  |  |  |
| 1.1 | Existing CCTV |  |  |  |  |  |
| 1.2 | There is currently a CCTV installation to rooms KW032A, KW032B and KW032C. By the time works are required to commence on Wednesday 31 May 2017, City College will have had this installation relocated. Therefore, ignore that installation for the purposes of the works except for making good where equipment has been removed. That in 36B will be removed by 31st May. 32A will stay until the second phase |  |  |  |  |  |
| 2.0 | Overview |  |  |  |  |  |
| 2.1 | Existing rooms KW032C (to become new room designation KW032B) & KW036D to be linked via a new, lined aperture (ie a doorway with no door in it) allowing access between the two rooms (aperture to be no larger than that for a standard single door set utilising existing lintel over former hatch) and to form the new “L-shaped” Admin Hub accommodation. This is confusing leave the room numbers the same |  |  |  |  |  |
| 3.0 | New Room KW032B |  |  |  |  |  |
| 3.0 | Decoration |  |  |  |  |  |
| 3.1 | Decorate room KW032B as described below to a height of 2m FFL to all walls. Make good, fill and rub down, preparing all walls as required prior to decoration. Include decoration of all woodwork as described below. |  |  |  |  |  |
| 3.2 | All paint shall be Johnstone’s, Crown, Dulux or equivalent approved and shall be used in accordance with manufacturer’s most recent instructions. |  |  |  |  |  |
| 3.3 | Decorate walls using 1 No undercoat, 2 No topcoats vinyl matt, colour “Tuscan Treasure 6”, code 25YY83103 (Dulux coding - other equivalent brand of paint shall match this colour as closely as possible). |  |  |  |  |  |
| 3.4 | Woodwork: 1 No primer coat, 1 No undercoat, 2 No topcoats Paint Mixing Eggshell, colour “Stonewashed Blue”, code 70BG28169 (Dulux coding – other equivalent brand of paint shall match this colour as closely as possible). |  |  |  |  |  |
| 4.0 | Lighting |  |  |  |  |  |
|  | NB - see Appendix for Fitzgerald specification – equivalent approved specification will be considered. |  |  |  |  |  |
| 4.1 | Carefully take down existing lighting and dispose of all arising from site (check with CA prior to disposal). Amend the height of the baffle boards suspended from the ceiling where required so as not to obstruct the plane of the light from the new fittings. Replace light fittings with 8 No Fitzgerald Spectrum 1595 x 208 x 77mm (L x W x H) mounted on 3m suspension kit as supplied by the manufacturer to provide 300 lux at desk height to all work stations. For the purposes of pricing, assume units to be suspended 0.5m from the ceiling. 2No of the new fittings shall be emergency lights and 6 No shall be standard fittings. |  |  |  |  |  |
| 4.2 | Split the electrical lighting circuit as required to separate the lights to the individual room as required and replace the existing light switch with 1 No new white plastic faced triple switch light switch and mounting box screw-fixed into the wall behind and connected to the existing wiring in the room. All light switches and electrical outlets/components shall be MK or equivalent approved. Where additional wiring is required, this shall be provided in white surface mounted trunking and screw fixed into the wall as needed. For the purposes of pricing, assume 3 linear metres will be required. |  |  |  |  |  |
| 4.3 | Allow for all necessary adjustments to fire alarm sensors and emergency lighting where required. For the purposes of pricing, allow £500.00 for this work. |  |  |  |  |  |
| 5.0 | Ceiling |  |  |  |  |  |
| 5.1 | There are no anticipated works to the ceiling save for making good around new lighting installation fixing points etc. |  |  |  |  |  |
| 6.0 | Aperture Reopening Between Rooms (New) KW032B & KW036D |  |  |  |  |  |
| # 28.4.1 | It is proposed to re-use the existing lintel visible in room KW036D beneath the noticeboard to the north-west wall into room KW032B (a former hatchway). To this end and having removed the noticeboards to either side as part of the preparatory works to room KW036D, carefully break out the existing blockwork to the underside of the lintel. Ensure that sufficient end-bearing is left to either end of the existing lintel (a minimum of 150mm at each end). For the purposes of pricing assume the partition is single skin standard 100mm thick concrete block, and required aperture size to be 900 x 2000mm (WxH, but where the height in reality is the existing available height of the existing lintel). Make good as required to leave a neat opening ready to receive a new door lining and architraves. |  |  |  |  |  |
| 28.4.2 | Supply and fix a new MDF pre-formed and self-finished/ ready painted door lining (colour white) and architraves to both room KW032B and room KW036D (there will not be a door fitted into the new aperture). See below for treatment of new threshold. |  |  |  |  |  |
|  | Make good around door aperture to room KW036D prior to decoration as required. |  |  |  |  |  |
| 28.5 | Flooring |  |  |  |  |  |
| 28.5.1 | Carefully take up and dispose from site of the existing flooring to room KW032B only (ie do not take up the flooring in room KW036D as it is new). Make good any areas of the concrete floor requiring repairs using a proprietary concrete repair compound as previously described. For the purposes of pricing, assume 1m2 of repairs. Supply and lay proprietary self-levelling latex to the entire floor of the room prior to laying new carpet and installing new skirtings. |  |  |  |  |  |
| 28.5.2 | To the whole of room KW032B (including the new threshold between offices), supply and fix new carpet tiles - Forbo Tessura Teviot loop pile floor tiles or equivalent approved, colour “Nightsky” or nearest equivalent. Where possible, lay right up to the carpet of room KW036D and provide all necessary threshold coverings/ strip where required to leave no trip hazard (for the purposes of pricing assume colour black, check with CA prior to ordering). NB – the carpet to room KW036D is relatively new and is not to be replaced. Where an alternative product is proposed, a sample will be required to be delivered to the Plymouth office of Vickery Holman for client approval prior to ordering. |  |  |  |  |  |
| 28.6 | Skirtings |  |  |  |  |  |
| 28.6.1 | It is the intention to recycle the existing timber skirting boards to the room and to splice-in new timber skirting sections where needed to the existing prior to decorating all as previously described. Any new sections shall match the profile of the existing as closely as possible and all surfaces to be level and flush prior to decoration. For the purposes of pricing, allow to supply and fix as required 10 linear metres of new timber skirting, 100mm FFL mitred profile. All new timber shall be (FSC) softwood timber skirting boards. All skirtings shall be decorated as previously described. |  |  |  |  |  |
| 28.7 | Dado Perimeter Trunking |  |  |  |  |  |
| 28.7.1 | It is the intention to alter trunking and electrical installations as little as possible and on no account is the removal of existing electrical and CAT5e points to be permitted without express consultation with the CA and the client. Where existing installations can be economically amended to accommodate the number of workstations and the remaining installation can be left looking neat and presentable/ tidy, this shall be the methodology implemented. Where new trunking is required this shall be Marshall Tufflex Monoplus 20 trunking or equivalent approved, colour white, with Krone CAT5e data points or equivalent approved, colour white. All sockets shall be standard model as manufactured by MK, MEM, Schneider or equivalent approved. See Appendix “Structured Cabling Infrastructure Specification Category 5e and 6a” document for reference. Where an alternative manufacturer is proposed, a sample of the product proposed shall be deposited in the Plymouth office of Vickery Holman and the CA shall seek the approval of the client prior to ordering. There is power and data set into the walls between room KW032B and KW032 as well as the wall between KW032B and KW036D and the existing installations are required to effectively be brought forward from the surface of the wall and set into new white wall mounted trunking as described above and to augment the number of outlets to accommodate 10 No workstations. Supply and fix new perimeter dado trunking to the whole of the perimeter of the room and DSO and Cat5e outlets to provide 10 No workstations plus an additional DSO for cleaning appliances, ie11 No DSO and 10 No Cat5e data points are required in all. Connect all data points back to the appropriate switch point and leave all installations networked, commissioned, NICEIC certified and in full working order. |  |  |  |  |  |
| 28.8 | Replacement External Door |  |  |  |  |  |
| 28.8.1 | To increase the available daylight into room KW032B, it is proposed to replace the existing timber fire door and frame complete. The contractor shall remove the existing lock etc for recycling/ installation into the new door. The contractor shall check with the CA/ CCP prior to disposal whether this is to be handed back to the College. For the purposes of pricing, include to dispose of door and frame from site complete. |  |  |  |  |  |
| 28.8.2 | To existing opening, supply and fix 1 No new metal powder coated external quality fully toughened, double-glazed metal door with mid rail, metal frame and threshold suitable for a fire escape. Door shall be mounted on good quality stainless steel ball bearing type hinges and shall have integral hold-open and automatic closer facility. Ref-fit KABA lock complete to new door. Door shall also have metal thumb-turn emergency door release and shall have 1 No metal pull handle externally with push plate internally fixed 1050mm GL and 1000mm FFL. Ensure there are no fittings to the door which could catch when exiting the door. NB – key access from the outside is essential. For the purposes of pricing assume glass to the door is required to be frosted/ obscured. |  |  |  |  |  |
|  | Provisional Item |  |  |  |  |  |
| 28.8.3 | Where it is not possible to recycle the old door lock or this is not a KABA suited lock, supply and install 1 No new KABA 20 System, KABA 22mm cyclinder, round profile with thumb turn and 1 No new KABA lock case, profile cylinder mortice sash lock (76mm). Supply 5 No keys to new lock to be handed to the client at completion. |  |  |  |  |  |
|  | End of Provisional Item |  |  |  |  |  |
| 28.9 | Existing Double Doorset Between Existing Room KW032A & (New Room) KW032B |  |  |  |  |  |
| 28.9.1 | The double doors between existing rooms KW032A and KW032C are to remain in position but do not currently shut in a satisfactory fashion or perform as fire doors. Carefully remove the existing master and slave leaves complete and allow to re-hang both doors within the existing frame following completion of the refurbishment works. |  |  |  |  |  |
| 28.9.2 | In use, the slave leaf of the double door set shall be bolted most of the time to secure it in position. Ease and adjust the existing shoot bolt to the top of the existing slave leaf and on re-hanging the door, ensure that the bolt matches up to the existing retaining hole/ keep. Supply and fix 1 No additional shoot bolt (to match the existing as closely as possible) to the bottom of the slave leaf and to line up with the existing hole/ keep to the floor. For the purposes of pricing, assume new bolt shall be a necked satin chrome finished 150mm bolt and keep, screw-fixed as needed onto the base of the door (the kick plate will require drilling prior to fixing new bolt). |  |  |  |  |  |
| 28.9.3 | Remove from the existing door and frame the existing hinges and dispose of from site. To slave and master leaf, supply and fix 3 No new fire door hinges per door, with 2 No fixed to the top of the door and one to the bottom: top and bottom hinges shall be 200mm from the head and the bottom of the door respectively and the third hinge shall be set 200mm down from the top hinge. Hinges shall be a standard twin ball bearing 100 x 75 x 3mm (L x W x D) grade 201 stainless steel as available at Screwfix or equivalent approved. |  |  |  |  |  |
| 28.9.4 | Before re-hanging the existing fire doors, rout out both leaves as needed ready to accommodate new intumescent seals and cold smoke seals. NB, rubber blade type fire door seals will not be adequate for this application and new seals shall be brush type such as 10mm rebated fire door seals, colour brown. Seals shall be fitted to the head and rails of the door and the re-hung doors shall have a 2-4mm gap between the head and rails and the frame and between slave and master leaf and with 3mm gap between the floor and the foot of the doors. |  |  |  |  |  |
| 28.9.5 | Provisional Item Where it is not possible to achieve the 3mm gap between the foot of the re-hung doors and FFL, supply and fix a threshold seal suitable for fire doors across the whole of the threshold to the double doorset to limit the spread of smoke in the event of a fire (eg for the purposes of pricing, 2 No 930mm AFASDBX/930 Rebated Drop Down Smoke Seal as available at Safelincs or equivalent approved, sized to suit doors). End of Provisional Item |  |  |  |  |  |
| 28.9.6 | To both the slave and master leaves, supply and fix 1 No new heavy duty door closers – 2 No in total (eg Briton 2003, colour silver (widely available) or equivalent approved) and leave doors in full working order such that they fully close and the fire door seals engage to prevent the spread of smoke. |  |  |  |  |  |
| 28.9.7 | Ease and adjust the existing door furniture to the existing doors and leave all in full working order. Recycle the existing KABA cylinder in the door. Supply and fix 1 No new KABA lock case, profile cylinder mortice sash lock (76mm). Supply and fit new escutcheon of the non-covered variety where missing, SAA finish to match the other door furniture. |  |  |  |  |  |
| 29.0 | Room KW036D |  |  |  |  |  |
| 29.1.0 | Reopened Aperture to KW032B |  |  |  |  |  |
| 29.1.1 | Make good around door aperture to room KW036D prior to decoration as required. |  |  |  |  |  |
| 29.2.0 | New Suspended Ceiling |  |  |  |  |  |
| 29.2.1 | To the whole of room KW036D complete, supply and fix a new slimline, low void 600 x 600mm tiled suspended ceiling system with sit in tile, such as Ecophon Focus A or equivalent approved Due to the existing low ceiling height and the need to access the services attached to the existing ceiling once the new ceiling is installed, unless otherwise agreed with CA, the void height to the new ceiling shall be a maximum of 100mm and the new ceiling is required to be at a uniform level across the whole of room KW036D. It is believed that the artex to part of the ceiling in this room is not asbestos containing and this will be confirmed prior to commencement (via Vickery Holman). |  |  |  |  |  |
| 29.3.0 | Lighting |  |  |  |  |  |
| 29.3.1 | Remove the existing light fittings complete and extend electrical cabling as required to facilitate the installation of the new LED light fittings within the suspended ceiling (see below). Reconfigure the existing light switching (recycling the existing light switches), and reconfigure light switching to suit the new workstation configuration. |  |  |  |  |  |
| 29.3.1 | In accordance with the specification by Fitzgerald (see Appendix) or equivalent approved, supply and fix 14 No Celestial 600 x 600mm 35w recessed LED panel light fittings to provide a minimum of 300 lux at desk level. 9 No fittings shall be standard and 5 No shall be emergency light fittings. |  |  |  |  |  |
| 29.4 | Fire Detection and Emergency Lights |  |  |  |  |  |
| 29.4.1 | The new ceiling will require the relocation of the existing fire detection devices and ceiling mounted emergency lighting to comply with current legislation and guidance. |  |  |  |  |  |
| 29.5 | Air Cooling Unit |  |  |  |  |  |
| 29.6 | External Window |  |  |  |  |  |
|  | Provisional Items |  |  |  |  |  |
| 29.6.1 | Carefully unbolt and remove/dispose from site of the existing metal bars to the window. Make good all surfaces disturbed and holes using mortar of a colour to match the brickwork as closely as possible. |  |  |  |  |  |
| 29.6.2 | Remove and dispose from site of the original window and frame complete. Windows looks okay |  |  |  |  |  |
| 29.6.3 | To the existing aperture, supply and fix 1 No new metal powder coated casement window with double glazed unit configured as existing and complete with new lockable window handle (3 No keys for which to be supplied to the client at completion) and integral trickle vent. Window shall also be fitted with integral restrictor device to allow 100mm opening unless released by user. |  |  |  |  |  |
|  | End of Provisional Items |  |  |  |  |  |
| 29.6.4 | Ease and adjust the existing window and leave all in full working order. |  |  |  |  |  |
| 29.7 | External Half Glazed Door |  |  |  |  |  |
| 29.7.1 | The external door is to provide access via a key to users of the new Admin Hub. This will preferably interface with the existing push bar configuration to the existing door. |  |  |  |  |  |
| 29.7.2 | Supply and install to interface with the existing push bar mechanism 1 No new Briton 1413.LElSE lockable outside access device (or equivalent approved) as available at Ironmongery Direct. |  |  |  |  |  |
|  | Provisional Items |  |  |  |  |  |
| 29.7.3 | Remove existing push bar emergency exit equipment complete from existing door and make good all surfaces disturbed. |  |  |  |  |  |
| 29.7.4 | Supply and fix 1 no new Briton 1438E single door push pad latch or equivalent approved as available at Ironmongery Direct. Ensure that installed device interfaces with new external access handle and that all is in full working order. |  |  |  |  |  |
|  | End of Provisional Items |  |  |  |  |  |
| 29.7.5 | Indicate the extra/over cost to supply and fix 1 No new fully double glazed metal powder coated door and frame to match new door to room KW032B in design (door to have mid rail) complete with integral hold open and closing device and with fire exit mechanism internally with keyed external access. For the purposes of pricing, assume glazing shall be frosted. Check with CA prior to ordering. |  |  |  |  |  |
| 29.7.6 | New lock to new door is required to be KABA suited lock as described under room KW032B. Supply and fix 5 No keys for new lock to be handed to the client at completion. |  |  |  |  |  |
|  | Decoration |  |  |  |  |  |
| 29.7.7 | Having first liaised with the CCP ICT department as to what can and cannot be temporarily disconnected, carefully ease the surface mounted trunking to the ceiling soffit away from the wall to allow the following works to take place and ensure all is sufficiently supported to prevent damage to cabling etc. Reinstate existing services following completion. |  |  |  |  |  |
| 29.7.8 | There is an extensive area of green wallpaper to one end of room KW036D which appears to be mounted on plasterboard. Having first isolated the electrics and services as needed, carefully remove all such boards and all such wallpaper and dispose of from site. Make good all surfaces disturbed. It is believed that there remains a former hatch and possibly glazing behind the wall boarding to the north-east wall of the room (ie it was just boarded over). Assuming a void of 3m x 2m, construct new softwood or metal studwork 100mm walling, cavity to be packed with sound attenuating quilt such as Knauff Earthwool Acoustic Roll (widely available) or equivalent approved. All joints to plasterboard shall be scrim taped and skimmed with finishing plaster prior to decoration. |  |  |  |  |  |
| 29.7.9 | Decorate the whole of room KW036D throughout as previously described under room KW032B. |  |  |  |  |  |
| 29.8 | Carpet |  |  |  |  |  |
| 29.8.1 | No works to the carpet itself. Provide all necessary protection prior to commencement of the works in general. |  |  |  | - |  |
| 29.9 | Skirtings |  |  |  |  |  |
| 29.9.1 | It is the intention to recycle the existing timber skirting boards to the room and to splice-in new timber skirting sections where needed to the existing prior to decorating all as previously described. Any new sections shall match the profile of the existing as closely as possible and all surfaces to be level and flush prior to decoration. For the purposes of pricing, allow to supply and fix as required 5 linear metres of new timber skirting, 100mm FFL mitred profile. All new timber shall be (FSC) softwood timber skirting boards. All skirtings shall be decorated as previously described. |  |  |  |  |  |
| 29.10 | Dado Perimeter Trunking |  |  |  |  |  |
| 29.10.1 | See general commentary under the same element paragraph for room KW032B in relation to overall intentions. Amend, extend and adjust existing dado trunking installations to provide in total, 9 No DSO and 8 No Cat5e data points to provide 8 No workstations and 1 No additional DSO for cleaning appliances. Connect all data points back to the appropriate switch point and leave all installations networked, commissioned, NICEIC certified and in full working order. For the purposes of pricing, assume that an additional 6 linear metres of trunking will be required. |  |  |  |  |  |

REFURBISHMENT OF ROOMS kw032b & kw036d

CITY college plymouth

COLLECTION SHEET

| SECTION |  | PAGE No. | £ p |
| --- | --- | --- | --- |
| 1 | PRELIMINARIES |  |  |
| 2 | GENERAL WORKS |  |  |
| 3 | THE WORKS |  |  |
| ## Total Excluding VAT |  |  |  |

Date:  …………………………………………………………………………..

In the Capacity of:  …………………………………………………………….

Authorised to sign

on behalf of :  ………………………………………………………………….

Registered Address:  …………………………………………………………..

…………………………………………………………..

SPW/CF/71609

Date May 2017

APPENDIX I

TRade preambles

APPENDIX II

Drawings

APPENDIX III

STRUCTURED CABLING INFRASTRUCTURE SPECIFICATION CATEGORY 5e & 6a