Area 7

Technical Surveys and Testing

M1 SB MP175/5 to 168/1 – Resurfacing

Scheme ID VM 001

Site Information

Table of Contents

1	SITE INFORMATION	3

1.1	Site boundary, extents and access arrangements	3

1.2	Pavement	3

1.3	Drainage	3

1.4	Geotechnical	3

1.5	Soft Estate and Environment	3

1.6	Traffic Signs, Road Markings	4

1.7	Lighting	4

1.8	Structures and Buildings	4

1.9	Tunnels	4

1.10	Technology	4

1.11	Statutory Undertakers	4

1.12	Traffic	4

# SITE INFORMATION

## Site boundary, extents and access arrangements

Scheme is located between J23 and J22 of M1 on southbound carriageway

It includes J23 SB entry slip road and J22 SB exit slip road.

Between Marker Posts 175/5 and 168/1. Chart sections:

2400M1/554 (slip road)

2400M1/556;546;526;516;506;496;488

2400M1/486 (slip road)

Static traffic management with lane closures will be provided for cores and DCP and FWD works.

Full slip road closures will be provided for slip road works and works on the mainline carriageway, in the vicinity of the slip road tapers.

See document titled “VM001 – Scheme Location Details” for schematic plans

## Pavement

Surveys works to pavement carriageway in lanes 1, 2 and 3, plus slip roads.

Some hard shoulder coring required.

Road pavement is constructed of tarmacadam based products.

## Drainage

No works anticipated to be in vicinity of drains. If required location appears to be near a drain or pipe, core and/or DCP to be relocated to avoid drainage.

## Geotechnical

Not used

## Soft Estate and Environment

It is not anticipated that any works will be carried out in the verge.

Environment checks will be carried out and results issued to Tenderers by Principal Contractor prior to start of work.

Cores/DCP’s to be relocated/cancelled if there could be adverse environmental impacts. This to be agreed with scheme Engineer and/or Highways England Site Supervisor.

## Traffic Signs, Road Markings

Not used

## Lighting

Street lighting present in the vicinity of J23. No street lighting anywhere else.

Tenderer to provide working lights for own activities.

## Structures and Buildings

No cores or DCP to be taken over structures.

Double check to be made on site in all instances.

If any have been requested over structures in error, these are to be relocated away from the structure.

## Tunnels

Not used

## Technology

No cores or DCP to be taken in vicinity of loops or linking cables.

Double check to be made on site in all instances.

If any have been requested near loops in error, these are to be relocated away from the loops.

## Statutory Undertakers

C2 NRSWA enquiry results to follow and be issued to tenderer by Principal Contractor prior to start on site.

Any cores or DCP tests in vicinity of Statutory Undertakers plant to be relocated.

## Traffic

Traffic flow in the region of 68,000 AADT each way.

Time of start on site to be determined each night by Traffic management contractor and/or Principal Contractor. This will be based on traffic counts and the traffic management contractors assessment of what constitutes a safe traffic count in this location.

Therefore scheduled start on site time is based on usual conditions and anticipated access time for this section of road but could be later.