Kingstown Works Limited

Document Reference

8 - Specifications

Supply of Manufactured Joinery

01 April 2019 – 31 March 2021 with an option to extend a further year

CLOSING DATE AND TIME FOR RETURN OF THE TENDER:

12 Noon, On Friday 15th February 2019

Procurement Contact

Wendy Rhind

Kingstown Works Ltd
Connaught Road
Kingswood
Hull
HU7 3AP

tendering@kingstownworks.co.uk

DOCUMENT 8 – SPECIFICATIONS

KWL CODE: DOR01

BATTEN DOOR ONLY, PRIMED, SOFTWOOD, GRADE 5TH

Door size UP TO 2200mm Height x 950mm Width

(ANY SIZES OVER AND ABOVE THIS WOULD BE PRICED INDIVIDUALLY)

Door

22mm T&G Match Boarding

22mm x 120mm Rails & Braces

Glue and screw both sides

Use Knotting Solution were required

Fill & sand screw holes

I coat white primer

KWL CODE: DOR02

BATTEN DOOR & FRAME, PRIMED, SOFTWOOD, GRADE 5TH

Door size UPTO 2500mm Height X 1000mm Width

(ANY SIZES OVER AND ABOVE THIS WOULD BE PRICED INDIVIDUALLY)

Frame

Mortice & Tenoned 45mm x 95mm PSE softwood timber with loose stop lath 22mm x 45mm, finished with water based primer

Door

22mm T&G Match Boarding

22mm x 120mm Rails & Braces

Glue and screw both sides

Use Knotting Solution were required

Fill & sand screw holes

I coat white primer

KWL CODE: DOR03

INTERNAL FLUSH PLYWOOD HOLLOW CORE DOOR, 35MM THICK, PRIMED, SOFTWOOD, GRADE 5TH

Door size UP TO 2040mm Height x 836mm Width

(ANY SIZES OVER AND ABOVE THIS WOULD BE PRICED INDIVIDAULLY)

Door

Standard door cut down and lath replaced to fit unusual size casing

All laths to be glued filled and sanded

1 coat white primer

KWL CODE: DOR04

INTERNAL PSE DOOR CASING, PRIMED, SOFTWOOD, GRADE 5TH

Rebated as required

Size UP TO 2500mm Height x 1000mm Width

(ANY SIZES OVER AND ABOVE WOULD BE PRICED INDIVIDUALLY)

Casing dimensions up to maximum size 45mm x 125mm (Common Size 45mm x 100mm)

Glued and Screwed

Knotting Solution were required

1 coat white primer

KWL CODE: DOR05

EXTERNAL SOLID CORE DOOR ONLY, PRIMED, SOFTWOOD, GRADE 5TH

(Excluding Letterbox, handle & lock)

Cut from a 7ft x 3ft door blank

Door

45mm External solid core door cut to size

All laths to be glued filled and sanded

1 coat white primer

KWL CODE: DOR06

EXTERNAL SOLID CORE DOOR & FRAME WITH OR WITHOUT SILL, PRIMED, SOFTWOOD, GRADE 5TH

(Excluding Letterbox, handle & lock)

Door size UPTO 2070mm Height x 838mm Width

Door

1 External PREMDOR FD30 Fire Door, 44mm

Frame to be rebated for 10mm/15mm smoke seals

Frame to be morticed and tenoned

Glue screw and sand

Use knotting solution were required

1 coat white primer

KWL CODE: DOR07

INTERNAL FD30 SOLID CORE DOOR ONLY, PRIMED, SOFTWOOD, GRADE 5TH

(Excluding Letterbox, handle & lock)

Door size UPTO 2070mmH x 838mmW

Door

35mm FD30 Premdor Flush Ply

All laths to be glued screwed and sanded

1 coat white primer

Use knotting solution were required

KWL CODE: DOR08

10mm or 15mm REBATES ALL SIDES OF DOOR FOR INTRUMSCENT STRIP ANY SIZE DOOR

KWL CODE: DOR09

CUT OUT LETTER BOX OR VENT FOR ANY DOOR

KWL CODE: DOR10

STORM DOOR & FRAME WITH FANLIGHT, PLYWOOD OR GLAZED FANLIGHT, PRIMED, SOFTWOOD, GRADE 5TH

Frame size up to 3000mm Height x 1200mm Width

Frame

Mortice & Tenoned 45mm x 95mm PSE softwood timber with loose stop lath 22mm x 45mm, finished with water based primer

Door

22mm T&G Match Boarding

22mm x 120mm Rails & Braces

Glue and screw both sides

Use Knotting Solution were required

Fill & sand screw holes

I coat white primer

KWL CODE: DOR11

FRAMED, LEDGED & BRACED DOOR, SOFTWOOD, GRADE 5TH

PRIMED & ASSEMBLED

ANYSIZE

45mm x 95mm stiles and top rail

22mm x 95mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer

KWL CODE: DOR12

FRAMED, LEDGED & BRACED DOOR & FRAME, SOFTWOOD, GRADE 5TH

TOPLIGHT WITH PLYWOOD INFIL

PRIMED AND ASSEMBLED

Size up to 3000mm Height x 1000mm Width

Frame

Mortice & Tenoned 45mm x 95mm PSE softwood timber with loose stop lath 22mm x 45mm, finished with water based primer

Door

45mm x 95mm stiles and top rail

22mm x 95mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer

KWL CODE: DOR13

INTERNAL PLYWOOD CUPBOARD DOOR, 25MM THICK, PRIMED,

SOFTWOOD, GRADE 5TH

Door size UP TO 2040mm Height x 900mm Width

(ANY SIZES OVER AND ABOVE THIS WOULD BE PRICED INDIVIDUALLY)

DOOR MANUFACTURED FROM

1”x4” PSE frame

Covered with 6mm paint grade ply

12mm timber laths

Glued screwed filled and sanded

1 coat white primer

KWL CODE: DOR14

PAIR OF SOLID CORE DOORS 35MM WITH REBATES, PRIMED, SOFTWOOD, GRADE 5TH

SIZE OF APERTURE NO MORE THAN 2200MM Height x 1800MM Width

Door

1Pr External Grade FD30 Flush Fire Door, 35mm

Rebated

1 coat white primer

KWL CODE: DOR15

PAIR OF GARAGE DOORS, APPROXIMATELY 2180 WIDE X 2165 HIGH,

ALL PRIMED, SOFTWOOD, GRADE 5TH

45mm x 145mm stiles and top rail

22mm x 145mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer

KWL CODE: DOR16

PAIR OF GARAGE DOORS AND FRAME 2180 WIDE 2165 HIGH OVERALL SIZE ALL PRIMED

Frame

Mortice and Tenoned 3”x4” PSE Timber

1”x2” PSE Timber stop laths

Doors

45mm x 145mm stiles and top rail

22mm x 145mm Rails and braces

22mm T&G match boarding

Rails morticed and tenoned in to stiles

Use knotting solution were required

1 coat white primer

KWL CODE: DOR17

GARAGE DOOR FRAME, 3X4 PSE AND PRIMED, APPROXIMATELY 2.2 METERS WIDE X 2.1 METER HIGH, SOFTWOOD, GRADE 5TH

All supplied dry to assemble on site due to size of frame

Frame

Morticed and Tenoned 3”x4” PSE Timber

1”x2” PSE Timber Stop Laths

Use knotting solution were required

1 coat of white primer

KWL CODE: DOR18

BIN AND METER CUPBOARD DOOR AND FRAME, ANY SIZE, ASSEMBLED AND PRIMED, SOFTWOOD, GRADE 5TH

Frame

Morticed and tenoned

45mm x 95mm Square or bevelled timber

45mm x 12mm Rebates

Doors

44mm Solid Flush doors cut from standard blank and edged in PSE timber

Use knotting solution were required

1 coat white primer

Meter door approx. 900mm high x 600mm wide

Dustbin door approx. 1100mm high x 600mm wide

KWL CODE: DOR19

BIN OR METER CUPBOARD DOOR ONLY, PRIMED, 44MM THICK UPTO 1300MM HIGH AND 620MM WIDE, SOFTWOOD, GRADE 5TH

44mm Solid Flush Door

44mm Solid flush door cut from solid blank

12mm Timber laths to all sides

Use knotting solution were required

1 coat white primer

KWL CODE: DOR20

DOOR FRAME WITH FANLIGHT, ANY SIZE, EXTERNAL USE, UPTO 1 METER WIDE WITH TOP LIGHT AND PLYWOOD INFILL 2X4 PSE AND PRIMED, SOFTWOOD, GRADE 5TH

PLY INFILL

Morticed and tenoned

Hardwood sill if required

Door frame to be square or bevelled as required

Normal section size 45mm x 95mm PSE

Use knotting solution were required

1 coat of white primer

KWL CODE: DOR21

DOOR FRAME, ANY SIZE, EXTERNAL USE, UPTO 1 METER WIDE 2X4 PSE AND PRIMED, SOFTWOOD, GRADE 5TH

Morticed and tenoned

Rebated timber

Hardwood Sill if required

Door frame to be square or bevelled as required

Normal section size 45mm x 95mm PSE

Use knotting solution were required

1 coat of white primer

KWL CODE: DOR22

EXTERNAL PREMDOR, FD30 DOOR & FRAME, WITH OR WITHOUT SILL, PRIMED, SOFTWOOD, GRADE 5TH

Frame to be altered to fit standard door. INTUMESCENT STRIPS REBATED IN TO FRAME, FIRE RETARDANT BOARD FOR INFIL TO FANLIGHT.

Door to be fitted on 3no CE fire approved 100mm hinges in frame.

(Excluding Letterbox, handle & lock)

Door size UPTO 2400mm Height x 850mm Width

Frame

Morticed and tenoned

Hardwood sill if required

Door frame to be square or bevelled as required

Normal section size 45mm x 95mm PSE but up to 125mm

Use knotting solution were required

1 coat of white primer

Door

1 External PREMDOR FD30 Fire Door, 44mm

Frame to be rebated for 10mm/15mm smoke seals

Frame to be morticed and tenoned

Glue screw and sand

Use knotting solution were required

1 coat white primer

KWL CODE: GATE01

TREATED LARGE GATE WITH MORTICE TENON JOINTS, SOFTWOOD, GRADE 5TH, SIZE AROUND 1000mm WIDE AND 2000mm HIGH

50mm x 100mm stiles tannalised timber

25mm x 150mm rails, braces pales

Glued and screwed

KWL CODE: GATE02

TREATED SMALL GATE AS PER ORIGINAL SPEC AGREED, 750MM WIDE X 710 HIGH, SOFTWOOD, GRADE 5TH

50mm x 100mm stiles tannalised timber

25mm x 100mm rails, braces pales

Glued and screwed

KWL CODE: STAIR03

6 TREAD STAIR CASE, SOFTWOOD, GRADE 5TH

SITE VISIT REQUIRED TO MEASURE ORIGINAL STAIRS

String approx. size 220mm x 32mm PSE

Treads 22mm MDF

Glue, screw, fill and sand

Use knotting solution were required

KWL CODE: STAIR04

UPTO 3 TREAD STAIRCASE BOX, SOFTWOOD, GRADE 5TH

SITE VISIT REQUIRED TO MEASURE ORIGINAL STAIRS

Left or Right handed

String approx. size 220mm x 32mm PSE

Treads 22mm MDF

Glue, screw, fill and sand

Use knotting solution were required