Quotation reference: ENVFRA12S065

Quotation title: Royal Military Canal Fencing

| Supply and install the following |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ITEM | DESCRIPTION | QUANTITY | UNIT | RATE | Cost £ Excluding VAT |
| Restraining existing fencing | Carry out restraining of existing fencing | 400 | metre |  |  |
| Replacing individual fencing Items | Take down remove and replace 50mm x 75mm machine round stakes with 75mm round stakes | 39 | No. |  |  |
|  | Take down remove and replace 100mm machine round stakes | 300 | No. |  |  |
|  | Take down, remove and replace 1no strut only within existing fencing runs. | 1 | No. |  |  |
|  | Take down, remove and replace 1no two way strainer within existing fencing runs, straining posts and struts to be concreted into the ground. | 11 | No. |  |  |
| Replacing gates and stiles | Remove existing broken gate posts and replace with new 175x175mm 4 way weathered softwood gate posts (using existing gate fittings). | 3 | No. |  |  |
|  | Remove existing broken gate posts for single leaf metal field gate and replace with new 175x175mm 4 way weathered softwood gate posts (using existing gate fittings). | 2 | No. |  |  |
|  | Replace existing broken stiles with new softwood timber stiles as per Highways Drawing H42. | 2 | No. |  |  |
|  | Straighten existing timber field gate | 1 | No. |  |  |
| Replacement of entire fencing runs | Take down and remove from site existing stock fencing across various runs. To include clearing of existing hedge material as required to install fencing. Hedge material to be left on site | 1600 | metre |  |  |
|  | 1.2m high stock fencing with 75mm / 100mm stakes driven into the ground @ 3m centres, comprising of 0.8m high stock fencing and 2no rows of 2.5mm line wire. 1 two way strainer every 49m. | 1600 | metre |  |  |
| Traffic management | Sections may require traffic management. Health and Safety check MUST be carried out along entire fencing line prior to works to ensure safety of fencing workers. | 5000 | metre |  |  |
| LUMP SUM TOTAL |  |  |  |  |  |

| Name | Title/Grade | Day Rate | No of Days |
| --- | --- | --- | --- |
| Total |  | £ |  |

Note to bidders:

- All costs/ rates are to be exclusive of Value Added Tax

- Sums quoted are fully inclusive of all requirements as detailed in the specification, see Section 2 of this quotation document

- No work shall be carried out at day work rates without the prior agreement of the Environment Agency’s nominated representative

- The sums shown shall be fixed for the duration of the contract

- Receipts for all rail travel, hotel and food expenses will be required as proof of expenditure

- Payment will be made on satisfactory completion of the work in accordance with the Conditions of Contract, see Section 4.

Signed:			___________________________________________

Name:			___________________________________________

Date:				___________________________________________

Position:                  	___________________________________________

Company:	           	___________________________________________

Telephone number:	___________________________________________