CIVIL WORKS CONTRACT

DIO SDT (B)

BATSUB

BOOKLET 3 SPECIFICATION LUMP SUM FIRM PRICE CONTRACT FOR BEL/GE/1612 ESTATE WORKS PACKAGE 1 BATSUB, PRICE BARRACKS, BELIZE, BFPO 12

References

- The Health & Safety at Work Act 1974 (HASWA 74).

- Construction (Design & Management) Regulations 2015 (CDM 2015).

- The Latest Edition of the UK Building Regulations.

# The drawing attached to this document, is to be read in conjunction with this PS for the proposed construction.  It should be noted that this drawing is ‘Concept Only’ and the Contractor is to produce a full set of design documentation and confirm quantities and distances.

# Contractors Design

# The Contractor is to advise the Project Manager (PM) of all design consultants / Sub-contractors who will be employed on this Contract and the areas of their design responsibility.  Any Works undertaken without prior issue of full information for the necessary approvals will be entirely at the Contractor’s own risk.

# The Contractor shall not commence any work in accordance with any Design Document until the PM has agreed the relevant Design Document.  The Contractor shall keep one copy of each Design Document on the Site, to which the PM or his representatives may have access at all reasonable times.

# Design Responsibilities

# Contractor’s Design Responsibilities.   The Contractor is responsible for the design of all elements contained within the schedule of Works.  Where required, all designs are to be submitted to the Authority for concurrence and authorisation prior to implementation of Works.  Where the nature of Works do not warrant full design (refurbishment of fencing, installation of area delineation fencing (as opposed to security fencing)), a descriptive proposal complete with proposal sketch should be submitted for concurrence.

# The Contractor shall ensure all designs are in accordance with, and includes the applicable requirements as contained within References A to C and all other design documents specified within this PS.  NOTE that these references are provided as guidance and minimum requirement only; the responsibility remains with the Contractor to ensure that all design and construction Works comply with the latest methods, requirements and guides, and generally accepted practice as relates to the Works, geography, climate, environment and Theatre of operations.  Where there are conflicts between the Contractors design drawings and the Specification (Booklet 3) the PS will take precedence in all cases.  Any queries should always be addressed to the Authority in the first instance.  The references and specified British Standards (BS) / EuroNorms (EN), within this PS are to be used (or similar Authority approved standards) as a minimum standard, throughout the Works.

# UK Building Regulations.  Where applicable all Works shall comply with the latest UK Building Regulations where possible.  Should any variation be required, approval from the Authority shall be obtained in writing prior to the commencement of any Works.

# Health and Safety (H&S).  All Works shall be carried out in accordance with the HASWA 74.  The Contractor shall submit a Construction Health and Safety Plan (CHASP) to the Authority for approval, including all risk assessments, method statements, hazardous material procedures, etc. to comply fully with CDM 2015.

# This Contract contains no requirement for the submission of a H&S file (normally required upon completion and presented to the Authority 10 days before handover) in compliance with CDM 2015.

# Construction Drawings.  All Works shall be constructed in accordance with the final approved Contractor furnished construction drawings (or written proposal in this case).  Where a conflict in design is found, authorisation shall be obtained in writing prior to any changes in design.  No construction shall commence prior to obtaining written approval of the final construction drawings / proposal by the Authority.

# Design Life.  The design life of all new civil and structural components and assemblies to first major overhaul, repair or replacement shall be a minimum of 10 years.  The Contractor shall supply and install all components, elements and systems / structures to satisfy this requirement, and provide documentary evidence as per CDM 2015 and any additional Contractual requirements that may apply.

# Existing Services.  The Contractor is responsible for ensuring all services on or adjacent to the site that will be affected by the Works are identified, located and appropriate action taken to prevent damage before work commences.  The Contractor shall ensure that any existing services and concrete slabs are reinstated to a condition at least similar to existing, with the minimum of disruption to existing facilities and services during construction.  All Works that could possibly influence existing services may only commence with written approval – this includes but is not limited to a Permit to Dig.  The Authority must be informed immediately if any unknown services are discovered that will have an impact on the Works.

# The Contractor shall ensure that any existing services and affecting Works – including all connections and services tied into – are of an acceptable standard to ensure the required performance over the life of the facility.  Should it be found that one or more of the services (including but not limited to existing water, sewerage, drainage and electrical reticulation) is not considered acceptable to the requirements, the Contractor shall inform the Authority and obtain written approval / instruction prior to taking any actions to rectify or make good the pertinent unacceptable situation.

# Construction Preliminaries

# Location Plan.  The location of the proposed Works is within Price Barracks shown at Dwg DIO/PB/1612/G/001.

# Existing Ground Conditions.  The Contractor is responsible for confirming the existing ground conditions, which are to be taken into account whilst designing and compiling the CHASP.

# Obstructions.  As the proposed Works are to be carried out around existing infrastructure and surrounding areas, there are obstructions in the form of fences and buildings etc, the Contractor is to review the site before commencing Works.

# H&S Restrictions, Precautions and Monitoring.  The Contractor shall implement the requirements described in the HASWA 74 to protect members of the public and persons visiting the site from risks arising from the use of equipment, materials or substances defined therein.

# Control of Noise and Vibration.  The Contractor shall comply with the recommendations for practical measures to reduce noise set out in BS 5228: Parts 1, 2 and 4.

# It is expected that the Contractor will not require a forward resources area other than the site. Should one be required a location within Price Barracks shall be made available, as agreed by the PM.  Packaging shall be to a good standard capable of withstanding transportation and handling loads.  The Contractor shall be solely responsible for storage and movement of all equipment to site.  The Contractor shall remain at all times responsible for the security of equipment including prevention of theft.

# Scope of the Overall Works

# General.  This PS outlines the requirements for the general estate Works required within the BATSUB estate.  These Works are of a varied type.  These can be seen in detail at serial 24 – Proposed Works.

# The Authority.  The Project Manager (PM) has overall Authority.  For the purpose of this PS, the PM is any person from within the Authority acting on behalf of the PM, the Superintending Officer (SO) in most cases.

# There are 9 Works Packages in total all in separate locations within Price Barracks.

# Use of the Site.  The Site shall not be used for any purpose other than undertaking the specified Works.  If required, the Contractor shall erect the site and storage compound in a location agreed with the Authority for no longer than is required to complete the Works.  No storage of materials, parking of vehicles, temporary accommodation or any other use of areas beyond the boundaries shall be permitted.  Under no circumstances shall it be permissible for the Contractor to cause an obstruction to normal pedestrian or vehicular movements within the vicinity of the site.  The sites and site compounds are to be suitably fenced off.

# Restrictions to the Works.  The Works are to be undertaken without interfering with every day running of the Authority’s operations within the area.  The demarcation of the Site boundary is to be obvious and clearly marked to restrict access to the Site whilst the Works are being undertaken.

# Proposed Works.  The Schedule of Works for the Contract is as follows:

## The 9 marked locations where the Works are required are shown on Dwg No. DIO/PB/1612/G/001.  It is the Contractor’s responsibility to attend site and confirm quantities and descriptions detailed below.  A site visit will be conducted during the ITT meeting and Contractors are also free to arrange access at a later date.

## The Works are as follows:

### Works Package 1:

#### Reset the DIO sign outside Bldg 114.

#### Remove  2 No. road sign posts within the vicinity.

#### Make good ground.

### Works Package 2:

#### Repaint and repair (corroded leg) the culvert railings opposite Bldg 82.

#### Reinstate the damaged bollards in the same area.

### Works Package 3:

#### Reinstate and paint damaged railings at Bldg 290.

#### Install 2 No. impact bollards.

### Works Package 4:

#### Repair damage to the corner of the roof at Bldg 49.

### Works Package 5:

#### Deconstruct the Attap opposite Bldg 263.  The contractor shall deconstruct and remove arisings from site.  The structures are to be disassembled methodically by hand.

### Works Package 6:

#### Deconstruct and remove shed 224A.  The contractor shall deconstruct the structure and remove arisings from site.  The structure is to be disassembled methodically by hand.

### Works Package 7:

#### Reinstate the damaged bollard outside Bldg 261.

#### Repaint both bollards for visibility.  Colours to be approved by the Authority.

#### Works Package 8:

#### Install 5 ft fence along road side opposite BDF MTFI (approx. 60m). Fencing constituents shall be approved by the Authority prior to installation. Fence poles and chainlink shall be factory treated to resist corrosion and posts shall be of a suitable heavy duty grade (the fence and constituents should be of the same type and grade as that recently installed at the rear of the LoGWO Dept, Bldg 260).  Fence line as depicted by Authority installed survey pegs.

#### 100mm box section Gate posts shall be installed at the access points to the MTFI, but no gates are to be fitted.

#### Works Package 9:

#### Break out and return to topsoil all former rubberised play areas within the proximity of Bldg 24.

#### Use of Design Codes and Specifications

# In addition to Clause 47 of the Conditions of Contract, all designs must conform to the most current editions of British Standards (BS) or EuroNorms (EN).  Other Design Codes may, on occasion, be used provided that they meet the minimum requirements and the Contractor shall submit proof of equivalency to the Authority for approval of use.  The Authority can be approached for clarification of the relevant BS if required.  On no account is the Contractor to use any design information other than that stated or approved by the Authority.

# All Contractors will be required to certify their designs for conformity to the specification and to the relevant BS / EN.  Only appropriately qualified and experienced engineers are to certify designs.  The Authority reserves the right to request copies of all calculations and drawings for analysis prior to the commencement of any construction on site.  The Contractor will provide copies of all calculations and ‘As Built Drawings’ for the H&S File.

# The Contractor is to ensure that the specified output of their design expressly meets the parameters outlined within this PS.

# Management of the Works

# Personnel.  The Contractor is fully responsible for ensuring all personnel within the workforce are authorised to gain access to and work within Price Barracks.  The Contractor is solely responsible for the submission and approval of passes and the Authority will not grant extensions to performance periods based on delays obtaining passes for access to the Base.  The Contractor shall be responsible for the transportation of Contractor personnel at all times both to and from the site.  Contractor management staff shall ensure that all employees and Sub-contractor employees’ ID documentation shall remain current during the life of the Contract.

# Liaison with the Authority.  The Contractor shall designate one person from within his organisation who will be responsible for liaising with the PM or his representative (SO) on a day to day basis and as the need arises.  The Liaison Officer must have the capability of reading, writing, speaking and receiving instructions in the English Language, including being able to understand and interpret technical drawings and specifications.  The Liaison Officer must be able to explain the work operations to persons performing the work in a language that those performing the work are capable of understanding.  The person so designated shall be responsible for communicating with the Authority regarding notification of intended work and explaining the effect that the Works will or may have on the operation or systems within the site.  The Liaison Officer will be responsible for responding to the Authority's enquiries and dealing with any issues or complaints.  It is a requirement that the Contractor's Liaison Officer makes daily contact with the Authority such that an active rather than a responsive attitude to liaison is maintained.  The Contractor's Liaison Officer is a key person in achieving the successful execution of this Contract.

# Supervision.  The Contractor shall accept responsibility for design, co-ordination, supervision and administration of the Works, including all sub-contracts.  They shall arrange and monitor a Programme with each Sub-contractor, supplier, local Employer and any statutory undertaker, and obtain and supply information as necessary for co-ordination of the work.  In addition to constant management and supervision of the Works provided by the Liaison Officer, all significant types of work must be under the close control of competent trade supervisors to ensure maintenance of satisfactory progress and quality.

# Co-ordination of Engineering Services.  The site organisation staff must include one or more persons with appropriate knowledge and experience of mechanical and electrical engineering services to ensure compatibility between engineering services, one with another and each in relation to the Works generally.

# Approvals.  Where products or work are specified to be approved or the PM instructs or requires that they are to be approved, the same must be supplied and executed to comply with all requirements.

# Photographs.  No photographs shall be taken of the site without the express Authority of the PM.

# Programme of Works.  Prior to the commencement of the Works, the Contractor shall liaise with the Authority, in order to produce a detailed Programme of Works.  This Programme shall be agreed and approved and shall form the baseline Programme for the Contract.  Any deviations from the agreed Programme shall be communicated to the Authority, with Programme updates being produced if required and approved by the SO.  The Programme must be daily resolution and show when stores equipment and plant deliveries will occur.

# Programme Status.  The Contractor shall also show on this Programme each state of both design and construction of the various elements of the Works so as to illustrate the latest dates by which instructions requiring changes can be accommodated in each part of the Works without effecting the completion thereof.  Thereafter, the Contractor shall amend and revise the Programme as required by the Conditions of Contract and as required by the PM.  The submission of a Programme will not relieve the Contractor of his responsibility to apply in writing for instructions, drawings etc. in accordance with the Conditions of Contract.

# Commencement of Work.  Inform the PM at least five days before the proposed date for commencement of work on site.

# Monitoring.  Record progress on a copy of the Programme kept on site. If any circumstances arise, that may affect the progress of the Works, put forward proposals or take other action as appropriate to minimise any delay and to recover any lost time.  A daily site diary in an appropriate format is to be kept by the Contractor.  The site diary is to include details of personnel and plant / equipment on site and is also to include delays and the reason for such delay, visitors to site and Works progress.

# Site Meetings.  The PM, or his representative, will hold regular site meetings to review progress and other matters arising from the administration of the Contract.  It will be the Contractors responsibility to ensure they are available and attend such meetings.

# Notice of Completion.  The Contractor is to provide the PM at least seven days notice of the anticipated dates of Practical Completion of the whole or parts of the Works.

# Extension of Time.  When it is necessary to report a cause of a delay or likely delay in the progress of the Works this shall be given in the form of a written notice that must also be given of all other causes, which apply concurrently.  The notice shall state the following details.

## Relevant particulars of the expected effects if appropriate related to the concurrent causes.

## An estimate of the extent, if any, of the expected delay in the completion of the Works beyond the Date of Completion.

## Any other relevant information required by the PM.

# Quality Standards / Control.  The Contractor will ensure quality standards and controls.

## General Quality of Products.  All products shall conform to the following subparagraphs:

### Products to be new unless otherwise specified by the Authority.

### For products specified to a BS or EN obtain certificates of compliance from manufacturers when requested.

### Where a choice of manufacturer or source is allowed for any particular product, the whole quantity required must be of the same type, manufacture and / or source unless otherwise approved.  Produce written evidence of sources of supply when requested by PM.

### Ensure that the whole quantity of each product required to complete the work is of consistent type, size, quality and overall appearance.

### Where consistency of appearance is desirable ensure consistency of supply from the same source.  Do not use different colour batches where they can be seen together.

### If products are prone to deterioration or have a limited shelf life, order in suitable quantities to a Programme and use in appropriate sequence.  Do not use if there are any signs of deterioration, setting or other unsatisfactory condition.

## Proprietary Products.  All products shall conform to the following subparagraphs.

### Handle, store, prepare and use or fix each product in accordance with its manufacturer's current printed or written recommendations / instructions.  Inform PM if these recommendations / instructions conflict with any other specified requirement. Submit copies to PM when requested.

### The tender will be deemed to be based on the products specified and recommendations on their use given in the manufacturer’s literature current at the date of tender.

## Checking Compliance of Products.  The Contractor shall check all delivery tickets, labels, identification marks and where appropriate the products themselves to ensure that all products comply with the Contract Documents. In particular, check that the products comply with the following sub-paragraphs.

### The sources, types, qualities, finishes and colours are correct, and match any approved samples.

### All accessories and fixings that should be supplied with the products have been supplied.

### Sizes are correct.  Where tolerances are critical, measure a sufficient quantity to ensure compliance.

### The delivered quantities are correct, to ensure that shortages do not cause delays in the Works.

### The products are clean, undamaged and otherwise in good condition.

### Any products with a limited shelf life are not out of date.

## Protection of Products.  All products shall be protected to ensure that they remain in the condition they are required to be in.  In particular the Contractor is to ensure that products are to be prevented from overstressing, kept clean, protected from the elements and kept in original wrappings until required for the Works.

# Setting Out.  The Contractor is responsible for all setting out and shall check out the dimensions of the site against those detailed by the Authority, and record the results.  The Contractor shall inform the PM when overall setting out is complete and before commencing construction.

# Defects in Existing Construction / Services.  This shall be reported to the PM without delay. Obtain instructions before proceeding with work which may be covered up or otherwise hinder access to the defective construction, or be rendered abortive by the carrying out of remedial work.  This is particular relevant in relation to the expansion of the existing service runs.

# Timing of Tests and Inspections.  The Contractor is to agree dates and times of tests and inspections with PM four days in advance, to enable the PM and other affected parties to be present. On the previous working day to each such test or inspection the Contractor is to confirm to the PM that the work or sample in question will be ready or, if not ready, agree a new date and time.

# Proposals for Rectification of Defective Work / Products.  As soon as possible after any part(s) of the work or any products are known to be not in accordance with the Contract, or appear that they may not be in accordance, the Contractor is to submit proposals to the PM for opening up, inspection, testing, making good, or removal and re-execution. Such proposals may be unacceptable to the PM and he may issue contrary instructions.

# Quality Control.  The Contractor is to establish and maintain procedures to ensure that the Works, including the work of all Sub-contractors, comply with specified requirements.  This is to include all testing of materials that are to be incorporated into the Works (e.g. concrete).  The Contractor is to maintain full records, keep copies on site for inspection by the PM, and submit copies of particular parts of the records on request.  The records must include the following:

## Identification of the element, item, batch or lot including location in the Works.

## The nature and dates of inspections by the Contractor or PM, tests and approvals.

## The nature and extent of any non-conforming work found.

## Details of any corrective action.

# Work at or after completion.  The Contractor is required to undertake the following Works prior to handover.

## Make good all damage consequent upon the work.

## Remove all temporary markings, coverings and protective wrappings unless otherwise instructed.

## Adjust, ease and lubricate moving parts as necessary to ensure easy and efficient operation, including doors, windows, drawers, ironmongery, appliances, valves and controls.

# Making Good Defects.  The Contractor is to make arrangements with the PM and give reasonable notice of the precise dates for access to the various parts of the Works for purposes of making good defects.  The PM is to be informed by the Contractor when remedial Works to the various parts of the Works are completed and ready for approval.

# Documents to be provided by Contractors

# Design Information.  For the purposes of the proposed Contract, the Contractor shall provide as a minimum a written scheme detailing the proposed design / configuration and method of works for each element of the proposed Works. Due to the simple nature of the proposed Works, design drawings will not be required, though pictorial representation of proposals included within the written scheme will be preferable.  The Contractor is to submit all design information to the Authority prior to commencement of the construction phase.  The PM will examine design documentation and shall be entitled to reject a design as unsatisfactory where it is not in accordance with the specification, statutory regulations, or if it would be unfit for purpose.

# The signed and approved design documentation shall be used as the construction documents on site for all work purposes and to satisfy the requirements as specified by the Construction (Design & Management) Regulations 2015 (CDM 2015).

# Site Diary.  The Contractor shall keep an up to date, daily, site diary.  This document is to be used to record all decisions made on the site both verbal and written.  The document is also to be used to record visits to site and note anything, which has a direct effect on the Contract in terms of cost and extensions to time, or any other occurrence that affects the Works Programme.  All changes must have been agreed and authorised by the Authority.  The site diary shall list in writing all issued Variation Orders (VO) and site instructions, including amended drawings, and verbal decisions made on site.  All verbal decisions must be recorded in writing no less than 24hrs after they have been made and entered in the site diary.  The Contractor shall on request make the site diary available for the Authority to inspect.  This document will be used as the audit trail in light of any disputes, concerning the Contract.

# General Conditions

# Permits to Work (PTW) and Authorisation.  The Contractor shall comply with the Client’s PTW system and the JSP 375, Volume 3, (details of which are obtainable from the Authority SO), including the provision of method statements, risk assessments, switching / isolating safety Programmes, Permit to Dig, etc.

# The responsibility remains with the Contractor to obtain all necessary authorisations including but not limited to a Permit to Dig, security and access to the camp and site, fencing / screens to isolate runways, taxiways, and other existing facilities from the Works, etc. required to commence and complete the Works.

# The Contractor shall at all times ensure that full coordination, cooperation and liaising with the SO is achieved and maintained.

# Site Instructions.  Where there is a change in design or specification, which does not have a cost or time implication a Site Instruction shall be issued by the SO to authorise the proposed change at nil cost to the Authority.  The site instruction shall also be used to authorise a change in a specified material / item / equipment as long as the replacement meets the standards required by the original specification and is authorised by the SO.

# Variation Order (VO).  Where there is a change in design or specification, which does have a cost or time implication a VO shall be raised by the SO to describe the proposed change, this may have a cost implication to the Authority.  The WCO will issue the VO to the Contractor for pricing; the Contractor will price the variation and return it to the WCO.  All VOs must be authorised by the PM, Financial Officer and issued via the WCO prior to the commencement of any variation Works on site.  All VOs are incorporated into the Contract by the issue of a Contract Amendment.

# Rejection of Work.  The Authority reserves the right to reject or condemn any area of the Works that he considers to be below an acceptable standard and the Contractor shall replace or repair the said Works within 7 days of being notified in writing of the rejection by means of a Non Conformance Report.

# Regulations.  It is the Contractors duty to be fully conversant with all local / MoD regulations and requirements in respect of fire, safety, security and occupational health, etc.  These are to be fully complied with throughout the Contract period.

# Defects Liability Period.  A defects liability period of 12 months shall apply for the Works.  The Contractor will be responsible for making good at his own expense defects in the Works for a 12 month period from handover.

DIO SDT (B) is the Authority for this Contract.