Resurfacing of the Car Parks and Roads

Location

MHRA, Blanche Lane, South Mimms, Potters Bar, Hertfordshire EN6 3QG, UK.

Introduction:

The South Mimms site requires some designated roads and many of our carparks to be re-surfaced. The carparks were built over 12 years ago and some of the car park has come up due to weather and vehicles. Parts of the road surface has become cracked; this could be caused by poor foundation bearing in mind that the under surface is mainly clay. It is envisaged to re-surface the car parks and roads as shown on the site layout drawing.

On the West side road, this has become severely cracked due to poor substrate and will need to have a replaced substrate foundation laid.

The car park resurfacing will be carried out in various phases.  Phase 1 top carparks, 2, 3, 4, 5, 6, 7, where we have solar carports, this will be carried out this financial year (by end March 2026).

The remainder of the site carparks and roads will be undertaken as budget becomes available over the next 5 years.  This is to include car parks 8,9,11,12, and the visitor car park and the two (East & West) road areas.

Please provide a fixed cost for Phase 1 and separate costs for each of the other car parks and roads with indicative cost increases over the 5-year period.

General Requirements

Please make your tender return MHRA site specific.

A comprehensive Gantt chart is to be provided with the tender return showing the work phases. This should show all lead times. It is preferable that this is supplied in Microsoft Project but an excel spreadsheet would be accepted. Please give realistic timings.

Emergency access at the top of site must at all times be clear of vehicles. This will be pointed out during the site visit.

During a fire alarm, MHRA staff will gather at the muster points in the carpark. The card readers at the end of the carparks must be accessible at all times.

Within your return, please state the length of warranty for defects for both the roads and carparks.

Any spoil or loose waste spilt during the works or transit must be cleared up immediately. This goes for new surface asphalt brought to site ready for laying.

Please detail any expected weekend work. Please make sure that this is captured on your schedule.

Site Visits

The Supplier is to carry out a thorough and extensive survey to establish and verify actual site requirements and equipment, prior to further design work and the subsequent construction phase. We expect a fully costed tender return, if there are assumptions, these must be fully explained, and a costed range supplied. The cost that you are to enter must be in total cost and not per square meter etc.

Note: Please check the availability of the products you will be supplying, prior to submitting your bid. Out of stock and superseded products would not constitute a variant cost during the construction phase.

Scope

The Supplier shall:

Conduct a site survey to review the proposed designs and locations and shall confirm suitability of their proposed solution. Any initial observations on the current installation should be noted on the tender return.

Comply with the Agency H&S guidelines at all times while on site.  The H&S advisor on site has responsibility for ensuring compliance on the Agency’s behalf and will form part of the project team.

All Supplier employees will be required to go through an Agency security background check prior to attending site.

All Suppliers employees will be given the site induction when attending site for the first time.

Should, as a result of the contract, an incident or accident occurs to either a member of the Agency’s staff, property or Supplier’s employees, the person responsible for you on site (generally the Project Manager) must be informed as soon as possible after the immediate emergency has been dealt with. The responsible person will then inform the H&S team.

The Principal Supplier (Contractor) will manage health and safety on the site with established rules and procedures. These will be displayed, and all operatives and visitors to the site are required to undergo a site-specific induction to be familiarised with them. A signature of understanding of the rules and procedures together with an undertaking to abide by them will be retained in a register maintained on site for that purpose.

Site rules, practices, and procedures to be established and enforced will include but not necessarily be limited to the following: -

- Site working hours are 9:00 am to 17:15pm, Monday to Friday, any work outside of this time would need to be agreed in advance with the Agency

- Suppliers’ personnel must comply fully with the client’s security arrangements and procedures

- Operatives and visitors report to the site supervisor are inducted and sign in and sign out

- Smoking is not permitted on the site except in defined areas

- Radios and personal stereos are not permitted

- Personal protective equipment must be worn as required by their risk assessment

- Debris removal and site clearance are regular

- All existing mains services are located, identified and marked

- Fire precautions and procedures are maintained including the provision of firefighting equipment and means of escape

- Site security is to be maintained at all times in accordance with Agency policies and procedures

- Sufficient resources are to be provided and maintained to ensure that operatives do not lose concentration or become fatigued resulting in injury

- Only trained personnel to operate mechanical plant or electrical equipment

- Risk and Method statements will be agreed and available to read or review in the site office with a copy held in the project office.

The Agency will continue to operate around site as normally as practical. However, it will be necessary for some operational areas to be vacated for periods of time to allow works to be carried out. The Principal Contractor (Supplier) will be required to liaise with the Agency so that a program can be established to suit operational requirements.

Some areas will be harder to carry out the install in normal working hours so you must allow for some weekend work for these areas, Line and logo marking may need to be carried out at the weekend. This will need to be carefully planned with our neighboring sites and the houses that are in our catchment area.

The Principal Contractor (Supplier) must ensure that his operations do not pose any risk to the Agency personnel or visitors to the site.

There are no health risks to Supplier’s contractor personnel from the Agency activities as long as the Agency security controls are complied with fully.

Whilst the work is happening, appropriate signage and barriers must be put in place to divert staff and deliveries onto site.  Persons at work signs are to be placed close to where Supplier’s contractors are operating. Please detail how you will safely manage staff, vehicles & deliveries during the works.

At no point should the road be blocked off without prior arrangement. Emergency vehicles must always have access to the site.

Precautions must be taken to protect site operatives and general public from hazards associated with vibration, dangerous fumes (diesel exhaust) and dust (ground cutting) Noise from plant, arising during the course of the works. There are extremely sensitive areas around site that will need careful consideration when using percussion drills etc. Some areas are Home Office regulated and as such all the work around these areas must be planned and agreed prior to starting.

If appropriate, all landscaped areas and grass land are to be levelled and fully made good at the end of the works. Allowance must be made to back fill vehicle track marks with topsoil and completely re-grass seeded.

The site roads and pavements must be kept clean at all times. Please allow for cleaning in the costings.

If you need to carry out a ground survey to determine the quality of the ground or underground services.  This needs to be allowed for in the costings.

Provide all design documentation, manufacturers literature and O&M manuals.

Detailed requirements.

Electric Vehicle Signage

Please provide cost for 11 x Reflective 3mm Aluminium Composite signs with wording like this. The sign in the visitor’s car park will go back-to-back so it can be seen from the roadside as well as in the carpark. An aluminium channel will act as a pole which will then be attached to the back of the chargers high enough to be seen.

Surface

The planed old surface spoil will be removed from site and a waste transfer note submitted to MHRA for the records.

The parking areas are to be constructed using a permeable surface material. The preferred solution is for the use of porous asphalt; an example of this solution can be seen at  http://www.tarmacdry.co.uk/.

The required top surface thickness will be 40mm, please clarify this in your return documentation.

The colour is to closely match the existing surface. If there are suitable, equivalent products that give the same durability and characteristics as the Tarmac product, please cost these separately.

Please provide a breakdown in square meterage for each of the areas shown on the drawing.

New Bays

Carpark 4. Form new car bay markings in the areas shaded GREEN where the old motorcycle area was. This is to allow 3 x car bays. White line markings to suit.

Carpark 6. Remove shrubbery and ground to form 2 new bays in the green shaded areas. Reinstate curbs to align with existing.

Carpark 6. Remove section of carpark at the end of row 6 (highlighted Red) to allow for 2 cars between base steel and curb line. At present only 1 car can park there.

Carpark 7. Remove shrubbery and ground to form 1 new bay as shown in the green shaded areas.

Carpark 7. Please provide a separate cost to extend the carpark by 2 bays (shown in RED) This will include curb stones and re-surfacing, line marking. This extension may not form part of the contract to re-surface.

Carpark 7. Motorcycle bays. Provide equally divided, offset, motorcycle bays highlighted in PURPLE. Please allow for 3 bikes to be parked there.

Carpark Re-surfacing

Main car park bays. These will be completed in turn. A decision will be made by the project team as to which carparks can be started and finished before moving on.  Note due to parking limits we can only accommodate two smaller carparks out at once with carpark 6, 7, 9, having to be done individually. The principal contractor and subsequent sub-contractors must clearly understand this requirement prior to starting.

Note we can have up to 50 car bays out at one time.

At each phase of works completed, new yellow lines will need to be marked both on the roads and around the carpark entrances. This will include all other requested markings and wording.

Phase 1 Top carparks (2,3,4,5,6,7)

All lines, hatchings, parking bays, wording and markings are to be replaced using the hot thermo lining paint, colour to suit purpose. For the Electric Vehicle and Disabled ground logo’s, please submit your example of this within the tender return.

The line marking will consist of:

- Disabled parking logo and hatching. 4 times.

- Disabled parking wording at the front of the bays as per regulations.

- Electric vehicle logos. 9 off in the main carpark and 2 off in the visitors.

- Give way markings coming out of the carparks. 5 times, as per regulations.

- White lines spaced equally, with two shorter white lines at the ends of the steel bases. Car bay width 2400mm spaced equally for standard size.

- Motorcycle markings as shown on the drawing. 2 locations.

- Motorcycle parking wording. To suit the number of bays.

The wording must be the standard “Transport” font, with the size of font to match existing road markings. An example of this must be agreed with the PE, prior to carrying out this part of the works.

Careful allowance must be made around the lightning protection pits as some of these are in the carpark itself.

The carport base steels have a strengthening grout between the steel base and the carpark surface. This grout must be left in situ and be carefully cut round, with the new asphalt surface meeting the cut line.

Phase 2 Lower carparks (8, 9,11,12, visitors)

All lines, hatchings, parking bays, wording and markings are to be replaced using the hot thermo lining paint, colour to suit purpose.

The line marking will consist of:

- Give way markings exiting the carparks. 5 times, as per regulations.

- White lines spaced equally. Car bay width 2400mm for standard size.

- Car parks 11 & 12, please allow a separate cost to replace the timber sleeper curbs with standard concrete curbs stones. These will be pointed out on the site visit. Marked in a red line on the drawing.

East Road

The main drag North to South will need to be planed and resurfaced. This resurfacing will include the section in front of reception, the small lower car park spaces and the hammerhead at FCS & canteen back doors.

In the FCS & Stores courtyard, a yellow box hatching is required with the words “No Parking” marked on the ground, the wording will be inside the hatching.

FCS courtyard- The new road surface will need to be formed around the gas cylinders that are located there.

Between FCS/Reception and carpark 8, a zebra crossing will need to be marked (please see drawing).

The island at reception will have “Give Way” markings on both exits.

East road. Please detail how you will manage planing, resurfacing plant movements during working hours especially between the busy hours of 8 to 10am and 3 to 5pm. Detail where plant and other machinery will be sited when not in use.

Please provide a separate cost to re-surface the paths on this section of site, highlighted on the drawing.

West Road

Parts of the road surface has become cracked; this could be caused by poor foundation bearing in mind that the under surface is mainly clay.

The West side road has become severely cracked due to this and will need to have a new substrate foundation laid. The worst section is between the dispatch delivery road and the cold room (North to South). Please allow for test pits and surveys to be carried out. We will be expecting a compacted sub base with a minimum 300mm reinforced concrete slab prior to surfacing. Roads plates to suit length of excavations. Please detail your proposal accurately.

All Drainage

The car parks are to fall towards the surface drains where possible and the wearing surface must not encourage rainwater to collect, but to flow to the nearest drain.  It is recognised that it is not possible to ensure absolute flatness using asphalt as a medium; however, the contractor must ensure that the car parks do not allow the collection of any rainwater.  Should this not be the case, the contractor will be requested to return and rectify the problem.  If the ‘full filtration’ option is not adequate, then the contractor is to design the drainage system to remove storm / rainwater from the car park and deliver the water to the existing storm water drainage.  Drawing attached shows the drains in the area.

The surface must have a slight fall towards the surface drains and must not form large puddles.

All manholes, gulleys and chambers are to be reset.

Drains must be covered prior to works and any material that enters removed after resurfacing. Any spoil must be removed prior to hand back of that section. Should the drain be blocked, the contractor is responsible to have it cleared at their expense.

Permit to Work

The Agency operates a permit-to-work system these include the following as required by the work: -

- General Permit to work

- Excavation Permit

All permits will be issued as necessary by authorised staff. No work is to commence without the possession of the relevant permit to work. These must be returned to the issuer on completion of the work for filing.

The Supplier must ensure that his operations do not pose any risk to the Agency personnel or visitors to the site.

Waste removal and recycling

The Supplier will be responsible for planing off the existing surface and the removal of the old tarmac, concrete surface from site and disposed of in accordance with the following:

All waste materials are the responsibility of the Supplier. All waste associated with the project will be disposed of in an environmentally acceptable way and in compliance with the Duty of Care under the Environmental Protection Act 1990.  All disposed waste from The Agency must have a certificate stating that the waste items have been transferred to an authorised waste handler and disposed of accordingly.  This certificate must be handed to the Agency for our records.

A Supplier’s skip can be located in the local area with agreement of the Agency but must be maintained so no rubbish is left around it or be blown around by the wind.

CDM

This project will be run alongside CDM regulations. Our appointed CDM supervisor is :

Calfordseaden.

Joshua Hutchins...

Health & Safety Consultant jhutchins@calfordseaden.com

for and on behalf of calfordseaden health and safety:.

St John's House, 1A Knoll Rise, Orpington, Kent, BR6 0JX...

Office:.; 01689 888222...; Mobile:.; 07525 812154....

Asbestos

An Asbestos Register for the site was compiled in 2003/2004, 2009 and updated 2010. This register is available on request. It should be noted that the asbestos survey may not have identified all the asbestos containing materials on site and the Supplier’s personnel must have received asbestos awareness training and proceed with caution.

There is no anticipated exposure to asbestos because of these works.

Site setup Information

A full survey is to be undertaken to establish and verify actual site dimensions, prior to further design work and the subsequent construction phase.

Site Management

Please allow for a site contact/foreman to be available onsite throughout the duration of all the works.

Please detail the credentials of the qualified foreman (SMSTS etc) you have allowed to oversee the day-to-day operations.

Welfare facilities

The site has toilets, power and water which will be provided to the Supplier with the Agency's permission. The site also has a staff restaurant that the Suppliers staff may use subject to persons being properly dressed and in clean and tidy clothing. We have first aiders on site, but we expect each main supplier to supply a first aider as part of their team.

Storage of materials and tools

The Supplier is to allow for onsite storage or any site setup if this is required.

The site is very limited to external space and therefore any storage requirements (including all vehicles, material, tools and equipment) must be agreed in advance of the works starting.

Any damage to any part of the site caused during the undertaking of works by the Supplier must be made good by the Supplier, at their expense, before handover is accepted. Any such damage will need to be repaired to the satisfaction of the Agency. This includes, but is not limited to.

- Roads & Paths

- Curbs

- Shrubbery

- Cables

- Vehicles

- Structures including solar panel steel carports.

Design Change Post Contract Award

Any change to this specification after the tender has been received and the contract awarded will be controlled using the Agency’s Design Change Control form that is signed by both parties as detailed on the form. The form will identify the change and its effect on costs and timescales.

An example of this form can be found in the attachments (Design Change Control Form).

Confidentiality

Suppliers are expected to keep any information about the work of the Agency or staff details totally confidential.

O & M’s

Electronic O and M manuals will be provided at the end of the works listing manufacturer’s literature and product details.

Confidence in your tender return

Please make sure that what you have quoted against can be achieved and you have made allowance for each and every part of the specification and the subsequent parts of it. If you are unsure, please check with the Agency or your suppliers. If you wish to re-visit site to confirm or check any clarifications, we would be happy to accommodate as long as it is within the tender window. All correspondence can be carried out via Atamis e-tendering portal.

Price Breakdown

Please provide a detailed price breakdown of each of the car parks and roads, shown on the drawing layout, through the Atamis commercial envelope. When returning the commercial costs, please breakdown all the costs associated for each carpark and each section of road. These costs must be fully installed total and not per square meter.