A34	SECURITY/SAFETY/PROTECTION

GENERALLY

A34/110	THE PRE-TENDER HEALTH AND SAFETY PLAN is integral with the project Preliminaries, including but not restricted to the sections set out below.

Nature of the project: Sections A10 and A13.

The existing environment: Section A12.

Existing drawings: Sections A11 and A12.

The design: Section A34.

Construction materials: Section A34.

Site-wide elements: Section A12.

Overlap with client's undertaking: Sections A34 and A36.

Site Rules: Sections A34 and A35.

Continuing liaison: Section A31.

A34/114	CONSTRUCTION HAZARDS arising from the design of the project include those identified below. Common hazards which should be controlled by good management and site practice are not listed.

-	Hazard: Refer to the risk assessments carried out by each designer noted on drawings

Precautions assumed: refer to residual risk register

Specification reference(s): refer to each trade section of the specification for specific comments

-

A34/117	HEALTH HAZARDS:

-	Hazardous substances: Site personnel levels must not exceed occupational exposure standards and maximum exposure limits stated in HSE document EH40/ 2005 Occupational Exposure Limits as recommended in the HSE document. Follow guidance in HSE document G409 for any monitoring and air sampling as required.

-	Health hazards arising from specified construction materials include those identified below:

-	Common hazards, which should be controlled by good management and site practice, are not listed.

-	Hazard: Fire & skin irritation

Material: Use of Lime

Specification reference(s): Refer to each specification section for comments

-	Hazard: Fire & inhalation / skin irritation

Material: Resins / bonding agents / liquid chemicals

Specification reference(s): Refer to each specification section for comments

- Hazard: Inhalation / touch

Material: Asbestos – All identified asbestos has been removed with the exception of existing panel below sink in Kitchenette. Should any other asbestos be identified works in that immediate area are to stop and the CA notified.

Specification reference(s): Refer to the documents issued by Asbestos Contractor

A34/120	THE CONSTRUCTION PHASE HEALTH AND SAFETY PLAN, developed from the Outline Construction Phase Health and Safety Plan (see section A30) must be submitted to the CA not less than one week before the proposed date for start of construction work. Do not start construction work until the Employer has confirmed in writing that the Construction Phase Health and Safety Plan includes the procedures and arrangements required by CDM Regulation 15(4).

A34/125	HSE APPROVED CODES OF PRACTICE: Comply with the following:

-	Management of health and safety at work.

-	Managing construction for health and safety.

A34/130	SECURITY: Adequately safeguard the site, the Works, products, materials, plant, and any existing buildings affected by the works from damage and theft. Take all reasonable precautions to prevent unauthorised access to the site, the Works and adjoining properly.

A34/140	STABILITY: Accept responsibility for the stability and structural integrity of the works during the Contract, and support as necessary. Prevent overloading: details of design loads may be obtained from CA.

A34/150	OCCUPIED PREMISES:

-	Existing buildings will be occupied and/or used during the Contract as follows:

NONE

A34/160	PASSES will be required for access to Buildings on site.

Submit to the CA a list of the names of all persons requiring passes together with any other related information the CA may reasonably require. Passes must be returned to the CA when required and in any case on completion of the work to which the pass relates.

A34/168	MOBILE TELEPHONES must not be switched on or used in the following area(s):

A34/170	EMPLOYER'S REPRESENTATIVES SITE VISITS:

Inform the CA in advance of all safety provisions and procedures (including those relating to materials which may be deleterious) which will require the compliance of the Employer or the Employer's representatives when visiting the site. Provide protective clothing and/or equipment for the Employer and the Employer's representatives as appropriate.

A34/180	WORK IN HAZARDOUS AREAS: Operatives must take the following precautions when working in the area(s) listed below:

-	Work area: Basement due to lack of ventilation

Precautions: See previous comments regarding asbestos

-	Work area: Existing room no. B02,03,04 & 05

Precautions: Wear protective masks

-	Work area: High level installing smoke vent and works to flat roof over wc block

Precautions: Contractor to provide safety measures as required (scaffolding/fall arrest)

A34/185	PERMIT to work procedures will operate for all work carried out in the following area(s): No permits are required to work in any specific areas. Noise related work at street elevations (adjacent the residential /commercial boundary) are to be limited in duration as much as possible and within the working hours agreed.

PROTECT AGAINST THE FOLLOWING:

A34/220	NOISE: The Local Authority has granted a consent under Part III of the Control of Pollution Act 1974 relating to the Works and laying down the following condition(s):

Refer to the Planning and Listed Building consent which are issued as part of the tender information

A34/221	NOISE:

-	Comply generally with the recommendations of BS 5228:Part 1, clause 9.3 for minimising noise levels during the excavation of the works.

-	Noise levels from the works are to be kept below recommended dB(A) levels when measured from neighbouring property boundary

-	Fit all compressors, percussion tools and vehicles with effective silencers of a type recommended by manufacturers of the compressors, tools or vehicles.

-	Do not use pneumatic drills and other noisy appliances during working hours without consent of the CA.

-	Do not use or permit employees to use radios or other audio equipment in ways or at times which may cause nuisance.

A34/230	POLLUTION: Take all reasonable precautions to prevent pollution of the site, the Works and the general environment including streams and waterways. If pollution occurs inform the appropriate Authorities and the CA without delay and provide them with all relevant information.

A34/235	USE OF PESTICIDES:

-	Use only where specified or approved, and then only suitable products as listed in the UK Pesticide Guide.

-	Where work is near water, drainage ditches or land drains, comply with the MAFF guidelines for the use of herbicides on weeds in or near water courses and lakes'.

-	Observe all precautions recommended by the manufacturer and remove containers from site immediately they have been emptied or are no longer required.

-	Operatives must hold a BASIS Certificate of Competence, or work under the supervision of a Certificate holder.

A34/236	USE OF PESTICIDES will not be permitted.

A34/240	NUISANCE: Take all necessary precautions to prevent nuisance from smoke, dust, rubbish, vermin and other causes.

A34/250	ASBESTOS BASED MATERIALS: Report immediately to the CA any suspected asbestos based materials discovered during demolition/refurbishment work. Avoid disturbing such materials. Agree with the CA methods for safe removal or encapsulation.

A34/260	FIRE PREVENTION: Take all necessary precautions to prevent personal injury, death, and damage to the Works or other property from fire. Comply with Joint Code of Practice 'Fire Prevention on Construction Sites' published by the Building Employers Confederation and the Loss Prevention Council.

A34/263	FIRE PREVENTION: Smoking will not be permitted on the site except in designated areas which must be carefully controlled, equipped with fire fighting equipment and receptacles for the safe disposal of smokers materials and inspected to guard against risk of fire.

A34/265	BURNING ON SITE of materials arising from the work will not be permitted.

A34/266	BURNING ON SITE of materials arising from the works will be permitted subject to compliance with relevant regulations. Ensure that fires:

-	Do not contain plastics, flammable liquids or other hazardous materials likely to generate dark or toxic smoke or toxic residues.

-	Are not situated within 10 metres of any tree canopy, shrubs, buildings, structures, combustible materials, etc or a public highway.

-	Are not lit if there is a danger of sparks reaching adjacent flammable areas, e.g. heathland, forests or ripe cereal crops.

-	Do not produce drifting smoke where it may be hazardous, e.g. across highways.

-	Will not interfere with amenity enjoyment of a site.

-	Are never left unattended.

-	Are extinguished at the end of each working day.

A34/270	WATER: Prevent damage from storm and surface water. (Items for keeping the site and excavations free of water are given elsewhere).

A34/280	MOISTURE: Prevent the work from becoming wet or damp where this may cause damage. Dry out the Works thoroughly. Control the drying out and humidity of the Works and the application of heat to prevent:

-	Blistering and failure of adhesion.

-	Damage due to trapped moisture.

-	Excessive movement.

A34/285	INFECTED TIMBER: Where instructed to remove timber affected by fungal/insect attack from the building, do so in a way which will minimise the risk of infecting other parts of the building.

A34/290	WASTE:

-	Remove rubbish, debris, surplus material and spoil regularly and keep the site and Works clean and tidy.

-	Remove all rubbish, dirt and residues from voids and cavities in the construction before closing in.

-	Ensure that non-hazardous material is disposed of at a tip approved by a Waste Regulation Authority.

-	Remove all surplus hazardous materials and their containers regularly for disposal off site in a safe and competent manner as approved by a Waste Regulation Authority and in accordance with relevant regulations.

-	Retain waste transfer documentation on site.

A34/320	MILLENNIUM COMPLIANCE: Certify that equipment containing computer chips complies with PD 2000-1: 'A definition of Year 2000 conformity requirements' published by BSI.

PROTECT THE FOLLOWING:

A34/410	WORK IN ALL SECTIONS: Adequately protect all types of work and all parts of the Works, including work carried out by others, throughout the Contract. Wherever work is of an especially vulnerable nature or is exposed to abnormal risks provide special protection to ensure that damage does not occur.

A34/420	EXISTING SERVICES:

-	Notify all service authorities and/or adjacent owners of the proposed works not less than one week before commencing site operations.

-	Before starting work check and mark positions of existing mains/ services. Where positions are not shown on drawings obtain relevant details from service authorities or other owners.

-	Observe service authority's recommendations for work adjacent to existing services.

-	Adequately protect, and prevent damage to all services. Do not interfere with their operation without consent of the service authorities or other owners.

-	Identify below ground services with signboards, giving type and depth, and overhead services with headroom markers.

-	If any damage to services results from the execution of the Works, immediately notify CA and appropriate service authority. Make arrangements for the work to be made good without delay to the satisfaction of the service authority or other owner as appropriate. Any measures taken by the CA to deal with an emergency will not affect the extent of the Contractor's liability.

-	Replace marker tapes or protective covers disturbed during site operations to the service authority's recommendations.

A34/430	ROADS AND FOOTPATHS: Adequately maintain roads and footpaths within and adjacent to the site and keep clear of mud and debris. Any damage to roads and footpaths caused by site traffic or otherwise consequent upon the Works must be made good to the satisfaction of the Local Authority or other owner. Bear any costs arising.

A34/435	EXISTING TOPSOIL/SUBSOIL: Before starting work, submit to the CA, proposals for protecting existing topsoil and subsoil from overcompaction in those areas which may be damaged by construction traffic, parking of vehicles, temporary site accommodation or storage of materials and which will require reinstatement prior to completion of the Works.

A34/440	RETAINED TREES/HEDGES/SHRUBS/GRASSED AREAS:

-	Adequately protect and preserve, except those which are to be removed.

-	Replace to approval or treat as instructed any species or areas damaged or removed without approval.

-	Mature trees and shrubs which, due to the Contractor's negligence, are uprooted, destroyed, or in the opinion of the CA, damaged beyond reasonable chance of survival in their original shape, must be replaced with those of a similar type and age at the Contractor's expense.

A34/445	TREES TO BE RETAINED: Unless agreed otherwise by the CA, do not:

-	Dump spoil or rubbish, excavate or disturb topsoil, park vehicles or plant, store materials or place temporary accommodation within the branch spread.

-	Sever roots exceeding 25 mm in diameter.

-	Change level of ground within an area 3 m beyond the branch spread.

A34/450	EXISTING FEATURES: Prevent damage to existing buildings, fences, gates, walls, roads, paved areas and other site features which are to remain in position during the execution of the Works.

A34/460	EXISTING WORK: Prevent damage to existing property undergoing alteration or extension and make good to match existing any defects so caused. Remove existing work the minimum necessary and with care to reduce the amount of making good to a minimum.

A34/465	BUILDING INTERIORS: Protect building interiors exposed to weather during the course of alteration work with temporary enclosures of sufficient size to permit execution of the work and which will remain weathertight in severe weather.

A34/470	EXISTING FURNITURE, FITTINGS AND EQUIPMENT: Prevent damage to any furniture, fittings or equipment left in the existing property. Move as necessary to enable the Works to be executed, cover and protect as necessary and replace in original positions. The following items will be removed by the Employer before work in each room starts:

Room		Items to be removed by Employer - NONE

All rooms	Existing doors set aside

A34/475	ESPECIALLY VALUABLE/VULNERABLE ITEM(S): Ensure the provision and maintenance of special protective measures to prevent damage to the following:

-	Item: Stairs

Method of protection: Ply facings to protect throughout the course of works

-	Item: windows generally

Method of protection: Ply facings as required

-	Item: original ceilings & joinery/doors, balustrade to main concert hall

Method of protection: Plywood protection as required, especially during demolitions

A34/481	ADJOINING PROPERTY: Prevent trespass of workpeople. Take all reasonable precautions to prevent damage to adjoining property. Obtain permission as necessary from the owners if requiring to erect scaffolding on or otherwise use adjoining property, and pay all charges. Remove and make good on completion

A34/490	EXISTING STRUCTURES:

-	Check proposed methods of work for effects on adjacent structures inside and outside the site boundary.

-	Provide and maintain during the execution of the Works all incidental shoring, strutting, needling and other supports as may be necessary to preserve the stability of existing structures on the site or adjoining, that may be endangered or affected by the Works.

-	Support existing structure as necessary during cutting of new openings or replacement of structural parts.

-	Monitor adjacent structures and immediately report excessive movement to the CA.

-	Do not remove supports until new work is strong enough to support the existing structure. Prevent overstressing of completed work when removing supports.