TENDER DOCUMENT FOR Wroxham Parish Cemetery

Extension - Phase One

Supporting the attached plan.

Background. Cemetery location o/s location 177.297

The planned site is 36.57mtr x 45.72mtr flat, sloping to the south-westerly corner.   The site was previously ploughed agricultural land which has been cleared, fenced with a provisional entrance to the south east corner of the proposed site. The ground has been allowed to settling since September 2019 with minimal disturbance.

Planning consent has been awarded by Broadland District Council (BDC) ref 20191872 and Broads Authority (BA) ref BA/2020/0017/CU. Environmental Agency (EA) has also given approved for the site.

Heavy vehicles will not be allowed entry to the site through the existing Cemetery.  There is a direct access to the new site via an entry to the south east corner elevation of the extension. Currently the entry point from Noble Crescent is across agricultural land and therefore upon completion of work the land to be reinstated back to farming land, with any damage making good.

Phase one specification (In two parts)

Part One:

Create roadway as shown on plan.

- From approved planning drawings mark out the roadway as shown to, 3.65mtrs x 16mtrs.The new road to join to the east with the present roadway through the existing Cemetery.

- The roadway to be dug out to a depth of 200mm and spoil to be redistribute evenly to the lower south westerly corner.

- Installation of concrete pre-cast edgings to set in concrete flush to allow drainage with a pea shingle edge. The roadway to have a camber to allow water run-off.

- Lay Teram matting with 150mm of Type 1 granite and compact.

- Surface with 50mm of stone mastic asphalt and roll.  Include a crown in the cross section to assist surface drainage.

- Allow suitable settlement time at each stage as appropriate for the ground conditions.

- Avoid heavy vehicle activities on the completed roadway.

- Remove soil and debris in existing concrete spoil area, and spread soil to the southwest corner of the cemetery extension. All unwanted debris to be removed and environmentally disposed of.

- Remove concrete structure, disposing of concrete posts, saving concrete panels for further use.

- Construct new concrete spoil area in the northeast corner of the extension using new concrete pillars and existing concrete panels to an approximate size of 5.482 mtrs leaving a 2mtr access point.

Note: At all stages marked layouts to be agreed by Wroxham Parish Council on site prior to final work commencing.

Part two:

Upon completion of part one, turn and level all ground surrounding road and plant general grass seed

Prepare the ground for seed planting, ensuring there are no compacted areas and the ground is even.

- Spread seed across the total field area.

- Avoid heavy vehicle activities on the completed roadway and seeded areas.

Make good any damaged areas to the adjacent field, from road entry to extension access point.

The area is accessed through a residential area and therefore due care and consideration is required at all times for undue noise, road clearing, and traffic activities. All road debris to be cleared on a daily basis.

2nd June 2020

PJM/ten 4