Annex A to

700978368

STATEMENT OF REQUIREMENT

Purpose

- This document states the requirements to be met by a supplier to provide 5 x Ultrasonic Cleaning Systems (UCS) for HQ 12 AI Bde, Bulford Camp. It provides a background to the product description and specifications required from any potential supplier.

Background

- UCS has been used in limited trials within the Army but has only been done in light role organisations. As an Armoured Infantry Formation,12 AI Bde has a wide variety of both small, medium, and large calibre weapon systems, all of which are fired regularly to maintain competency. Additionally, equipped with CVRT, Warrior, and CR2 there are many parts of these vehicles that require regular servicing. While the key activity is to maintain Combat Power, vast numbers of man hours are used cleaning manually. This occurs as part of routine equipment care on returning from exercises.

Product Description

- The UCS should include the following features:

- Built with a heavy duty 316 stainless steel tank with stainless steel lid and basket.

- Digital control for time and temperature – Allows the user to set both the fluid temperature and duration of the cleaning cycle

- Fluid level protection

- Lockable drain valve

- Safety Thermal Cut Off

- Plug Type G – British 3-pin

- 3Yr Warranty

Product Specifications

| Description | Specification |
| --- | --- |
| Internal Dimensions (L x W x H)(Min-Max) mm | 1000-1500 x 300-500 x 300-500 |
| Basket Dimensions (L x W x H)(Min-Max) mm | 950-1450 x 250-450 x 250-450 |
| Maximum Tank Capacity (Litres) | 90 |
| Ultrasonic Power (Watts) | 1kw |
| Heating Power (Watts) | 2kw |
| Operating Frequency (kHz) | 30khz |
| Timer | Yes - digital |
| Temperature Range | 0-80 deg c |
| Main input/ frequency | 220v 50/60 htz |
| Drain facility | Yes ¾”BSP |