SPECIFICATION OF WORKMANSHIP AND MATERIALS

CONTENTS

GENERAL	3

GLAZING	8

# GENERAL

GENERAL

Applicability

001	This initial general section applies to all subsequent sections of this Specification of Workmanship and Materials (“this Specification”).

002	This Specification is drafted as a series of instructions that the Service Provider must ensure are complied with in relation to the Works.  Each instruction includes all tasks necessary to comply fully with the instruction and the Schedule of Rates item(s) to which it relates.

003	The Schedule of Rates payments, as adjusted by the Service Provider’s tendered Rates, include for carrying out all tasks required by this Specification.  No further payment is due to the Service Provider in respect of any such tasks beyond the payments provided for in the Schedule of Rates.

004	Specifications across a number of trades may be relevant to each Schedule of Rates item.  The Service Provider must comply with all requirements of this Specification applicable to the specific type of Works to be undertaken.

005	References to Paragraphs and Sections in this Specification are to the applicable Paragraph and Section of this Specification. If any contradiction appears within the Specification sections, Schedules of Rates, the Client’s Policy documents etc., the most rigorous standard takes precedent.

Standards of workmanship and Materials

006	Carry out and complete all Works:

in accordance with Good Industry Practice;

in accordance with statutory Regulations;

in accordance with the Client’s Policies;

in accordance with the Client’s Codes of Practice;

in accordance with any specific requirements for those Works in this Specification; and

to the satisfaction of the Client (acting reasonably).

007	To the extent that the standard of any Works has not been specified in this Contract, agree the relevant standard for the Works with the Client before their execution.  Where particular Works or working methods are to be “Approved by” “Agreed with” or are indicated to be “subject to the Approval of” the Client, give the Client adequate notice when such Approval or Agreement is needed and retain evidence of all Approvals given, and items that have been Agreed, by the Client.

008	To the extent that it is necessary to design any aspect of the Works, in preparing those Designs use the reasonable skill and care to be expected of an experienced maintenance Service Provider that is skilled in undertaking works similar to the Works.

009	Maintain all existing lines and levels at all times and carry through new work to the same lines and levels unless otherwise instructed by the Client.

European and British Standards & Codes of Practice

010	Ensure all Works undertaken and all Materials used in those Works comply with all applicable International, European and/or British Standards and Codes of Practice and the Client’s Codes of Practice that are current at the time of their use.

011	References in this Specification of Workmanship and Materials to any International, European and/or British Standard or Code of Practice are to be construed as references to the version current at the time the Order is undertaken.

012	Where a specific International, European and/or British Standard or a Code of Practice is referred to, this sets out the minimum acceptable standard of Materials or workmanship.

013	Any requirement in this Specification of Workmanship and Materials to use material or an article which is defined by reference to a specified Quality Assurance Scheme, Agreement Certificate, British Standard Specification or other approval, may be met by a material or article which has received equivalent approval in another Member State of the European Union or an equivalent international standard recognised but not yet adopted in the UK.

014	A Service Provider offering a product on the basis of compliance with any such approval shall notify the Client of all such substitutions in advance of placing any order and will be required to provide, in English, technical or other details of the approval and its qualifying tests.

Materials

015	The Client wishes to standardise the use of Materials across its Properties.  This is in order to simplify parts requirements and van stock loads, to improve its repairs processes and to reduce maintenance costs.  Wherever possible, match all Materials used to materials currently used in the Properties, particularly in terms of their parts requirements and repair procedures.  In this Specification the Client has set out details of its current Materials to which the Service Provider is required to standardise.

016	Where this Specification indicates that Materials are to be “Approved by the Client”, provide samples of the proposed Materials to the Client for Approval.  Any Materials that comply with the functionality and compatibility (including aesthetic compatibility) requirements of this Specification may be proposed.  No further Approval is required for any Materials listed in this Specification as being the Client’s currently used Materials.  The purpose of the Client’s decision on the use and Approval of such Materials is to ensure that they meet the Client’s requirements for functionality and compatibility.  The decision of the Client on this is final. No future precedence will be set by individual Client representatives, with exception to those authorised to do so within the Clients Headquarters. A ‘common sense’ and ‘best practice’ approach it is the recommended method of approval.

017	Where this Specification requires Materials to be matched to existing Materials or finishes, this match is subject to the Approval of the Client.

018	Do not use any Prohibited Materials in carrying out the Works.  Prohibited Materials are those Materials which are generally accepted or (having regard to Good Industry Practice) are reasonably suspected of:

being harmful in themselves;

being harmful when used in a particular situation or in combination with other Materials;

becoming harmful with the passage of time; or

being damaged by or causing damage to the structure in which they are to be affixed.

019	Materials are to be regarded as harmful if, in the context of their use in the Works (whether alone or in combination with other materials) they:

are prejudicial to health and safety;

may pose a threat to the structural stability or the physical integrity of any Property; or

could materially reduce the normal life expectancy of any part of the Property.

020	Sustainable Timber: In compliance with Public Procurement Policy, all timber and wood derived products referred to throughout this document and which are supplied to the Client, or used by the Service Provider, his agents and Sub-contractorin the performance of any contract to which this document relates, must be procured in accordance with the latest edition of the European Union Timber Regulation (EUTR)

021	CE Marked Products: In compliance with Construction Products Regulation, all products referred to throughout this document and supplied to the Client, or used by the Service Provider, his agents and Sub-contractorin the performance of the works, must be supplied with a Declaration of Performance (DoP) and carry the CE marking (European Conformity marking).

Performance Standards on the CE Mark must comply with relevant Building Regulations as required.

The CE Mark must be fixed visibly, legibly and indelibly either to the product or to a label attached to the product. If this is not possible or not warranted, then it must be fixed to the packaging or within the accompanying documentation.

The DoP must be made available by the manufacture for 10 years after the product was first placed on the EEA market (this may be via a website)

022	Use, fix and apply all Materials strictly in accordance with the manufacturer’s recommendations, directions or instructions.

023	Participate in joint initiatives with the Client and other Service Providers to establish supply chain agreements.

024	Where appropriate suggest (economically viable) amendments to this Specification where those amendments may lead to an improvement in environmental performance or sustainability.

025	At the Client’s request provide all information the Client reasonably requests regarding the environmental impact of the supply and use of any Materials and goods the Service Provider selects for use in the Works.

026	[optional clause] In the event that the Service Provider considers that decanting elderly, vulnerable, people with disabilities and other occupiers and carers from a dwelling whilst intrusive works or disruption to washing and sanitary facilities are required to be undertaken, the Service Provider may be required at his own cost to provide the following facilities:

Decant Mobile - Daytime Decant

Temporary Accommodation shall conform to the requirements of BS EN 1645-1:2012, BS 1646-1:2012 and BS EN 1647:2012

If it is agreed, prior to a scheme start, that daytime facilities are required this may either be a Touring caravan used outside homes between 9am and 5pm and then removed, or a mobile unit located in a fixed position supplied with at least the following.

- External door

- Bedroom

- A toilet compartment with wc suite, wash-handbasin and shower unit

- A flued gas fire/electric heater (note: gas is the preferred option )

- A flued gas fire multi-point water heater or electric water heater.

- Electrical installation to BS 7671:2008+A1:2011

- Mattresses with fireproof removable covers (which shall be thoroughly cleaned and changed after each decant)

- A cooking appliance and fridge

- Warning notice for health and safety advice to users

- Fire blanket (to be located by the cooking appliance)

- 1kg-powder fire extinguisher (to be located by the main door)

- Smoke Detector

- Carbon Monoxide Detector

- User’s handbook (to be used by Service Provider when demonstrating the mobile to new occupants).

The Service Provider shall also supply the following:

- A security cabinet for two 13kg bottles of propane gas if gas is to be used (red gas bottle).

- Entrance steps, handrails, level access ramp (maximum 1:12) to be provided for people with disabilities to the satisfaction of the Client.

- Water supply

- Mains sewerage connection

Daytime decanting shall have agreement between the tenant, the Client and the Service Provider on the hours required for the daytime facility, its location and siting. The provision of car parking by Customers of adjoining dwellings must be considered in all circumstances. The form of provision may vary in different situations.

The daytime facility is not required to provide laundry and storage facilities, telephone connections or television aerials or a dedicated car parking facility.

A chemical toilet to BS 2081-1:1998 and BS 2081-2:1998 may be used where no sewer connection is feasible.

All work during the duration of the contract shall conform to the Health and Safety Legislation, which includes installation and checking and is the responsibility of the Service Provider.

Service Providers should be aware of the Code of Practice for the transportation, siting and commissioning of caravans published by the National Caravan Council (www.thecaravan.net).

The location of the day-time mobile decant facility must conform to Health and Safety standards and be agreed with the Client.

Service checks are to be carried out by suitably qualified personnel after each decant, checks should cover:

- Electrical,

- Gas,

- Water,

- Fire prevention equipment,

- Warning Notices

- Steps and Handrails.

The facility is to be cleaned between each change of user.

The following notice must be displayed in the Temporary Mobile Accommodation.

A permanent warning notice not less than 200mm x 130mm with the heading printed in red is to be fixed in a prominent position.

ADVICE TO OCCUPIERS Ventilation Do not obstruct the ventilators, which are fitted; your safety depends on them. In Case of Fire Get everyone out. Turn off the outside gas valve Raise the alarm and call the Fire Brigade Do not stay behind to put the fire out yourself Do not put yourself at risk Fire Precautions Children - must not be left alone in the caravan. When cooking never leave a cooker unattended Do not use multi-adaptors. If you smoke use metal or glass ashtrays-not plastic. Make sure cigarettes are put out properly Do not smoke in bed. Means of Escape Make sure you know the location and operation of the emergency windows and doors, Keep door and window keys handy. Keep all escape routes clear. If there is smoke, keep low where the air is clearer Do not go back into the caravan. Combustible Materials Keep them clear of all heating and cooking appliances. Fire Fighting Equipment In addition to the 1kg powder fire extinguisher by the main exit door, a fire blanket is provided next to the cooker. Make yourself familiar with the instructions on your fire extinguisher and fire blanket and the fire precautions arrangements on site. Do not stay behind to put the fire out yourself. Do not put yourself at risk. The use of chip pans in mobiles is strictly prohibited.

# GLAZING

GLAZING

MATERIALS

Glass

001	Clear float, obscured pattern and Georgian wired glass shall comply with BS 952-1:1995

002	Laminated safety, toughened glass and polycarbonate sheet shall comply with the requirements of BS EN 12600

003	Laminated safety glass shall comply with and meet the requirements of BS 6206:1981

004	Laminated safety glass shall comply with and meet the requirements of BS EN ISO 12543:2011

005	Panes to be accurately and squarely cut, with clean, undisfigured, free from all specks, bubbles, bladders and all other defects, and having undamaged edges and surfaces, to suit the sizes of the openings. Dimensions of edge cover and clearance, positions and materials of distance pieces, setting and location blocks to be to BS 6262:2005 and glass and sealant manufacturer’s recommendations.

006	For clear float glass, use 'ordinary glazing quality'.

007	For obscure/patterned glass, use clear cast glass either to match the existing glass or of a pattern Approved by the Client.

008	For polished plate glass, use 'glass for glazing quality'.

009	For wired glass, use Georgian wired cast or Georgian wired polished plate glass, as specified.  Ensure the wire extends to the edges of the glass and is free from rust. Cut Georgian wired glass to ensure that any edge is parallel to the alignment of the wires. Care to be taken to ensure that the wires in adjacent panes line up either horizontally or vertically.

Double and triple glazed units

010	Ensure flat dual/hermetically sealed double glazing units are manufactured to the same standards as the other windows within the property and have a minimum 5 (five) year guarantee. Provide details of the guarantee to the Client. Units to comply with BS EN 1279-1:2004, with units to be clearly marked on at least one section, the spacer bar with the BS kitemark or equivalent European standard or compliance with BS EN 1279-5:2005+A2:2010, the manufacturer’s name and number and the date of manufacture, to the month. All units to have argon gas filling.

Secure double glazed units into rebates with double-sided PVC foam closed cell high density security glazing to PAS 24:2012.

Install units in accordance with BS EN 572-2:2012, BS 6262:2005, BS 8000-0:2014, the glass and Glazing Federation Glazing Manual or European equivalents.

Putty

011	For glazing to wood use timber slips and sealants.

012	On timber windows the bottom slip shall be a proprietary drained aluminium bead complete with end caps and spacers.use linseed oil putty or equivalent.

Intumescent mastic

013	Ensure mastic to fire doors/windows is of a type of fire protection sealant Approved by the Client.

Plastic Protection Channels:

014	Preformed proprietary protection channel are to suit particular glass thickness, and fitted securely to bottom edge of glass.

Condensation Channels:

015	Proprietary pre-formed PVC/metal condensation channel are to be complete with stop ends, glazing gaskets and proprietary fixings. Channel to be bedded in recommended sealant to bottom member of window.

Ventilated Glazing Beads:

016	Proprietary pre-formed PVC or aluminium ventilated glazing bead are to be complete with stop ends, glazing gaskets and proprietary fixings and fixed in accordance with manufacturer’s technical data sheet.

Glass Locking System:

017	Proprietary plastic/metal body and clip and fixed in accordance with manufacturer’s technical data sheet.

WORKMANSHIP

Glazing generally

018	Undertake all glazing in accordance with the Glass and Glazing Federation Code of Practice and the current Building Regulations.

019	Accurately cut glass to size with a small clearance. Provide a clearance of 3mm on timber, 5mm on PVC-U windows all round between the edge of the double glazed unit and the frame to permit drainage and ventilation.

020	Ensure glass, except that bedded in patent glazing strip, is bedded back and front and around the perimeter with mastic neatly trimmed and cleaned off.

021	Ensure glazing is sprigged to wood, or fixed with aluminium, timber beads or PVCu beads and security clips or double edged security tape.

022	If gasket glazing is required, ensure the glazing gaskets and weather seals are extruded from EPDM (Ethylene Propylene Diene Monomer).

023	Install obscured glass in single glazing with the "rough" side to the inside of the Property. Install obscure glass to sealed double glazed units with the "rough" side to the inside of the unit and the obscure glass to the inside of the Property.

024	Seal and prime rebates and beads before glass is inserted.

Double glazing units

025	Glazing packers are to in accordance with BPF COP 362 and should not obstruct the drainage with profile specific bridging packers used. Use setting blocks and distance pieces so as to centralise the glass, unit or infill within the opening and ensure that it cannot move in the wind.  Use setting blocks at the bottom edge of the frame. For fixed windows, position them as near the quarter points as possible. Where it is necessary to avoid undue deflection of the frame, place them nearer the sides, but never less than 85mm from the corner. Use setting blocks that are 3mm wider than the glass unit or infill and as thick as the designed edge clearance. Allow for toeing and healing.

026	Use setting blocks that are at least 25mm in length and approximately 25mm in length for each metre of major glass dimension.  For vertically pivoted windows, use setting chocks that are at least 75mm in length.

027	Use location blocks between the edges of the glass unit or infill and at the top and sides of the frame in an opening light.

028	Use distance pieces 25mm long and 3mm less in height than the rebate depth. Ensure the thickness is at least 3mm and such as to ensure that the glass is held firmly in the glazing rebate. Insert rigid PVC shims if necessary, to ensure that the distance pieces are a tight fit between the face of the glass and rebate. For beads which fit into continuous grooves, insert the first distance pieces 75mm from each corner, and the remainder at approximately 30mm centres. For beads fixed by screws or other studs, insert the distance pieces at the fixing points provided.

Neoprene glazing gaskets

029	Fit glass to PVCu windows using glazing gaskets appropriate to the window.  Angle all glazing gaskets if possible, but in any event mitre all corners.

Cleaning:

030	Remove cement and plaster based spillage whilst wet. Remove all smears and excess glazing materials. Leave glazing clean and free from scratches inside and out.

Damage:

031	Replace all glass and fixing materials broken or damaged before Completion, and redecorate.

Client’s current manufacturers/suppliers/products

032	Ensure all Materials are compatible with and standardised to the Client’s current products specified in the table below (listed by manufacturers, suppliers and/or brand names).

| Product | Brand name | Manufacturer’s details |
| --- | --- | --- |

[complete table as appropriate]