Invitation to Tender

Brewing and Canning Equipment

REF: BIGTP150

# Background/Introduction

St.Ives Brewery was founded in 2010 and has steadily grown, experimented and learned a few lessons along the way. Boilers Golden Cornish Ale was our original session ale and remains as popular today as it was back then.

Recently we have consolidated a decade of experience and launched a new core range and have ambitions for further growth that requires new brewing equipment (LOT 1) and canning facilities (LOT 2).

The purchase of the equipment detailed in this tender is part of a grant funded application process and therefore procurement will be subject to grant approval of the project as a whole. We will compare tenders received on a compliance basis only.

# Project Specifications

## LOT 1 Brewing equipment

## The following equipment is required; detailed specifications requirements follow.

| See | Item | Minimum capacity | Quantity |
| --- | --- | --- | --- |
| 2.1.1 | Fermentation tank | 2000L | 3 |
| 2.1.2 | Cold liquor tank | 2000L | 1 |
| 2.1.3 | Glycol water tank | 1000L | 1 |
| 2.1.4 | Glycol water pump | 3T/h | 1 |
| 2.1.5 | Chiller | 5HP | 1 |
| 2.1.6 | Control panel | - | 1 |
| 2.1.7 | Connecting Pipelines | - | As required |
| 2.1.8 | Kegs (S-type spears) | 30L | 100 |
| 2.1.9 | One head keg filling machine (S-type spears) | 20-40 kegs/h | 1 |
| 2.1.10 | One station keg washing machine (S-type spears) | 20-40 kegs/h | 1 |
| 2.1.11 | Candle type diatomite filter | 500L | # 1 |

### 2.1.1 	Fermentation Tanks

| Useful capacity: | Minimum of 2000L |
| --- | --- |
| Outline size: | Maximum diameter 700x3000mm |
| Cooling way: | Dimple jacket plate |
| Cooling media: | Glycol water or alcohol water or tap water |
| Heat exchange area: | No more than 4m2 |
| Inner thickness: | 3.0mm Minimum Stainless Steel 304 |
| External-Jacket thickness: | 2.0mm Minimum Stainless Steel 304 |
| Oval head cone bottom, thickness: | Minimum 2mm. |
| Insulation: | Polyurethane or equivalent |
| Insulation thickness: | Minimum of 70mm |
| Sampling method: | 1 piece of aseptic sampling valve |
| Manhole: | Side manhole minimum 330*430mm |
| Machine polishing accuracy: | Minimum of Ra0.4µm |
| Fermentation tank test pressure: | Maximum 0.5Mpa |
| Fermentation tank working pressure: | Maximum 0.4Mpa |
| Cleaning System: | CIP washing unit or equivalent. |
| Beer outlet: | Raking arms or equivalent |
| Vacuum valve: | Protecting the tank, 0.-0.07MPa |
| Dry hop: | Dry hop adding port which can work in fermentation process |

# 2.1.2	Cold liquor tank

| Gross capacity: | 2250L |
| --- | --- |
| Useful capacity: | 2000L |
| Outline size: | Maximum diameter 1500x3000mm |
| Inner-Jacket thickness: | 3.0mm Minimum Stainless Steel 304 |
| External-Jacket thickness: | 2.0mm Minimum Stainless Steel 304 |
| Dimple jacket plate thickness: | 1.5mm Minimum Stainless Steel 304 |
| Heating way: | Dimple cooling jacket |
| Insulation layer: | Polyurethane or equivalent |
| Insulation thickness: | Minimum of 70mm |
| Diameter of the top manhole: | Minimum of 400mm |
| Temperature way: | Temperature probe PT100 or equivalent temperature transmitting. |
| CIP cleaning device: | Minimum Stainless Steel 304 OR equivalent |

# 2.1.3	Glycol water tank

| Gross capacity: | 1150L |
| --- | --- |
| Useful capacity: | 1000L |
| Outline size: | Diameter maximum1100x2600mm |
| Cooling way: | Dimple jacket plate |
| Cooling media: | Glycol water or alcohol water. |
| Inner thickness: | 3.0mm Minimum Stainless Steel 304 |
| External-Jacket thickness: | 2.0mm Minimum Stainless Steel 304 |
| Oval head cone bottom, thickness: | 4mm. |
| Insulation: | Polyurethane or equivalent |
| Insulation thickness: | Minimum of 70mm |
| Manhole: | Top manhole: Minimum of 500*500mm |
| Cleaning System: | CIP washing unit |

# 2.1.4	Glycol water pump

| Capacity: | Minimum 3m3/h, |
| --- | --- |
| Lift: | Minimum 20m |
| Material: | Minimum Stainless Steel 304 OR equivalent |
| Impeller: | Open impeller, no sanitary dead angle |
| Maximum inlet pressure: | 0.5MPa |
| Temperature range: | Minimum -5C to 100C (EPDM) |

# 2.1.5	Chiller

| Power supply |  |  | 3 PHASE |
| --- | --- | --- | --- |
| Water temperature control |  |  | Minimum (low) -5deg C |
|  |  |  | Minimum (high) +30 deg C |
| Cooling capacity | -5C outlet | KW | Minimum 70 |
|  |  | KCal/hr | Minimum 60000 |
|  | -10C outlet | KW | Minimum 55 |
|  |  | KCal/hr | Minimum 50000 |
|  | Air volume | m3/h | Minimum 35000 |
| Water pump | Rated flow | m3/h | Minimum 15m3/h@ 2.0bar |
|  | Power | kW | Maximum 5.0 |
| Machine Size | L*W*H | mm | Maximum 2500*1500*2500 |
| Net weight |  |  | Maximum 1400Kg |
| Safety Protection 1 | Compressor over current protection |  |  |
| Safety Protection 2 | High/low pressure protection |  |  |
| Safety Protection 3 | Phase sequence/phase missing protection |  |  |
| Safety Protection 4 | Anti freezing protection |  |  |
| Safety Protection 5 | Over temperature protection |  |  |
| Safety Protection 6 | water flow protection |  |  |

# 2.1.6	Control panel

| Material | Stainless steel |
| --- | --- |
| Safety Protection 1 | Circuit breakers |
| Safety Protection 2 | Quick stop button |

# 2.1.7	Connecting Pipelines

# All interconnecting system pipelines are to be included.

# 2.1.8	Kegs (S-type spears)

| VOLUME (L) | 30L |
| --- | --- |
| MATERIAL | Minimum Stainless steel 304 OR equivalent |
| Keg Spear | S-type keg spear |

# 2.1.9	One head keg filling machine (S-type spears)  (compatible with 2.1.8)

| Production capacity | Minimum 25 kegs/h |
| --- | --- |
| Rate of beer waste | Less than 1% |
| Material | Minimum Stainless Steel 304 OR equivalent |
| Kegs size | Range of barrels from 10-60L stainless steel and plastic kegs |

# 2.1.10	One station keg washing machine (S-type spears)(compatible with 2.18)

| Capacity | Minimum 25 kegs/h |
| --- | --- |
| Material of keg | SUS304 |
| Washing scope | 10-60L plastic and stainless steel beer kegs |

# 2.1.11	Candle type diatomite filter

| Filtering capacity | Minimum 500L per hour |
| --- | --- |

## LOT 2 Canning Line

| See | Item | Minimum capacity | Quantity |
| --- | --- | --- | --- |
| 2.2.1 | Can filling and sealing Line | 700 – 1000 CPH | 1 |
| 2.2.2 | Rotary Labelling Line and Date Coder | 800-1600 CPH | 1 |
| 2.2,3 | Collecting table | - | 1 |

## 2.2.1	Can Filling & Sealing Line

| Required capacity | Minimum 700-1000CPH |
| --- | --- |
| Diameter of Can Height of Can | Minimum 50mm Minimum 70mm |
| Material | Minimum Stainless Steel 304 OR equivalent |
| Control System | Speed can be adjusted and set automatically. Common faults to have automatic alarming to show the fault place and reason and then automatically decides if the main engine can continue the operation or stop according to the faults. |
| Footprint | 1500 w x 800 D x 2000H Max footprint. |

## 2.2.2	Rotary Labelling Line and Date Coder

| Labeling usage |  | Cans, Bottles |
| --- | --- | --- |
| Material |  | Minimum Stainless Steel 304 OR equivalent |
| Control System |  | Speed can be adjusted and set automatically. Common faults to have automatic alarming to show the fault place and reason and then automatically decides if the main engine can continue the operation or stop according to the faults. |
| Date Coder |  | Date, Time, Gyle printed coding |
| Applicable containers |  | 500ml can, 440ml Can, 330ml Can, Bottles |
| Power / 240V |  | Pneumatic / 3 pin 240V |
| Capacity |  | 800-1000 CPH |
| Suitable can diameter |  | Minimum range 55-65mm |
| Compressed air | Pressure(Mpa) | 0.3MPa |
|  | Air consumption(Nm3/h) | 0.3m3/min |

## 2.2.3	Collecting table

## To be compatible with 2.2.1 and 2.2.2

## Factory Acceptance Test Brewing Equipment

## The tenderer should include the costs of setting to work the equipment and for a full factory acceptance test.  Costs to be included in the assessment of this tender will be for the labour with any expenses being invoiced separately at cost.

# 2.4 Factory Acceptance Canning Line

## The tenderer should include the costs of setting to work the equipment and for a full factory acceptance test.  Costs to be included in the assessment of this tender will be for the labour with any expenses being invoiced separately at cost.

## Delivery of Brewing

## The costs should be Ex-works.  However, non-UK suppliers are to provide CIF costs which will be included in the overall costs.

## Delivery of Canning Line

## The costs should be Ex-works.  However, non-UK suppliers are to provide CIF costs which will be included in the overall costs.

## The tenderer may bid for either LOT 1 comprising 2.1.1 - 2.1.11; 2.3 and 2.5 or LOT 2 comprising 2.2.1 – 2.2.3; 2.4 and 2.6 or both.

# ITT Timetable

The anticipated timetable for submission of the tender and commission milestones are set out below:

| Activity | Date |
| --- | --- |
| Date ITT available on Contracts Finder | 3 August 2021 |
| Last date for raising queries | 9 August 2021 |
| Last date for clarifications to queries | 10 August 2021 |
| Deadline to return ITT | 19 August 2021 |
| Evaluation of ITT | 20 August 2021 |
| Award of Contract | This is subject to successfully obtaining grant funding and will normally be no later than 90 days from contract evaluation |

# Conflicts of Interest

Please provide a statement with regards to a conflict of interest for this procurement through the provision of either:-

A Declaration that to your knowledge there is no conflict of interest between your company and the St Ives Brewery Limited that is likely to influence the outcome of this procurement either directly or indirectly through financial, economic or other personal interest which might be perceived to compromise their impartiality and independence in the contexts of this procurement procedure.

Or

A Declaration that there is a likely conflict of interest between your company and St Ives Brewery Limited that is likely to influence the outcome of this procurement either directly or indirectly through financial, economic or other personal interest which might be perceived to compromise their impartiality and independence in the contexts of this procurement procedure, please provide details of this connection.

This will permit St Ives Brewery Limited, that in the event of a conflict of interest, appropriate steps are taken to ensure that the evaluation of any submission will be undertaken by an independent and impartial party.

Exclusion

St Ives Brewery Limited shall exclude applicants from participation in this procurement procedure where they have established or are otherwise aware that the applicant, to include administrative, management or supervisory staff that have powers of representation, decision or control of the applicants company, has been the subject of a conviction by final judgment of one of the following reasons:-

Participation in a criminal organisation

Corruption

Fraud

Terrorist offences or offences linked to terrorist activities

Money laundering or terrorist financing

Child labour and other forms of trafficking in human beings

# Consortium or sub-contracting

Where a consortium or sub-contracting approach is proposed, all information requested should be given in respect of the proposed prime contractor or consortium leader. Relevant information should also be provided in respect of consortium members or sub-contractors who will play a significant ( greater than 25%) role in the delivery of the services under any ensuing Contract.

# Tender Application Requirements

Please provide paper copies of your application which should include:

## Confirmation that you the supplier are able to meet the requirements outlined in the brief above.

## Dated your response, used our company’s full postal address (albeit you submission might be by email) and included the Reference:  BIGTP150

## Details of who to contact in your company in relation to this tender

## Company registration Number and VAT number (if appropriate)

## Total cost of providing the goods/services requested in the format required with a breakdown to include:

- LOT 1: 2.1.1 – 2.1.11

- LOT 1: 2.3

- LOT 1: 2.5

- LOT2: 2.2.1 – 2.2.3

- LOT2: 2.4

- LOT 2: 2.6

# Tender Scoring Criteria

The tender will be scored only on their compliance to the specification set out in section 2 and awarded to the lowest compliant tender for each of the LOTS.

# Tender Returns

Tenders may be returned by email.

Tenders are to be returned by:

Latest date to be returned:	19 August 2021

Latest time to be returned:	5:00 pm

If submitting by email, tenders should be sent electronically to:

marco@stives-brewery.co.uk

with the following message clearly noted in the Subject box; ‘Brewing and Canning Equipment REF: BIGTP150”

Tenderers are advised to request an acknowledgement of receipt when submitting by email.

# Clarification

There will not be any negotiations of any of the substantive terms of the Tender Documents.  Only clarification queries will be answered. Any clarification queries arising from the Tender Documents which may have a bearing on the offer should be raised as soon as possible in writing. The deadline for clarification questions is 9 August 2021 before the submission date. All e-mailed queries should be sent to:-

Name: Marco Amura

E-mail: marco@stives-brewery.co.uk

No representation by way of explanation or otherwise to persons or corporations tendering or desirous of tendering as to the meaning of the tender, Contract or other Tender Documents or as to any other matter or thing to be done under the proposed contract shall bind us unless such representation is in writing and duly signed by Marco Amura of St Ives Brewery Limited. All such correspondence shall be returned with the Tender Documents and shall form part of the Contract.

Tenderers must provide a single point of contact in their organisation for all contact between the Tenderer and St Ives Brewery Limited Responses to any queries will be shared  through Contracts Finder website

# Disclaimer

The issue of this documentation does not commit St Ives Brewery Limited to award any contract pursuant to the bid process or enter into a contractual relationship with any provider of the service.  Nothing in the documentation or in any other communications made between St Ives Brewery Limited or its agents and any other party, or any part thereof, shall be taken as constituting a contract, agreement or representation between St Ives Brewery Limited and any other party (save for a formal award of contract made in writing by or on behalf of St Ives Brewery Limited).

Bidders must obtain for themselves, at their own responsibility and expense, all information necessary for the preparation of their tender responses.  Information supplied to bidders by St Ives Brewery Limited or any information contained in St Ives Brewery Limited’s publications are supplied only for general guidance in the preparation of the tender response.  Bidders must satisfy themselves by their own investigations as to the accuracy of any such information and no responsibility is accepted by St Ives Brewery Limited for any loss or damage of whatever kind and howsoever caused arising from the use by bidders of such information.

Bidders shall be responsible for their own costs and expenses in connection with or arising out of their response St Ives Brewery Limited reserves the right to vary or change all or any part of the basis of the procedures for the procurement process at any time or not to proceed with the proposed procurement at all.

Cancellation of the procurement process (at any time) under any circumstances will not render St Ives Brewery Limited liable for any costs or expenses incurred by bidders during the procurement process.