Part Two: Specification

2 x UAV’s (Unmanned Aerial Vehicles)

Contract Reference: RBGKEW/437

(This document is for information)

Part two of this Invitation To Tender sets out the requirement for which the Customer invites Tenderers to submit a Tender Response.

- Summary

RBG Kew requires two replacement UAV’s (Unmanned Aerial Vehicles), plus software, and accessories. The UAV’s will be used for vegetation survey of plant metrics, health and produce high resolution surface models at various sites around the world and within the UK.  These two units will augment an existing eBee unit: two fixed wing UAV’s, with extra batteries, controllers, imaging accessories and software.

The new units would ideally be compatible with our existing UAV accessories (batteries = 2200mAh, Lipo Battery, Sequoia sensor, Cannon S110 red edge and NIR camera), to optimise cost savings through reuse of accessories and reduced training needs. We will accept proposals which provide equivalent accessories, where the parts and costs are itemised.

Two units are required for resilience in the field (back up in case of breakage) and to enable dual flights to cover larger areas, accessories required are to allow for remote field working and to cover breakages and replacements.  The system is to be used within an academic/charity/conservation institution with no commercial gain.

- Scope of requirements

The UAV must have the following essential features:

| Requirement reference | Specification | Scoring methodology / Weighting |
| --- | --- | --- |
| Technical |  |  |
| 1 | 2 x UAV’s units of which individually must be able to survey an area of 20 km2 or more in one flight | Pass/Fail |
| 2 | UAV’s to be Hand launched | Pass/Fail |
| 3 | UAV’s to be small enough to be able to be transported by one person (approximately 56 x 56 x 28 cm) | Pass/Fail |
| 4 | Software to process imagery compatible with Photoscan or Pix4D, outputs to be compatible with ESRI software | Pass/Fail |
| 5 | 2 x UAV Controllers/radios receiver software: PC or Android | Pass/Fail |
| 6 | Sensors: RBG resolution of 12 Megapixels or greater. Of ground resolution 2 cm or more. Horizontal accuracy of 1-5 Metres with no GCP (ground control points) and below 5cm with GCP. | 10% |
| 7 | Sensors: NIR (near infra-red) sensor with discrete bands in the green, red, red edge and NIR, plus upwards light sensor. | 10% |
| 8 | UAV Weight below 1.5 kg | 10% |
| 9 | UAV Flight time must be 40 minutes or more (with lightest sensor) | 20% |
| 10 | UAV Automated Landing within 30 metres of take off | 5% |
| 11 | UAV Radio range over 3 km | 5% |
| 12 | UAV to fly in winds up to 20 mph | 5% |
| 13 | Spares parts for remote working, equivalent to at least: - Spare wings x 4 pairs - Propellers x 10 - Batteries x 12 - Lenses/sensors - Protectors/covers x 2 - Additional battery chargers x 2 - Any other UAV parts required for remote working (e.g. skid plates, etc.) x 2 | 5% |
| 14 | Manufacturer’s warranty/maintenance of 2 years on UAV, to include parts, labour, annual servicing and all available updates to the control software | Pass/Fail |
| 15 | Processing software supplied, supported and updated for 2 years | Pass/Fail |
| Commercial |  |  |
| 16 | Price to meet requirements 1 - 15, fully inclusive of delivery and the following accessories (or their equivalents as needed by the UAV you offer): - 1 x Sequoia sensor - 1 x Cannon S110 red edge - 1 x Cannon S110 NIR | 24% |
| 17 | Additional years of maintenance and warranty as described in requirement 14 | 3% |
| 18 | Additional years of processing software support as described in requirement 15 | 3% |
| Total |  | 100% |

- Maintenance & Support

We expect compliant paperwork and certificates to be supplied with the equipment.

- Delivery

The equipment must be delivered to the Jodrell Laboratory, Royal Botanic Gardens Kew, Jodrell Gate, Kew, Richmond, Surrey, TW9 3DS, within 1 month of purchase.

Please provide a minimum of 1 week notice prior to delivery.