Standard Container Programme and Technical Brief

Programme

Container quantity					~760 off

Initial Prototype Delivery 				5 off, in 2026

Contract Start –

  - Design and Development			Q3 2024

  - Manufacturing					Post completion of development

Contract Terms and Conditions will be based on an MoD flow-down template with relevant DEFCONs

Technical

Missile External Dimensions				L~6000mm x W~600mm x H~600mm

Container Total Mass (incl. missile and equipment)	In excess of 2000kg

Estimated Maximum Mass of Missile			In excess of 1000kg

Key Design Features:

- Container to be made up of a base, removable lid and a removable equipment cradle

- Cradle to interface with RAF Aircraft Loader VAP60

Hazard classifications (explosive classification / environmentally hazardous (fuel) etc)

- Inert Missile – Expected to be Environmentally Hazardous

- Live Missile – Expected to be 1.4S

Transportable by air/sea/land

Main Mechanical Environmental Requirement

- Shock and vibration covering Road, Rail,  Sea and Air transport

Main Climatic Environmental Requirement

- Temperature Range				-46°C to +71°C

Design Life						25+ years