Specification – Demountable Defences

This specification is for the design and build of demountable defences against coastal flooding and associated supporting elements on the islands of St Mary’s and St Agnes, Isles of Scilly.

The demountable defences are systems that can be assembled and/or installed before predicted flooding events and removed after the event has passed.

Some supporting elements of the demountable defences may be relatively extensive and permanent, including works such as earth banks or solid walls. The demountable elements in these cases will be required to be installed to close each more permanent defence completely during coastal flooding events.

The locations at which these defences are likely to be required the following (all St Mary’s unless noted):

- Porthcressa – Little Porth

- Porthcressa Bank – park between Tourist Information Centre and laundrette building

- Porthcressa – east of the library

- Town Beach – Rechabite Slip area

- Town Beach – Holgate’s Green

- Town Beach – West of Holgate’s Green

- Mermaid Inn

- Old Town – Trench Lane and Old Town Road

- Lifeboat slip, Periglis St Agnes

- Town Beach stormwater and seawater removal systems

The structures that may be used to close these gaps and protect against a design flood vary in style from simple stop logs to a range of self-supporting demountable structures. Systems to evacuate stormwater and any influx of seawater from the area behind Town Beach defences when the demountable systems are in place are also required.

Specification of the structures varies, based on the type of structure to be installed. For simple stop logs, the design is likely to be straightforward but more complex demountables may require more extensive design.

Since the demountables and other structures are intended to work in defence of flooding from the sea, normal seepage and piping may occur for a short period only over the highest part of the tide. However, hydraulic analysis should be completed to consider whether this is likely to affect the integrity of any defence founded on unsealed ground.

The specification includes the following items:

- Hydraulic design, for example to determine the height of the design flood event and its expected energy, and to determine (for structures founded on unsealed ground) likely seepage rates over a high tide

- Engineering analysis of the type, size and function of a demountable and associated features, including drainage systems behind the existing sea wall at Old Town

- Specification of ‘off-the-shelf’ or bespoke standard demountables, to meet the hydraulic and engineering specifications

- Detailed design of special demountables that cannot be bought ‘off-the-shelf’ or supplied as a bespoke standard demountable

- Obtaining any licences necessary to install the works, if required

Timing

All works on St Mary’s must be completed by the end of May 2023 to allow funding to be claimed from the European Regional Development Fund (ERDF) before the fund closes in June 2023. The design and construction of these works must be prioritised to achieve this timetable.

For each of the demountable sites, the following approaches are possible solutions:

| No. | Site | Style | Hydraulic design | Engineering design | Standard/ Special |
| --- | --- | --- | --- | --- | --- |
| 1 | Little Porth | Fence/wall | Yes | Yes | spc |
| 2 | Porthcressa Bank | Bund/wall/demountable | Yes | Yes | spc/spc/std |
| 3 | East of library | Bund/demountable | Yes | Yes | spc/std |
| 4 | Rechabite Slip | Demountables x4 | No | Yes | std x4 |
| 5 | Holgate’s Green | Bund or wall | Yes | Yes | spc |
| 6 | W of Holgate’s Gn | Demountable | No | Yes | spc |
| 7 | Mermaid Inn | Demountable | No | Yes | std |
| 8 | Old Town | Demountables x3, drainage | No | Yes | std x3/spc |
| 9 | Lifeboat slip | Demountable x1, drainage | Yes | Yes | std |

- Hydraulic design

Significant information is available from the JBA Flood Study for the islands (2016) on the design height of events affecting these frontages. However, for some sites the information in the JBA study is unlikely to be sufficient to understand the hydraulic design standard.

In addition, early designs are available for Site 8 at Old Town, although the designs provided were inappropriate for the site.

The provision of hydraulic design by the contractor is likely to be required for the following:

(a) the Little Porth fence/wall designed to close an opening between the existing buildings that themselves provide flood defences at this site. In the February 2014 storm water travelled between the houses and into the rest of Hugh Town behind. The design at this site needs to cope with water ponding in the seaward garden of the property and moving to the landward side of the property. It must allow continued access (when not flooding) for the residents but be strong enough to resist the force of waves and wind during flood events. This requires discussion with the residents. Any fixed fence/wall is likely to need to be hydraulically designed to avoid seepage or piping occurring below it during a high tide event.

(b) Demountable and fixed defences at Porthcressa Bank park, where the area to the west of the laundrette needs protection against high sea levels flowing from Porthcressa during storms. The area to the west of this park is elevated (‘Porthcressa Bank’), but east of the pathway over the bank the ground is lower. The design outcome should retain the natural feel of the park, but also provide protection against blue water flooding. One feasible approach may be a bund to elevate the ground level, together with a low stone wall at the seaward margin of the park to provide additional protection, and a demountable to close the gap between the park and the laundrette. Hydraulic design is required for the design elevation to achieve an appropriate standard. The bund formed in the park must be hydraulically designed to avoid seepage or piping occurring below the bund during a high tide event, and to prevent wave runup overtopping.

(c) East of the library, a bund that forms the end point of a demountable is likely to need to be hydraulically designed to avoid seepage or piping occurring below the bund during a high tide event, and to prevent wave runup overtopping.

(d) At the Rechabite Slip and Holgate’s Green area, where hydraulic design is required to determine the optimum height of demountable and permanent structures to reduce risk to the area behind the works. The options include works at the seaward side of the Green and the Slip, or potentially works at the rear of the Green, close to the road. Additionally, any design should not preclude the Green being used as a storage area for gigs during the annual gig championships and other events. Piping and seepage on any bunded structure must be considered.

(e) Hydraulic analysis of the requirements for a system for pumping stormwater from behind the defences on Hugh Street may be required. This should examine the volume of stormwater that could not escape from – and seawater that might flow to – the landward side of the demountable defences to the sea.

- Engineering design

The provision of engineering design by the contractor is likely to be required for installation of demountables, bunds and associated works designed to anchor or support the end of the demountables. Minor additional works such as additional drainage also may be required.

In considering the engineering design, the energy each location may be subject to from wave, surge and wind forces during storms must be considered by the contractor. Some locations are very exposed to these forces, while others are less exposed. The JBA report on coastal flooding for the Isles of Scilly discusses static sea levels, storm surge and expected wave overtopping of existing ground levels.

Demountable designs must consider how the structures will be transported to site and fitted. In general, most structures will require to be transported by up to 2 people to the actual site, but 1 person portability is preferred.

- At Little Porth the engineering design should provide construction diagrams and specifications for the wall/fence for infilling the gap between the buildings.

- At Porthcressa Bank park, the design should provide diagrams and specs for the bund, stone wall and demountables. Key constraints are avoiding any attachment to the wall of the laundrette building, but the attachment point in the park can include the existing stone wall.

- East of the library the design is for the bund that continues the ground level from the existing park across to an upright provided for one end of the demountable structure, and the receiving structure of the other end fixed against the wall of the library.

- At the Rechabite slip area, the design is for the 4 demountable elements that i) cross the Rechabite Slip (about 6.5 m), ii) and iii) close both gaps either side of the existing shelter (about 2.5 m each) and close iv) the gap from the end of the wall east of the slip to the slip edge (3.25 m). The attachment points can be fixed to the various stone structures and the shelter itself.

- On Holgate’s Green, the potential for overtopping through the Green should be managed by a natural feature such as a bund at the rear (south) and east side of the Green that extends the existing walls, but does not exclude towed gig boats on trailers from accessing the Green during gig week. This element should join to element (d) above

- The accessway west of Holgate’s Green needs a design for a demountable structure that fits the irregular-shaped gap, about 2.2 m wide. The attachment points can fix to the stone wall present already.

- The 5 m demountable in front of the Mermaid Inn is to close the gap on Hugh Street at the crest, where a low structure of 0.5 m high will be sufficient to stop water running up from the Mermaid Wall area. The attachment points need to stand alone.

- At Old Town demountable barriers are needed to close off i) Old Town Road between the Duchy cottages and the Old Town Café wall ii) Trench Lane, iii) Old Town Lane east of ‘Nowhere’, between the existing rock armour and the low concrete and stone wall north of the road. The demountables at the centre of Old Town could be continuous, if that is appropriate, or they could end against existing structures.

- At the old lifeboat station on Periglis during storms seawater lifts between the two slipways (the old lifeboat slipway and the modern slipway) to spread over the concrete apron outside the old lifeboat station. This water then runs down the falling track besides the old lifeboat station into the low-lying land around the settlement behind. Rock armour will be used (by others) to break up the waves where they hit the shoreline; a demountable structure should be designed for the 5 m-wide concrete apron to direct overtopping water back into the sea and prevent it running inland. The concrete apron may require repair and reconstruction to provide a solid surface for the demountable structure to rest on. Enhanced drainage is also required behind the Periglis road frontage to direct stormwater to an appropriate soakaway point. It may not be feasible to use the sea for this disposal during storms or high-water level.

- Stormwater may pond behind demountable defence systems behind Town Beach at high tide. Despite a one-way flap being fitted to the local combined sewer overflow, some seawater also enters Hugh Street during very high sea levels, particularly during storms. If hydraulic analysis shows this is likely to be a problem, to avoid this water causing flooding on the landward side of the demountable defences because it cannot drain to the sea anymore, a system for removing that water should be designed to be installed to be available during high sea levels, particularly when heavy rain also is occurring.

- Permitting, construction and installation of special demountables/protection works

At some sites in Hugh Town, it will be necessary for the contractor to design, construct and install special demountables or protection works because the local context (ground level, materials) doesn’t allow standard structures. These special works include a defence against flooding between two houses at Little Porth; a bund and wall placed on Porthcressa Bank to link to the standard demountable at the same location; the bund east of the library to link to the demountable; the complex demountable west of Holgate’s green and drainage at the bottom of Trench Lane, Old Town.

It is not thought that planning permission is required for any demountable flood defence structures, since they are not permanent, and their support elements are minor in extent. Since some elements of the Porthcressa Bank and Holgate’s Green solutions may be permanent in nature and require ground works, all design solutions proposed must be checked with the Chief Planning Officer of Council of the Isles of Scilly to determine whether they require planning permission.

It will be the responsibility of the supplier to obtain all regulatory permissions required for the works to be completed. Council of the Isles of Scilly will assist in providing information and contacts for this component of the project. These permissions may include highway modification orders obtained from Council of the Isles of Scilly for any structures based in the road or footpath.

Any demountable or temporary flood defences procured should be tested to BS 851188-2+A1:2021, where appropriate. Note that BS 851188 does not apply to marine settings, but performance under these conditions is described. Materials used in the design and construction of the demountable systems designed should comply with the recommendations in BS 6349.

All elements of the project must be undertaken in a manner which allows the client to obtain a CEEQUAL rating of ‘Very Good’. The methods by which the tenderer proposes to help the client achieve this rating must be explained in their tender submission.

- Specification of ‘off-the-shelf’ or bespoke standard demountables and other systems

The specification of demountable and other flood defence systems by the contractor should use off-the-shelf systems where feasible. Bespoke standard systems may be used where appropriate, but full bespoke engineering is discouraged for any structures. It is recognised that bunds and support systems may need to be engineered specifically to support such off-the-shelf and bespoke standard systems.

Demountable structures that could be supplied by the contractor using off-the-shelf suppliers are likely to be those where both upright posts are the same height and orientation, and the ground is level in between the posts. This includes sites at Porthcressa Bank, east of the library, Rechabite slip, Mermaid Inn, Old Town and St Agnes old lifeboat station, totalling 12 individual structures.

Each site will require the specifications for the demountables to be identified by the contractor before the demountables are manufactured and the upright posts fitted. In some cases, intermediate posts will be required, because the gap to be closed is longer than a standard demountable plank. There are expected to be 10 individual gaps to be closed using these structures. In some cases, the ground may require levelling to provide a flat base against which the bottom unit of the defence can seal effectively.

The engineering design element by the contractor will provide the specifications necessary for procurement of the demountable structures and their fittings. Each potential supplier has a slightly different approach to configuring and locking the demountable planks into position, so it is preferable to include the engineering design in the procurement of these standard demountables as a single contract, to avoid multiple types of systems being installed.

- Specification of bunds and non-standard systems

The engineering designs by the contractor must include all plans and specifications required for the construction of all elements of the demountable defences and associated systems, including all ancillary works required to prepare the site for installation of those defences.

- Construction of all elements

Construction of all demountable systems and associated systems by the contractor must proceed as soon as possible following acceptance of the proposed design solutions by the Council of the Isles of Scilly. Individual sites may be designed and built separately, rather than waiting for all designs to be completed before proceeding with construction.

Summary – Specification

The specification includes all the elements above. A single contract for design, licensing and construction is expected to be formed.

The schedule of design and licensing works will be required from the tenderer to deliver all the elements above before the end of January 2023, for construction before the end of May 2023.

The schedule for this tender process and its associated outputs is expected to be:

- Tender returns – 28 October 2022

- Appraisal completed – 4 November 2022

- Hydraulic design – 2 December 2022

- Engineering design – 24 December 2022

- Any permitting of works – completed by end of January 2023

- Works completion, invoiced and defrayed – 26 May 2023

Site Photos

Site 1: Porthcressa-Little Porth

The site is within a private garden, so will require close liaison with the owner for design and installation.

Site 2: Porthcressa Bank

Figure 2: Porthcressa Bank looking west from the laundrette

Figure 3: Porthcressa Bank looking towards the laundrette

Figure 4: The gap between the laundrette and Porthcressa Bank park

Site 3: East of the library

Figure 5: East of the library building, showing the gap to be closed

Site 4: Rechabite slip

Figure 6: Rechabite slip, showing the 6.5 m gap to be closed at the slip

Figure 7: the 2.5 m gap west of the shelter at the Rechabite slip

Figure 8: the 2.5 m gap east of the shelter at the Rechabite slip

Figure 9: the 3.25 m gap east of the shelter at the Rechabite slip

Site 5: Holgate’s Green

Figure 10: Holgate’s Green, looking east from its W boundary, showing short wall at beach edge

Figure 11: Holgate’s Green, looking south-east from W boundary, showing boundary with road

Site 6: W of Holgate’s Green

Figure 12: the approx. 2.2 m gap in the beach accessway west of Holgate’s Green, from land

Figure 13: the approx. 2.2 m gap in the beach accessway west of Holgate’s Green, from beach

Site 7: Mermaid Inn road crest

Figure 14: the approx. 5 m gap outside the Mermaid Inn, towards the crest of the view in the photo

Site 8: Old Town

Figure 15: the 7.8 m gap at the top of Trench Lane, from Old Town Road

Figure 16: the 7.8 m gap at the top of Trench Lane

Site 9: Periglis slips on St Agnes

Figure 17: The gap between the two slips that focuses waves up to the slips’ crest

Figure 18: Crest of the slips, showing the track falling alongside the old lifeboat station

Figure 19: The track alongside the old lifeboat station down which flooding water flows

Figure 20: Closeup of track alongside the old lifeboat station down which flooding water flows