Call-Off Schedule 20 (Call-Off Specification)

This Schedule sets out the characteristics of the Deliverables that the Supplier will be required to make to the Buyers under this Call-Off Contract

# Statement of requirementS

- INTRODUCTION AND BACKGROUND TO THE AUTHORITY

  - Permanent Joint Headquarters (PJHQ) commands joint military operations on behalf of the Ministry of Defence. A division of PJHQ, J3 is responsible for global operations.

- OVERVIEW of requirements

  - The Authority is seeking the provision of two requirements for a period of 2 years from 1 June 2021 to 31 May 2023. For both requirements equipment will be leased.

  - The Potential Provider must make sure (at its own cost) that each piece of equipment is delivered in a new and unused condition. The Authority can at its sole discretion reject a piece of equipment which is not in a new condition.

Requirement 1 – Knowledge Wall. PJHQ J3 requires the ability to simultaneously display multiple video feeds on a large display solution, from a range of classifications from Unclassified to Secret UK Eyes only (as a minimum), Above Secret preferably.

Requirement 2 – Transportable device. PJHQ J3 requires 15 transportable devices that can download, store and upload classified information. The device will be used to provide up to date information to senior personnel within the HQ. Supply of these items will be subject to governance approval in line with section 6 of this SOR. Where the appropriate governance documentation and subsequent accreditation cannot be achieved, this contract will be for the provision of requirement 1 only.

- Key user requirement 1

Requirement 1 – Knowledge Wall. PJHQ J3 requires the ability to simultaneously display multiple video feeds on a large display solution, from a range of classifications from Unclassified to Secret UK Eyes only (as a minimum), Above Secret preferably).

  - Operating with the Operations Control Room (OCR) Projectors. The Potential Provider must provide a capability that is able to operate alongside the OCR projectors and utilise the same subordinate system machines that are already installed and detailed in Table 1.

  - The Knowledge Wall. The Potential Provider must provide a capability that will enable at least 8 video and audio feeds to be displayed on a knowledge wall and additional screens in the wider floorplate.

| For Information: Subordinate Systems. The subordinate systems in use in the OCR. |  | Requirement applicable to |  |
| --- | --- | --- | --- |
| Subordinate System | Output protocol | Knowledge wall | Transportable Device |
| AS – System | DVI | Yes | No |
| DII(S) | DVI | Yes | Yes |
| MODNET(S) | To be confirmed | Yes | Yes |
| BICES | DVI | Yes | No |
| SIPR | DVI | Yes | No |
| MODNET | DVI | Yes | No |
| PJHQ.com x 2 | DVI | Yes | No |
| dVTC | HDMI | Yes | No |

Table 1 - Subordinate Systems

  - Visualisation.

    - Multiple classification visualisation.  The Potential Provider must provide a capability that is able to receive and manipulate video feeds in a single electronic workspace, with a classification range of Unclassified to SUKEO as a minimum, Above Secret (preferable).

    - Number of video feeds selectable. The Potential Provider must provide at least 8 (minimum) video feeds to be selectable at any one time. Each shall be selectable from the display management solution (see 3.6).

    - Output visualisation. The capability will need to be able to display the video feeds in the following manner.

      - The Potential Provider must provide a solution which enables video or audio feeds to be selectable from the display management solution. As a minimum the solution should display 4 separate video feeds concurrently. A single video feed should be able to be repeated across the knowledge wall. It should also be able to display 4 video feeds into any of the single knowledge walls areas see Figure 1.

Figure 1 - Display area requirements

  - Display interaction. User interaction will be minimal with the displays but subject to the following interactions.

    - The Potential Provider must provide a capability that can be controlled by the subordinate systems.

    - The video feed does not need to be stored on the solution, all feeds can be supplied directly from the subordinate systems.

  - Physical display screens.  The Potential Provider must provide a capability that is able to display the video feeds in the following physical areas and further details in Appendix A.

    - A video wall, equivalent to 4 (3) x 70” high definition display screens.

      - The capability must be able to display as a minimum Secret UK Eyes Only (SUKEO – 3 display areas). It is highly desirable for the proposed solution to display information Above Secret (4 display areas.)

    - Two wall-mountable displays, equivalent to 55” high definition display screens.

    - A single wall mountable display, equivalent to a 55” high definition display screen on the wider floorplate.

  - Display management.  The Potential Provider must provide a capability that has the ability to be operated by an intuitive user interface, to select and manipulate the feeds being displayed from the bird table area, whether by physical matrix selection, zero client or a touch pad solution.

    - If a touchpad solution is provided, it must be physically tethered to the capability, but could be able to be moved around up to 5m from around the Duty Ops Controllers desk / bird table areas. All must be accreditable to operate within in a highly secure area.

  - Change of protocol. If the technical solution requires a change of protocol (DVI to HDMI) then the solution must provide the equipment to convert the protocol.

  - Accreditation of the knowledge wall.  The Potential Provider must provide a capability that shall comply with MOD security policies in accordance with JSP 440 and be assessed by Ministry of Defence security accreditors (Cyber Defence and Risk CyDR) with a classification range of Unclassified to SUKEO as a minimum, Above Secret (preferable).

    - Classification separation. Lessons Identified have shown that in order to achieve AS accreditation for this type of system, the video feeds must be kept separate so that lower classification systems are not able to access to higher classified video streams. PJHQ is not dictating video separation as the only solution, but the Potential Provider MUST provide a commentary on how their proposed solution could be accredited to operate with such classified video streams.

    - Classification of compartment. As a minimum, the solution must not employ, wireless communication methods, cameras or microphones (the VTC device is exempt from this.

    - Disposal Plan. The Potential Provider must sanitise equipment, returning the device to its original security classification (unclassified).  The Potential Provider must provide a disposal plan which states how components that have been exposed to SC data will be disposed in a manner which follows NSCS policy. This plan should also consider any components that are to be re-used by the Potential Provider.

    - Accreditation lessons learned.  Due to the highest classification feed being Above Secret, the accreditors require each subsystem to have a one-way fibre optic network path and a change of protocol i.e. output DVI TX-FO RX-HDMI before the signal reaches the switch, this ensures that the data streams remain separated. A penetration test to identify cyber vulnerabilities will be conducted on the equipment by the Authority once installed. Depending on the findings, the supplier may or may not be notified of the results.

  - Out of scope for Knowledge Wall. The following items are out of scope.

      - The subordinate systems found in Table 1.

- KEY User Requirement 2

Requirement 2 – Transportable device. The Authority requires 15 transportable devices that can download, store and upload classified information. The devices will be used to provide up to date information to senior personnel within the HQ.

  - Physical devices. The Potential Provider must provide transportable devices that fulfil the following criteria:

    - The Authority requires all devices to be updatable in a single data transfer.

    - The devices must be accredited for use within the HQ. It is highly desirable for the devices to be accredited for use outside the HQ.

    - It should not be identifiable as a military piece of equipment.

    - As a minimum the device must have a keyboard, mouse and display element all within a single pane and a single pane protective cover (which allows the device to be stood-up).

  - Headquarters considerations. The HQ is physically split into a mix of security classified floorplates (rooms). The Potential Provider must provide a device that can be manually transported in and around the building and approved to be used in all security areas.

  - Device network connectivity. The Potential Provider must provide a device that has approved connections to both DII(S) and MODNET(S). It must be able to download, store and upload the data from these specific networks upto the classification of Secret UK Eyes Only (SUKEO).

    - The devices will be presented with a PDF document, the devices must be able to as a minimum:

      - Be able to view and edit PDF document.

      - Be able to make electronic notes, either using a stylus, keypad or touchscreen inputs.

      - Each device could have annotated notes provided by each of the users (max 15), these notes should have a method of being able to be combined into a single document, with minimal administration.

  - Accreditation of the Transportable Devices. The Potential Provider must provide transportable devices which, complies with policies for the connection, downloading, storage and uploaded of classified material:

    - Physical security measures:

      - The device must have Bitlocker password control as well as a user logon.

      - As a minimum, the solution must not employ, wireless communication methods, cameras or microphones.

      - Auditable proof that the devices cannot be connected to another networks or devices (other than those described in 4.3) by either physical or wireless means. i.e. the transportable device can only be connected to the proposed solution.

      - If the device uses USB as its connection mechanism, the USB ports must be locked down and must be on an approved list ethernet adapter.

      - Malware and virus protection must be installed and updated by the supplier as part of this contract.

    - Security Governance measures. The Potential Provider must provide appropriate security governance documentation to: download, store and upload data of a classified nature from the subordinate systems (DII(S) and MODNET(S)). There are security conditions that must meet before the connections can be made, (i.e. Anti-Virus, Operating System updates, Vetting levels etc).

    - Security Governance Documentation. The Subordinate systems Project Teams will be able to advise on their security requirements, it will not be the responsibility of the customer (PJHQ) to coordinate the security governance creation and acceptance of these documentation sets, but the customer MUST be provided will copies of all documentation once approved. PJHQ’s previous knowledge of security documentation has seen requests for the following documentation (not limited to):

      - Risk Management and Accreditation Documents Set (RMADS).

      - Security Operating Procedures (SyOps) only which will identify, amongst other things, acceptable use of the system, password policies, antivirus update frequency (associated with 4.4.1.4).

      - Code of Connection (CoCo).

      - Data Protection Impact Assessment (DPIA)

      - Disposal Plan. The Potential Provider must sanitise equipment, returning the device to its original security classification (unclassified). The Potential Provider must provide a disposal plan which states how components that have been exposed to SC data will be disposed in a manner which follows NSCS policy. This plan should also consider any components that are to be re-used by the Potential Provider.

- Associated Services - Service Desk

  - The service desk must include as a minimum:

    - In the case of equipment breakdown or accidental damage, the Potential Provider must provide an off-office site service desk operating on a weekday only provision from 0800-1700 UK time. See Call off Schedule 14 Appendix 1 to Section 2 Service Tables. The customer must be able to have a ticket raised or initial assistance provided with 1 hour of the originating phone call.

    - In times of non-urgent operational working, the diagnosis of the fault must be identified within 2 working days of the ticket creation. All cases will be assumed to be a non-urgent operational matter unless PJHQ J6 informs the Supplier by telephone that the case must be dealt with as urgent operational case.

    - In times of urgent operational working, the diagnosis of the fault must be identified within 2 days of the ticket creation. If these days fall over the weekend then para 5.1.5 will be enacted.

    - All faults are to be rectified to the satisfaction of the customer within 2 weeks of the original ticket being raised.

    - If an urgent issue extends over a weekend, 10 engineering days per year shall be included in this support envelope. PJHQ J6’s approval to utilise these engineering days will be required and available through the PJHQ J6 duty officer (number will be supplied on award of contract).

    - A one-day outage for routine maintenance is acceptable.

    - Any routine outages need to be notified to PJHQ J6 no later than 20 working days in advance and PJHQ J6 shall be able to accept or decline this outage before the outage is agreed.

- Logistics and Delivery Capabilities

  - Due to the nature of PJHQ’s business, any time without a capability should be minimised. With the current contract expiring 31 May 21, the suggested timeline of activities post contract award is provided in Table 2.

| ### Date | ### Event |  |
| --- | --- | --- |
| ### 12 May 2021 | Start of the mobilisation period |  |
| ### w/e 14 May 2021 | Project Meetings begin. |  |
|  | ### Key User Requirement 1. ### Participants: PJHQ, Skanska and winning Provider. ### What infrastructure is required to be installed? Skanska is the PFI site lead and will conduct any works which are required to affect, ceilings, walls and floors. Should the current electrical provision be deficient for the proposed solution this also needs to be included in the infrastructure requirements. | ### Key User Requirement 2. ### The provider to begin governance process with MOD Networks (DII(S) and MODNET(S)). |
| ### Operational stops | ### NOTE 1: PJHQ could be at any time drawn into an urgent operational situation. At very short notice the installation maybe asked to stop or any on site visited cancelled or postponed. It is unknown how long this stop could be for. ### The provider will not be held to account if installation is delayed because of this. A recommencement date will be negotiated if a stop occurs. |  |
| ### w/c 31 May ### 1 week | ### Key User Requirement 1. ### Target date for installation – It should be assumed that installation will only take 1 week. ### NOTE 2: It should be assumed that installation can be conducted during the working week, but if operationally essential this maybe altered to night shift work. | ### Key User Requirement 2. ### It is highly desirable that the transportable devices are available and accredited for use alongside KUR1 timescales. ### Accreditation. The Potential Provider must submit all governance documentation and gain approval from the appropriate security accreditor and copies of all documentation must be provided in hard and soft copy to the customer. |
| ### 01 Oct 21 |  | ### Key User Requirement 2. ### The customer recognises that ### achieving accreditation for the transportable devices could be protracted. Therefore, the customer will set a No Later Than date for the availability and approval of the devices of 1 Oct 21. No payments under lines 4, 5 and 6 of the pricing schedule will be made for KUR 2 until accreditation is satisfied. ### If the supplier fails to satisfy KUR 2 by the 01 Oct 21, the Customer will withdraw the requirement from the supplier and seek an alternative solution for supply. |

Table 2 - Outline project timetable

- Planning assumptions

  - For planning assumptions, the following are provided.

    - Training requirement.  The capability shall be intuitive and not require a formal competency or course to operate the system. All equipment provided must have User Guides (minimum) and maintenance manuals (preferably) provided as a part of the equipment delivery. The documents will be received in electronic format. The documents can be direct from the original equipment manufacturer.

    - Security. All personnel who need access to the building either for project management requirements or are involved with the installation and maintenance of the capability must have as a minimum SC clearance. It is the Potential Provider’s responsibly to supply this.

# Appendix A – PJHQ J3 Floorplate layouts

## Floorplate layout of PJHQ J3 Division (OCR)

https://www.ncsc.gov.uk/guidance/secure-sanitisation-storage-media

Extracted from the Defence ICT Accreditation Guide 21 Aug 21

https://www.ncsc.gov.uk/guidance/secure-sanitisation-storage-media