STATEMENT OF SERVICE REQUIREMENT FOR THE PROVISION OF TECHNICAL SUPPORT TO PERFORM INDEPENDENT CONFIRMATORY ANALYSIS OF UK ABWR DESIGN BASIS REACTOR FAULTS

1	BACKGROUND TO THE PROJECT

1.1	ONR is undertaking a phased Generic Design Assessment (GDA) of the Hitachi General Electric (Hitachi-GE) UK Advanced Boiling Water Reactor (UK ABWR). Hitachi-GE’s submissions to ONR are similarly phased, providing increasing detail and evidence as the design is finalised and the UK regulatory requirements are better understood. In November 2015, the UK ABWR GDA entered its fourth and final step, focussed on an examination of the evidence that supports safety case claims and arguments put forward by Hitachi-GE.

1.2	During Steps 2 and 3, ONR commissioned GRS to develop an ATHLET thermal hydraulic model of the UK ABWR, independent of that used by Hitachi-GE. This is complemented by a COCOSYS UK ABWR containment model and a 3D QUABOX-CUBBOX reactor core model, both of which can be coupled to ATHLET to allow detailed consideration of a range of transients.

1.3	To date, the UK ABWR ATHLET model has only been applied to a single fault transient (“turbine load rejection without bypass”). This transient was chosen early in GDA before ONR had time to identify its regulatory priorities for targeted assessment.

1.4	This piece of work is for the UK ABWR ATHLET model (and supporting codes) developed during Steps 2 and 3 to be adapted and run to consider a range of carefully selected faults. These will inform ONR’s fault studies (design basis analysis) and probabilistic safety analysis (PSA) Step 4 assessments.

- SCOPE OF THE SERVICES REQUIRED

2.1	The required task is for a technical support contractor on the Lot 5 framework to:

- Request information as required from Hitachi-GE to further develop the independent UK ABWR computer models.

- Develop the UK ABWR models using the information received from Hitachi-GE and the learning from the earlier development work.

- Model a selection of fault transients agreed in discussion with ONR, comparing the results with the equivalent cases run by Hitachi-GE.

- Undertake a small number of sensitivity cases to investigate areas of regulatory interest emerging from the main cases run, or at the request of ONR.

2.2	Any updates to the UK ABWR models should be documented. The fault transients analysed should be reported in one or more documents supplied to ONR at the end of the project.

2.3	The specific fault transients to consider could be subject to change. They will be confirmed at the start of the contract and kept under review. For the purposes of responding to this specification, the following faults (from nominal full power) should be assumed:

- In-containment loss of coolant accidents (feedwater and steamline breaks), in part to investigate the containment response to design basis events.

- Further investigations of loss of turbine related faults, extending the initial studies undertaken during Step 3 as part of original ATHLET model development work.

- Partial and full loss of reactor internal pumps.

- Station blackout events with a view to investigating containment venting.

- A small number of anticipated transients without scram (CFD modelling of boron injection should not be considered within scope).

- OBJECTIVES

3.1	The objective of this project is to further develop and then apply the independent UK ABWR computer models developed during earlier Steps of GDA to a number of carefully selected fault sequences.

3.2	The aim of the analyses to be undertaken is not to directly support the UK ABWR safety case. Rather, they are to assist ONR in coming to a regulatory judgement on the adequacy of Hitachi-GE’s transient analysis for the UK ABWR.

3.3	Any updates to the UK ABWR models will need to be documented. In addition, the results of the transient analyses will need to reported, along with any insights or recommendations that come from the work and comparisons against Hitachi-GE’s fault analyses.

4	CONSTRAINTS

4.1	The following constraints will apply:

- The work is expected to start in February 2016 and concluded in approximately 12 months.

- The analysis is to be done with the ATHLET, QUABOX-CUBBOX and COCOSYS codes (www.grs.de/en/content/development-and-validation-simulation-codes). Input decks commissioned by ONR during Steps 2 and 3 will be supplied to the selected organisation with supporting documentation.

- The selected organisation will need to have appropriate code licenses from GRS and export control licenses from the German Federal Office for Economic Affairs and Export Control. ONR understands that there is a 2500 Euro administrative fee associated with each code to obtain these licenses.

- The latest versions of the UK ABWR pre-construction safety report (PCSR) and supporting Topic Report on Design Basis Analysis will be provided at the start of the contract. Further information will need to be identified and requested by the Technical Support Contractor through Regulatory Queries (via ONR).

- The work will be undertaken at the Technical Support Contractor's offices. "Kick-off" and "close-out" meetings with ONR are envisaged, along with approximately monthly progress meetings. Some of these could be done face-to-face, either at ONR’s or the Technical Support Contractor's offices. However, pragmatic use of conferencing technology can also be used for meetings.

- No travel to Japan or the US should be assumed for this contract. At least one meeting with Hitachi-GE present should be planned for. However, this should be assumed to be either in Liverpool or by video/telephone.

- All information received from Hitachi-GE will need to be treated in accordance with the ONR Technical Support Contact Framework agreement and non-disclosure agreement. It will need to be returned to ONR at the end of the contract.

- The arrangements for security vetting and IT security established in the Technical Support Contract framework agreement will need to be followed. No information with a security marking above UK OFFICAL - SENSITIVE is anticipated as part of this contract. Most information will carry a Hitachi-GE commercial marking.

- The transmittal of all documents between ONR, Hitachi-GE and the successful TSC will be through ONR's Joint Programme Office (JPO). This will be through encrypted DVDs, regardless of the security marking (unless the documents are publically available on the internet).

- Any conflict of interest should be declared.  Please fully complete Schedule C.

- The aim of this work package is to inform and advise ONR. However, it is ONR’s intention to brief Hitachi-GE on the conclusions of the work. Hitachi-GE will be provided with copies of any reports produced for information and will be invited to comment on their factual accuracy.

- It is not ONR’s intention to publish the reports produced through this contract. However, it is likely they will be referenced in publically available reports and therefore subject to freedom of information requests. This should not constrain or limit the produced reports but should be taken into account with the style and format.

5	TENDER SUBMISSIONS

5.1 	In addition to the generic requirements established in ONR’s Technical Support framework, a proposal in response to this specification is required which:

- Clearly addresses all aspects of the specification (including objectives, scope and constraints).

- Details of the project team members, including grades and rates and short curriculum vitae (CV) summarising relevant experience.

- Details the organisation’s and team members’ experience in running the ATHLET, COCOSYS and QUABOX-CUBBOX computer codes

- Identifies the anticipated engagement with ONR, including progress meetings.

- Provides timescales for performing the various aspects of the contract.

- Provides a budget price for the whole task (limit of liability), with a monthly spend profile set against key milestones/deliverables identified against which progress can be monitored/linked. It should be clear how the budget price is broken down between the different fault sequences, project management costs, reporting, travel etc.

- The names and contact details of the project and contract managers for this work package.

6	CONTRACT MANAGEMENT

6.1	ONR will require to be kept updated about progress and delivery of the required work via (approximately) monthly meetings, including "Kick off" and "close-out" meetings.

6.2	Invoicing is expected to be monthly, but linked to specific milestones.

7	SECURITY MARKINGS OF DOCUMENTS

7.1	No information with a security marking above UK OFFICAL - SENSITIVE is anticipated as part of this contract. Most information will carry a Hitachi-GE commercial marking.