Technical Specifications

- Ground Level Reserve Water Tank

- Dimensions: Minimum 6m Long x 2.5m Wide x 2.6m High

- Dimensions: Maximum 7.5m Long x 4.8m Wide x 3.6m High

- Hold volume of up to 100m³

- Tanks Sides, Base and Top – Maximum 5mm thick

- 600mm – 800mm diameter Access Manway with sealed hinged lid

- Externally Stiffened Panel Flanges

- Tank to be well braced to avoid Bulging

- Roof Vent with insect screen.

- Screened cowl ventilator

- Insect screens for overflow/warning pipes to comply with UK By Law 30

- Durable galvanised internal Access ladder

- Galvanised external ladder & handrails to BS 4211

- Galvanised Steel Bolts and Washers

- Non Toxic Sealants

- Ball valve boxes

- Spill Over Weir boxes to provide Air Gaps – Type A – BS 6281 Part 1.

- Self- draining base with sluice valve to short pipe prior to joining  the overflow

- Float Level Indicator (Connecting Rope/String to be well-weathered)

- Bell mouth Overflow- tank bottom – below Supply Valve Level.

- Outlet Services thru Rose Strainer

- Matching Pipe-works & Control valves

- Flow Control Mechanism

- Resistance to Ultra Violet Light

- Uniform Panel Dimensions

- Smooth Surface finishes

- Tank Cover imposed Loading to comply with BS 6399, Part 3, 1988

- Tank Cover Side & Base Wind Loading to BS 6399 Part 2 – BS 5950 Part 2  1990.

- WRAS Compliant

- Protective Coating

- Warranty for up to 20 years

- Quality Assurance BS EN ISO 9001 2000

- Materials delivered to site should be Blast Cleaned to BS 4232

- Long Life Service

- Low Maintenance

All standards specified to be as requested ‘or equivalent’.

Installation and commissioning of the tank to be carried out within the Gambia and provided by the supplier or an agent of the supplier.

O&M (Operations & Maintenance) Training to be provided for staff within the Gambia following commissioning.