Product Description

An ATV fitted with specialist weed spraying equipment designed for highway weed control, fully conforming to the current British and European Standard for spraying equipment, with the ability to spot treat and provide targeted application in-line with best practice techniques.  Essentially the control of the spray can be done with both hands on the handlebars. Vehicles to be delivered to Beverley Depot Annie Reed Road HU17 0LF please state approximate delivery.

- Full automatic transmission, with front- and rear wheel engine braking

- Selectable Two / Four wheel drive / Four wheel drive diff lock

- Hi and Low gear ranges

- Electronic Power Steering

- Push-button electric start

- Reliable, low-maintenance shaft drive

- Rear Rack Capacity 80kgs minimum

- Front Rack Capacity 40kgs minimum

- Towing capacity 600kgs minimum

- Full Road legal lighting kit

- PLG registration (as originally supplied by the manufacturer or tested through a VOSA testing station)

- Front E marked road legal ATV road type tyres

- Rear E marked road legal ATV road type tyres

- Heated Handle Bar Grips

- Charging Point to allow easy battery charging

- 50mm Towball

- 2 x 7 pin trailer lighting socket

- Polypropylene sprayer tank with a 70 litre capacity fitted with a basket filter incorporating a baffle

- Polypropylene front clean water tank with a 22 litre capacity (mandatory requirement)

- Full filtration system

- Full Pressure Control and Agitation

- 11.3 litre/min pump

- Glycerine Pressure Gauge

- Flashing Amber Beacon (xenon type)

- Front mounted rectangular 3 bank LED pulse beacons

- Rear mounted rectangular 3 bank LED pulse beacons

- Front Mounted Control Valves

- Two Foot Rest Nozzles c/w off centre very low drift nozzles, angle of spray can be adjusted by driver on seat – mounted within the width of the quad bike

- Very Low drift nozzle technology, all nozzles are of an Air Inclusion design

- 3 quads with rear mounted 2 nozzle boom with individual nozzle isolation

- 1 quad with 2m rear folding boom with nozzle isolation

- Telescopic Hand lance and 5m of Hose c/w metal holding bracket and lower cup

- Handlebar mounted on/off switch for Hands on the handlebar control of spray

- Compliant with EN ISO 4254 and CE marked with the ability to target apply herbicide inline the product labels and codes of practice